3GPP TSG-RAN WG3 Meeting #114b-e	R3-220318

E-meeting, 17 – 26 Jan 2022

Title: 	(TP for SON BLCR for 38.423, 38.300) MRO for SN Change Failure

Source: 	Huawei

Agenda item:	10.2.1.6

Document Type:	other

# 1. Introduction

The MRO for SN Change Failure was discussed during RAN4#114-e meeting.

# 2. Background

In RAN3#114-e meeting, there were some progresses for Pre-R17 UE and open issues on the MRO for SN Change Failure. Some issues as listed below still need more discussion:

Class 2 procedure is used to transmit SCGFailureInformation from the MN to the last serving SN.

Agree B1-1 as the procedure between the MN and the last serving SN.

Solution B1-1: MN always forward SCG failure report to last serving SN. If the problem is not introduced by the last serving SN (not too late PScell change and no intra-SN Pscell change), last serving SN sends the second message to MN. Two class 2 procedures should be defined. If the failure is brought by the last serving SN, the second class 2 procedure is not needed.

Open issues:

- Whether the same signaling flow should be used for Pre-R17 and R17 UE

- Whether the source SN may have no UE context when the source SN performs MRO.

The IEs in the new Xn messages.

In this paper, we’ll give our considerations based on the agreements and open issues.

# 3. Discussion

## 3.1 Different Signalling flow for both Pre-R17 and R17 UE

-	Whether the same signaling flow should be used for Pre-R17 and R17 UE.

According to the description of solution B1-1, we can summarize the below signalling flow for Pre-R17 UEs.

Fig.1 Signalling flow for Pre-R17 UE

For Pre-R17 UE, the MN is unware of whether there was an intra-SN PSCell change in the last serving cell, illustrated as T-SN in figure 1. The MN is mandated to forward SCG failure information to last serving SN.

According the LS [1] from RAN2, it worth noting that RAN2 has agreed to provide

RAN2 has the following agreement:

- RAN2 confirms that the 5 information requested by RAN3 LS ‎ R3-211332 ‎ are needed, and how to report them to the network could be further discussed.

The detailed 5 information requested by RAN3 LS R3-211332 [2] are quoted as below:

- CGI of the Source PSCell: the source PSCell of the last SN change. The source PSCell could be E-UTRA cell or NR cell.

- CGI of the Failed PSCell: the PSCell in which SCG failure is detected or the target PSCell of the failed PScell change. The Failed PSCell could be E-UTRA cell or NR cell.

- timeSCGFailure: the time elapsed since the last PSCell change initialization until SCG failure.

- connectionFailureType: radio link failure or SN change failure.

5)    random-access related information set by the PSCell

Besides, RAN3 has agreed that in RAN3#111-e meeting:

MN performs initial analysis to identify the node that caused the failure. The node that caused the failure performs root cause analysis.

Based on the previous progresses on the new information reported from UE, it is obvious that different from Pre-R17 UE, the R17 UE can include CGI of the Source PSCell, CGI of the Failed PSCell and timeSCGFailure. Technically speaking, with these information and the neighbour cell relationship, the MN can know whether the source PSCell and Failed PSCell belong to the same SN or the last serving cell.

Furthermore, if both of them are in the charge of the last serving SN, the MN can identify that there was an intra-SN PSCell change. Otherwise, there is no intra-SN PSCell change in the last serving SN. The MN can perform more analysis based on the timeSCGFailure. If it is above than the threshold, e.g., the Tstore_UE_cntxt, this is kind of too late PSCell. The MN considers the last serving cell as the right node. If it is below than the threshold, it is reasonable for the MN consider the source SN as the right node.

Fig.2 initial analysis in MN

Accounting for the above analysis, it is desirable that different from the solution for Pre-R17 UE the MN should not be mandated to forward the SCG Failure Information from the UE to the last serving SN node. Instead, the MN can decide the right SN node based on the initial analysis on the SCG failure and just forward the SCG Failure Information to the right node.

As a small summary on the aforesaid consideration, it is an intuitive idea that:

Proposal 1: The signalling flow for R17 UE is different from the one for Pre-R17 UE.

Fig.3 Signalling flow for R17 UE

## 3.2 IEs in the new Xn message

The IEs in the new Xn messages.

Here we’ll give our analysis on the required IEs among the above information from the MN to the SN.

First of all, as described in section 3.1, the MN can decide PSCell failure type, e.g., Too late PSCell change, Too early PSCell change or Triggering PSCell change to wrong PSCell, based on the timeSCGFailure and the possibly suitable cell(s).

Besides, upon the PSCell change failure, if the MN decides to continue the DC for the UE, it is highly anticipated that the MN selects a new target PSCell based on the measurement results in the SCGFailureInformation message. This can quickly recover the SCG configuration and ensure the UE experience compared to the SN decision solution.

If we wait for the source SN to re-decide the new target PSCell, it will bring additional latency due to the delivery of the SCGFailureInformation and the SN Modification Required messages. Besides, it is a bit questionable how the source SN can know whether the new PSCell belonging to a new SN is a suitable candidate for SCG connection, e.g. whether there is an Xn connection between the MN and the new SN. In this way, the MN can also send the new suitable target PSCell decision to the initiating SN node.

In other hand, if the MN decides not to continue the DC configuration for the UE. It depends on MN implementation whether to include the new suitable target PSCell in the XnAP message SCG Failure Information.

Proposal 2: Includes PSCell failure type in the new XnAP message to the right SN.

Proposal 3: Includes Suitable PSCell CGI in the new XnAP message to the right SN, if the MN decides to configure DC.

Whether the source SN may have no UE context when the source SN performs MRO.

After the SCG failure, the SCG Failure Information is reported immediately. If the timeSCGFailure indicating the time elapsed since the last PSCell change initialization until SCG failure is below the threshold and there is no intra-PSCell change within the last serving cell, it is the source SN to perform MRO. In my understanding, the source SN can keep the UE context at least for some time period, e.g., about the time indicated by the threshold. In this implementation way, it ensures that the source SN can always have the UE context upon reception of the new Xn message to preform MRO.

Proposal 4: The source SN maintains the UE context by implementation.

To assist the right SN to retrieve more relevant information collected from the UE context, the MN can also provide the UE ID for the right SN to identify the UE. But in MR-DC, the MN does not know the C-RNTI of UE in the initiating SN. Therefore it is preferred to use the S-NG-RAN node UE XnAP ID as a reference to the UE context in the source SN.

For M-NG-RAN node UE XnAP ID, this is another kind of UE ID and can work similar as the S-NG-RAN node UE XnAP ID to assist the failure SN node to identify the UE. If the S-NG-RAN node UE XnAP ID is provided, it is not needed to transmit duplicate M-NG-RAN node UE XnAP ID.

According to the above analysis and consideration, it is preferred to include the following information:

Proposal 5: Include S-NG-RAN node UE XnAP ID for the initiating SN to identify the UE in the new XnAP message.

Here we show one possible flow chart in case of too early PSCell change if the MN decides to continue the DC configuration.

Fig.4 the IEs the new Xn message

For the other remaining IEs, all of them are not needed. For the Mobility Information, if the SCG failure information is reported immediately, it is desirable that the failure SN node can keep the UE context, including the mobility information. For PSCell selection assistant information, e.g., UE history information, this is being discussed in the UE history information topic to support the report of the PSCell related UHI. Besides, there are the latest measurements results provided from the UE, which is workable for the SN node to perform the PSCell selection. For Initiating node type, if the failure is caused by the MN, no SCGFailureInformation will be delivered to the SN node. The transmission of the SCGFailureInformation is kind of implicit indicator of the initiating node type.

# 4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: The signalling flow for R17 UE is different from the one for Pre-R17 UE.

Proposal 2: Includes PSCell failure type in the new XnAP message to the right SN.

Proposal 3: Includes Suitable PSCell CGI in the new XnAP message to the right SN, if the MN decides to configure DC.

Proposal 4: The source SN maintains the UE context by implementation.

Proposal 5: Include S-NG-RAN node UE XnAP ID for the initiating SN to identify the UE in the new XnAP message.

# 5. Reference

- R2-2109208, Reply LS on RACH report for SgNB and information needed for MRO in SCG Failure Report

- R3-211332, LS on information needed for MRO in SCG Failure Report

# Annex 1 – TP on TS 38.423

Start of the first change

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| SCG Failure Report | SCG FAILURE REPORT |

Start of the next change

### 8.4.x	SCG Failure Report

#### 8.4.x.1	General

The purpose of the SCG Failure Report procedure is to transfer SCG mobility related information from M-NG-RAN node to S-NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SCG Failure Report, successful operation

An M-NG-RAN node initiates the procedure by sending an SCG FAILURE REPORT message to S-NG-RAN node. By sending the message M-NG-RAN node indicates to S-NG-RAN node that a PSCell mobility-related problem was detected.

If the PSCell Change Failure Type IE is set to "too early PSCell change" or "triggering PSCell change to wrong PSCell", then the M-NG-RAN node indicates to S-NG-RAN node that, following a PSCell change from a source PSCell to a target PSCell, an SCG failure occurred and the UE established connection either at the original PSCell (too early PSCell change), or at another PSCell (triggering PSCell change to wrong PSCell). If the PSCell Change Failure Type IE is set to "too late PSCell change", then the M-NG-RAN node indicates to S-NG-RAN node that an SCG failure occurred and there was no recent PSCell change before the SCG failure.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

Void.

Start of the next change

#### 9.1.3.x	SCG FAILURE REPORT

This message is sent by the M-NG-RAN node to report a PSCell change failure event to S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| PSCell UE Context ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PSCell Change Failure Type | C- ifUESCGfailureinformationReportContainerabsent |  | ENUMERATED (too early PSCell change, too late PSCell change, triggering PSCell change to wrong PSCell, …) |  |  |  |
| Source PSCell CGI | C- ifUESCGfailureinformationReportContainerabsentand ifPSCellChangeFailureType tooearlyPSCellchangeortriggeringPSCellchangetowrongPSCell |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failure PSCell CGI | C- ifUESCGfailureinformationReportContainerabsent |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where the failure occurred | YES | ignore |
| Suitable PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of the new suitable PSCell | YES | ignore |
| UE SCGfailureinformation Report Container | O |  | OCTET STRING | The received SCGFailureInformation or SCGFailureInformationEUTRA message defined in TS 38.331 [10] from UE. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifUESCGfailureinformationReportContainerabsent | This IE shall be present if the UE SCGfailureinformation Report Container IE is absent. |
| ifUESCGfailureinformationReportContainerabsentandifPSCellChangeFailureType tooearlyPSCellchangeortriggeringPSCellchangetowrongPSCell | This IE shall be present if the UE SCGfailureinformation Report Container IE is absent and if the PSCell Change Failure Type IE is set to the value "too early PSCell change" or "triggering PSCell change to wrong PSCell". |

Start of the next change

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication,

SCGFailureReport

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-sCGFailureReport

-------- skip unchanged part ----------

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport 			|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

sCGFailureReport							,

...

}

-------- skip unchanged part ----------

accessAndMobilityIndication XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY 				ignore

}

sCGFaiureReport XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SCGFailureReport

PROCEDURE CODE			id-sCGFailureReport

CRITICALITY				ignore

}

END

-- ASN1STOP

Start of the next change

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

-------- skip unchanged part ----------

HandoverReportType,

TargetCellinEUTRAN,

C-RNTI,

UERLFReportContainer,

Measurement-ID,

RegistrationRequest,

ReportCharacteristics,

CellToReport,

ReportingPeriodicity,

CellMeasurementResult,

UEHistoryInformationFromTheUE,

MobilityParametersInformation,

MobilityParametersModificationRange,

RACHReportInformation,

IABNodeIndication,

SNTriggered,

SCGIndicator,

UESpecificDRX,

SuccessfulHOReportInformation,

UESCGFailureInformationReportContainer,

PSCellChangeFailureType

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

-------- skip unchanged part ----------

id-UEHistoryInformationFromTheUE,

id-HandoverReportType,

id-HandoverCause,

id-SourceCellCGI,

id-TargetCellCGI,

id-ReEstablishmentCellCGI,

id-TargetCellinEUTRAN,

id-SourceCellCRNTI,

id-UERLFReportContainer,

id-NGRAN-Node1-Measurement-ID,

id-NGRAN-Node2-Measurement-ID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReport,

id-ReportingPeriodicity,

id-CellMeasurementResult,

id-NG-RANnode1CellID,

id-NG-RANnode2CellID,

id-NG-RANnode1MobilityParameters,

id-NG-RANnode2ProposedMobilityParameters,

id-MobilityParametersModificationRange,

id-RACHReportInformation,

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

id-SuccessfulHOReportInformation,

id-PSCellChangeFailureType,

id-SourcePSCellCGI,

id-FailurePSCellCGI,

id-SuitablePSCellCGI,

id-UESCGFailureInformationReportContainer,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows

FROM XnAP-Constants;

-------- skip unchanged part ----------

-- **************************************************************

--

#### -- ACCESS AND MOBILITY INDICATION

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ AccessAndMobilityIndication-IEs}},

...

}

AccessAndMobilityIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RACHReportInformation			CRITICALITY ignore		TYPE RACHReportInformation			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SCG FAILURE REPORT

--

-- **************************************************************

SCGFailureReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SCGFailureReport-IEs}},

...

}

SCGFailureReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-PSCellChangeFailureType			CRITICALITY ignore		TYPE PSCellChangeFailureType				PRESENCE mandatory}|

{ ID id-SourceCellCGI				CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE conditional }|

-- This IE shall be present if the UESCGFailureInformationReportContainer IE is absent and if the PSCell Change Failure Type IE is set to the value "too early PSCell change" or "triggering PSCell change to wrong PSCell". --

{ ID id-FailureCellCGI            	CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE conditional }|

-- This IE shall be present if the UESCGFailureInformationReportContainer IE is absent.

{ ID id-SuitablePSCellCGI   	CRITICALITY ignore		TYPE GlobalNG-RANCell-ID					PRESENCE conditional }|

-- This IE shall be present if the UESCGFailureInformationReportContainer IE is absent.

{ ID id-UESCGFailureInformationReportContainer   		CRITICALITY ignore		TYPE UESCGFailureInformationReportContainer			PRESENCE optional },

...

}

END

-- ASN1STOP

Start of the next change

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

-------- skip unchanged part ----------

#### -- P

-------- skip unchanged part ----------

PDUSession-List ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-ID

PDUSession-List-withCause ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-List-withCause-Item

PDUSession-List-withCause-Item ::= SEQUENCE {

pduSessionId		PDUSession-ID,

cause				Cause				OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withCause-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSession-List-withDataForwardingFromTarget ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingFromTarget-Item

PDUSession-List-withDataForwardingFromTarget-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

dataforwardinginfoTarget			DataForwardingInfoFromTargetNGRANnode,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse		CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional},

...

}

PDUSession-List-withDataForwardingRequest ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingRequest-Item

PDUSession-List-withDataForwardingRequest-Item ::= SEQUENCE {

pduSessionId							PDUSession-ID,

dataforwardingInfofromSource			DataforwardingandOffloadingInfofromSource					OPTIONAL,

dRBtoBeReleasedList						DRBToQoSFlowMapping-List									OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PSCellChangeFailureType ::= ENUMERATED {

tooEarlyPSCellChange,

tooLatePSCellChange,

triggeringPSCellChangeToWrongPSCell,

...

}

-------- skip unchanged part ----------

#### -- U

-------- skip unchanged part ----------

UERLFReportContainer ::= CHOICE {

nR-UERLFReportContainer			UERLFReportContainerNR,

lTE-UERLFReportContainer		UERLFReportContainerLTE,

choice-Extension		ProtocolIE-Single-Container { {UERLFReportContainer-ExtIEs} }

}

UERLFReportContainer-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UERLFReportContainerLTE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-r9 IE contained in the UEInformationResponse message (TS 36.331 [14])

UERLFReportContainerNR ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the nr-RLF-Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10])

UESecurityCapabilities ::= SEQUENCE {

nr-EncyptionAlgorithms					BIT STRING {nea1-128(1),

nea2-128(2),

nea3-128(3)} (SIZE(16, ...)),

nr-IntegrityProtectionAlgorithms		BIT STRING {nia1-128(1),

nia2-128(2),

nia3-128(3)} (SIZE(16, ...)),

e-utra-EncyptionAlgorithms				BIT STRING {eea1-128(1),

eea2-128(2),

eea3-128(3)} (SIZE(16, ...)),

e-utra-IntegrityProtectionAlgorithms	BIT STRING {eia1-128(1),

eia2-128(2),

eia3-128(3)} (SIZE(16, ...)),

iE-Extension			ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} } OPTIONAL,

...

}

UESCGFailureInformationReportContainer ::= CHOICE {

nR-UESCGFailureInformationReportContainer		UESCGFailureInformationReportContainerNR,

lTE-UESCGFailureInformationReportContainer		UESCGFailureInformationReportContainerLTE,

choice-Extension		ProtocolIE-Single-Container { {UESCGFailureInformationReportContainer-ExtIEs} }

}

UESCGFailureInformationReportContainer-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UESCGFailureInformationReportContainerLTE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the SCGFailureInformationEUTRA message (TS 38.331 [10])

UESCGFailureInformationReportContainerNR ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the SCGFailureInformation message (TS 38.331 [10])

Start of the next change

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-sCGFailureReport												ProcedureCode ::= xx

-------- skip unchanged part ----------

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

-------- skip unchanged part ----------

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-PagingeDRXInformation																			ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-SuccessfulHOReportInformation																		ProtocolIE-ID ::= xxx

id-SliceRadioResourceStatus-List																		ProtocolIE-ID ::= yyy

id-CompositeAvailableCapacitySupplementaryUplink														ProtocolIE-ID ::= zzz

id-SCGUEHistoryInformation																	ProtocolIE-ID ::= www

id-PSCellChangeFailureType																				ProtocolIE-ID ::= xxy

id-SourcePSCellCGI																					ProtocolIE-ID ::= xxz

id-FailurePSCellCGI																					ProtocolIE-ID ::= xyy

id-SuitablePSCellCGI																			ProtocolIE-ID ::= xyz

id-UESCGFailureInformationReportContainer																				ProtocolIE-ID ::= xzz

END

-- ASN1STOP

End of the last change

# Annex 2– TP on TS 38.300

Start of the first change

#### 15.5.2.x	PSCell change failure

##### 15.5.2.x.1	General

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. The MN may perform an initial analysis and transfers the XnAP message SCG Failure Information to the relevant SN (see the section 13.x in TS 37.340 [21]).

End of the last change