3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220498

E-meeting, 17 - 26 January 2022

Agenda item:	  24.3

Source:		Lenovo, Motorola Mobility

Title:			On Remaining Issues of CG based SDT

Document for:		Discussion and Decision

1	Introduction

This contribution discusses the following remaining issues on CG-SDT:

- Awareness of SDT bearers in gNB-DU;

- How to provide the CG-SDT resource configuration;

- How to indicate that CG-SDT configuration should be kept in gNB-DU;

- CG-SDT resource configuration release due to SDT-TAT expiry;

- CG-SDT fallback to RA-SDT/non-SDT (CG-SDT is configured but the UE selects RA-SDT/non-SDT).

2	Discussion

Awareness of SDT bearers in gNB-DU

It is still FFS on if gNB-DU needs be aware of which bearers are SDT bearers. In order that the gNB-DU allocates the CG-SDT resource according to the QoS requirements of each related DRB, it makes sense that the gNB-DU is aware of which bearers are SDT bearers. When the gNB-CU sends the CG-SDT Query Indication IE, it would be better to indicate the DRB information together to the gNB-DU. In other words, the CG-SDT Query Indication IE is per DRB basis.

In order that the gNB-DU allocates the CG-SDT resource according to the QoS requirements of each related DRB, the CG-SDT Query Indication IE should be per DRB basis.

How to provide the CG-SDT resource configuration

According to RRC running CR [1], RAN2 has already defined a new IE (i.e., SDT-MAC-PHY-Config IE) to provide the CG-SDT resource configuration

SDT-Config-r17 ::= SEQUENCE { sdt-DRB-List SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity OPTIONAL, -- Need S sdt-SRB2-Indication ENUMERATED {allowed} OPTIONAL, -- Need R sdt-MAC-PHY-Config OCTET STRING {CONTAINING SDT-MACPHY-Config} OPTIONAL, -- Need R sdt-DRB-ContinueROHC ENUMERATED { cell, rna } OPTIONAL -- Need R sdt-DataVolumeThreshold FFS OPTIONAL -- Need R } SDT-MAC-PHY-Config ::= SEQUENCE { -- CG-SDT specific configuration -- FFS on BSR configuration (e.g. i.e. for the FFS on the logicalChannelSR-DelayTimer) -- FFS on delta signalling (We need to clarify how this works, for instance, whether initial BWP dedicated can be considered as baseline to enable delta configuration or not etc). cg-SDT-Config-Initial-BWP-NUL SetupRelease {BWP-Uplink-Dedicated-SDT} OPTIONAL, -- Need M cg-SDT-Config-Initial-BWP-SUL SetupRelease {BWP-Uplink-Dedicated-SDT} OPTIONAL, -- Need M cg-SDT-Config-Initial-BWP-DL BWP-Downlink-Dedicated-SDT OPTIONAL, -- Need M cg-SDT-timeAlignmentTimer TimeAlignmentTimer OPTIONAL, -- Need N cg-TA-ValidityThresholdSSB RSRP-Range OPTIONAL, -- Need R cg-nrofSS-BlocksToAverage INTEGER (1.. maxRS-IndexCellQual-r15) OPTIONAL, -- Need R ... }

There are three opitons for providing the CG-SDT resource configuration from gNB-DU to gNB-CU in the UE CONTEXT MODIFICATION RESPONSE message:

- Option 1: a new RRC container including CG-SDT resource configuration is included outside DU to CU RRC Information IE

- Option 2: a new RRC container including CG-SDT resource configuration is included within DU to CU RRC Information IE

- Option 3: CG-SDT resource configuration is included in the CellGroupConfig IE of the DU to CU RRC Information IE

Since the SDT-MAC-PHY-Config IE is a new IE other than the CellGroupConfig IE, then the option 3 is not workable. And the CellGroupConfig IE is mandatory in the DU to CU RRC Information IE, option 2 needs more clarifications, e.g. on which case the mandatory CellGroupConfig IE should be ignored, which is more complicated than option 1.

A new RRC container including CG-SDT resource configuration (i.e. the SDT-MAC-PHY-Config IE is) is included outside the DU to CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

How to indicate that CG-SDT configuration should be kept in gNB-DU

There is an open issue on how to indicate the CG-SDT resource configuration should be kept in gNB-DU. The following options were discussed in last meeting:

- Option 1: UE CONTEXT RELEASE COMMAND message containing a new indication to keep the CG-SDT resource configuration in gNB-DU.

- Option 2: UE CONTEXT MODIFICATION REQUEST message containing a new indication to keep the CG-SDT resource configuration in gNB-DU.

- Option 3: DL RRC MESSAGE TRANSFER message containing a new indication to keep the CG-SDT resource configuration in gNB-DU.

- Option 4: UE CONTEXT RELEASE COMMAND message containing a list of unreleased SDT bearers to keep some of SDT bearers.

- Option 5: DL RRC MESSAGE TRANSFER message without a new indication to keep the CG-SDT resource configuration in gNB-DU.

Since the gNB-CU needs to send the UE CONTEXT RELEASE COMMAND message including the RRCRelease message, which includes the CG-SDT resource configuration anyway, it is not necessary to use additional message to indicate the gNB-DU keeps the CG-SDT resource configuration.

A new indication to keep the CG-SDT resource configuration in gNB-DU is included in the UE CONTEXT RELEASE COMMAND message. FFS on whether the DRB IDs of SDT bearers should be included or not.

- CG-SDT resource configuration release due to SDT-TAT expiry

Regarding the release if CG-SDT resource configuration due to SDT-TAT expiry, there is an FFS:

FFS on whether there is F1 impact to support the release of CG-SDT resource configuration due to TAT-SDT expiry.

In RAN2#113e, it was agreed that

Agreements

- TAT-SDT is started upon receiving the TAT-SDT configuration from gNB, i.e. RRCrelease message, and can be (re)started upon reception of TA command.

- UE releases CG-SDT resources when TAT expires in RRC_Inactive state

In the SDT RRC running CR [1], it has been specified that

5.3.x UE actions upon CG-SDT resource release request Upon receiving a CG-SDT resource release request from lower layers, the UE shall: 1> release the stored CG-SDT configuration in sdt-MAC-PHY-Config. Editor’s Note: The exact IE name and the parameters to release above are FFS – we can finalise this after the ASN.1 is stable since this will depend on the final structure (including the structure used for delta-configuration)

- Observation 1: RAN2 agreed that UE releases CG-SDT resource configuration when SDT-TAT expiries.

Same with UE’s behaviour, the gNB-DU also needs to release the CG-SDT resource configuration due to SDT-TAT expires. Besides the CG-SDT resource configuration, how to release, the kept UE associated logical F1 connection and F1-U resource between gNB-CU and gNB-DU needs to be addressed.

One solution is the gNB-DU starts the SDT-TAT upon it is configured or upon sending RRCRelease message to the UE. When the SDT-TAT expires, the gNB-DU triggers a UE Context Release Request procedure to request the gNB-CU releases the kept UE-associated logical F1 connection. For example, the gNB-DU sends a UE CONTEXT RELEASE REQUEST message to the gNB-CU, and within the message, a new cause value ‘SDT-TAT expiry’ is included to indicate the release request is caused by the TAT-SDT expiry.

Another solution is the gNB-DU sends SDT-TAT to the gNB-CU. The gNB-CU starts the SDT-TAT timer upon sending RRCRelease message to the UE. When TAT-SDT expires, both the gNB-DU and gNB-CU release the kept UE associated logical F1 connection, F1-U resource and related CG-SDT resource configuration locally.

When SDT-TAT expiry, the kept CG-SDT resource configuration, the UE associated logical F1 connection and F1-U resource should be released. RAN3 selects one of the following solutions:

-	Solution 1: gNB-DU starts the SDT-TAT. When the SDT-TAT expires, gNB-DU triggers the UE Context Release Request procedure with a new cause ‘SDT-TAT expiry’.

-	Solution 2: gNB-DU provides the SDT-TAT to gNB-CU when providing CG-SDT resource configuration. When TAT-SDT expires, both the gNB-DU and gNB-CU release it locally.

CG-SDT fallback to RA-SDT/non-SDT (CG-SDT is configured but the UE selects RA-SDT/non-SDT)

In RAN2#113e, it was agreed that CG-SDT may fallback to RA-based SDT:

-	Upon arrival of data only for DRB/SRB(s) for which SDT is enabled, the high level procedure for selection between SDT and non SDT procedure is as follows:

-	If CG-SDT criteria is met: UE selects CG-SDT. UE initiate SDT procedure

-	Else if RA-SDT criteria is met: UE selects RA-SDT. UE initiate SDT procedure

-	Else: UE initiate non SDT procedure.

Figure 2. issue in case CG-SDT fallbacks to RA-SDT/non-SDT

When CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure, a completely new UE associated logical F1 connection is setup for the UE during RA-SDT or non-SDT procedure. In the legacy behavior, when a new UE associated logical F1 connection is setup, the old UE associated logical F1 connection is released. But it does not allow delta configuration of CG-SDT resource configuration due to the old CG-SDT resource is completely released by the gNB-DU.

- Observation 2: In case that CG-SDT is configured but the UE selects RA-SDT or non-SDT procedure, the legacy behavior does not allow delta configuration of CG-SDT resource configuration due to the old CG-SDT resource is completely released.

In order to support delta configuration of CG-SDT resource configuration in case that CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure, the following solutions should be considered:

Solution 1: The gNB-CU stores the old CG-SDT resource configuration. when CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure, the gNB-CU provides the old CG-SDT resource configuration to the gNB-DU when a new CG-SDT resource configuration is needed.

Solution 2: when CG-SDT is configured but the UE selects RA-SDT or non-SDT procedure, the gNB-CU does not trigger to release the old UE association F1 connection immediately and provides the old gNB-DU F1AP UE ID to the gNB-DU. The gNB retrieves the old CG-SDT resource configuration based on the old gNB-DU F1AP UE ID.

In case that CG-SDT is configured but the UE selects RA-SDT/non-SDT procedure, in order to support delta configuration of CG-SDT resource configuration, RAN3 discusses and selects one of the following solutions:

-	Solution 1: The gNB-CU stores the old CG-SDT resource configuration. When CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure, the gNB-CU provides the old CG-SDT resource configuration to the gNB-DU when a new CG-SDT resource configuration is needed.

-	Solution 2: when CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure, the gNB-CU does not trigger to release the old UE association F1 connection immediately and provides the old gNB-DU F1AP UE ID to the gNB-DU when a new CG-SDT resource configuration is needed. The gNB retrieves the old CG-SDT resource configuration based on the old gNB-DU F1AP UE ID.

3	Conclusion

This contribution discusses the remaining issues on CG based SDT. The following observations are made:

- Observation 1: RAN2 agreed that UE releases CG-SDT resource configuration when SDT-TAT expiries.

- Observation 2: In case that CG-SDT is configured but the UE selects RA-SDT or non-SDT procedure, the legacy behavior does not allow delta configuration of CG-SDT resource configuration due to the old CG-SDT resource is completely released.

And the following proposals are made:

- In order that the gNB-DU allocates the CG-SDT resource according to the QoS requirements of each related DRB, the CG-SDT Query Indication IE should be per DRB basis.

- A new RRC container including CG-SDT resource configuration (i.e. the SDT-MAC-PHY-Config IE is) is included outside the DU to CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

- A new indication to keep the CG-SDT resource configuration in gNB-DU is included in the UE CONTEXT RELEASE COMMAND message. FFS on whether the DRB IDs of SDT bearers should be included or not.

- F1 connection and F1-U resource should be released. RAN3 selects one of the following solutions:

-	Solution 1: gNB-DU starts the SDT-TAT. When the SDT-TAT expires, gNB-DU triggers the UE Context Release Request procedure with a new cause ‘SDT-TAT expiry’.

-	Solution 2: gNB-DU provides the SDT-TAT to gNB-CU when providing CG-SDT resource configuration. When TAT-SDT expires, both the gNB-DU and gNB-CU release it locally.

In case that CG-SDT is configured but the UE selects RA-SDT/non-SDT procedure, in order to support delta configuration of CG-SDT resource configuration, RAN3 discusses and selects one of the following solutions:

-	Solution 1: The gNB-CU stores the old CG-SDT resource configuration. When CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure, the gNB-CU provides the old CG-SDT resource configuration to the gNB-DU when a new CG-SDT resource configuration is needed.

-	Solution 2: when CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure, the gNB-CU does not trigger to release the old UE association F1 connection immediately and provides the old gNB-DU F1AP UE ID to the gNB-DU when a new CG-SDT resource configuration is needed. The gNB retrieves the old CG-SDT resource configuration based on the old gNB-DU F1AP UE ID.

Reference

[1]. R2-2200050. SDT RRC Running CR. ZTE.