3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220503

# Electronic meeting, 17th – 26th Nov 2021

Agenda item:		18.4.3

Source:	Intel Corporation

Title:	AI/ML based mobility optimization

Document for:		Discussion and Decision

# Introduction

During RAN3 #114e meeting, potential signaling flows and required input and output data were discussed and captured in TR37.817 [1]. In this contribution, we continue discuss FFS in the solutions and standard impact of AI-based mobility optimization.

# Discussion

## UE Selection for AI-based Mobility Optimization

It was agreed in [1] that UE location information is needed as input to generate AI/ML based mobility decision. To get the location of one UE, either network or UE need to perform positioning function. When UE is performing positioning, it is very power consuming for a UE, as, e.g. for DL positioning, it needs to monitor PRS, estimate the different angles of arriving signaling and perform positioning algorithm calculation. Positioning is not a mandatory feature at the UE side. This makes the UE selection for AI-based mobility optimization more necessary and realistic, as only the selected UE(s) need to perform positioning and AI/ML based location prediction. This can save a lot of power at the innocent UE(s).

- Performing positioning at UE side is power consuming, UE selection can help saving powers at the innocent UEs.

For mobility optimization, UE selection may be considered based on following criteria:

- Statistic of unintended mobility events

If the successful handover events collected for certain UE is very low, using AI/ML model with predicted information may greatly increase handover successful rate. The unintended events may include: Number of failed handover preparations, Number of failed handover resource allocation, Number of failed handover execution, etc. If the failed event collected for one UE is higher than a certain threshold (e.g. the HO failure ratio is larger than certain value), the network can select this UE to use AI/ML prediction and improve the handover success rate.

Alternatively, the threshold can also be set by the ratio between above event counts and the corresponding requested handover events.

- UE location information accuracy status

The accuracy of the AI/ML model outcome highly depends on the coverage of the training data set. Sometimes, for a certain environment or scenario, the AI/ML model may not be able to make the right/precise decision (as the input is far beyond the data set coverage) and has low accuracy results. In mobility use cases, the wrong decision will lead to severe service interruption caused by failed handover. Hence, in the trigger condition for the gNB to seek additional AI support, a measure of past accuracy result for these scenarios along with the UE information could be incorporated. In this case, it is proposed the AI/ML based mobility model is used for the UEs with higher accuracy results which may be for more common scenarios for, e.g. moving along the road with a fixed speed/direction, etc. The easier UE’s trajectory can be predictable, the higher accuracy of AI/ML generated predicted handover. Hence, the network may select the UE based on, for example, UE’s current/future speed under a threshold, UE’s current/future speed is not changed for a certain time, UE’s current/future moving direction is unchanged for a certain time, etc.

- Network should be able to select UEs to perform AI/ML based mobility optimization based on the number of unintended mobility events, or UE location information accuracy status.

## Input data of Mobility optimization use case

In legacy handover procedure, handover is normally triggered by measurement report, i.e. UE is experience RLF, low RSRP, RSRQ, SINR, etc. Except for L3 measurements, it is also important to handover UEs who are experiencing poor channel condition to a new target cell if the channel parameters are poor. Hence, it is important for AI/ML based mobility use case to predict channel condition of one UE, so that AI/ML model can handover the UE to a suitable target cell before user experience goes down. Channel condition, UE-specific radio-related information, etc can be used as input for AI/ML based mobility optimization, so that network can predict the best timing of UE’s handover and avoid poor user experience caused by the predicted poor channel condition in the source cell. Following radio-related information at the source NG-RAN node for the specific UE are proposed to be used as input for AI/ML based mobility optimization:

- CQI information (e.g. averaged rate of CQI)

- SRS

- Input data of AI/ML-based mobility optimization includes UE’s CQI and SRS from source NG-RAN node, which is used to predicted handover before channel condition becomes poor.

## Output data of Mobility optimization use case

UE trajectory prediction

As discussed in RAN3 #114e, it is FFS whether the UE trajectory prediction is an internal output to the node hosting the Model Inference function or not [1]:

- Note:FFS whether the UE trajectory prediction is an internal output to the node hosting the Model Inference function

The UE trajectory prediction is mainly used to predict the target NG-RAN node based on the future UE location. If the predicted UE location falls into the coverage of the neighbour NG-RAN node, this NG-RAN node can be considered as the target NG-RAN node for mobility optimization. It can be considered as a mid-term output which is used within the source NG-RAN node.

Additionally, the predicted target NG-RAN node does not need to know the predicted UE trajectory, as all the mobility decisions are done at the source NG-RAN node.

- UE trajectory prediction is an internal output to the node hosting Model Inference function.

As captured in [1], except UE trajectory prediction, following output data are considered for AI/ML-based mobility optimization:

- Estimated arrival probability in CHO and relevant confidence interval - Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication

However, it is not clear whether the predicted target node is configured with or without CHO. From our understanding, the predicted target node(s) can either be considered as output without configuring CHO or candidate cell for CHO configuration. Hence, in following context, we discuss the predicted HO strategy and CHO strategy separately.

HO strategy

AI/ML based mobility optimization model can generate one/multiple candidate target cells as output of the AI/ML model. The network can select the target cell with the highest probability, which representing the best candidate cell for handover. Together with the predicted target cells, the AI/ML model can also provide when the handover should be performed (i.e. the best time to perform handover) based on the QoS requirement and channel condition of the user. Then the network can trigger the handover command to the UE at the corresponding time.

To reduce service interruption, gNB further sends the predicted timing and selected target cell of predicted handover to the UE. Furthermore, gNB also sends the configuration of the selected UEs to the selected target cell. Hence, both target gNB and UE can prepare for the handover before it actually happens. There’s no additional condition need to be met before handover is executed, which is different from CHO..

- Besides the predicted handover cell(s), the predicted handover timing corresponding to each predicted cell is also generated as output of AI/ML based mobility optimization.

- To reduce service interruption, the source gNB configures the UE with predicted handover target cell and corresponding handover execution time. The source gNB also sends the handover execution time to the predicted target gNB.

CHO strategy

Predicted handover target node, candidate cells in CHO is agreed as one of the outputs for mobility optimization use case.

According to the UE’s predicted location, besides the arrival probability, AI/ML model can generate a list of candidate target cells together with priority and handover execution timestamp. Network can select the CHO candidate cells based on the probability of this list of candidate cells, then configure the list to the UE as part of conditional handover. Hence, UE can select one target cell in the list and perform handover at the corresponding handover execution timestamp. The selected handover target cell is the one which has the highest priority among candidate cells in the list.

- The source NG-RAN node configures UE with the probability, priority of predicted candidate target cells in CHO configuration, together with handover execution timing of each candidate cells.

Validity Time

Another key information that needs to be considered is validity time of model inference output. Depending on environment changes, an output from model inference can be outdated sooner or later. Hence, it is essential to include one “validity time” to indicate how long the prediction results can be valid. Model inference of mobility optimization use case can select a suitable target handover cell for a certain UE, and further send a list of candidate cells to UE as CHO configuration. Meanwhile, considering UE is still moving and channel status is changing, it may result in inaccuracy of previous predicted candidate cells. Hence, for each predicted candidate cells, validity time of each candidate cell should be generated together with each option as the output of Model Inference.

When the validity time expires, UE may no longer consider the corresponding candidate cell for handover. This could also avoid network sending another deactivation configuration to the UE to disable certain predicted candidate cell(s) from CHO list.

- Validity time corresponding to predicted handover cell and predicted candidate cell is the output of AI/ML based mobility optimization.

## Feedback of Moblity optimization use case

Performance of handed-over UE from neighbouring NG-RAN node

As defined in RAN intelligence functional framework [1], Actor shall provide feedback to Data Collection function. In both deployment scenarios, the target NG-RAN node is the Actor node in the functional framework. To continuously improve AI/ML model performance and accuracy, performance of handed-over UE should be reported from neighbouring NG-RAN node.

- For deployment scenario “Model Training” at OAM, “Model Inference” at NG-RAN, the neighbouring NG-RAN node reports performance of handed-over UE to OAM. If online training is performed at NG-RAN node, the performance feedback is also reported to the source NG-RAN node.

- For deployment scenario “Model Training” and “Model Inference” at NG-RAN, the neighbouring NG-RAN node reports performance of handed-over UE to the source NG-RAN node via Xn interface.

The performance of handed-over UE may include throughput, packet delay, etc.

- The neighboring target NG-RAN node reports performance (e.g. throughput, packet delay, etc) of handed-over UE to the source NG-RAN node or OAM.

A text proposal for the above solutions and standard impact is provided in Annex A.

- Agree the corresponding text proposal to TR 37.817 in Annex A.

# Conclusion

In this contribution, we further discussed the required input/output data used for AI/ML based mobility optimization.

We propose the following observations and proposals:

Observation 1: Performing positioning at UE side is power consuming, UE selection can help saving powers at the innocent UEs.

Proposal 1: Network should be able to select UEs to perform AI/ML based mobility optimization based on the number of unintended mobility events, or UE location information accuracy status.

Proposal 2: Input data of AI/ML-based mobility optimization includes UE’s CQI and SRS from source NG-RAN node, which is used to predicted handover before channel condition becomes poor.

Proposal 3: UE trajectory prediction is an internal output to the node hosting Model Inference function.

Proposal 4: Besides the predicted handover cell(s), the predicted handover timing corresponding to each predicted cell is also generated as output of AI/ML based mobility optimization.

Proposal 5: To reduce service interruption, the source gNB configures the UE with predicted handover target cell and corresponding handover execution time. The source gNB also sends the handover execution time to the predicted target gNB.

Proposal 6: The source NG-RAN node configures UE with the probability, priority of predicted candidate target cells in CHO configuration, together with handover execution timing of each candidate cells.

Proposal 7: Validity time corresponding to predicted handover cell and predicted candidate cell is the output of AI/ML based mobility optimization.

Proposal 8: The neighboring target NG-RAN node reports performance (e.g. throughput, packet delay, etc) of handed-over UE to the source NG-RAN node or OAM.

Proposal 9: Agree the corresponding text proposal to TR 37.817 in Annex A.

# References

- R3-216278, Draft TR37.817, Study on enhancement for Data Collection for NR and EN-DC

# Annex A – Text Proposal to TR 37.817

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# 5       Use Cases and Solutions for Artificial Intelligence in RAN

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 5.3	Mobility Optimization

### 5.3.1	Use case description

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 5.3.2 Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

          - 5.3.2.1			Locations for AI/ML Model Training and AI/ML Model Inference

Considering the locations of AI/ML Model Training and AI/ML Model Inference for mobility solution, the following two options are considered:

- The AI/ML Model Training function is deployed in OAM, while the Model Inference function resides within the RAN node

- Both the AI/ML Model Training function and the AI/ML Model Inference function reside within the RAN node

Furthermore, for CU-DU split scenario, following option is possible:

- AI/ML Model Training is located in CU-CP or OAM, and AI/ML Model Inference function is located in CU-CP

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM.

          - 5.3.2.2			AI/ML Model Training in OAM and AI/ML Model Inference in NG-RAN node

NG-RAN node

OAM

UE

4. Model Training

3. Measurement Report

5. ML Model Deployment

7. Model Inference

9. Action:

Mobility Optimization

6. Measurement Report

8. ML Performance Feedback

2. Measurement

1.

Measurement  Configuration

3. Measurement Report

Figure 5.3-1  AI/ML Model Training in OAM and AI/ML Model Inference in NG-RAN node

Step 1. The NG-RAN node configures the measurement information on the UE side and sends configuration message to UE including configuration information. Network may select proper UE to perform AI/ML-based mobility optimization based on e.g. the number of unintended mobility events, or UE location information accuracy status, etc.

Step 2. The UE collects the indicated measurement, e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3. The UE  sends measurement report message to OAM via NG-RAN node including the required measurement.

Step 4. Model Training.  Required measurements are leveraged to training ML model for UE mobility optimization.

Step 5. OAM sends ML Model Deployment Message to deploy the trained/updated ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Note:This step is out of RAN3 Rel-17 scope.

Step 6. The NG-RAN node obtains the measurement report as inference data for UE mobility optimization.

Step 7. Model Inference. Required measurements are leveraged into Model Inference to output the prediction, e.g.  UE trajectory prediction, target cell prediction, target NG-RAN node prediction, etc.

Step 8. The NG-RAN sends the AI/ML model performance feedback to OAM(FFS).

Step 9. According to the prediction, recommended actions or configuration are executed for Mobility Optimization.

          - 5.3.2.3			AI/ML Model Training and AI/ML Model Inference in a NG-RAN node

Figure 5.3-2: Model Training and Model Inference both located in RAN node

Step 1. NG-RAN node1 configures the measurement information on the UE side and sends configuration message to UE including configuration information. Network may select proper UE to perform AI/ML-based mobility optimization based on e.g. the number of unintended mobility events, or UE location information accuracy status, etc.

Step 2. UE collects the indicated measurement, e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3. UE sends measurement report message to NG-RAN node1 including the required measurement.

Step 4. Model training. Required measurements are leveraged to training ML model for mobility optimization.

Step 5. NG-RAN node1 obtains the measurement report as inference data for real-time UE mobility optimization.

Step 6. Model Inference. Required measurements are leveraged into Model Inference to output the prediction, including e.g., UE trajectory prediction, target cell prediction, target NG-RAN node prediction, etc.

Step 7. According to the prediction, recommended actions are executed for Mobility Optimization.

Step 8. The NG-RAN node 1 sends handover request message to the NG-RAN node 2.

Step 9. NG-RAN node 2 sends feedback to NG-RAN node 1.

          - 5.3.2.4			Input data

The following data is required as input data for mobility optimization.

Input Information from the UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available.

- Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

- UE historical serving cells and their locations

- Moving velocity

- UE radio-related information, e.g. CQI information, SRS

- FFS predicted traffic

Input Information from the neighbouring RAN nodes:

- UE’s successful handover information in the past and received from neighboring RAN nodes

- UE’s history information from neighbor

- Position, resource status, FFS QoS parameters of historical HO-ed UE (e.g., loss rate, delay, etc.)

- Resource status and utilization prediction/estimation

- SON Reports of handovers that are successful, too-early, too-late, or handover to wrong (sub-optimal) cell

- Information about the performance of handed over UEs

- Resource status prediction

Input Information from the local node:

- UE trajectory prediction output (will be used by the RAN node internally)

- Local resource status prediction

If existing UE measurements are needed by a gNB for AI/ML-based mobility optimization, RAN3 shall reuse the existing framework (including MDT and RRM measurements). FFS on whether new UE measurements are needed.

          - 5.3.2.5			Output data

- UE trajectory prediction (Latitude, longitude, altitude, cell ID of UE over a future period of time). This is an internal output to the node hosting the Model Inference function

- HO strategy, including predicted handover target node(s), and confidence of the prediction, predicted handover execution timing, validity time

- CHO strategy, including predicted candidate CHO cells, and corresponding estimated arrival probability, priority, confidence of prediction, handover execution timing, validity time

          - 5.3.2.6			Standard impact

To improve the mobility decisions at a gNB (gNB-CU), a gNB can request mobility feedback from a neighbouring node. Details of the procedure are FFS.

- Potential Xn interface impact:

  - Predicted resource status info and performance info from candidate target NG-RAN node to source NG-RAN node

- Potential Uu interface impact

  - Predicted CHO configurations

#### 5.3.2.5 Feedback

- Throughput, packet delay of the handed-over UE, etc