3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220584

# Online, 17th-26th Jan 2022

Agenda Item:	10.3.2.1

Source:	Ericsson

Title:	(TP for MDT BL CR for TS 38.413, TS38.423) Introduction of the on Beam level measurement report configuration

Document for:	Other

# 1	Introduction

In R2-2111476, RAN2 confirmed that:

- includeBeamMeasurements is mandatorily configured by gNB as part of ReportConfigNR with the value TRUE or FALSE.

- the OAM could indicate operator’s preference the beam level measurement inclusion (includeBeamMeasurements) in the measurement reports associated to the M1 measurement so that the beam level coverage map can be generated by the OAM.

- an indicator (includeBeamMeasurements) can be added in the M1 Configuration signaled over the RAN interfaces (e.g., NG, Xn, F1) to indicate to RAN whether the beam measurements are preferred be included for MDT purposes by the UE for M1 measurements or not.

In addition, RAN2 wanted to inform RAN3 that the reference signal type (i.e. SSB and CSI-RS) used for reporting the beam level measurement information is the same as that of cell level measurement information.

In S5-216628, SA5 informed RAN3 that it has agreed to address the beam level configuration in the case of immediate MDT measurement in NR. Such agreements include requirement and configuration parameters for beam level measurements.

This contribution proposes the introduction of Beam measurement report configurations on M1 in accordance with the agreed contributions.

# 2	Discussion

RAN3 has discussed the Beam level report configuration for MDT measurements in previous meetings and in particular during RAN3#113[1], where the referenced LS[2] has been sent to RAN2 and SA5.

While TS32.422 specifies that the M1 measurements may include cell and/or beam level measurements, the M1 configuration signalled over the NG and Xn interfaces does not specify whether beam measurements should be reported or not.

In the LS RAN3 asked RAN2 and SA5 to indicate whether the M1 Configuration signalled over the RAN interfaces should be corrected with the addition of information describing whether and how beam measurements should be configured at the UE for M1 measurements, and in particular whetherit is feasible to include in the M1 Configuration signalled over the RAN interfaces information describing whether and how beam measurements should be configured at the UE.

Both RAN2 and SA5 confirmed their acceptance in introducing the beam level measurement report configuration for M1 measurements as was proposed in [1] and liaised in [2].

The M1 measurement configuration currently defined, e.g. in TS38.413 is as follows:

#### 9.3.1.171	M1 Configuration

This IE defines the parameters for M1 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, A2event-triggered periodic, …) |  |
| M1 Threshold Event A2 | C-ifM1A2trigger |  |  |  |
| >CHOICE Threshold Type | M |  |  |  |
| >>RSRP |  |  |  |  |
| >>>Threshold RSRP | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| >>RSRQ |  |  |  |  |
| >>>Threshold RSRQ | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| >>SINR |  |  |  |  |
| >>>Threshold SINR | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| M1 Periodic Reporting | C-ifperiodicMDT |  |  |  |
| >Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60) | This IE is defined in TS 38.331 [18]. |
| >Report Amount | M |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity) | Number of reports. |

| Condition | Explanation |
| --- | --- |
| C-ifM1A2trigger | This IE shall be present if the M1 Reporting Trigger IE is set to “A2event-triggered” or to “A2event-triggered periodic”. |
| C-ifperiodicMDT | This IE shall be present if the M1 Reporting Trigger IE is set to “periodic”, or to “A2event-triggered periodic”. |

As it can be seen, there is no information in the M1 configuration regarding whether beam level measurements should be reported by the UE. This contradicts the requirements from TS32.422 that the M1 measurements should be able to report cell and/or beam level measurements.

This also goes in contrast with the M1 measurement configuration provided by the RAN to the UE over RRC. In fact, over RRC, the following measurement configuration is provided (excerpt from TS38.331):

ReportConfigNR information element

-- ASN1START

-- TAG-REPORTCONFIGNR-START

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16

}

}

-Text Omitted-

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

eventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA3                                     SEQUENCE {

a3-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA5                                     SEQUENCE {

a5-Threshold1                               MeasTriggerQuantity,

a5-Threshold2                               MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA6                                     SEQUENCE {

a6-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

...

},

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                     MeasReportQuantity                                            OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

reportAddNeighMeas                          ENUMERATED {setup}                                             OPTIONAL,   -- Need R

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

useT312-r16                                 BOOLEAN                                                        OPTIONAL,   -- Need M

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]]

}

-Text Omitted-

As it can be seen from the text highlighted above, the RRC protocol offers the possibility to configure M1 measurements with or without beam measurements by configuring the “includeBeamMeasurements” IE.

Note that the includeBeamMeasurements IE is a mandatory IE in the measurement configuration signalled to the UE. It is therefore evident that the current M1 configuration over the NG and Xn interfaces does not offer any information explaining how the includeBeamMeasurements IE should be configured. This leads to an interoperability problem because:

- Configuration of beam measurements is left to RAN implementation and not specified

- A RAN implementation may choose not to configure beam measurements, even if the operator needs such measurements as part of its M1 measurement reports

- The OAM system, responsible for configuring M1 measurements for MDT, has no control on how to trigger reporting of beam level measurements from the UE

The following is therefore proposed:

Proposal 1: it is proposed to enhance the M1 Measurement configuration with the possibility of beam measurement level reporting

TPs mirroring the proposals above are presented in the Annexes below.

# 3	Conclusion

In this contribution the following conclusions and proposals were derived

Proposal 1: it is proposed to enhance the M1 Measurement configuration with the possibility of beam measurement level reporting

TPs mirroring the proposals above are presented in the Annexes below.

# References

- R3-213826, “(TP for MDT BL CR for TS 38.413, TS38.423) TPs for the introduction of the on Beam level measurement report configuration in TS38.413 and TS38.423”, Ericsson

- R3-214475, "LS on the Beam measurement reports for the MDT measurements"

- R2-2111476, “Reply LS on Beam measurement reports for the MDT measurements”

- S5-216628, “Reply LS on the Beam measurement reports for the MDT measurements”

# TP for TS38.413

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.171	M1 Configuration

This IE defines the parameters for M1 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, A2event-triggered periodic, …) |  |
| M1 Threshold Event A2 | C-ifM1A2trigger |  |  |  |
| >CHOICE Threshold Type | M |  |  |  |
| >>RSRP |  |  |  |  |
| >>>Threshold RSRP | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| >>RSRQ |  |  |  |  |
| >>>Threshold RSRQ | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| >>SINR |  |  |  |  |
| >>>Threshold SINR | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| M1 Periodic Reporting | C-ifperiodicMDT |  |  |  |
| >Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60) | This IE is defined in TS 38.331 [18]. |
| >Report Amount | M |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity) | Number of reports. |
| Include Beam Measurements Indication | M |  | ENUMERATED (true, …) | To configure whether the UE should include the beam level measurements. This IE is defined in TS 38.331 [18]. |
| Beam Measurements Report configuration | C-ifM1BeamMeasInd |  |  | Included in case of include the beam level measurements indication for measurement M1. |
| >M1 reportQuantityRS-Indexes | O |  |  | This IE indicates which measurement information per RS index the UE shall include in the measurement report. It is defined in TS 38.331 [18]. |
| >>RSRP | M |  | ENUMARATED (true, …) |  |
| >>RSRQ | M |  | ENUMARATED (true, …) |  |

| Condition | Explanation |
| --- | --- |
| C-ifM1A2trigger | This IE shall be present if the M1 Reporting Trigger IE is set to “A2event-triggered” or to “A2event-triggered periodic”. |
| C-ifperiodicMDT | This IE shall be present if the M1 Reporting Trigger IE is set to “periodic”, or to “A2event-triggered periodic”. |
| C- ifM1BeamMeasInd | This IE shall be present if the Include Beam Measurements Indication IE is set to “true”. |

| Range bound | Explanation |
| --- | --- |
| maxNrofIndexesToReport | Maximum no. Max number of beam related measurements to To Report. Value is 32. |

<<<<<<<<<<<<<<<<<<<< End of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-ConfiguredTACIndication,

id-CurrentQoSParaSetIndex,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-EndpointIPAddressAndPort,

id-ExtendedPacketDelayBudget,

id-ExtendedRATRestrictionInformation,

id-ExtendedSliceSupportList,

id-ExtendedTAISliceSupportList,

id-ExtendedUEIdentityIndexValue,

id-GlobalCable-ID,

id-GlobalRANNodeID,

id-GlobalTNGF-ID,

id-GlobalTWIF-ID,

id-GlobalW-AGF-ID,

id-GUAMIType,

id-LastEUTRAN-PLMNIdentity,

id-LocationReportingAdditionalInfo,

id-MaximumIntegrityProtectedDataRate-DL,

id-MDTConfiguration,

id-MicoAllPLMN,

id-NetworkInstance,

id-NID,

id-NPN-MobilityInformation,

id-NPN-PagingAssistanceInformation,

id-NPN-Support,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-PagingAssisDataforCEcapabUE,

id-PagingeDRXInformation,

id-PDUSessionAggregateMaximumBitRate,

id-PduSessionExpectedUEActivityBehaviour,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceReleaseResponseTransfer,

id-PDUSessionType,

id-PSCellInformation,

id-QosFlowAddOrModifyRequestList,

id-QosFlowFailedToSetupList,

id-QosFlowFeedbackList,

id-QosFlowParametersList,

id-QosFlowSetupRequestList,

id-QosFlowToReleaseList,

id-QosMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-RAT-Information,

id-RedundantCommonNetworkInstance,

id-RedundantDL-NGU-TNLInformationReused,

id-RedundantDL-NGU-UP-TNLInformation,

id-RedundantDLQosFlowPerTNLInformation,

id-RedundantPDUSessionInformation,

id-RedundantQosFlowIndicator,

id-RedundantUL-NGU-UP-TNLInformation,

id-SCTP-TLAs,

id-SecondaryRATUsageInformation,

id-SecurityIndication,

id-SecurityResult,

id-SgNB-UE-X2AP-ID,

id-S-NSSAI,

id-SONInformationReport,

id-TNLAssociationTransportLayerAddressNGRAN,

id-TargetRNC-ID,

id-TraceCollectionEntityURI,

id-TSCTrafficCharacteristics,

id-UEHistoryInformationFromTheUE,

id-UERadioCapabilityForPaging,

id-UERadioCapabilityForPagingOfNB-IoT,

id-UL-NGU-UP-TNLInformation,

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

id-UsedRSNInformation,

id-UserLocationInformationTNGF,

id-UserLocationInformationTWIF,

id-UserLocationInformationW-AGF,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofBluetoothName,

maxnoofBPLMNs,

id-m1includeBeamMeasurements,

id-M1BeamMeasurementsReportConfiguration,

maxnoofCAGSperCell,

maxnoofCandidateCells,

maxnoofCellIDforMDT,

maxnoofCellIDforWarning,

maxnoofCellinAoI,

maxnoofCellinEAI,

maxnoofCellsingNB,

maxnoofCellsinngeNB,

maxnoofCellinTAI,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEmergencyAreaID,

maxnoofEAIforRestart,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofE-RABs,

maxnoofErrors,

maxnoofExtSliceItems,

maxnoofForbTACs,

maxnoofFreqforMDT,

maxnoofMDTPLMNs,

maxnoofMultiConnectivity,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighPCIforMDT,

maxnoofNGConnectionsToReset,

maxNRARFCN,

maxnoofNRCellBands,

maxnoofPC5QoSFlows,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofQosFlows,

maxnoofQosParaSets,

maxnoofRANNodeinAoI,

maxnoofRecommendedCells,

maxnoofRecommendedRANNodes,

maxnoofAoI,

maxnoofSensorName,

maxnoofServedGUAMIs,

maxnoofSliceItems,

maxnoofTACs,

maxnoofTAforMDT,

maxnoofTAIforInactive,

maxnoofTAIforPaging,

maxnoofTAIforRestart,

maxnoofTAIforWarning,

maxnoofTAIinAoI,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofWLANName,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxNrofIndexesToReport

FROM NGAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NGAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Container{},

NGAP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

NGAP-PROTOCOL-IES

FROM NGAP-Containers;

-- TEXT OMITTED –

MeasurementsToActivate ::= BIT STRING(SIZE(8))

<<<<<<<<<<<<<<<<<<<< 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

M1BeamMeasurementsReportConfiguration	::= SEQUENCE {

reportQuantityRS-Indexes	M1measReportQuantity	OPTIONAL,

maxNrofRS-IndexesToReport	M1maxNrofRS-IndexesToReport	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { M1BeamMeasurementsReportConfiguration-ExtIEs} } OPTIONAL,

...

}

M1BeamMeasurementsReportConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M1Configuration ::= SEQUENCE {

m1reportingTrigger			M1ReportingTrigger,

m1thresholdEventA2			M1ThresholdEventA2									OPTIONAL,

--	The above IE shall be present if the M1 Reporting Trigger IE is set to “A2event-triggered” or “A2event-triggered periodic”

m1periodicReporting			M1PeriodicReporting									OPTIONAL,

--	The above IE shall be present if the M1 Reporting Trigger IE is set to “periodic” or “A2event-triggered periodic”

iE-Extensions		ProtocolExtensionContainer { { M1Configuration-ExtIEs} } 	OPTIONAL,

...

}

M1Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-m1includeBeamMeasurements		CRITICALITY ignore	EXTENSION M1includeBeamMeasurements		PRESENCE mandatory		}|

{ ID id-M1BeamMeasurementsReportConfiguration		CRITICALITY ignore	EXTENSION M1BeamMeasurementsReportConfiguration		PRESENCE optional		},

-- Included in case of M1includeBeamMeasurements set to true

...

}

M1includeBeamMeasurements ::= ENUMERATED{

True,

...

}

M1maxNrofRS-IndexesToReport	::= INTEGER (1..maxNrofIndexesToReport)

M1measReportQuantity

M1MeasReportQuantity ::= SEQUENCE {

rSRP						ENUMERATED {true, ...},

rSRQ						ENUMERATED {true, ...},

sINR						ENUMERATED {true, ...},

iE-Extensions		ProtocolExtensionContainer { { M1MeasReportQuantity-ExtIEs} } OPTIONAL,

...

}

M1MeasReportQuantity-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M1ReportingTrigger ::= ENUMERATED{

periodic,

a2eventtriggered,

a2eventtriggered-periodic,

...

}

-- TEXT OMITTED –

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-AMFConfigurationUpdate					ProcedureCode ::= 0

id-AMFStatusIndication						ProcedureCode ::= 1

id-CellTrafficTrace							ProcedureCode ::= 2

id-DeactivateTrace							ProcedureCode ::= 3

id-DownlinkNASTransport						ProcedureCode ::= 4

id-DownlinkNonUEAssociatedNRPPaTransport	ProcedureCode ::= 5

id-DownlinkRANConfigurationTransfer			ProcedureCode ::= 6

id-DownlinkRANStatusTransfer				ProcedureCode ::= 7

id-DownlinkUEAssociatedNRPPaTransport		ProcedureCode ::= 8

id-ErrorIndication							ProcedureCode ::= 9

id-HandoverCancel							ProcedureCode ::= 10

id-HandoverNotification						ProcedureCode ::= 11

id-HandoverPreparation						ProcedureCode ::= 12

id-HandoverResourceAllocation				ProcedureCode ::= 13

id-InitialContextSetup						ProcedureCode ::= 14

id-InitialUEMessage							ProcedureCode ::= 15

id-LocationReportingControl					ProcedureCode ::= 16

id-LocationReportingFailureIndication		ProcedureCode ::= 17

id-LocationReport							ProcedureCode ::= 18

id-NASNonDeliveryIndication					ProcedureCode ::= 19

id-NGReset									ProcedureCode ::= 20

id-NGSetup									ProcedureCode ::= 21

id-OverloadStart							ProcedureCode ::= 22

id-OverloadStop								ProcedureCode ::= 23

id-Paging									ProcedureCode ::= 24

id-PathSwitchRequest						ProcedureCode ::= 25

id-PDUSessionResourceModify					ProcedureCode ::= 26

id-PDUSessionResourceModifyIndication		ProcedureCode ::= 27

id-PDUSessionResourceRelease				ProcedureCode ::= 28

id-PDUSessionResourceSetup					ProcedureCode ::= 29

id-PDUSessionResourceNotify					ProcedureCode ::= 30

id-PrivateMessage							ProcedureCode ::= 31

id-PWSCancel								ProcedureCode ::= 32

id-PWSFailureIndication						ProcedureCode ::= 33

id-PWSRestartIndication						ProcedureCode ::= 34

id-RANConfigurationUpdate					ProcedureCode ::= 35

id-RerouteNASRequest						ProcedureCode ::= 36

id-RRCInactiveTransitionReport				ProcedureCode ::= 37

id-TraceFailureIndication					ProcedureCode ::= 38

id-TraceStart								ProcedureCode ::= 39

id-UEContextModification					ProcedureCode ::= 40

id-UEContextRelease							ProcedureCode ::= 41

id-UEContextReleaseRequest					ProcedureCode ::= 42

id-UERadioCapabilityCheck					ProcedureCode ::= 43

id-UERadioCapabilityInfoIndication			ProcedureCode ::= 44

id-UETNLABindingRelease						ProcedureCode ::= 45

id-UplinkNASTransport						ProcedureCode ::= 46

id-UplinkNonUEAssociatedNRPPaTransport		ProcedureCode ::= 47

id-UplinkRANConfigurationTransfer			ProcedureCode ::= 48

id-UplinkRANStatusTransfer					ProcedureCode ::= 49

id-UplinkUEAssociatedNRPPaTransport			ProcedureCode ::= 50

id-WriteReplaceWarning						ProcedureCode ::= 51

id-SecondaryRATDataUsageReport				ProcedureCode ::= 52

id-UplinkRIMInformationTransfer				ProcedureCode ::= 53

id-DownlinkRIMInformationTransfer			ProcedureCode ::= 54

id-RetrieveUEInformation					ProcedureCode ::= 55

id-UEInformationTransfer					ProcedureCode ::= 56

id-RANCPRelocationIndication				ProcedureCode ::= 57

id-UEContextResume							ProcedureCode ::= 58

id-UEContextSuspend							ProcedureCode ::= 59

id-UERadioCapabilityIDMapping				ProcedureCode ::= 60

id-HandoverSuccess							ProcedureCode ::= 61

id-UplinkRANEarlyStatusTransfer				ProcedureCode ::= 62

id-DownlinkRANEarlyStatusTransfer			ProcedureCode ::= 63

id-AMFCPRelocationIndication				ProcedureCode ::= 64

id-ConnectionEstablishmentIndication		ProcedureCode ::= 65

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas					INTEGER ::= 16

maxnoofAllowedCAGsperPLMN			INTEGER ::= 256

maxnoofAllowedS-NSSAIs				INTEGER ::= 8

maxnoofBluetoothName				INTEGER ::= 4

maxnoofBPLMNs						INTEGER ::= 12

maxnoofCAGSperCell					INTEGER ::= 64

maxnoofCellIDforMDT					INTEGER ::= 32

maxnoofCellIDforWarning				INTEGER ::= 65535

maxnoofCellinAoI					INTEGER ::= 256

maxnoofCellinEAI					INTEGER ::= 65535

maxnoofCellinTAI					INTEGER ::= 65535

maxnoofCellsingNB					INTEGER ::= 16384

maxnoofCellsinngeNB					INTEGER ::= 256

maxnoofCellsinUEHistoryInfo			INTEGER ::= 16

maxnoofCellsUEMovingTrajectory		INTEGER ::= 16

maxnoofDRBs							INTEGER ::= 32

maxnoofEmergencyAreaID				INTEGER ::= 65535

maxnoofEAIforRestart				INTEGER ::= 256

maxnoofEPLMNs						INTEGER ::= 15

maxnoofEPLMNsPlusOne				INTEGER ::= 16

maxnoofE-RABs						INTEGER ::= 256

maxnoofErrors						INTEGER ::= 256

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofForbTACs						INTEGER ::= 4096

maxnoofFreqforMDT					INTEGER ::= 8

maxnoofMDTPLMNs						INTEGER ::= 16

maxnoofMultiConnectivity			INTEGER ::= 4

maxnoofMultiConnectivityMinusOne	INTEGER ::= 3

maxnoofNeighPCIforMDT				INTEGER ::= 32

maxnoofNGConnectionsToReset			INTEGER ::= 65536

maxnoofNRCellBands					INTEGER ::= 32

maxnoofPC5QoSFlows 					INTEGER ::= 2048

maxnoofPDUSessions					INTEGER ::= 256

maxnoofPLMNs						INTEGER ::= 12

maxnoofQosFlows						INTEGER ::= 64

maxnoofQosParaSets					INTEGER ::= 8

maxnoofRANNodeinAoI					INTEGER ::= 64

maxnoofRecommendedCells				INTEGER ::= 16

maxnoofRecommendedRANNodes			INTEGER ::= 16

maxnoofAoI							INTEGER ::= 64

maxnoofSensorName					INTEGER ::= 3

maxnoofServedGUAMIs					INTEGER ::= 256

maxnoofSliceItems					INTEGER ::= 1024

maxnoofTACs							INTEGER ::= 256

maxnoofTAforMDT						INTEGER ::= 8

maxnoofTAIforInactive				INTEGER ::= 16

maxnoofTAIforPaging					INTEGER ::= 16

maxnoofTAIforRestart				INTEGER ::= 2048

maxnoofTAIforWarning				INTEGER ::= 65535

maxnoofTAIinAoI						INTEGER ::= 16

maxnoofTimePeriods					INTEGER ::= 2

maxnoofTNLAssociations				INTEGER ::= 32

maxnoofWLANName						INTEGER ::= 4

maxnoofXnExtTLAs					INTEGER ::= 16

maxnoofXnGTP-TLAs					INTEGER ::= 16

maxnoofXnTLAs						INTEGER ::= 2

maxnoofCandidateCells				INTEGER ::= 32

maxNRARFCN							INTEGER ::= 3279165

maxNrofIndexesToReport				INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

id-AMFName												ProtocolIE-ID ::= 1

id-AMFOverloadResponse									ProtocolIE-ID ::= 2

id-AMFSetID												ProtocolIE-ID ::= 3

id-AMF-TNLAssociationFailedToSetupList					ProtocolIE-ID ::= 4

id-AMF-TNLAssociationSetupList							ProtocolIE-ID ::= 5

id-AMF-TNLAssociationToAddList							ProtocolIE-ID ::= 6

id-AMF-TNLAssociationToRemoveList						ProtocolIE-ID ::= 7

id-AMF-TNLAssociationToUpdateList						ProtocolIE-ID ::= 8

id-AMFTrafficLoadReductionIndication					ProtocolIE-ID ::= 9

id-AMF-UE-NGAP-ID										ProtocolIE-ID ::= 10

id-AssistanceDataForPaging								ProtocolIE-ID ::= 11

id-BroadcastCancelledAreaList							ProtocolIE-ID ::= 12

id-BroadcastCompletedAreaList							ProtocolIE-ID ::= 13

id-CancelAllWarningMessages								ProtocolIE-ID ::= 14

id-Cause												ProtocolIE-ID ::= 15

id-CellIDListForRestart									ProtocolIE-ID ::= 16

id-ConcurrentWarningMessageInd							ProtocolIE-ID ::= 17

id-CoreNetworkAssistanceInformationForInactive			ProtocolIE-ID ::= 18

id-CriticalityDiagnostics								ProtocolIE-ID ::= 19

id-DataCodingScheme										ProtocolIE-ID ::= 20

id-DefaultPagingDRX										ProtocolIE-ID ::= 21

id-DirectForwardingPathAvailability						ProtocolIE-ID ::= 22

id-EmergencyAreaIDListForRestart						ProtocolIE-ID ::= 23

id-EmergencyFallbackIndicator							ProtocolIE-ID ::= 24

id-EUTRA-CGI											ProtocolIE-ID ::= 25

id-FiveG-S-TMSI											ProtocolIE-ID ::= 26

id-GlobalRANNodeID										ProtocolIE-ID ::= 27

id-GUAMI												ProtocolIE-ID ::= 28

id-HandoverType											ProtocolIE-ID ::= 29

id-IMSVoiceSupportIndicator								ProtocolIE-ID ::= 30

id-IndexToRFSP											ProtocolIE-ID ::= 31

id-InfoOnRecommendedCellsAndRANNodesForPaging			ProtocolIE-ID ::= 32

id-LocationReportingRequestType							ProtocolIE-ID ::= 33

id-MaskedIMEISV											ProtocolIE-ID ::= 34

id-MessageIdentifier									ProtocolIE-ID ::= 35

id-MobilityRestrictionList								ProtocolIE-ID ::= 36

id-NASC													ProtocolIE-ID ::= 37

id-NAS-PDU												ProtocolIE-ID ::= 38

id-NASSecurityParametersFromNGRAN						ProtocolIE-ID ::= 39

id-NewAMF-UE-NGAP-ID									ProtocolIE-ID ::= 40

id-NewSecurityContextInd								ProtocolIE-ID ::= 41

id-NGAP-Message											ProtocolIE-ID ::= 42

id-NGRAN-CGI											ProtocolIE-ID ::= 43

id-NGRANTraceID											ProtocolIE-ID ::= 44

id-NR-CGI												ProtocolIE-ID ::= 45

id-NRPPa-PDU											ProtocolIE-ID ::= 46

id-NumberOfBroadcastsRequested							ProtocolIE-ID ::= 47

id-OldAMF												ProtocolIE-ID ::= 48

id-OverloadStartNSSAIList								ProtocolIE-ID ::= 49

id-PagingDRX											ProtocolIE-ID ::= 50

id-PagingOrigin											ProtocolIE-ID ::= 51

id-PagingPriority										ProtocolIE-ID ::= 52

id-PDUSessionResourceAdmittedList						ProtocolIE-ID ::= 53

id-PDUSessionResourceFailedToModifyListModRes			ProtocolIE-ID ::= 54

id-PDUSessionResourceFailedToSetupListCxtRes			ProtocolIE-ID ::= 55

id-PDUSessionResourceFailedToSetupListHOAck				ProtocolIE-ID ::= 56

id-PDUSessionResourceFailedToSetupListPSReq				ProtocolIE-ID ::= 57

id-PDUSessionResourceFailedToSetupListSURes				ProtocolIE-ID ::= 58

id-PDUSessionResourceHandoverList						ProtocolIE-ID ::= 59

id-PDUSessionResourceListCxtRelCpl						ProtocolIE-ID ::= 60

id-PDUSessionResourceListHORqd							ProtocolIE-ID ::= 61

id-PDUSessionResourceModifyListModCfm					ProtocolIE-ID ::= 62

id-PDUSessionResourceModifyListModInd					ProtocolIE-ID ::= 63

id-PDUSessionResourceModifyListModReq					ProtocolIE-ID ::= 64

id-PDUSessionResourceModifyListModRes					ProtocolIE-ID ::= 65

id-PDUSessionResourceNotifyList							ProtocolIE-ID ::= 66

id-PDUSessionResourceReleasedListNot					ProtocolIE-ID ::= 67

id-PDUSessionResourceReleasedListPSAck					ProtocolIE-ID ::= 68

id-PDUSessionResourceReleasedListPSFail					ProtocolIE-ID ::= 69

id-PDUSessionResourceReleasedListRelRes					ProtocolIE-ID ::= 70

id-PDUSessionResourceSetupListCxtReq					ProtocolIE-ID ::= 71

id-PDUSessionResourceSetupListCxtRes					ProtocolIE-ID ::= 72

id-PDUSessionResourceSetupListHOReq						ProtocolIE-ID ::= 73

id-PDUSessionResourceSetupListSUReq						ProtocolIE-ID ::= 74

id-PDUSessionResourceSetupListSURes						ProtocolIE-ID ::= 75

id-PDUSessionResourceToBeSwitchedDLList					ProtocolIE-ID ::= 76

id-PDUSessionResourceSwitchedList						ProtocolIE-ID ::= 77

id-PDUSessionResourceToReleaseListHOCmd					ProtocolIE-ID ::= 78

id-PDUSessionResourceToReleaseListRelCmd				ProtocolIE-ID ::= 79

id-PLMNSupportList										ProtocolIE-ID ::= 80

id-PWSFailedCellIDList									ProtocolIE-ID ::= 81

id-RANNodeName											ProtocolIE-ID ::= 82

id-RANPagingPriority									ProtocolIE-ID ::= 83

id-RANStatusTransfer-TransparentContainer				ProtocolIE-ID ::= 84

id-RAN-UE-NGAP-ID										ProtocolIE-ID ::= 85

id-RelativeAMFCapacity									ProtocolIE-ID ::= 86

id-RepetitionPeriod										ProtocolIE-ID ::= 87

id-ResetType											ProtocolIE-ID ::= 88

id-RoutingID											ProtocolIE-ID ::= 89

id-RRCEstablishmentCause								ProtocolIE-ID ::= 90

id-RRCInactiveTransitionReportRequest					ProtocolIE-ID ::= 91

id-RRCState												ProtocolIE-ID ::= 92

id-SecurityContext										ProtocolIE-ID ::= 93

id-SecurityKey											ProtocolIE-ID ::= 94

id-SerialNumber											ProtocolIE-ID ::= 95

id-ServedGUAMIList										ProtocolIE-ID ::= 96

id-SliceSupportList										ProtocolIE-ID ::= 97

id-SONConfigurationTransferDL							ProtocolIE-ID ::= 98

id-SONConfigurationTransferUL							ProtocolIE-ID ::= 99

id-SourceAMF-UE-NGAP-ID									ProtocolIE-ID ::= 100

id-SourceToTarget-TransparentContainer					ProtocolIE-ID ::= 101

id-SupportedTAList										ProtocolIE-ID ::= 102

id-TAIListForPaging										ProtocolIE-ID ::= 103

id-TAIListForRestart									ProtocolIE-ID ::= 104

id-TargetID												ProtocolIE-ID ::= 105

id-TargetToSource-TransparentContainer					ProtocolIE-ID ::= 106

id-TimeToWait											ProtocolIE-ID ::= 107

id-TraceActivation										ProtocolIE-ID ::= 108

id-TraceCollectionEntityIPAddress						ProtocolIE-ID ::= 109

id-UEAggregateMaximumBitRate							ProtocolIE-ID ::= 110

id-UE-associatedLogicalNG-connectionList				ProtocolIE-ID ::= 111

id-UEContextRequest										ProtocolIE-ID ::= 112

id-UE-NGAP-IDs											ProtocolIE-ID ::= 114

id-UEPagingIdentity										ProtocolIE-ID ::= 115

id-UEPresenceInAreaOfInterestList						ProtocolIE-ID ::= 116

id-UERadioCapability									ProtocolIE-ID ::= 117

id-UERadioCapabilityForPaging							ProtocolIE-ID ::= 118

id-UESecurityCapabilities								ProtocolIE-ID ::= 119

id-UnavailableGUAMIList									ProtocolIE-ID ::= 120

id-UserLocationInformation								ProtocolIE-ID ::= 121

id-WarningAreaList										ProtocolIE-ID ::= 122

id-WarningMessageContents								ProtocolIE-ID ::= 123

id-WarningSecurityInfo									ProtocolIE-ID ::= 124

id-WarningType											ProtocolIE-ID ::= 125

id-AdditionalUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 126

id-DataForwardingNotPossible							ProtocolIE-ID ::= 127

id-DL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 128

id-NetworkInstance										ProtocolIE-ID ::= 129

id-PDUSessionAggregateMaximumBitRate					ProtocolIE-ID ::= 130

id-PDUSessionResourceFailedToModifyListModCfm			ProtocolIE-ID ::= 131

id-PDUSessionResourceFailedToSetupListCxtFail			ProtocolIE-ID ::= 132

id-PDUSessionResourceListCxtRelReq						ProtocolIE-ID ::= 133

id-PDUSessionType										ProtocolIE-ID ::= 134

id-QosFlowAddOrModifyRequestList						ProtocolIE-ID ::= 135

id-QosFlowSetupRequestList								ProtocolIE-ID ::= 136

id-QosFlowToReleaseList									ProtocolIE-ID ::= 137

id-SecurityIndication									ProtocolIE-ID ::= 138

id-UL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 139

id-UL-NGU-UP-TNLModifyList								ProtocolIE-ID ::= 140

id-WarningAreaCoordinates								ProtocolIE-ID ::= 141

id-PDUSessionResourceSecondaryRATUsageList				ProtocolIE-ID ::= 142

id-HandoverFlag											ProtocolIE-ID ::= 143

id-SecondaryRATUsageInformation							ProtocolIE-ID ::= 144

id-PDUSessionResourceReleaseResponseTransfer			ProtocolIE-ID ::= 145

id-RedirectionVoiceFallback								ProtocolIE-ID ::= 146

id-UERetentionInformation								ProtocolIE-ID ::= 147

id-S-NSSAI												ProtocolIE-ID ::= 148

id-PSCellInformation									ProtocolIE-ID ::= 149

id-LastEUTRAN-PLMNIdentity								ProtocolIE-ID ::= 150

id-MaximumIntegrityProtectedDataRate-DL					ProtocolIE-ID ::= 151

id-AdditionalDLForwardingUPTNLInformation				ProtocolIE-ID ::= 152

id-AdditionalDLUPTNLInformationForHOList				ProtocolIE-ID ::= 153

id-AdditionalNGU-UP-TNLInformation						ProtocolIE-ID ::= 154

id-AdditionalDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 155

id-SecurityResult										ProtocolIE-ID ::= 156

id-ENDC-SONConfigurationTransferDL						ProtocolIE-ID ::= 157

id-ENDC-SONConfigurationTransferUL						ProtocolIE-ID ::= 158

id-OldAssociatedQosFlowList-ULendmarkerexpected			ProtocolIE-ID ::= 159

id-CNTypeRestrictionsForEquivalent						ProtocolIE-ID ::= 160

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-AdditionalULForwardingUPTNLInformation				ProtocolIE-ID ::= 172

id-SCTP-TLAs											ProtocolIE-ID ::= 173

id-SelectedPLMNIdentity									ProtocolIE-ID ::= 174

id-RIMInformationTransfer								ProtocolIE-ID ::= 175

id-GUAMIType											ProtocolIE-ID ::= 176

id-SRVCCOperationPossible								ProtocolIE-ID ::= 177

id-TargetRNC-ID											ProtocolIE-ID ::= 178

id-RAT-Information										ProtocolIE-ID ::= 179

id-ExtendedRATRestrictionInformation					ProtocolIE-ID ::= 180

id-QosMonitoringRequest									ProtocolIE-ID ::= 181

id-SgNB-UE-X2AP-ID										ProtocolIE-ID ::= 182

id-AdditionalRedundantDL-NGU-UP-TNLInformation			ProtocolIE-ID ::= 183

id-AdditionalRedundantDLQosFlowPerTNLInformation		ProtocolIE-ID ::= 184

id-AdditionalRedundantNGU-UP-TNLInformation				ProtocolIE-ID ::= 185

id-AdditionalRedundantUL-NGU-UP-TNLInformation			ProtocolIE-ID ::= 186

id-CNPacketDelayBudgetDL								ProtocolIE-ID ::= 187

id-CNPacketDelayBudgetUL								ProtocolIE-ID ::= 188

id-ExtendedPacketDelayBudget							ProtocolIE-ID ::= 189

id-RedundantCommonNetworkInstance						ProtocolIE-ID ::= 190

id-RedundantDL-NGU-TNLInformationReused					ProtocolIE-ID ::= 191

id-RedundantDL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 192

id-RedundantDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 193

id-RedundantQosFlowIndicator							ProtocolIE-ID ::= 194

id-RedundantUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 195

id-TSCTrafficCharacteristics							ProtocolIE-ID ::= 196

id-RedundantPDUSessionInformation 						ProtocolIE-ID ::= 197

id-UsedRSNInformation									ProtocolIE-ID ::= 198

id-IAB-Authorized										ProtocolIE-ID ::= 199

id-IAB-Supported										ProtocolIE-ID ::= 200

id-IABNodeIndication									ProtocolIE-ID ::= 201

id-NB-IoT-PagingDRX										ProtocolIE-ID ::= 202

id-NB-IoT-Paging-eDRXInfo								ProtocolIE-ID ::= 203

id-NB-IoT-DefaultPagingDRX								ProtocolIE-ID ::= 204

id-Enhanced-CoverageRestriction							ProtocolIE-ID ::= 205

id-Extended-ConnectedTime								ProtocolIE-ID ::= 206

id-PagingAssisDataforCEcapabUE							ProtocolIE-ID ::= 207

id-WUS-Assistance-Information							ProtocolIE-ID ::= 208

id-UE-DifferentiationInfo								ProtocolIE-ID ::= 209

id-NB-IoT-UEPriority									ProtocolIE-ID ::= 210

id-UL-CP-SecurityInformation							ProtocolIE-ID ::= 211

id-DL-CP-SecurityInformation							ProtocolIE-ID ::= 212

id-TAI													ProtocolIE-ID ::= 213

id-UERadioCapabilityForPagingOfNB-IoT					ProtocolIE-ID ::= 214

id-LTEV2XServicesAuthorized								ProtocolIE-ID ::= 215

id-NRV2XServicesAuthorized								ProtocolIE-ID ::= 216

id-LTEUESidelinkAggregateMaximumBitrate					ProtocolIE-ID ::= 217

id-NRUESidelinkAggregateMaximumBitrate					ProtocolIE-ID ::= 218

id-PC5QoSParameters										ProtocolIE-ID ::= 219

id-AlternativeQoSParaSetList							ProtocolIE-ID ::= 220

id-CurrentQoSParaSetIndex								ProtocolIE-ID ::= 221

id-CEmodeBrestricted									ProtocolIE-ID ::= 222

id-PagingeDRXInformation								ProtocolIE-ID ::= 223

id-CEmodeBSupport-Indicator								ProtocolIE-ID ::= 224

id-LTEM-Indication										ProtocolIE-ID ::= 225

id-EndIndication										ProtocolIE-ID ::= 226

id-EDT-Session											ProtocolIE-ID ::= 227

id-UECapabilityInfoRequest								ProtocolIE-ID ::= 228

id-PDUSessionResourceFailedToResumeListRESReq			ProtocolIE-ID ::= 229

id-PDUSessionResourceFailedToResumeListRESRes			ProtocolIE-ID ::= 230

id-PDUSessionResourceSuspendListSUSReq					ProtocolIE-ID ::= 231

id-PDUSessionResourceResumeListRESReq					ProtocolIE-ID ::= 232

id-PDUSessionResourceResumeListRESRes					ProtocolIE-ID ::= 233

id-UE-UP-CIoT-Support									ProtocolIE-ID ::= 234

id-Suspend-Request-Indication							ProtocolIE-ID ::= 235

id-Suspend-Response-Indication							ProtocolIE-ID ::= 236

id-RRC-Resume-Cause										ProtocolIE-ID ::= 237

id-RGLevelWirelineAccessCharacteristics					ProtocolIE-ID ::= 238

id-W-AGFIdentityInformation								ProtocolIE-ID ::= 239

id-GlobalTNGF-ID										ProtocolIE-ID ::= 240

id-GlobalTWIF-ID										ProtocolIE-ID ::= 241

id-GlobalW-AGF-ID										ProtocolIE-ID ::= 242

id-UserLocationInformationW-AGF							ProtocolIE-ID ::= 243

id-UserLocationInformationTNGF							ProtocolIE-ID ::= 244

id-AuthenticatedIndication								ProtocolIE-ID ::= 245

id-TNGFIdentityInformation								ProtocolIE-ID ::= 246

id-TWIFIdentityInformation								ProtocolIE-ID ::= 247

id-UserLocationInformationTWIF							ProtocolIE-ID ::= 248

id-DataForwardingResponseERABList						ProtocolIE-ID ::= 249

id-IntersystemSONConfigurationTransferDL				ProtocolIE-ID ::= 250

id-IntersystemSONConfigurationTransferUL				ProtocolIE-ID ::= 251

id-SONInformationReport									ProtocolIE-ID ::= 252

id-UEHistoryInformationFromTheUE						ProtocolIE-ID ::= 253

id-ManagementBasedMDTPLMNList							ProtocolIE-ID ::= 254

id-MDTConfiguration										ProtocolIE-ID ::= 255

id-PrivacyIndicator										ProtocolIE-ID ::= 256

id-TraceCollectionEntityURI								ProtocolIE-ID ::= 257

id-NPN-Support											ProtocolIE-ID ::= 258

id-NPN-AccessInformation								ProtocolIE-ID ::= 259

id-NPN-PagingAssistanceInformation						ProtocolIE-ID ::= 260

id-NPN-MobilityInformation								ProtocolIE-ID ::= 261

id-TargettoSource-Failure-TransparentContainer			ProtocolIE-ID ::= 262

id-NID													ProtocolIE-ID ::= 263

id-UERadioCapabilityID									ProtocolIE-ID ::= 264

id-UERadioCapability-EUTRA-Format						ProtocolIE-ID ::= 265

id-DAPSRequestInfo									ProtocolIE-ID ::= 266

id-DAPSResponseInfoList								ProtocolIE-ID ::= 267

id-EarlyStatusTransfer-TransparentContainer 			ProtocolIE-ID ::= 268

id-NotifySourceNGRANNode									ProtocolIE-ID ::= 269

id-ExtendedSliceSupportList								ProtocolIE-ID ::= 270

id-ExtendedTAISliceSupportList							ProtocolIE-ID ::= 271

id-ConfiguredTACIndication								ProtocolIE-ID ::= 272

id-Extended-RANNodeName									ProtocolIE-ID ::= 273

id-Extended-AMFName										ProtocolIE-ID ::= 274

id-GlobalCable-ID										ProtocolIE-ID ::= 275

id-QosMonitoringReportingFrequency						ProtocolIE-ID ::= 276

id-QosFlowParametersList									ProtocolIE-ID ::= 277

id-QosFlowFeedbackList									ProtocolIE-ID ::= 278

id-BurstArrivalTimeDownlink								ProtocolIE-ID ::= 279

id-ExtendedUEIdentityIndexValue							ProtocolIE-ID ::= 280

id-PduSessionExpectedUEActivityBehaviour					ProtocolIE-ID ::= 281

id-MicoAllPLMN											ProtocolIE-ID ::= 282

id-QosFlowFailedToSetupList								ProtocolIE-ID ::= 283

id-m1includeBeamMeasurements							ProtocolIE-ID ::= xxx

id-M1BeamMeasurementsReportConfiguration				ProtocolIE-ID ::= xxy

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# TP for TS38.423

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< 1st set of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.128	M1 Configuration

This IE defines the parameters for M1 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, A2event-triggered periodic, …) |  |
| M1 Threshold Event A2 | C-ifM1A2trigger |  |  | Included in case of event-triggered or event-triggered periodic reporting for measurement M1. |
| >CHOICE Threshold | M |  |  |  |
| >>RSRP |  |  |  |  |
| >>>Threshold RSRP | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| >>RSRQ |  |  |  |  |
| >>>Threshold RSRQ | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| >>SINR |  |  |  |  |
| >>>Threshold SINR | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| M1 Periodic reporting | C-ifperiodicMDT |  |  | Included in case of periodic or event-triggered periodic reporting for measurement M1. |
| >Report interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60) | This IE is defined in TS 38.331 [18]. |
| >Report amount | M |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity) | Number of reports. |
| Include Beam Measurements Indication | M |  | ENUMERATED (true, …) | To configure whether the UE should include the beam level measurements. This IE is defined in TS 38.331 [18]. |
| Beam MeasurementsReportConfiguration | C-ifM1BeamMeasInd |  |  | Included in case of include the beam level measurements indication for measurement M1. |
| >M1 reportQuantityRS-Indexes | O |  |  | This IE indicates which measurement information per RS index the UE shall include in the measurement report. It is defined in TS 38.331 [18]. |
| >>RSRP | M |  | ENUMARATED (true, …) |  |
| >>RSRQ | M |  | ENUMARATED (true, …) |  |
| >>SINR | M |  | ENUMARATED (true, …) |  |
| >M1 maxNrofRS-IndexesToReport | O |  | INTEGER (1..maxNrofIndexesToReport) | Indicates how many beam related measurements are to be included. This IE is defined in TS 38.331 [18]. |

| Condition | Explanation |
| --- | --- |
| C-ifM1A2trigger | This IE shall be present if the Measurements to Activate IE has the first bit set to "1" and the M1 Reporting Trigger IE is set to "A2event-triggered" or to "A2event-triggered periodic". |
| C-ifperiodicMDT | This IE shall be present if the M1 Reporting Trigger IE is set to "periodic", or to "A2event-triggered periodic". |
| C- ifM1BeamMeasInd | This IE shall be present if the Include Beam Measurements Indication IE is set to “true”. |

| Range bound | Explanation |
| --- | --- |
| maxNrofIndexesToReport | Maximum no. Max number of beam related measurements to To Report. Value is 32. |

<<<<<<<<<<<<<<<<<<<< End of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-IntendedTDD-DL-ULConfiguration-NR,

id-MaxIPrate-DL,

id-SecurityResult,

id-OldQoSFlowMap-ULendmarkerexpected,

id-PDUSessionCommonNetworkInstance,

id-BPLMN-ID-Info-EUTRA,

id-BPLMN-ID-Info-NR,

id-DRBsNotAdmittedSetupModifyList,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-ULForwardingProposal,

id-DRB-IDs-takenintouse,

id-SplitSessionIndicator,

id-NonGBRResources-Offered,

id-MDT-Configuration,

id-TraceCollectionEntityURI,

id-NPN-Broadcast-Information,

id-NPNPagingAssistanceInformation,

id-NPNMobilityInformation,

id-NPN-Support,

id-LTEUESidelinkAggregateMaximumBitRate,

id-NRUESidelinkAggregateMaximumBitRate,

id-ExtendedRATRestrictionInformation,

id-QoSMonitoringRequest,

id-QoSMonitoringDisabled,

id-QosMonitoringReportingFrequency,

id-DAPSRequestInfo,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-NBIoT-UL-DL-AlignmentOffset,

id-TDDULDLConfigurationCommonNR,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRCellPRACHConfig,

id-Redundant-UL-NG-U-TNLatUPF,

id-Redundant-DL-NG-U-TNLatNG-RAN,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-Additional-Redundant-UL-NG-U-TNLatUPF-List,

id-RedundantCommonNetworkInstance,

id-TSCTrafficCharacteristics,

id-RedundantQoSFlowIndicator,

id-Additional-PDCP-Duplication-TNL-List,

id-RedundantPDUSessionInformation,

id-UsedRSNInformation,

id-RLCDuplicationInformation,

id-CSI-RSTransmissionIndication,

id-UERadioCapabilityID,

id-secondary-SN-UL-PDCP-UP-TNLInfo,

id-pdcpDuplicationConfiguration,

id-duplicationActivation,

id-NPRACHConfiguration,

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-QoS-Mapping-Information,

id-AdditionLocationInformation,

id-dataForwardingInfoFromTargetE-UTRANnode,

id-Cause,

id-m1includeBeamMeasurements,

id-M1BeamMeasurementsReportConfiguration,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEPLMNsplus1,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

maxnoofForbiddenTACs,

maxnoofMBSFNEUTRA,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighbours,

maxnoofNIDs,

maxnoofNRCellBands,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofProtectedResourcePatterns,

maxnoofQoSFlows,

maxnoofQoSParaSets,

maxnoofRANAreaCodes,

maxnoofRANAreasinRNA,

maxnoofSCellGroups,

maxnoofSCellGroupsplus1,

maxnoofSliceItems,

maxnoofExtSliceItems,

maxnoofSNPNIDs,

maxnoofsupportedTACs,

maxnoofsupportedPLMNs,

maxnoofTAI,

maxnoofTAIsinAoI,

maxnoofTNLAssociations,

maxnoofUEContexts,

maxNRARFCN,

maxNrOfErrors,

maxnoofRANNodesinAoI,

maxnooftimeperiods,

maxnoofslots,

maxnoofExtTLAs,

maxnoofGTPTLAs,

maxnoofCHOcells,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofRACHReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationstate,

maxnoofBluetoothName,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofTAforMDT,

maxnoofWLANName,

maxnoofSensorName,

maxnoofNeighPCIforMDT,

maxnoofFreqforMDT,

maxnoofNonAnchorCarrierFreqConfig,

maxnoofDataForwardingTunneltoE-UTRAN,

maxNrofIndexesToReport

FROM XnAP-Constants

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

#### -- M

M1BeamMeasurementsReportConfiguration	::= SEQUENCE {

reportQuantityRS-Indexes	M1measReportQuantity	OPTIONAL,

maxNrofRS-IndexesToReport	M1maxNrofRS-IndexesToReport	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { M1BeamMeasurementsReportConfiguration-ExtIEs} } OPTIONAL,

...

}

M1BeamMeasurementsReportConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M1Configuration ::= SEQUENCE {

m1reportingTrigger			M1ReportingTrigger,

m1thresholdeventA2			M1ThresholdEventA2				OPTIONAL,

-- Included in case of event-triggered, or event-triggered periodic reporting for measurement M1

m1periodicReporting			M1PeriodicReporting				OPTIONAL,

-- Included in case of periodic or event-triggered periodic reporting

iE-Extensions				ProtocolExtensionContainer { { M1Configuration-ExtIEs} } OPTIONAL,

...

}

M1Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-m1includeBeamMeasurements		CRITICALITY ignore	EXTENSION M1includeBeamMeasurements		PRESENCE mandatory		}|

{ ID id-M1BeamMeasurementsReportConfiguration		CRITICALITY ignore	EXTENSION M1BeamMeasurementsReportConfiguration		PRESENCE optional		},

-- Included in case of M1includeBeamMeasurements set to true

...

}

M1includeBeamMeasurements ::= ENUMERATED{

True,

...

}

M1maxNrofRS-IndexesToReport	::= INTEGER (1..maxNrofIndexesToReport)

M1measReportQuantity

M1MeasReportQuantity ::= SEQUENCE {

rSRP						ENUMERATED {true, ...},

rSRQ						ENUMERATED {true, ...},

sINR						ENUMERATED {true, ...},

iE-Extensions		ProtocolExtensionContainer { { M1MeasReportQuantity-ExtIEs} } OPTIONAL,

...

}

M1MeasReportQuantity-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M1PeriodicReporting ::= SEQUENCE {

reportInterval				ReportIntervalMDT,

reportAmount				ReportAmountMDT,

iE-Extensions				ProtocolExtensionContainer { { M1PeriodicReporting-ExtIEs} } OPTIONAL,

...

}

M1PeriodicReporting-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 3rd set of Changes >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs						INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI 							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRACHReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN    	INTEGER ::= 256

maxNrofIndexesToReport						INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold 																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs 																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 																				ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse																				ProtocolIE-ID ::= 133

id-SplitSessionIndicator																			ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent																	ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing																		ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= 137

id-ULForwardingProposal																				ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort 																		ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR																ProtocolIE-ID ::= 140

id-TNLConfigurationInfo																				ProtocolIE-ID ::= 141

id-PartialListIndicator-NR																			ProtocolIE-ID ::= 142

id-MessageOversizeNotification																		ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR																	ProtocolIE-ID ::= 144

id-NG-RANTraceID																					ProtocolIE-ID ::= 145

id-NonGBRResources-Offered																			ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN																ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																				ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= 158

id-CHOinformation-Ack																				ProtocolIE-ID ::= 159

id-targetCellsToCancel																				ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= 161

id-procedureStage																					ProtocolIE-ID ::= 162

id-DAPSRequestInfo																					ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List																			ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator																				ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN															ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN															ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset																		ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized																			ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized																			ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 172

id-PC5QoSParameters																					ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList																		ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex																			ProtocolIE-ID ::= 175

id-MobilityInformation 																				ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication															ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE																	ProtocolIE-ID ::= 178

id-HandoverReportType																				ProtocolIE-ID ::= 179

id-HandoverCause																					ProtocolIE-ID ::= 180

id-SourceCellCGI																					ProtocolIE-ID ::= 181

id-TargetCellCGI																					ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI																			ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN																				ProtocolIE-ID ::= 184

id-SourceCellCRNTI																					ProtocolIE-ID ::= 185

id-UERLFReportContainer																				ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID																		ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID																		ProtocolIE-ID ::= 188

id-RegistrationRequest																				ProtocolIE-ID ::= 189

id-ReportCharacteristics																				ProtocolIE-ID ::= 190

id-CellToReport																								ProtocolIE-ID ::= 191

id-ReportingPeriodicity																					ProtocolIE-ID ::= 192

id-CellMeasurementResult																				ProtocolIE-ID ::= 193

id-NG-RANnode1CellID																						ProtocolIE-ID ::= 194

id-NG-RANnode2CellID																				ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters																	ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters															ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange																ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR																		ProtocolIE-ID ::= 199

id-CarrierList																						ProtocolIE-ID ::= 200

id-ULCarrierList																					ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz																				ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst																				ProtocolIE-ID ::= 203

id-NRCellPRACHConfig																				ProtocolIE-ID ::= 204

id-RACHReportInformation																			ProtocolIE-ID ::= 205

id-IABNodeIndication																				ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF																		ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink																		ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink																		ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List														ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance																	ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics																		ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator																		ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN																	ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget																		ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List																ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation																	ProtocolIE-ID ::= 217

id-UsedRSNInformation																				ProtocolIE-ID ::= 218

id-RLCDuplicationInformation																		ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information																		ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation																	ProtocolIE-ID ::= 221

id-NPNMobilityInformation																			ProtocolIE-ID ::= 222

id-NPN-Support																						ProtocolIE-ID ::= 223

id-MDT-Configuration																				ProtocolIE-ID ::= 224

id-MDTPLMNList																						ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI																			ProtocolIE-ID ::= 226

id-UERadioCapabilityID																				ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication																		ProtocolIE-ID ::= 228

id-SNTriggered  																					ProtocolIE-ID ::= 229

id-DLCarrierList																					ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList																		ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA																			ProtocolIE-ID ::= 232

id-ConfiguredTACIndication																			ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo																	ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated													ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-PagingeDRXInformation																			ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-m1includeBeamMeasurements																		ProtocolIE-ID ::= xxx

id-M1BeamMeasurementsReportConfiguration															ProtocolIE-ID ::= xxy

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>