3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220692

E-meeting, 17-26 Jan 2022

Title: 	(TP to TS 38.413 BL CR) Shared NG-U tunnel establishment and release

Source: 	Huawei, CBN, China Unicom, China Telecom, Lenovo, Motorola Mobility, Qualcomm Incorporated

Agenda item:	22.2.2

Document Type:	Other

# 1. Introduction

- In the RAN3#114e meeting, there were more agreements about multicast session management achieved:

Multicast Session Management - About providing mapped QoS flow and associated QoS flow information from CN to RAN, update UE associated NGAP: PDU Session Resource Modify Request Transfer IE and PDU Session Resource Setup Request Transfer IE. - Introduce a non-UE associated NGAP Class1 Multicast Session Update procedure triggered by MB-SMF to support multicast session update in case the change of some of QoS parameters and/or service area. - Introduce non-UE associated NGAP Class1 Multicast Session Activation procedure and Multicast Session Deactivation procedure, FFS on whether a single procedure or separate procedures should be used for activation/update/deactivation. - WA: introduce new MB-SMF containers in TS 38.413, subject to SA2/CT4. Applicable for both MC and BC. - Introduce one or two non-UE associated Class1 NGAP procedure(s), triggered by the gNB to implement Multicast Distribution Setup function, and Multicast Distribution Release function. - For location dependent Multicast MBS service, the MBS Distribution Setup/Release procedure is used to setup/release the NG-U tunnel for an area Session.

- In this paper, we discuss the NG-U tunnel establishment and release, and provide the text proposal to BL CR of TS TS 38.413 in Section 4.

# 2. Discussion

- In the last meeting, whether to implement in one procedure or two procedures is still FFS. For this issue, there are two alternatives to support NG-U tunnel establishment and release, as follow:

- Alt 1: two separate procedures, i.e. Multicast Distribution Setup procedure and Multicast Distribution Release procedure

- Alt 2: a single procedure, i.e. Multicast Distribution Update procedure

- For both alternatives, the NG-U tunnel can be established or released via MB-SMF container well. While, the AMF should also be aware of the NG-U tunnel status, for it will store the NG-RAN node IDs in MBS context for the subsequent signaling related to the multicast MBS Session. If the NG-U tunnel is released, the AMF removes the information of the RAN node from the context of the multicast MBS session. Similar with PDU Session Resource, there is no special gain in merging procedures just to reduce the number of procedures. What’s more, during NG-U tunnel establishment, some other MBS informations can be provided to the NG-RAN, that is different from NG-U tunnel release. From the point view of RAN3, the procedure should be better for two separate procedures, directly.

- To establish the shared NG-U tunnel, the following procedures should be considered:

Figure 1

Based on the Figure above, to setup the shared NG-U transport, the NG-RAN node will trigger the Multicast Distribution Setup Request by providing the DL GTP-U Tunnel information if it is configured to use unicast transport for multicast distribution sessions, and if the DL GTP-U Tunnel information is not provided in the Multicast Distribution Setup Request, the SMF will provide the DL tunnel info in the Response message to enable IP multicast transport of the multicast distribution.

Proposal 1: Introduce two separate non-UE associated class1 NGAP procedures:

- Multicast Distribution Setup procedure, triggered by the gNB

  - in case the gNB decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info in the Multicast Distribution Setup Request.

  - in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the core network shall provide IP multicast DL tunnel info to the gNB, to enable IP multicast transport.

- Multicast Distribution Release procedure, triggered by the gNB or CN

Considering UP-CP split architecture, the DL tunnel info should be transferred between CU-UP and CU-CP, it is necessary to introduce a non UE associated Class1 E1AP procedure, e.g. named as Multicast Distribution Setup procedure.

Proposal 2: Introduce two separate non-UE associated Class1 E1AP procedures：

- Multicast Distribution Setup procedure, triggered by the gNB-CU-UP

  - in case the gNB-CU-UP decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info in the Multicast Distribution Setup Request.

  - in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the gNB-CU-CP shall provide IP multicast DL tunnel info to the gNB-CU-UP, to enable IP multicast transport.

- Multicast Distribution Release procedure, triggered by the gNB-CU-UP or gNB-CU-CP

- In TS 23.247, section 7.2.1.4 shows the establishment procedure of shared delivery toward RAN node.

6. The MB-SMF sends Nmbsmf_MBSSession_ContextUpdate response (MBS Session ID, [Area Session ID], N2 SM information ([TMGI], multicast QoS flow information, [multicast DL tunnel Info], [MBS service area])) to the AMF. If the MB-SMF did not receive unicast DL tunnel Info in step 3, it provides the multicast DL tunnel info that includes transport multicast address (e.g. a LL SSM) and a GTP tunnel endpoint for multicast transport of the shared delivery. 7. The AMF sends an N2 MBS Session response message (MBS Session ID, [Area Session ID], N2 SM information) to the NG-RAN node. If the NG-RAN node receives the multicast DL tunnel Info of the shared delivery, it uses the transport multicast address included in the multicast DL tunnel info to join the multicast transport distribution.

As the highlight word in the above description, the multicast QoS flow information should always be provided in NGAP: Multicast Distribution Setup Response message. Since the multicast QoS flow information, i.e. QoS Flow Level QoS Parameters, is per Session level. Thus, it should be better to provide it once to NG-RAN node, instead of provide the per session level parameters in PDU session modify response message for each UEs request for join the Session. Based on this, the PDU session modify response message only need to provide the QoS Flow Identifier for each UEs.

- Proposal 3: The MBS QoS Flow Level QoS Parameters should be provided in the Multicast Distribution Setup Response message.

According to the latest version of TS 23.247, for location dependent multicast service, the service area information can be provided to NG-RAN node as follow:

- The SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID and MBS service area associated with the cell where UE camps. - If the NG-RAN node supports MBS, the NG-RAN uses the received MBS Session ID and Area Session ID to determine the local multicast session context and whether the user plane for the local multicast session is already established. If the target RAN determines the shared delivery is not established for the MBS session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4. The MB-SMF provides MBS service area information (Area session ID(s), MBS service area(s)) associated with the same MBS session to NG-RAN in the shared delivery establishment response. Editor's note: It is FFS whether the MB-SMF provides only the service areas associated with the target RAN node or all service areas associated with the MBS session in the shared delivery establishment response message.

As the highlight word in the above description, the MBS service area associated with the cell where UE camps is provided to the NG-RAN node via per UE PDU Session management procedures, and the MBS service area information(s) associated with the same MBS session will be provided in non-UE associated NGAP: Multicast Distribution Setup Response message. The first one provided via UE associated signaling can cover the cells/TAs for the current MBS Area Session, and the latter one included in the non-UE associated signaling can cover full/bigger area for the current MBS Session which includes multiple MBS Area Session IDs.

- Proposal 4: The MBS service area information(s) associated with the same MBS Session should be provided in the Multicast Distribution Setup Response message.

- In eMBMS, the Network operators providing MBMS over LTE networks may need to broadcast services via networks that have a mix of IPv4 and IPv6 backhaul and MCE can send both IPv4 and IPv6 multicast addresses to RAN during session setup procedure. For NR MBS, the IP multicast address and the IP address of the multicast source can be also for both IPv4 and IPv6. Thus, it is necessary to introduce Alternative IP Multicast TNL Information IE rather than IP Multicast TNL Information IE to support IPv6 transport network.

- Proposal 5: Same as eMBMS, introduce alternative IP multicast TNL address to the MB-SMF container in Multicast Distribution Setup Response message to support mix IPv4 and IPv6.

# 3. Conclusion

Proposal 1: Introduce two separate non-UE associated class1 NGAP procedures:

- Multicast Distribution Setup procedure, triggered by the gNB

  - in case the gNB decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info in the Multicast Distribution Setup Request.

  - in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the core network shall provide IP multicast DL tunnel info to the gNB, to enable IP multicast transport.

- Multicast Distribution Release procedure, triggered by the gNB or CN

Proposal 2: Introduce two separate non-UE associated Class1 E1AP procedures：

- Multicast Distribution Setup procedure, triggered by the gNB-CU-UP

  - in case the gNB-CU-UP decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info in the Multicast Distribution Setup Request.

  - in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the gNB-CU-CP shall provide IP multicast DL tunnel info to the gNB-CU-UP, to enable IP multicast transport.

- Multicast Distribution Release procedure, triggered by the gNB-CU-UP or gNB-CU-CP

- Proposal 3: The MBS QoS Flow Level QoS Parameters should be provided in the Multicast Distribution Setup Response message.

- Proposal 4: The MBS service area information(s) associated with the same MBS Session should be provided in the Multicast Distribution Setup Response message.

- Proposal 5: Same as eMBMS, introduce alternative IP multicast TNL address to the MB-SMF container in Multicast Distribution Setup Response message to support mix IPv4 and IPv6.

# 4. Text Proposal to BL CR of TS 38.413

## *****************First change*******************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture description".

[3]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[6]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[7]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[8]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[9]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[11]	3GPP TS 32.422: "Trace control and configuration management".

[12]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode and in RRC inactive state".

[13]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[14]	3GPP TS 38.414: "NG-RAN; NG data transport".

[15]	3GPP TS 29.281: "General Packet Radio System (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[16]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[17]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[18]	3GPP TS 38.331: "NG-RAN; Radio Resource Control (RRC) Protocol Specification".

[19]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[20]	3GPP TS 23.007: "Technical Specification Group Core Network Terminals; Restoration procedures".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[25]	IETF RFC 5905 (2010-06): "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[26]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[27]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[28]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[29]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[30]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3".

[31]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[32]	3GPP TS 37.340: " Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[33]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[34]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[35]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[36]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[37]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[38]	3GPP TS 36.401: "E-UTRAN Architecture Description".

[39]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[40]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP) ".

[41]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA), Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[42]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[xx]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2”.

## *****************Next changes*******************

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |

## *****************Next changes*******************

## 8.xx	MBS Session Management Procedures

### 8.xx.c	Distribution Setup

#### 8.xx.c.1	General

The purpose of the Distribution Setup procedure is to assign NG-U resources for one MBS session and the corresponding MBS QoS flows. The procedure uses non-UE-associated signalling.

#### 8.xx.c.2	Successful Operation

Figure 8.xx.c.2-1: Distribution Setup: successful operation

The NG-RAN node initiates the procedure by sending a DISTRIBUTIONSETUP REQUEST message to the AMF. The AMF responds with a DISTRIBUTION SETUP RESPONSE message.

If the DL Transport Layer Information IE is included in the Distribution Setup Request Transfer IE in the DISTRIBUTION SETUP REQUEST message, the SMF shall consider that the included information as the downlink termination point for the included associated QoS flows for this MBS session.

If the DL Transport Layer Information IE is not included in the Distribution Setup Request Transfer IE in the DISTRIBUTION SETUP REQUEST message, the SMF shall interpret that the IP multicast is used for this MBS Session and provide IP Multicast Address Information IE or Alternative IP Multicast Address Information IE in the Distribution Setup Response Transfer IE in the DISTRIBUTION SETUP RESPONSE message.

#### 8.xx.c.3	Unsuccessful Operation

Figure 8.xx.c.3-1: Distribution Setup: unsuccessful operation.

In case the Multicast Distribution cannot be setup successfully, the AMF shall response with the DISTRIBUTION SETUP FAILURE message to the NG-RAN node with an appropriate cause value in the Cause IE.

#### 8.xx.c.4	Abnormal Conditions

Not applicable.

### 8.xx.d	Distribution Release

#### 8.xx.d.1	General

The purpose of the Distribution Release procedure is to enable the release of already established NG-U resources for a given MBS Session. The procedure uses non-UE-associated signalling.

#### 8.xx.d.2	Successful Operation

Figure 8.xx.d.2-1: Distribution Release initiated by the NG-RAN node

Figure 8.xx.d.2-2: Distribution Release initiated by the AMF

The NG-RAN node or the AMF initiates the procedure by sending a DISTRIBUTION RELEASE REQUEST message.

Upon receipt of the DISTRIBUTION RELEASE REQUEST message, the receiving node shall send the DISTRIBUTION RELEASE RESPONSE message after successfully remove the corresponding NG-U resource for the MSB Session.

#### 8.xx.d.3		Unsuccessful Operation

Not applicable.

#### 8.xx.d.4	Abnormal Conditions

Not applicable.

## *****************Next changes*******************

### 9.2.x	MBS Session Management Messages

#### 9.2.x.c1	DISTRIBUTION SETUP REQUEST

This message is sent by the NG-RAN node to request the setup of the NG-U Transport for a MBS Session.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.z |  | YES | reject |
| MBS Distribution Setup Request Transfer | M |  | OCTET STRING | Containing the MBS Distribution Setup Request Transfer IE specified in subclause 9.3.A.a. | YES | reject |

#### 9.2.x.c2	DISTRIBUTION SETUP RESPONSE

This message is sent by the AMF to confirm the setup of the NG-U Transport for a MBS Session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.z |  | YES | reject |
| MBS Distribution Setup Response Transfer | M |  | OCTET STRING | Containing the MBS Distribution Setup Response Transfer IE specified in subclause 9.3.A.b. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.c3	DISTRIBUTION SETUP FAILURE

This message is sent by the AMF to indicate that the setup of the NG-U Transport for a MBS Session was unsuccessful.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.z |  | YES | reject |
| MBS Distribution Setup Unsuccessful Transfer | M |  | OCTET STRING | Containing the MBS Distribution Setup Unsuccessful Transfer IE specified in subclause 9.3.A.c. |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.d1	DISTRIBUTION RELEASE REQUEST

This message is sent by the AMF and the NG-RAN node to request the release of the NG-U Transport for a MBS Session.

Direction: AMF  NG-RAN node and NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.z |  | YES | reject |
| MBS Distribution Release Request Transfer | M |  | OCTET STRING | Containing the MBS Distribution Release Request Transfer IE specified in subclause 9.3.A.d. | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.x.d2	DISTRIBUTION RELEASE RESPONSE

This message is sent by the NG-RAN node and the AMF to confirm the release of the NG-U Transport for a MBS Session.

Direction: NG-RAN node  AMF and AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.z |  | YES | reject |
| MBS Distribution Release Response Transfer | M |  | OCTET STRING | Containing the MBS Distribution Release Response Transfer IE specified in subclause 9.3.A.e. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

## *****************Next changes*******************

#### 9.3.1.51	QoS Flow Identifier

This IE identifies a QoS flow within a PDU Session, or a MBS QoS flow within a MBS Session. The definition and use of the QoS Flow Identifier is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0..63, …) |  |

## *****************Next changes*******************

#### 9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

#### 9.3.1.z	MBS Area Session ID

This IE identifies an Area Session. The definition and use of the Area Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Area Session ID | M |  | FFS |  |

## *****************Next changes*******************

### 9.3.A	MB-SMF Related IEs

#### 9.3.A.a	MBS Distribution Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.y |  |
| DL Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. |

#### 9.3.A.b	MBS Distribution Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.y |  | YES | reject |
| IP Multicast TNL Information | O |  |  |  | YES | reject |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  | - |  |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  | - |  |
| >GTP DL TEID | M |  | 9.3.2.5 |  | - |  |
| Alternative IP Multicast TNL Information | O |  |  |  | YES | ignore |
| >Alternative IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  | - |  |
| >Alternative IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  | - |  |
| >GTP DL TEID | M |  | 9.3.2.5 |  | - |  |
| MBS QoS Flows Information List |  | 1 |  |  | YES | reject |
| >MBS QoS Flows To Be Information Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  | - |  |
| MBS service area information List |  | 0 |  |  | YES | reject |
| > MBS service area information Item |  | 1 .. <maxnoofAreaSessions> |  |  | - |  |
| >>MBS Area Ssession ID | M |  | 9.3.1.z |  | - |  |
| >> MBS service area information | M |  | FFS |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAreaSessions | Maximum no. of MBS Area Sessions allowed within one MBS session. Value is FFS. |
| maxnoofMBSQoSFlows | Maximum no. of QoS Flows allowed within one MBS session. Value is 64. |

#### 9.3.A.c	MBS Distribution Setup Unsuccessful Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.y |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.A.d	MBS Distribution Release Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.y |  |
| Cause | M |  | 9.3.1.2 |  |

#### 9.3.A.e	MBS Distribution Release Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.y |  |

## *****************Next changes*******************

//asn.1 part to be added

## *****************End of the changes*******************