3GPP TSG-RAN3 Meeting #114-bis-e	R3-220921

Electronic meeting, 17 - 26 Jan 2022

Agenda item:	15.2.1.2

Source: 	Samsung

Title: 	Further discussion on per-slice QoE

Document for:	Discussion & Approval

Introduction

In last RAN3 meeting, RAN3 had discussed Configuration Details. Most issues had been solved, and there’re still some open issues below, we have no strong views on the NG messages issue and capabilities issue, but we would like to further discuss per-slice RVQoE.

Whether slice ID should be configured as an explicit IE to UE over Uu, at least for RAN visible QoE metric configuration

Whether to extend some NG messages, e.g. NGAP INITIAL UE MESSAGE, with a new QoE Measurement Capabilities IE including a UE Application Layer Measurement Capability and a Max Number of UE Application Layer Measurements

Details to be continued...

In this contribution, we’d like to further discuss whether slice ID can be explicit over Uu for RAN visible QoE.

Discussion

In last meeting, RAN3 had below agreement on per slice QoE:

There is no need to include slice ID as an explicit IE over Uu outside the QoE configuration and reporting container for legacy QoE. FFS whether and how to support per slice RVQoE configuration and reporting.

Based on the above text in the LS S4-211225, RAN3 agreed the following in RAN3#114e meeting:

- RAN3 assumes that slice ID is included inside the transparent QoE reporting container, which is up to SA4’s decision.

- For legacy QoE, there is no need to include slice ID as an explicit IE over Uu outside the QoE configuration and reporting container. FFS whether including the slice ID as an explicit IE over Uu outside the QoE configuration and reporting container is required to support per slice RAN visible QoE configuration and reporting.

And RAN3 had already sent the LS to SA4 to inform that the slice ID should be included in the container, so we assume that SA4 will support to include slice ID inside the reporting container for legacy QoE.

For RAN visible QoE, the slice ID should be transmitted as an explicit IE over Uu so that the NG-RAN node can be aware of the per-slice RVQoE, and reporting slice ID is enough and more straightforward, as NG-RAN node knows all the mappings, such as the mapping between the slice ID and PDU session ID, etc.

As UE Application is aware of slice ID, and SA4 will support to include slice ID in legacy QoE report container, it is also feasible for the UE Application to transmit the slice ID along with the RVQoE QoE metrics at the same time to UE AS, and the UE AS send the RVQoE report including RV QoE metrics and slice ID to the serving gNB. And the following is an example procedure for supporting per-slice RVQoE.

Proposal 1, for RAN visible QoE report, the slice ID should be included in the RVQoE report over Uu as an explicit IE.

Proposal 2, send LS to RAN2 and CT1 to ask the support of including slice ID in RAN visible QoE report as an explicit IE.

Conclusion

In this paper, we discussed per-slice QoE for RAN visible QoE, the following are proposals:

Proposal 1, for RAN visible QoE report, the slice ID should be included in the RVQoE report over Uu as an explicit IE.

Proposal 2, send LS to RAN2 and CT1 to ask the support of including slice ID in RAN visible QoE report as an explicit IE.

References