3GPP TSG-RAN WG3 #114bis-e													   				R3-220941

17-26 Jan 2022

Online

Agenda item:		21.2

Source:				CATT

Title:	TP for BLCR for 38.473 on Propagation Delay Compensation

Document for:		Endorsement

# 1	Introduction

As discussed in R3-220940, it is proposed to update BLCR for TS 38.423 to capture agreements on Propagation Delay Compensation

In the following text proposals, add Time Synchronisation Assistance Information in the message REFERENCE TIME INFORMATION REPORTING CONTROL

# 2 	TP for TS38.473

Change Begins

#### 9.2.11.1	REFERENCE TIME INFORMATION REPORTING CONTROL

This message is sent by the gNB-CU and is used to request the gNB-DU to deliver the accurate reference time information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Reporting Request Type | M |  | 9.3.1.147 |  | YES | reject |
| Time Synchronisation Assistance Information | O |  | 9.3.1.bbb |  | YES | ignore |

<Unchanged Text Omitted>

9.3.1.142	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [21]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.143 | Periodicity as specified in TS 23.501 [21]. | - |  |
| Burst Arrival Time | O |  | 9.3.1.144 | Burst Arrival Time as specified in TS 23.501 [21]. | - |  |
| Survival Time | O |  | 9.3.1.aaa | Survival Time as specified in TS 23.501 [21]. | YES | ignore |

<Unchanged Text Omitted>

9.3.1.aaa	Survival Time

This IE indicates the Survival Time of the TSC QoS flow as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0.. 1920000, …) | Expressed in units of 1 us. |

#### 9.3.1.bbb	Time Synchronisation Assistance Information

This IE indicates 5G access stratum time distribution parameters as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uu Time Synchronisation Error Budget | M |  | INTEGER (1..1000000, …) | Expressed in units of 1ns. |

<Unchanged Text Omitted>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<Unchanged Text Omitted>

PosMeasurementResultList,

PosReportCharacteristics,

TimeSyncAssistanceInfo,

TRPInformationTypeItem,

TRPInformationItem,

LMF-MeasurementID,

RAN-MeasurementID,

<Unchanged Text Omitted>

id-SRBs-SetupMod-List,

id-TimeSyncAssistanceInfo,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

<Unchanged Text Omitted>

-- **************************************************************

--

##### -- REFERENCE TIME INFORMATION REPORTING CONTROL

--

-- **************************************************************

ReferenceTimeInformationReportingControl::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ReferenceTimeInformationReportingControlIEs} },

...

}

ReferenceTimeInformationReportingControlIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-ReportingRequestType		CRITICALITY reject	TYPE ReportingRequestType		PRESENCE mandatory	}|

{ ID id-TimeSyncAssistanceInfo		CRITICALITY ignore	TYPE TimeSyncAssistanceInfo		PRESENCE optional		},

...

}

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-NRCGI,

id-SFN-Offset,

id-TransmissionStopIndicator,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-SurvivalTime,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

<Unchanged Text Omitted>

SupportedSULFreqBandItem ::= SEQUENCE {

freqBandIndicatorNr 			INTEGER (1..1024,...),

iE-Extensions				ProtocolExtensionContainer { { SupportedSULFreqBandItem-ExtIEs} } OPTIONAL,

...

}

SupportedSULFreqBandItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SurvivalTime ::= INTEGER (0.. 1920000,...)

<Unchanged Text Omitted>

TimeInformationType ::= ENUMERATED {localClock}

TimeStamp ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotIndex				TimeStampSlotIndex,

measurementTime			RelativeTime1900	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeStamp-ExtIEs} }	OPTIONAL

}

TimeStamp-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeStampSlotIndex ::= CHOICE {

sCS-15			INTEGER(0..9),

sCS-30			INTEGER(0..19),

sCS-60			INTEGER(0..39),

sCS-120			INTEGER(0..79),

choice-extension		ProtocolIE-SingleContainer { { TimeStampSlotIndex-ExtIEs} }

}

TimeStampSlotIndex-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TimeSyncAssistanceInfo ::= SEQUENCE {

uUTimeSyncErrorBudget			INTEGER (1..1000000, ...)							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TimeSyncAssistanceInfo-ExtIEs} }	OPTIONAL,

...

}

TimeSyncAssistanceInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

<Unchanged Text Omitted>

Transport-Layer-Address-Info ::= SEQUENCE {

transport-UP-Layer-Address-Info-To-Add-List		Transport-UP-Layer-Address-Info-To-Add-List							OPTIONAL,

transport-UP-Layer-Address-Info-To-Remove-List	Transport-UP-Layer-Address-Info-To-Remove-List					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Transport-Layer-Address-Info-ExtIEs } }								OPTIONAL

}

Transport-Layer-Address-Info-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime													OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SurvivalTime	CRITICALITY ignore	EXTENSION SurvivalTime	PRESENCE optional },

...

}

<Unchanged Text Omitted>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-SurvivalTime										ProtocolIE-ID ::= aaa

id-TimeSyncAssistanceInfo							ProtocolIE-ID ::= bbb

<Unchanged Text Omitted>

Change Ends