3GPP TSG-RAN3 Meeting #114-bis-e	R3-221424

Electronic meeting, 17 - 26 January 2022

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0745 | rev | 2 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to TS38.423) RAN slicing enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell, Huawei, ZTE,CATT, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Slice-Core |  |  |  |  |  |  | Date: |  | 2022-1-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR contains the related protocol changes for RAN slicing enhancement WID in RP-212534, e.g., Support of UE-Slice-MBR |  |  |  |  |  |  |  |  |
| Summary of change: |  | To support UE Slice MRB enforcement in mobility and DC scenario, the following changes are made 1. Add UE Slice Maximum Bit Rate IE in HANDOVER REQUEST message and UE Context Information – Retrieve UE Context Response IE. 2. Add S-NG-RAN node UE Slice Maximum Bit Rate List IE and corresponding descriptions in S-NODE ADDITION REQUEST message and S-NODE MODIFICATION REQUEST message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE Slice MBR cannot be enforced in mobility and DC scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS 38.463 CR1388 TS 38.413 CR 0682 TS 38.473 CR 0856 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | v1, updated according to the agreements RAN3 114-bis-e meeting |  |  |  |  |  |  |  |  |

<Start of change>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Target NG-RAN node UE XnAP ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node shall remove the existing prepared conditional HO identified by the Target NG-RAN node UE XnAP ID IE and the Target Cell Global ID IE. It is up to the implementation of the target NG-RAN node when to remove the HO information.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall stop the timer TXnRELOCprep and terminate the Handover Preparation procedure. If the procedure was initiated for an immediate handover, the source NG-RAN node shall start the timer TXnRELOCoverall. The source NG-RAN node is then defined to have a Prepared Handover for that Xn UE-associated signalling.

For each E-RAB ID IE included in the QoS Flow To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the content of the IE in the UE context and use it for subsequent inter-system handover.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

At reception of the HANDOVER REQUEST message the target NG-RAN node shall prepare the configuration of the AS security relation between the UE and the target NG-RAN node by using the information in the UE Security Capabilities IE and the AS Security Information IE in the UE Context Information IE, as specified in TS 33.501 [28].

Upon reception of the PDU Session Resource Setup List IE, contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as specified in TS 38.413 [5] for the PDU Session Resource Setup procedure. The target NG-RAN node shall report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU session resources. When the target NG-RAN node reports the unsuccessful establishment of a PDU session resource, the cause value should be precise enough to enable the source NG-RAN node to know the reason for the unsuccessful establishment.

For each PDU session if the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store the received PDU Session Aggregate Maximum Bit Rate in the UE context and use it when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [7].

For each QoS flow for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. The source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" for all the QoS flows mapped to a DRB, if it requests a DAPS handover for that DRB. For each PDU session that the target NG-RAN node decides to admit the data forwarding for at least one QoS flow, the target NG-RAN node includes the PDU Session level DL data forwarding GTP-U Tunnel Endpoint IE within the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resource Admitted Info IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

For each QoS flow for which the source NG-RAN node has not yet received the SDAP end marker packet if QoS flow re-mapping happened before handover, the source NG-RAN node shall include the UL Forwarding Proposal IE within the Data Forwarding and Offloading Info from source NG-RAN node IE in the HANDOVER REQUEST message, and if the target NG-RAN node decides to admit uplink data forwarding for at least one QoS flow, the target NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted Item IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the uplink data forwarding.

For each PDU session resource successfully setup at the target NG-RAN, the target NG-RAN node may allocate resources for additional Xn-U PDU session resource GTP-U tunnels, indicated in the Secondary Data Forwarding Info from target NG-RAN node List IE.

For each PDU session in the HANDOVER REQUEST message, if the Alternative QoS Parameters Set List IE is included in the GBR QoS Flow Information IE in the PDU Session Resources To Be Setup List IE, the target NG-RAN node may accept the setup of the involved QoS flow when notification control has been enabled if the requested QoS parameters set or at least one of the alternative QoS parameters sets can be fulfilled at the time of handover as specified in TS 23.501 [7]. In case the target NG-RAN node accepts the handover fulfilling one of the alternative QoS parameters it shall indicate the alternative QoS parameters set which it can currently fulfil in the Current QoS Parameters Set Index IE within the PDU Session Resources Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message while setting the QoS parameters towards the UE according to the requested QoS parameters set as specified in TS 23.501 [7].

For each DRB for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DRB ID IE and the mapped QoS Flows List IE within the Source DRB to QoS Flow Mapping List IE contained in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. The source NG-RAN node may include the QoS Flow Mapping Indication IE in the Source DRB to QoS Flow Mapping List IE to indicate that only the uplink or downlink QoS flow is mapped to the DRB. If the target NG-RAN node decides to use the same DRB configuration and to map the same QoS flows as the source NG-RAN node, the target NG-RAN node includes the DL Forwarding GTP Tunnel Endpoint IE within the Data Forwarding Response DRB List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this DRB.

The target NG-RAN node may additionally include the Redundant DL Forwarding UP TNL Information IE if at least one of the QoS flow mapped to the DRB is eligible to the redundant transmission feature as indicated in the Redundant QoS Flow Indicator IE within the PDU Session Resource To Be Setup List IE received in the HANDOVER REQUEST message for the QoS flow.

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL Forwarding GTP Tunnel Endpoint IE for a given DRB in the Data Forwarding Response DRB List IE within Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted List IE and the source NG-RAN node accepts the data forwarding proposed by the target NG-RAN node, the source NG-RAN node shall perform forwarding of uplink data for the DRB.

If the HANDOVER REQUEST includes PDU session resources for PDU sessions associated to S-NSSAIs not supported by target NG-RAN, the target NG-RAN shall reject such PDU session resources. In this case, and if at least one PDU Session Resource To Be Setup Item IE is admitted, the target NG-RAN shall send the HANDOVER REQUEST ACKNOWLEDGE message including the PDU Session Resources Not Admitted List IE listing corresponding PDU sessions rejected at the target NG-RAN.

If the Mobility Restriction List IE is

-	contained in the HANDOVER REQUEST message, the target NG-RAN node shall

-	store the information received in the Mobility Restriction List IE in the UE context;

-	use this information to determine a target for the UE during subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE, except when one of the PDU sessions has a particular ARP value (TS 23.501 [7]) in which case the information shall not apply;

-	use this information to select a proper SCG during dual connectivity operation.

-	use this information to select proper RNA(s) for the UE when moving the UE to RRC_INACTIVE.

-	not contained in the HANDOVER REQUEST message, the target NG-RAN node shall

-	consider that no roaming and no access restriction apply to the UE.

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23].

If the Index to RAT/Frequency Selection Priority IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the UE Context Reference at the S-NG-RAN IE is contained in the HANDOVER REQUEST message the target NG-RAN node may use it as specified in TS 37.340 [8]. In this case, the source NG-RAN node may expect the target NG-RAN node to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 37.340 [8].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource To Be Setup List IE and the Common Network Instance IE is not present, the target NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

Redundant transmission:

-	For each PDU session, if the Redundant UL NG-U UP TNL Information at UPF IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it as the uplink termination point for the user plane data for the redundant transmission for the concerned PDU session.

-	For each PDU session, if the Additional Redundant UL NG-U UP TNL Information at UPF List IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use them as the uplink termination points for the user plane data for the redundant transmission for the concerned PDU session.

-	For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [7].

-	For each PDU session, if the Redundant PDU Session Information IE is included in the PDU Session Resource To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received information in the UE context and set up the redundant user plane for the concerned PDU session, as specified in TS 23.501 [7].

If the TSC Traffic Characteristics IE is included in the QoS Flows To Be Setup List in the PDU Session Resource To Be Setup List IE, the target NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Be Setup List IE or in the Additional UL NG-U UP TNL Information at UPF List IE, or in the Additional Redundant UL NG-U UP TNL Information at UPF List IE, the target NG-RAN node shall, if supported, use it when selecting transport network resource for the concerned NG-U transport bearer as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", the target NG-RAN node shall perform user plane integrity protection or ciphering, respectively. If the NG-RAN node is not able to perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

If the NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required".

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or the Confidentiality Protection Indication IE is set to "preferred", the target NG-RAN node should, if supported, perform user plane integrity protection or ciphering, respectively and shall notify the SMF whether it succeeded the user plane integrity protection or ciphering or not for the concerned security policy.

For each PDU session for which the Maximum Integrity Protected Data Rate IE is included in the Security Indication IE in the PDU Session Resources To Be Setup List IE, the NG-RAN node shall store the respective information and, if integrity protection is to be performed for the PDU session, it shall enforce the traffic corresponding to the received Maximum Integrity Protected Data Rate IE, for the concerned PDU session and concerned UE, as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "not needed", the target NG-RAN node shall not perform user plane integrity protection or ciphering, respectively, for the concerned PDU session.

For each PDU session, if the Additional UL NG-U UP TNL Information List IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node may forward the UP transport layer information to the target S-NG-RAN node as the uplink termination point for the user plane data for this PDU session split in different tunnel.

If the Location Reporting Information IE is included in the HANDOVER REQUEST message, then the target NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

Upon reception of UE History Information IE in the HANDOVER REQUEST message, the target NG-RAN node shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

If the Trace Activation IE is included in the HANDOVER REQUEST message which includes

-	the MDT Activation IE set to "Immediate MDT and Trace", then the target NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the target NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

-	the MDT Location Information IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the target NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

-	the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the WLAN Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the Sensor Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall take it into account for MDT Configuration as described in TS 37.320 [43].

-	the MDT Configuration IE and if the target NG-RAN node is a gNB at least the MDT Configuration-NR IE shall be present, while if the target NG-RAN node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present. If the target NG-RAN node is a gNB receiving a MDT Configuration-EUTRA IE, or the target NG-RAN node is a ng-eNB receiving a MDT Configuration-NR IE, the target NG-RAN node shall store it as part of the UE context, and propagate it at the next Xn handover as described in TS 37.320 [43].

If the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

If the HANDOVER REQUEST message includes the Management Based MDT PLMN List IE, the target NG-RAN node shall take it into account if it includes information regarding the PLMN serving the UE in the target NG-RAN node.

If the Mobility Information IE is provided in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information. The target NG-RAN shall, if supported, store the C-RNTI assigned at the source cell as received in the HANDOVER REQUEST message.

Upon reception of the UE History Information from the UE IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

For each QoS flow which has been successfully established in the target NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information, and, if supported, use it for RAN part delay reporting.

If the 5GC Mobility Restriction List Container IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

V2X:

-	If the NR V2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the LTE V2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

-	If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

If the PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.287 [38].

If the DAPS Request Information IE is included for a given DRB in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a DAPS handover for that DRB, as described in TS 38.300 [9]. Accordingly, the target NG-RAN node shall include the DAPS Response Information IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Maximum Number of CHO Preparations IE is included in the Conditional Handover Information Acknowledge IE contained in the HANDOVER REQUEST ACKNOWLEDGE message, then the source NG-RAN node should not prepare more candidate target cells for a CHO for the same UE towards the target NG-RAN node than the number indicated in the IE.

If the Estimated Arrival Probability IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node may use the information to allocate necessary resources for the incoming CHO.

If the IAB Node Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the handover is for an IAB node.

If the UE Radio Capability ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

<Next change>

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, and decides to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message.

If the Index to RAT/Frequency Selection Priority IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the Location Reporting Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, then the new NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message which includes

-	the MDT Activation IE set to "Immediate MDT and Trace", then the target NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the target NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

-	the MDT Location Information IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the target NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

-	the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the WLAN Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the Sensor Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the MDT Configuration IE and if the target NG-RAN Node is a gNB at least the MDT Configuration-NR IE shall be present, while if the target NG-RAN Node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

For each QoS flow in the RETRIEVE UE CONTEXT RESPONSE message, if the QoS Monitoring Request IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall store this information, and, if supported, use it for RAN part delay reporting.

If the 5GC Mobility Restriction List Container IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

V2X:

-	If the NR V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the LTE V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

-	If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

-	If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

If the PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the target NG-RAN node shall, if supported, use it as defined in TS 23.287[38].

In case of RRC Re-establishment, the old NG-RAN may include the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message. Upon reception of the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

If the UE Radio Capability ID IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG- RAN node shall, if supported store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If the UE Slice-Maximum Bit Rate List IE is contained in RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

<Next change>

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the QoS Flow Level QoS Parameters IE for each QoS flow shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

The S-NG-RAN node shall choose the ciphering algorithm based on the information in the UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the S-NG-RAN node Security Key IE as specified in TS 33.501 [28].

If the TSC Traffic Characteristics IE is included for a QoS flow in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

If the Additional QoS Flow Information IE is included for a QoS flow in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

For each GBR QoS flow, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE, the S-NG-RAN node shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure specified in TS 38.413 [5].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE and the Common Network Instance IE is not present, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

For each GBR QoS flow, if the Offered GBR QoS Flow Information IE is included in the QoS Flows To Be Setup List IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node may request the M-NG-RAN node to configure the DRB to which that QoS flow is mapped with MCG resources.

For each PDU session, if the Non-GBR Resources Offered IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE and set to "true", the S-NG-RAN node may request the M-NG-RAN node to configure DRBs to which non-GBR QoS flows of the PDU session are mapped with MCG resources.

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

Redundant transmission:

-	For each PDU session, if the Redundant UL NG-U UP TNL Information at UPF IE is included in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall, if supported, use it as the uplink termination point for the user plane data for this PDU session for the redundant transmission and it shall include the Redundant DL NG-U UP TNL Information at NG-RAN IE in the PDU Session Resource Setup Response Info – SN terminated IE as described in TS 23.501 [9].

-	For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE the S-NG-RAN node shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [7].

-	For each PDU session for which the Redundant QoS Flow Indicator IE is include in QoS Flows To Be Setup List IE contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use it as specified in TS 23.501 [7].

-	For each PDU session, if the Redundant PDU Session Information IE is included in the PDU Session Resource Setup Info - SN terminated IE in the S-NODE ADDITION REQUEST message, the S-NODE-RAN node shall, if supported, store the received information in the UE context and setup the redundant user plane resources for the concerned PDU session, as specified in TS 23.501 [7].

-	For each PDU session resource successfully setup for which the Redundant PDU Session Information IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, include the Used RSN Information IE in the PDU Session Resource Setup Response Info – SN terminated IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the S-NODE ADDITION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE ADDITION REQUEST message contains the Expected UE Behaviour IE, the S-NG-RAN node shall, if supported, store this information and may use it to optimize resource allocation.

If the S-NODE ADDITION REQUEST message contains the Mobility Restriction List IE, the S-NG-RAN node, if supported, shall store this information and use it to select an appropriate SCG.

If the S-NODE ADDITION REQUEST message contains the Index to RAT/Frequency Selection Priority IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NG-RAN node is a gNB and the S-NODE ADDITION REQUEST message contains the PCell ID IE, the S-NG-RAN node shall search for the target NR cell among the NR neighbour cells of the PCell indicated, as specified in the TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the S-NG-RAN node PDU Session Aggregate Maximum Bit Rate IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE ADDITION REQUEST message contains the MR-DC Resource Coordination Information IE, the S-NG-RAN node should forward it to lower layers and it may use it for the purpose of resource coordination with the M-NG-RAN node, or to coordinate with sidelink resources used in the M-NG-RAN node. The S-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE. The S-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the S-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the S-NG-RAN node and the M-NG-RAN node.

If the S-NODE ADDITION REQUEST message contains the NE-DC TDM Pattern IE, the S-NG-RAN node should forward it to lower layers and use it for the purpose of single uplink transmission. The S-NG-RAN node shall consider the value of the received NE-DC TDM Pattern IE valid until reception of a new update of the IE for the same UE.

If the S-NODE ADDITION REQUEST message contains the QoS Flow Mapping Indication IE, the S-NG-RAN node may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

For each bearer for which allocation of the PDCP entity is requested at the S-NG-RAN node:

-	the M-NG-RAN node may propose to apply forwarding of downlink data by including the DL Forwarding IE within PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message. For each bearer that it has decided to admit, the S-NG-RAN node may include the DL Forwarding GTP Tunnel Endpoint IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer.

-	the S-NG-RAN node may include for each bearer in the PDU Session Resource Setup Response Info – SN terminated IE the UL Forwarding GTP Tunnel Endpoint IE to indicates it request data forwarding of uplink packets to be performed for that bearer.

-	the M-NG-RAN node shall include RLC Mode IE for each bearer offloaded from M-NG-RAN node to S-NG-RAN node in the DRBs to QoS Flow Mapping List IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDTION REQUEST message, and the RLC Mode IE indicates the mode that the M-NG-RAN used for the DRB when it was hosted at the M-NG-RAN node.

For each bearer for which the PDCP entity is at the M-NG-RAN node:

-	the M-NG-RAN node shall include the RLC mode IE for each bearer in the DRBs To Be Setup List IE within the PDU Session Resource Setup Info – MN terminated IE of the S-NODE ADDTION REQUEST message to indicate the RLC mode has been configured at the M-NG-RAN node, so that the S-NG-RAN node shall configure the same RLC mode for this MN terminated split bearer.

The M-NG-RAN node may also propose to apply forwarding of UL data when offloading QoS flows for which in-order delivery is requested by including the UL Forwarding Proposal IE in the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message. The S-NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding.

If the Masked IMEISV IE is contained in the S-NODE ADDITION REQUEST message the S-NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the UE Radio Capability ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

The S-NG-RAN node shall report to the M-NG-RAN node, in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the result for all the requested PDU session resources in the following way:

-	A list of PDU session resources which are successfully established shall be included in the PDU Session Resources Admitted To Be Added List IE.

-	A list of PDU session resources which failed to be established shall be included in the PDU Session Resources Not Admitted List IE.

Upon reception of the S-NODE ADDITION REQUEST ACKNOWLEDGE message the M-NG-RAN node shall stop the timer TXnDCprep.

If the S-NODE ADDITION REQUEST ACKNOWLEDGE message contains the MR-DC Resource Coordination Information IE, the M-NG-RAN node may use it for the purpose of resource coordination with the S-NG-RAN node. The M-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE. The M-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the M-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the M-NG-RAN node and the S-NG-RAN node.

The S-NG-RAN node may include for each bearer in the DRBs To Be Setup List IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message the PDCP SN Length IE to indicate the PDCP SN length for that DRB.

If the S-NG-RAN node UE XnAP ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information and use it as defined in TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the PDCP SN Length IE, the S-NG-RAN node shall, if supported, store this information and use it for lower layer configuration of the concerned MN terminated bearer.

If the S-NODE ADDITION REQUEST message contains the SN Addition Trigger Indication IE, the S-NG-RAN node shall include the RRC config indication IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message to inform the M-NG-RAN node if the S-NG-RAN node applied full or delta configuration, as specified in TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the S-NG-RAN node Maximum Integrity Protected Data Rate Uplink IE or the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE, the S-NG-RAN node shall use the received information when enforcing the maximum integrity protected data rate for the UE.

If the Security Indication IE is included in the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message, the behaviour of the S-NG-RAN node shall be the same as specified for the same IE in the PDU Session Resources To Be Setup List IE in the Handover Preparation procedure, for the concerned PDU session, and the S-NG-RAN node shall include the Security Result IE in the PDU Session Resource Setup Response Info – SN terminated IE.

If the Security Result IE is included in the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message, the S-NG-RAN node may take the information into account when deciding whether to perform user plane integrity protection or ciphering for the DRBs that it establishes for the concerned PDU session, except if the Split Session Indicator IE is included in the PDU Session Resource Setup Info – SN terminated IE and set to "split", in which case it shall perform user plane integrity protection or ciphering according to the information in the Security Result IE. If the S-NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required" as specified in TS 33.501 [28]. If either the S-NG-RAN node or the M-NG-RAN node is an ng-eNB, the S-NG-RAN node shall behave according to clause 6.10.4 of TS 33.501 [28] for PDU sessions for which the Integrity Protection Indication IE is set to "preferred".

The S-NG-RAN node may include the Location Information at S-NODE IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, if respective information is available at the S-NG-RAN node.

If the Location Information at S-NODE Reporting IE set to "pscell" is included in the S-NODE ADDITION REQUEST, the S-NG-RAN node shall, start providing information about the current location of the UE. If the Location Information at S-NODE IE is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE, the M-NG-RAN node shall store the included information so that it may be transferred towards the AMF.

If the Default DRB Allowed IE is included in the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message and set to "true", the S-NG-RAN node may configure the default DRB for the PDU session.

If the S-NODE ADDITION REQUEST ACKNOWLEDGE message includes the DRB IDs taken into use IE, the M-NG-RAN node, if applicable, shall act as specified in TS 37.340 [8].

If Trace Activation IE has previously been received for this UE, it shall be included in the S-NODE ADDITION REQUEST message. If the Trace Activation IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [23].

If the Requested Fast MCG recovery via SRB3 IE set to "true" is included in the S-NODE ADDITION REQUEST message and the S-NG-RAN node decides to configure fast MCG link recovery via SRB3 as specified in TS 37.340 [8], the S-NG-RAN shall, if supported, include the Available fast MCG recovery via SRB3 IE set to "true" in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the QoS Monitoring Request IE is included in the QoS Flow Level QoS Parameters IE for a QoS flow contained in the DRBs To Be Setup List IE of the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE is included in the QoS Flow Level QoS Parameters IE for a QoS flow contained in the DRBs To Be Setup List IE of the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall, if supported, use it for RAN part delay reporting.

For each QoS flow which has been successfully established in the S-NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and, if supported, use it for RAN part delay reporting. In case such a QoS flow is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring. If the QoS Monitoring Reporting Frequency IE is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it for RAN part delay reporting.

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted List IE in the PDU Session Resource Setup Response Info – MN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

If the S-NG-RAN node UE Slice Maximum Bit Rate IE for a specific S-NSSAI is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

<Next change>

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE MODIFICATION REQUEST message, it shall start the timer TXnDCprep.

The S-NODE MODIFICATION REQUEST message may contain

-	within the UE Context Information IE;

-	PDU session resources to be added within the PDU Session Resources To Be Added Item IE;

-	PDU session resources to be modified within the PDU Session Resources To Be Modified Item IE;

-	PDU session resources to be released within the PDU Session Resources To Be Released Item IE;

-	the S-NG-RAN node Security Key IE;

-	the S-NG-RAN node UE Aggregate Maximum Bit Rate IE;

-	the M-NG-RAN node to S-NG-RAN node Container IE;

-	the PDCP Change Indication IE;

-	the SCG Configuration Query IE;

-	the Requested split SRBs IE;

-	the Requested split SRBs release IE;

-	the Requested fast MCG recovery via SRB3 IE;

-	the Requested fast MCG recovery via SRB3 Release IE;

-	the Additional DRB IDs IE;

-	the MR-DC Resource Coordination Information IE.

If the S-NODE MODIFICATION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE MODIFICATION REQUEST message contains the Mobility Restriction List IE, the S-NG-RAN node shall

-	replace the previously provided Mobility Restriction List by the received Mobility Restriction List in the UE context;

-	use this information to select an appropriate SCG.

If the S-NG-RAN node UE Aggregate Maximum Bit Rate IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall:

-	replace the previously provided S-NG-RAN node UE Aggregate Maximum Bit Rate by the received S-NG-RAN node UE Aggregate Maximum Bit Rate in the UE context;

-	use the received S-NG-RAN node UE Aggregate Maximum Bit Rate for Non-GBR Bearers for the concerned UE as defined in TS 37.340 [8].

If the S-NG-RAN node UE Slice Maximum Bit Rate IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported:

-	store and replace the previously provided S-NG-RAN node UE Slice Maximum Bit Rate, if any, by the received S-NG-RAN node UE Slice Maximum Bit Rate for each S-NSSAI for the concerned UE;

-	use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

<Next change>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.x |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |

<Next change>

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | Slice Maximum Bit Rate List 9.2.3.x | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | Ignore |

<Next change>

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node |  | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. |  | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.3.49 |  |  | – |  |
| >S-NG-RAN node Security Key | O |  | 9.2.3.51 |  |  | – |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.3.17 |  |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  |  | – |  |
| >Lower Layer presence status change | O |  | 9.2.3.60 |  |  | – |  |
| >PDU Session Resources To Be Added List |  | 0..1 |  |  |  | – |  |
| >>PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  |  | – |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  | – |  |
| >>>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  |  | – |  |
| >>>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  |  | – |  |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. |  | YES | ignore |
| >PDU Session Resources To Be Modified List |  | 0..1 |  |  |  | – |  |
| >>PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE nor the PDU Session Resource Modification Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  | – |  |
| >>>PDU Session Resource Modification Info – SN terminated | O |  | 9.2.1.9 |  |  | – |  |
| >>>PDU Session Resource Modification Info – MN terminated | O |  | 9.2.1.11 |  |  | – |  |
| >>>S-NSSAI | O |  | 9.2.3.21 |  |  | YES | reject |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. |  | YES | ignore |
| >PDU Session Resources To Be Released List | O |  | PDU session List with Cause 9.2.1.26 |  |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10]. |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. |  | YES | ignore |
| Requested Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested to be released. |  | YES | ignore |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  |  | YES | ignore |
| Additional DRB IDs | O |  | DRB List 9.2.1.29 |  | Indicates additional list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) |  | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 |  | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  |  | YES | reject |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) |  | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| Requested Fast MCG recovery via SRB3 Release | O |  | ENUMERATED (true, ...) |  | Indicates that resources for fast MCG recovery via SRB3 are requested to be released. | YES | ignore |
| SN triggered | O |  | ENUMERATED (TRUE ...) |  |  | YES | ignore |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.x |  | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | Ignore |

<Next change>

#### 9.2.1.13	UE Context Information – Retrieve UE Context Response

This IE contains the UE context information within the RETRIEVE UE CONTEXT RESPONSE message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the old NG-C connection. | – |  |
| Signalling TNL Association Address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| AS Security Information | M |  | 9.2.3.50 |  | – |  |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| PDU Session Resources To Be Setup List | M |  | 9.2.1.1 |  | – |  |
| RRC Context | M |  | OCTET STRING | Includes the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331[10] if the old and new serving NG-RAN nodes are gNBs. Includes either the HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [14] or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the old and new serving NG-RAN nodes are ng-eNBs. | – |  |
| Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| 5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | Ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| UE Slice Maximum Bit Rate List | O |  | 9.2.3.x |  | YES | ignore |

<Next change>

#### 9.2.3.x	UE Slice Maximum Bit Rate List

The UE Slice Maximum Bit Rate List includes a list of UE Slice Maximum Bit Rate, each UE Slice Maximum Bit Rate is applicable for all PDU Sessions associated with a specific S-NSSAI of that UE, which is defined for the Downlink and the Uplink direction as specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Slice Maximum Bit Rate |  | 1..<maxnoofSMBR> |  |  |
| >S-NSSAI | M |  | 9.3.1.24 |  |
| >UE Slice Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.3.4 | This IE indicates the UE Slice Maximum Bit Rate as specified in TS 23.501 [7] in the downlink direction. |
| >UE Slice Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.3.4 | This IE indicates the UE Slice Maximum Bit Rate as specified in TS 23.501 [7] in the uplink direction. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSMBR | Maximum no. of SLICE MAXIMUM BIT RATE for a UE. Value is 8. |

<Next change>

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													|

resourceStatusReportingInitiation						|

mobilitySettingsChange									,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport 			|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

sNStatusTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-sNStatusTransfer

CRITICALITY				ignore

}

handoverCancel	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

PROCEDURE CODE			id-handoverCancel

CRITICALITY				ignore

}

retrieveUEContext	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEContextRequest

SUCCESSFUL OUTCOME		RetrieveUEContextResponse

UNSUCCESSFUL OUTCOME	RetrieveUEContextFailure

PROCEDURE CODE			id-retrieveUEContext

CRITICALITY				reject

}

rANPaging	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANPaging

PROCEDURE CODE			id-rANPaging

CRITICALITY				reject

}

xnUAddressIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnUAddressIndication

PROCEDURE CODE			id-xnUAddressIndication

CRITICALITY				reject

}

uEContextRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextRelease

PROCEDURE CODE			id-uEContextRelease

CRITICALITY				reject

}

sNGRANnodeAdditionPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeAdditionRequest

SUCCESSFUL OUTCOME		SNodeAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeAdditionRequestReject

PROCEDURE CODE			id-sNGRANnodeAdditionPreparation

CRITICALITY				reject

}

sNGRANnodeReconfigurationCompletion	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReconfigurationComplete

PROCEDURE CODE			id-sNGRANnodeReconfigurationCompletion

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequest

SUCCESSFUL OUTCOME		SNodeModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeModificationRequestReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequired

SUCCESSFUL OUTCOME		SNodeModificationConfirm

UNSUCCESSFUL OUTCOME	SNodeModificationRefuse

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequest

SUCCESSFUL OUTCOME		SNodeReleaseRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeReleaseReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequired

SUCCESSFUL OUTCOME		SNodeReleaseConfirm

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeCounterCheck	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeCounterCheckRequest

PROCEDURE CODE			id-sNGRANnodeCounterCheck

CRITICALITY				reject

}

sNGRANnodeChange		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeChangeRequired

SUCCESSFUL OUTCOME		SNodeChangeConfirm

UNSUCCESSFUL OUTCOME	SNodeChangeRefuse

PROCEDURE CODE			id-sNGRANnodeChange

CRITICALITY				reject

}

rRCTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				reject

}

xnRemoval	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnRemovalRequest

SUCCESSFUL OUTCOME		XnRemovalResponse

UNSUCCESSFUL OUTCOME		XnRemovalFailure

PROCEDURE CODE			id-xnRemoval

CRITICALITY				reject

}

xnSetup	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnSetupRequest

SUCCESSFUL OUTCOME		XnSetupResponse

UNSUCCESSFUL OUTCOME		XnSetupFailure

PROCEDURE CODE			id-xnSetup

CRITICALITY				reject

}

nGRANnodeConfigurationUpdate	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGRANNodeConfigurationUpdate

SUCCESSFUL OUTCOME		NGRANNodeConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	NGRANNodeConfigurationUpdateFailure

PROCEDURE CODE			id-nGRANnodeConfigurationUpdate

CRITICALITY				reject

}

e-UTRA-NR-CellResourceCoordination	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-UTRA-NR-CellResourceCoordinationRequest

SUCCESSFUL OUTCOME		E-UTRA-NR-CellResourceCoordinationResponse

PROCEDURE CODE			id-e-UTRA-NR-CellResourceCoordination

CRITICALITY				reject

}

cellActivation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellActivationRequest

SUCCESSFUL OUTCOME		CellActivationResponse

UNSUCCESSFUL OUTCOME	CellActivationFailure

PROCEDURE CODE			id-cellActivation

CRITICALITY				reject

}

reset	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResetRequest

SUCCESSFUL OUTCOME		ResetResponse

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

notificationControl			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NotificationControlIndication

PROCEDURE CODE			id-notificationControl

CRITICALITY				ignore

}

activityNotification		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ActivityNotification

PROCEDURE CODE			id-activityNotification

CRITICALITY				ignore

}

privateMessage			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

secondaryRATDataUsageReport	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-secondaryRATDataUsageReport

CRITICALITY				reject

}

deactivateTrace XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-deactivateTrace

CRITICALITY				ignore

}

traceStart XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-traceStart

CRITICALITY				ignore

}

handoverSuccess			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverSuccess

PROCEDURE CODE			id-handoverSuccess

CRITICALITY				ignore

}

conditionalHandoverCancel	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ConditionalHandoverCancel

PROCEDURE CODE			id-conditionalHandoverCancel

CRITICALITY				ignore

}

earlyStatusTransfer		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		EarlyStatusTransfer

PROCEDURE CODE			id-earlyStatusTransfer

CRITICALITY				ignore

}

failureIndication XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		FailureIndication

PROCEDURE CODE			id-failureIndication

CRITICALITY				ignore

}

handoverReport XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverReport

PROCEDURE CODE			id-handoverReport

CRITICALITY				ignore

}

resourceStatusReportingInitiation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ResourceStatusRequest

SUCCESSFUL OUTCOME				ResourceStatusResponse

UNSUCCESSFUL OUTCOME			ResourceStatusFailure

PROCEDURE CODE					id-resourceStatusReportingInitiation

CRITICALITY						reject

}

resourceStatusReporting XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

mobilitySettingsChange	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				MobilityChangeRequest

SUCCESSFUL OUTCOME				MobilityChangeAcknowledge

UNSUCCESSFUL OUTCOME			MobilityChangeFailure

PROCEDURE CODE					id-mobilitySettingsChange

CRITICALITY						reject

}

accessAndMobilityIndication XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY 				ignore

}

END

-- ASN1STOP

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

BitRate,

Cause,

CellAndCapacityAssistanceInfo-EUTRA,

CellAndCapacityAssistanceInfo-NR,

CellAssistanceInfo-EUTRA,

CellAssistanceInfo-NR,

CHOinformation-Req,

CHOinformation-Ack,

CHO-MRDC-EarlyDataForwarding,

CHO-MRDC-Indicator,

CPTransportLayerInformation,

TNLA-To-Add-List,

TNLA-To-Update-List,

TNLA-To-Remove-List,

TNLA-Setup-List,

TNLA-Failed-To-Setup-List,

CriticalityDiagnostics,

XnUAddressInfoperPDUSession-List,

DAPSResponseInfo-List,

DataTrafficResourceIndication,

DeliveryStatus,

DesiredActNotificationLevel,

DRB-ID,

DRB-List,

DRB-Number,

DRBsSubjectToDLDiscarding-List,

DRBsSubjectToEarlyStatusTransfer-List,

DRBsSubjectToStatusTransfer-List,

DRBToQoSFlowMapping-List,

E-UTRA-CGI,

ExpectedUEActivityBehaviour,

ExpectedUEBehaviour,

ExtendedUEIdentityIndexValue,

FiveGCMobilityRestrictionListContainer,

GlobalCell-ID,

GlobalNG-RANNode-ID,

GlobalNG-RANCell-ID,

GUAMI,

InterfaceInstanceIndication,

I-RNTI,

LocationInformationSNReporting,

LocationReportingInformation,

LowerLayerPresenceStatusChange,

LTEUESidelinkAggregateMaximumBitRate,

LTEV2XServicesAuthorized,

MR-DC-ResourceCoordinationInfo,

ServedCells-E-UTRA,

ServedCells-NR,

ServedCellsToUpdate-E-UTRA,

ServedCellsToUpdate-NR,

MAC-I,

MaskedIMEISV,

MDT-Configuration,

MDTPLMNList,

MobilityRestrictionList,

NG-RAN-Cell-Identity,

NG-RANnodeUEXnAPID,

NR-CGI,

NE-DC-TDM-Pattern,

NRUESidelinkAggregateMaximumBitRate,

NRV2XServicesAuthorized,

PagingDRX,

PagingeDRXInformation,

PagingPriority,

PartialListIndicator,

PLMN-Identity,

PDCPChangeIndication,

PDUSessionAggregateMaximumBitRate,

PDUSession-ID,

PDUSession-List,

PDUSession-List-withCause,

PDUSession-List-withDataForwardingFromTarget,

PDUSession-List-withDataForwardingRequest,

PDUSessionResourcesAdmitted-List,

PDUSessionResourcesNotAdmitted-List,

PDUSessionResourcesToBeSetup-List,

PDUSessionResourceChangeRequiredInfo-SNterminated,

PDUSessionResourceChangeRequiredInfo-MNterminated,

PDUSessionResourceChangeConfirmInfo-SNterminated,

PDUSessionResourceChangeConfirmInfo-MNterminated,

PDUSessionResourceSecondaryRATUsageList,

PDUSessionResourceSetupInfo-SNterminated,

PDUSessionResourceSetupInfo-MNterminated,

PDUSessionResourceSetupResponseInfo-SNterminated,

PDUSessionResourceSetupResponseInfo-MNterminated,

PDUSessionResourceModificationInfo-SNterminated,

PDUSessionResourceModificationInfo-MNterminated,

PDUSessionResourceModificationResponseInfo-SNterminated,

PDUSessionResourceModificationResponseInfo-MNterminated,

PDUSessionResourceModConfirmInfo-SNterminated,

PDUSessionResourceModConfirmInfo-MNterminated,

PDUSessionResourceModRqdInfo-SNterminated,

PDUSessionResourceModRqdInfo-MNterminated,

PDUSessionType,

PC5QoSParameters,

QoSFlowIdentifier,

QoSFlowNotificationControlIndicationInfo,

QoSFlows-List,

RANPagingArea,

ResetRequestTypeInfo,

ResetResponseTypeInfo,

RFSP-Index,

RRCConfigIndication,

RRCResumeCause,

SCGConfigurationQuery,

SecurityIndication,

S-NG-RANnode-SecurityKey,

SpectrumSharingGroupID,

SplitSRBsTypes,

S-NG-RANnode-Addition-Trigger-Ind,

S-NSSAI,

TargetCellList,

TAISupport-List,

Target-CGI,

TimeToWait,

TraceActivation,

UEAggregateMaximumBitRate,

UEContextID,

UEContextInfoRetrUECtxtResp,

UEContextKeptIndicator,

UEHistoryInformation,

UEIdentityIndexValue,

UERadioCapabilityForPaging,

UERadioCapabilityID,

UERANPagingIdentity,

UESecurityCapabilities,

UPTransportLayerInformation,

UserPlaneTrafficActivityReport,

XnBenefitValue,

RANPagingFailure,

TNLConfigurationInfo,

MaximumCellListSize,

MessageOversizeNotification,

NG-RANTraceID,

MobilityInformation,

InitiatingCondition-FailureIndication,

HandoverReportType,

TargetCellinEUTRAN,

C-RNTI,

UERLFReportContainer,

Measurement-ID,

RegistrationRequest,

ReportCharacteristics,

CellToReport,

ReportingPeriodicity,

CellMeasurementResult,

UEHistoryInformationFromTheUE,

MobilityParametersInformation,

MobilityParametersModificationRange,

RACHReportInformation,

IABNodeIndication,

SNTriggered,

SCGIndicator,

UESpecificDRX,

UESliceMaximumBitRateList

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AdditionalDRBIDs,

id-AMF-Region-Information,

id-AMF-Region-Information-To-Add,

id-AMF-Region-Information-To-Delete,

id-AssistanceDataForRANPaging,

id-AvailableDRBIDs,

id-Cause,

id-cellAssistanceInfo-EUTRA,

id-cellAssistanceInfo-NR,

id-CellAndCapacityAssistanceInfo-EUTRA,

id-CellAndCapacityAssistanceInfo-NR,

id-ConfigurationUpdateInitiatingNodeChoice,

id-UEContextID,

id-CriticalityDiagnostics,

id-XnUAddressInfoperPDUSession-List,

id-DesiredActNotificationLevel,

id-DRBsSubjectToStatusTransfer-List,

id-ExpectedUEBehaviour,

id-ExtendedUEIdentityIndexValue,

id-FiveGCMobilityRestrictionListContainer,

id-GlobalNG-RAN-node-ID,

id-GUAMI,

id-indexToRatFrequSelectionPriority,

id-List-of-served-cells-E-UTRA,

id-List-of-served-cells-NR,

id-LocationInformationSN,

id-LocationInformationSNReporting,

id-LocationReportingInformation,

id-LTEUESidelinkAggregateMaximumBitRate,

id-LTEV2XServicesAuthorized,

id-MAC-I,

id-MaskedIMEISV,

id-MDT-Configuration,

id-MDTPLMNList,

id-MN-to-SN-Container,

id-MobilityRestrictionList,

id-M-NG-RANnodeUEXnAPID,

id-new-NG-RAN-Cell-Identity,

id-newNG-RANnodeUEXnAPID,

id-NRUESidelinkAggregateMaximumBitRate,

id-NRV2XServicesAuthorized,

id-oldNG-RANnodeUEXnAPID,

id-OldtoNewNG-RANnodeResumeContainer,

id-PagingDRX,

id-PagingeDRXInformation,

id-PagingPriority,

id-PartialListIndicator-EUTRA,

id-PartialListIndicator-NR,

id-PCellID,

id-PDUSessionResourceSecondaryRATUsageList,

id-PDUSessionResourcesActivityNotifyList,

id-PDUSessionResourcesAdmitted-List,

id-PDUSessionResourcesNotAdmitted-List,

id-PDUSessionResourcesNotifyList,

id-PDUSessionToBeAddedAddReq,

id-PDUSessionToBeReleased-RelReqAck,

id-procedureStage,

id-RANPagingArea,

id-requestedSplitSRB,

id-RequiredNumberOfDRBIDs,

id-ResetRequestTypeInfo,

id-ResetResponseTypeInfo,

id-RespondingNodeTypeConfigUpdateAck,

id-RRCResumeCause,

id-selectedPLMN,

id-ServedCellsToActivate,

id-servedCellsToUpdate-E-UTRA,

id-ServedCellsToUpdateInitiatingNodeChoice,

id-servedCellsToUpdate-NR,

id-sourceNG-RANnodeUEXnAPID,

id-SpareDRBIDs,

id-S-NG-RANnodeMaxIPDataRate-UL,

id-S-NG-RANnodeMaxIPDataRate-DL,

id-S-NG-RANnodeUEXnAPID,

id-TAISupport-list,

id-Target2SourceNG-RANnodeTranspContainer,

id-targetCellGlobalID,

id-targetNG-RANnodeUEXnAPID,

id-TimeToWait,

id-TNLA-To-Add-List,

id-TNLA-To-Update-List,

id-TNLA-To-Remove-List,

id-TNLA-Setup-List,

id-TNLA-Failed-To-Setup-List,

id-TraceActivation,

id-UEContextInfoHORequest,

id-UEContextInfoRetrUECtxtResp,

id-UEContextKeptIndicator,

id-UEContextRefAtSN-HORequest,

id-UEHistoryInformation,

id-UEIdentityIndexValue,

id-UERANPagingIdentity,

id-UESecurityCapabilities,

id-UserPlaneTrafficActivityReport,

id-XnRemovalThreshold,

id-PDUSessionAdmittedAddedAddReqAck,

id-PDUSessionNotAdmittedAddReqAck,

id-SN-to-MN-Container,

id-RRCConfigIndication,

id-SplitSRB-RRCTransfer,

id-UEReportRRCTransfer,

id-PDUSessionReleasedList-RelConf,

id-BearersSubjectToCounterCheck,

id-PDUSessionToBeReleasedList-RelRqd,

id-ResponseInfo-ReconfCompl,

id-initiatingNodeType-ResourceCoordRequest,

id-respondingNodeType-ResourceCoordResponse,

id-PDUSessionToBeReleased-RelReq,

id-PDUSession-SNChangeRequired-List,

id-PDUSession-SNChangeConfirm-List,

id-PDCPChangeIndication,

id-PC5QoSParameters,

id-SCGConfigurationQuery,

id-UEContextInfo-SNModRequest,

id-requestedSplitSRBrelease,

id-PDUSessionAdmitted-SNModResponse,

id-PDUSessionNotAdmitted-SNModResponse,

id-admittedSplitSRB,

id-admittedSplitSRBrelease,

id-PDUSessionAdmittedModSNModConfirm,

id-PDUSessionReleasedSNModConfirm,

id-s-ng-RANnode-SecurityKey,

id-PDUSessionToBeModifiedSNModRequired,

id-S-NG-RANnodeUE-AMBR,

id-PDUSessionToBeReleasedSNModRequired,

id-target-S-NG-RANnodeID,

id-S-NSSAI,

id-MR-DC-ResourceCoordinationInfo,

id-RANPagingFailure,

id-UERadioCapabilityForPaging,

id-PDUSessionDataForwarding-SNModResponse,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-NE-DC-TDM-Pattern,

id-InterfaceInstanceIndication,

id-S-NG-RANnode-Addition-Trigger-Ind,

id-SNTriggered,

id-DRBs-transferred-to-MN,

id-TNLConfigurationInfo,

id-MessageOversizeNotification,

id-NG-RANTraceID,

id-FastMCGRecoveryRRCTransfer-SN-to-MN,

id-FastMCGRecoveryRRCTransfer-MN-to-SN,

id-RequestedFastMCGRecoveryViaSRB3,

id-AvailableFastMCGRecoveryViaSRB3,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-ReleaseFastMCGRecoveryViaSRB3,

id-CHOinformation-Req,

id-CHOinformation-Ack,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-DAPSResponseInfo-List,

id-CHO-MRDC-EarlyDataForwarding,

id-CHO-MRDC-Indicator,

id-MobilityInformation,

id-InitiatingCondition-FailureIndication,

id-UEHistoryInformationFromTheUE,

id-HandoverReportType,

id-HandoverCause,

id-SourceCellCGI,

id-TargetCellCGI,

id-ReEstablishmentCellCGI,

id-TargetCellinEUTRAN,

id-SourceCellCRNTI,

id-UERLFReportContainer,

id-NGRAN-Node1-Measurement-ID,

id-NGRAN-Node2-Measurement-ID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReport,

id-ReportingPeriodicity,

id-CellMeasurementResult,

id-NG-RANnode1CellID,

id-NG-RANnode2CellID,

id-NG-RANnode1MobilityParameters,

id-NG-RANnode2ProposedMobilityParameters,

id-MobilityParametersModificationRange,

id-RACHReportInformation,

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

id-UESliceMaximumBitRateList,

id-S-NG-RANnodeUE-Slice-MBR,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause										PRESENCE mandatory}|

{ ID id-targetCellGlobalID					CRITICALITY reject	TYPE Target-CGI									PRESENCE mandatory}|

{ ID id-GUAMI								CRITICALITY reject	TYPE GUAMI										PRESENCE mandatory}|

{ ID id-UEContextInfoHORequest				CRITICALITY reject	TYPE UEContextInfoHORequest						PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore	TYPE MaskedIMEISV								PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE mandatory}|

{ ID id-UEContextRefAtSN-HORequest			CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest				PRESENCE optional }|

{ ID id-CHOinformation-Req					CRITICALITY reject	TYPE CHOinformation-Req							PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized				CRITICALITY ignore	TYPE NRV2XServicesAuthorized					PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized			CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional }|

{ ID id-PC5QoSParameters					CRITICALITY ignore	TYPE PC5QoSParameters					PRESENCE optional }|

{ ID id-MobilityInformation				CRITICALITY ignore	TYPE MobilityInformation								PRESENCE optional}|

{ ID id-UEHistoryInformationFromTheUE	CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject	TYPE IABNodeIndication							PRESENCE optional},

...

}

UEContextInfoHORequest ::= SEQUENCE {

ng-c-UE-reference						AMF-UE-NGAP-ID,

cp-TNL-info-source						CPTransportLayerInformation,

ueSecurityCapabilities					UESecurityCapabilities,

securityInformation						AS-SecurityInformation,

indexToRatFrequencySelectionPriority	RFSP-Index															OPTIONAL,

ue-AMBR									UEAggregateMaximumBitRate,

pduSessionResourcesToBeSetup-List		PDUSessionResourcesToBeSetup-List,

rrc-Context								OCTET STRING,

locationReportingInformation			LocationReportingInformation										OPTIONAL,

mrl										MobilityRestrictionList												OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {UEContextInfoHORequest-ExtIEs} }	OPTIONAL,

...

}

UEContextInfoHORequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-FiveGCMobilityRestrictionListContainer	CRITICALITY ignore	EXTENSION FiveGCMobilityRestrictionListContainer		PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitRate		CRITICALITY ignore	EXTENSION NRUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION LTEUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-MDTPLMNList								CRITICALITY ignore	EXTENSION MDTPLMNList											PRESENCE optional }|

{ ID id-UERadioCapabilityID						CRITICALITY reject	EXTENSION UERadioCapabilityID								PRESENCE optional }|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	EXTENSION UESliceMaximumBitRateList							PRESENCE optional },

...

}

UEContextRefAtSN-HORequest ::= SEQUENCE {

globalNG-RANNode-ID				GlobalNG-RANNode-ID,

sN-NG-RANnodeUEXnAPID			NG-RANnodeUEXnAPID,

iE-Extensions					ProtocolExtensionContainer { {UEContextRefAtSN-HORequest-ExtIEs} }	OPTIONAL,

...

}

UEContextRefAtSN-HORequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

-- **************************************************************

--

#### -- HANDOVER REQUEST ACKNOWLEDGE

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequestAcknowledge-IEs}},

...

}

HandoverRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionResourcesAdmitted-List		CRITICALITY ignore	TYPE PDUSessionResourcesAdmitted-List			PRESENCE mandatory}|

{ ID id-PDUSessionResourcesNotAdmitted-List		CRITICALITY ignore	TYPE PDUSessionResourcesNotAdmitted-List		PRESENCE optional }|

{ ID id-Target2SourceNG-RANnodeTranspContainer	CRITICALITY ignore	TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore	TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-DRBs-transferred-to-MN					CRITICALITY ignore	TYPE DRB-List										PRESENCE optional }|

{ ID id-DAPSResponseInfo-List					CRITICALITY reject	TYPE DAPSResponseInfo-List						PRESENCE optional }|

{ ID id-CHOinformation-Ack						CRITICALITY reject	TYPE CHOinformation-Ack								PRESENCE optional },

...

}

-- **************************************************************

--

#### -- HANDOVER PREPARATION FAILURE

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverPreparationFailure-IEs}},

...

}

HandoverPreparationFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-requestedTargetCellGlobalID				CRITICALITY reject	TYPE Target-CGI									PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SN STATUS TRANSFER

--

-- **************************************************************

SNStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{SNStatusTransfer-IEs}},

...

}

SNStatusTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-DRBsSubjectToStatusTransfer-List		CRITICALITY ignore		TYPE DRBsSubjectToStatusTransfer-List		PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- UE CONTEXT RELEASE

--

-- **************************************************************

UEContextRelease ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{UEContextRelease-IEs}},

...

}

UEContextRelease-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- HANDOVER CANCEL

--

-- **************************************************************

HandoverCancel ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverCancel-IEs}},

...

}

HandoverCancel-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause										CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-targetCellsToCancel							CRITICALITY reject		TYPE TargetCellList								PRESENCE optional},

...

}

-- **************************************************************

--

#### -- HANDOVER SUCCESS

--

-- **************************************************************

HandoverSuccess ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverSuccess-IEs}},

...

}

HandoverSuccess-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-requestedTargetCellGlobalID					CRITICALITY reject		TYPE Target-CGI										PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- CONDITIONAL HANDOVER CANCEL

--

-- **************************************************************

ConditionalHandoverCancel ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ConditionalHandoverCancel-IEs}},

...

}

ConditionalHandoverCancel-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause										CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-targetCellsToCancel							CRITICALITY reject		TYPE TargetCellList								PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EARLY STATUS TRANSFER

--

-- **************************************************************

EarlyStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ EarlyStatusTransfer-IEs}},

...

}

EarlyStatusTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-procedureStage								CRITICALITY reject		TYPE ProcedureStageChoice						PRESENCE mandatory},

...

}

ProcedureStageChoice ::= CHOICE {

first-dl-count						FirstDLCount,

dl-discarding						DLDiscarding,

choice-extension					ProtocolIE-Single-Container { {ProcedureStageChoice-ExtIEs} }

}

ProcedureStageChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

FirstDLCount ::= SEQUENCE {

dRBsSubjectToEarlyStatusTransfer			DRBsSubjectToEarlyStatusTransfer-List,

iE-Extension								ProtocolExtensionContainer { {FirstDLCount-ExtIEs} }	OPTIONAL,

...

}

FirstDLCount-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DLDiscarding ::= SEQUENCE {

dRBsSubjectToDLDiscarding					DRBsSubjectToDLDiscarding-List,

iE-Extension								ProtocolExtensionContainer { {DLDiscarding-ExtIEs} }	OPTIONAL,

...

}

DLDiscarding-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- RAN PAGING

--

-- **************************************************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue				CRITICALITY reject		TYPE UEIdentityIndexValue							PRESENCE mandatory}|

{ ID id-UERANPagingIdentity					CRITICALITY ignore		TYPE UERANPagingIdentity							PRESENCE mandatory}|

{ ID id-PagingDRX							CRITICALITY ignore		TYPE PagingDRX										PRESENCE mandatory}|

{ ID id-RANPagingArea						CRITICALITY reject		TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-PagingPriority						CRITICALITY ignore		TYPE PagingPriority									PRESENCE optional }|

{ ID id-AssistanceDataForRANPaging			CRITICALITY ignore		TYPE AssistanceDataForRANPaging					PRESENCE optional }|

{ ID id-UERadioCapabilityForPaging			CRITICALITY ignore		TYPE UERadioCapabilityForPaging					PRESENCE optional }|

{ ID id-ExtendedUEIdentityIndexValue		CRITICALITY ignore		TYPE ExtendedUEIdentityIndexValue				PRESENCE optional }|

{ ID id-PagingeDRXInformation				CRITICALITY ignore		TYPE PagingeDRXInformation						PRESENCE optional }|

{ ID id-UESpecificDRX						CRITICALITY ignore		TYPE UESpecificDRX									PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT REQUEST

--

-- **************************************************************

RetrieveUEContextRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RetrieveUEContextRequest-IEs}},

...

}

RetrieveUEContextRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-UEContextID								CRITICALITY reject		TYPE UEContextID									PRESENCE mandatory}|

{ ID id-MAC-I									CRITICALITY reject		TYPE MAC-I											PRESENCE mandatory}|

{ ID id-new-NG-RAN-Cell-Identity				CRITICALITY reject		TYPE NG-RAN-Cell-Identity						PRESENCE mandatory}|

{ ID id-RRCResumeCause							CRITICALITY ignore		TYPE RRCResumeCause									PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-GUAMI									CRITICALITY reject		TYPE GUAMI											PRESENCE mandatory}|

{ ID id-UEContextInfoRetrUECtxtResp				CRITICALITY reject		TYPE UEContextInfoRetrUECtxtResp				PRESENCE mandatory}|

{ ID id-TraceActivation							CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-MaskedIMEISV							CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-LocationReportingInformation			CRITICALITY ignore		TYPE LocationReportingInformation				PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized			CRITICALITY ignore	TYPE NRV2XServicesAuthorized		PRESENCE optional}|

{ ID id-LTEV2XServicesAuthorized		CRITICALITY ignore	TYPE LTEV2XServicesAuthorized		PRESENCE optional}|

{ ID id-PC5QoSParameters				CRITICALITY ignore	TYPE PC5QoSParameters			PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE optional}|

{ ID id-UEHistoryInformationFromTheUE	CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-MDTPLMNList								CRITICALITY ignore		TYPE MDTPLMNList									PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT FAILURE

--

-- **************************************************************

RetrieveUEContextFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextFailure-IEs}},

...

}

RetrieveUEContextFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-OldtoNewNG-RANnodeResumeContainer		CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN-U ADDRESS INDICATION

--

-- **************************************************************

XnUAddressIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnUAddressIndication-IEs}},

...

}

XnUAddressIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-XnUAddressInfoperPDUSession-List		CRITICALITY reject		TYPE XnUAddressInfoperPDUSession-List		PRESENCE mandatory}|

{ ID id-CHO-MRDC-Indicator						CRITICALITY reject		TYPE CHO-MRDC-Indicator							PRESENCE optional }|

{ ID id-CHO-MRDC-EarlyDataForwarding			CRITICALITY ignore		TYPE CHO-MRDC-EarlyDataForwarding				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- The IE shall be present if there is at least one  PDUSessionResourceSetupInfo-SNterminated included --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional}|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional}|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional}|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional}|

{ ID id-TraceActivation						CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional}|

{ ID id-UERadioCapabilityID					CRITICALITY reject		TYPE UERadioCapabilityID							PRESENCE optional}|

{ ID id-S-NG-RANnodeUE-Slice-MBR			CRITICALITY reject		TYPE UESliceMaximumBitRateList					PRESENCE optional },

...

}

PDUSessionToBeAddedAddReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionToBeAddedAddReq-Item

PDUSessionToBeAddedAddReq-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

s-NSSAI					S-NSSAI,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate			OPTIONAL,

sn-terminated			PDUSessionResourceSetupInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE

-- nor the PDU Session Resource Setup Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.1.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionToBeAddedAddReq-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeAddedAddReq-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RequestedFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestAcknowledge-IEs}},

...

}

SNodeAdditionRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionAdmittedAddedAddReqAck	CRITICALITY ignore		TYPE PDUSessionAdmittedAddedAddReqAck		PRESENCE mandatory}|

{ ID id-PDUSessionNotAdmittedAddReqAck		CRITICALITY ignore		TYPE PDUSessionNotAdmittedAddReqAck		PRESENCE optional }|

{ ID id-SN-to-MN-Container					CRITICALITY reject		TYPE OCTET STRING								PRESENCE mandatory}|

{ ID id-admittedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes								PRESENCE optional }|

{ ID id-RRCConfigIndication					CRITICALITY reject		TYPE RRCConfigIndication						PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore		TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-LocationInformationSN				CRITICALITY ignore		TYPE Target-CGI									PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo		PRESENCE optional }|

{ ID id-AvailableFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE AvailableFastMCGRecoveryViaSRB3		PRESENCE optional },

...

}

PDUSessionAdmittedAddedAddReqAck ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedAddedAddReqAck-Item

PDUSessionAdmittedAddedAddReqAck-Item ::= SEQUENCE {

pduSessionId							PDUSession-ID,

sn-terminated			PDUSessionResourceSetupResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE

-- nor the PDU Session Resource Setup Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.1.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionNotAdmittedAddReqAck ::= SEQUENCE {

pduSessionResourcesNotAdmitted-SNterminated		PDUSessionResourcesNotAdmitted-List OPTIONAL,

pduSessionResourcesNotAdmitted-MNterminated		PDUSessionResourcesNotAdmitted-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionNotAdmittedAddReqAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionNotAdmittedAddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AvailableFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST REJECT

--

-- **************************************************************

SNodeAdditionRequestReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestReject-IEs}},

...

}

SNodeAdditionRequestReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RECONFIGURATION COMPLETE

--

-- **************************************************************

SNodeReconfigurationComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReconfigurationComplete-IEs}},

...

}

SNodeReconfigurationComplete-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-ResponseInfo-ReconfCompl				CRITICALITY ignore		TYPE ResponseInfo-ReconfCompl					PRESENCE mandatory},

...

}

ResponseInfo-ReconfCompl ::= SEQUENCE {

responseType-ReconfComplete		ResponseType-ReconfComplete,

iE-Extensions						ProtocolExtensionContainer { {ResponseInfo-ReconfCompl-ExtIEs} } OPTIONAL,

...

}

ResponseInfo-ReconfCompl-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResponseType-ReconfComplete ::= CHOICE {

configuration-successfully-applied			Configuration-successfully-applied,

configuration-rejected-by-M-NG-RANNode		Configuration-rejected-by-M-NG-RANNode,

choice-extension					ProtocolIE-Single-Container { {ResponseType-ReconfComplete-ExtIEs} }

}

ResponseType-ReconfComplete-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

Configuration-successfully-applied ::= SEQUENCE {

m-NG-RANNode-to-S-NG-RANNode-Container		OCTET STRING		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {Configuration-successfully-applied-ExtIEs} } OPTIONAL,

...

}

Configuration-successfully-applied-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Configuration-rejected-by-M-NG-RANNode ::= SEQUENCE {

cause											Cause,

m-NG-RANNode-to-S-NG-RANNode-Container		OCTET STRING		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {Configuration-rejected-by-M-NG-RANNode-ExtIEs} } OPTIONAL,

...

}

Configuration-rejected-by-M-NG-RANNode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST

--

-- **************************************************************

SNodeModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequest-IEs}},

...

}

SNodeModificationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDCPChangeIndication					CRITICALITY ignore		TYPE PDCPChangeIndication						PRESENCE optional }|

{ ID id-selectedPLMN							CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList					CRITICALITY ignore		TYPE MobilityRestrictionList					PRESENCE optional }|

{ ID id-SCGConfigurationQuery					CRITICALITY ignore		TYPE SCGConfigurationQuery						PRESENCE optional }|

{ ID id-UEContextInfo-SNModRequest				CRITICALITY reject		TYPE UEContextInfo-SNModRequest				PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-requestedSplitSRB						CRITICALITY ignore		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-requestedSplitSRBrelease				CRITICALITY ignore		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AdditionalDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL			CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL			CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting			CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional}|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-PCellID									CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern						CRITICALITY ignore		TYPE NE-DC-TDM-Pattern							PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3Release	CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3Release	PRESENCE optional }|

{ ID id-SNTriggered						CRITICALITY ignore		TYPE SNTriggered							PRESENCE optional}|

{ ID id-S-NG-RANnodeUE-Slice-MBR				CRITICALITY ignore		TYPE UESliceMaximumBitRateList		PRESENCE optional },

...

}

UEContextInfo-SNModRequest ::= SEQUENCE {

ueSecurityCapabilities							UESecurityCapabilities								OPTIONAL,

s-ng-RANnode-SecurityKey						S-NG-RANnode-SecurityKey							OPTIONAL,

s-ng-RANnodeUE-AMBR								UEAggregateMaximumBitRate							OPTIONAL,

indexToRatFrequencySelectionPriority			RFSP-Index											OPTIONAL,

lowerLayerPresenceStatusChange					LowerLayerPresenceStatusChange						OPTIONAL,

pduSessionResourceToBeAdded						PDUSessionsToBeAdded-SNModRequest-List				OPTIONAL,

pduSessionResourceToBeModified					PDUSessionsToBeModified-SNModRequest-List			OPTIONAL,

pduSessionResourceToBeReleased					PDUSessionsToBeReleased-SNModRequest-List			OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {UEContextInfo-SNModRequest-ExtIEs} }			OPTIONAL,

...

}

UEContextInfo-SNModRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionsToBeAdded-SNModRequest-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionsToBeAdded-SNModRequest-Item

PDUSessionsToBeAdded-SNModRequest-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

s-NSSAI					S-NSSAI,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate			OPTIONAL,

sn-terminated			PDUSessionResourceSetupInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE

-- nor the PDU Session Resource Setup Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionsToBeAdded-SNModRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionsToBeAdded-SNModRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-PDUSessionExpectedUEActivityBehaviour		CRITICALITY ignore	EXTENSION ExpectedUEActivityBehaviour		PRESENCE optional},

...

}

PDUSessionsToBeModified-SNModRequest-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionsToBeModified-SNModRequest-Item

PDUSessionsToBeModified-SNModRequest-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate				OPTIONAL,

sn-terminated			PDUSessionResourceModificationInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModificationInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE

-- nor the PDU Session Resource Modification Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionsToBeModified-SNModRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionsToBeModified-SNModRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-S-NSSAI		CRITICALITY reject	EXTENSION S-NSSAI		PRESENCE optional}|

{ID id-PDUSessionExpectedUEActivityBehaviour		CRITICALITY ignore	EXTENSION ExpectedUEActivityBehaviour		PRESENCE optional},

...

}

PDUSessionsToBeReleased-SNModRequest-List ::= SEQUENCE {

pdu-session-list		PDUSession-List-withCause				OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionsToBeReleased-SNModRequest-List-ExtIEs} }	OPTIONAL,

...

}

PDUSessionsToBeReleased-SNModRequest-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RequestedFastMCGRecoveryViaSRB3Release ::= ENUMERATED {true, ...}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeModificationRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequestAcknowledge-IEs}},

...

}

SNodeModificationRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionAdmitted-SNModResponse		CRITICALITY ignore		TYPE PDUSessionAdmitted-SNModResponse			PRESENCE optional }|

{ ID id-PDUSessionNotAdmitted-SNModResponse		CRITICALITY ignore		TYPE PDUSessionNotAdmitted-SNModResponse		PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY ignore		TYPE OCTET STRING										PRESENCE optional }|

{ ID id-admittedSplitSRB						CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-admittedSplitSRBrelease					CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics							PRESENCE optional }|

{ ID id-LocationInformationSN					CRITICALITY ignore		TYPE Target-CGI											PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo				PRESENCE optional }|

{ ID id-PDUSessionDataForwarding-SNModResponse	CRITICALITY ignore		TYPE PDUSessionDataForwarding-SNModResponse	PRESENCE optional }|

{ ID id-RRCConfigIndication						CRITICALITY reject		TYPE RRCConfigIndication								PRESENCE optional }|

{ ID id-AvailableFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE AvailableFastMCGRecoveryViaSRB3				PRESENCE optional }|

{ ID id-ReleaseFastMCGRecoveryViaSRB3	CRITICALITY ignore		TYPE ReleaseFastMCGRecoveryViaSRB3		PRESENCE optional },

...

}

PDUSessionAdmitted-SNModResponse ::= SEQUENCE {

pduSessionResourcesAdmittedToBeAdded			PDUSessionAdmittedToBeAddedSNModResponse 			OPTIONAL,

pduSessionResourcesAdmittedToBeModified			PDUSessionAdmittedToBeModifiedSNModResponse 		OPTIONAL,

pduSessionResourcesAdmittedToBeReleased			PDUSessionAdmittedToBeReleasedSNModResponse			OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmitted-SNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmitted-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAdmittedToBeAddedSNModResponse ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedToBeAddedSNModResponse-Item

PDUSessionAdmittedToBeAddedSNModResponse-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

sn-terminated			PDUSessionResourceSetupResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE

-- nor the PDU Session Resource Setup Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeAddedSNModResponse-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeAddedSNModResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAdmittedToBeModifiedSNModResponse::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedToBeModifiedSNModResponse-Item

PDUSessionAdmittedToBeModifiedSNModResponse-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

sn-terminated			PDUSessionResourceModificationResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModificationResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Response Info – SN terminated IE

-- nor the PDU Session Resource Modification Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.3.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeModifiedSNModResponse-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeModifiedSNModResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAdmittedToBeReleasedSNModResponse ::= SEQUENCE {

sn-terminated			PDUSession-List-withDataForwardingRequest		OPTIONAL,

mn-terminated			PDUSession-List-withCause						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeReleasedSNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeReleasedSNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionNotAdmitted-SNModResponse ::= SEQUENCE {

pdu-Session-List		PDUSession-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionNotAdmitted-SNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionNotAdmitted-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionDataForwarding-SNModResponse ::= SEQUENCE {

sn-terminated		PDUSession-List-withDataForwardingRequest,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionDataForwarding-SNModResponse-ExtIEs} }	OPTIONAL,

...

}

PDUSessionDataForwarding-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ReleaseFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST REJECT

--

-- **************************************************************

SNodeModificationRequestReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequestReject-IEs}},

...

}

SNodeModificationRequestReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUIRED

--

-- **************************************************************

SNodeModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequired-IEs}},

...

}

SNodeModificationRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDCPChangeIndication					CRITICALITY ignore		TYPE PDCPChangeIndication						PRESENCE optional }|

{ ID id-PDUSessionToBeModifiedSNModRequired		CRITICALITY ignore		TYPE PDUSessionToBeModifiedSNModRequired	PRESENCE optional }|

{ ID id-PDUSessionToBeReleasedSNModRequired		CRITICALITY ignore		TYPE PDUSessionToBeReleasedSNModRequired	PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-SpareDRBIDs								CRITICALITY ignore		TYPE DRB-List										PRESENCE optional }|

{ ID id-RequiredNumberOfDRBIDs					CRITICALITY ignore		TYPE DRB-Number										PRESENCE optional }|

{ ID id-LocationInformationSN					CRITICALITY ignore		TYPE Target-CGI										PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-RRCConfigIndication						CRITICALITY reject		TYPE RRCConfigIndication							PRESENCE optional }|

{ ID id-AvailableFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE AvailableFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-ReleaseFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE ReleaseFastMCGRecoveryViaSRB3				PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore		TYPE SCGIndicator										PRESENCE optional },

...

}

PDUSessionToBeModifiedSNModRequired::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF 	PDUSessionToBeModifiedSNModRequired-Item

PDUSessionToBeModifiedSNModRequired-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

sn-terminated			PDUSessionResourceModRqdInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModRqdInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Required Info – SN terminated IE

-- nor the PDU Session Resource Modification Required Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.4.4 apply.

iE-Extension		ProtocolExtensionContainer { {PDUSessionToBeModifiedSNModRequired-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeModifiedSNModRequired-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionToBeReleasedSNModRequired ::= SEQUENCE {

sn-terminated			PDUSession-List-withDataForwardingRequest		OPTIONAL,

mn-terminated			PDUSession-List-withCause						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionToBeReleasedSNModRequired-ExtIEs} } OPTIONAL,

...

}

PDUSessionToBeReleasedSNModRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION CONFIRM

--

-- **************************************************************

SNodeModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationConfirm-IEs}},

...

}

SNodeModificationConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionAdmittedModSNModConfirm		CRITICALITY ignore		TYPE PDUSessionAdmittedModSNModConfirm		PRESENCE optional }|

{ ID id-PDUSessionReleasedSNModConfirm			CRITICALITY ignore		TYPE PDUSessionReleasedSNModConfirm			PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-AdditionalDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional },

...

}

PDUSessionAdmittedModSNModConfirm ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedModSNModConfirm-Item

PDUSessionAdmittedModSNModConfirm-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sn-terminated			PDUSessionResourceModConfirmInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceModConfirmInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Modification Confirm Info – SN terminated IE

-- nor the PDU Session Resource Modification Confirm Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.4.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedModSNModConfirm-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedModSNModConfirm-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionReleasedSNModConfirm ::= SEQUENCE {

sn-terminated			PDUSession-List-withDataForwardingFromTarget						OPTIONAL,

mn-terminated			PDUSession-List														OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedToBeReleasedSNModConfirm-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedToBeReleasedSNModConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE MODIFICATION REFUSE

--

-- **************************************************************

SNodeModificationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRefuse-IEs}},

...

}

SNodeModificationRefuse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REQUEST

--

-- **************************************************************

SNodeReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequest-IEs}},

...

}

SNodeReleaseRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDUSessionToBeReleased-RelReq			CRITICALITY ignore		TYPE PDUSession-List-withCause					PRESENCE mandatory}|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore		TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-DRBs-transferred-to-MN					CRITICALITY ignore		TYPE DRB-List										PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeReleaseRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequestAcknowledge-IEs}},

...

}

SNodeReleaseRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID						CRITICALITY reject		TYPE NG-RANnodeUEXnAPID									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID						CRITICALITY reject		TYPE NG-RANnodeUEXnAPID									PRESENCE optional }|

{ ID id-PDUSessionToBeReleased-RelReqAck			CRITICALITY ignore		TYPE PDUSessionToBeReleasedList-RelReqAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore		TYPE CriticalityDiagnostics							PRESENCE optional },

...

}

PDUSessionToBeReleasedList-RelReqAck ::= SEQUENCE {

pduSessionsToBeReleasedList-SNterminated		PDUSession-List-withDataForwardingRequest												OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {PDUSessionToBeReleasedList-RelReqAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeReleasedList-RelReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REJECT

--

-- **************************************************************

SNodeReleaseReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseReject-IEs}},

...

}

SNodeReleaseReject-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE REQUIRED

--

-- **************************************************************

SNodeReleaseRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequired-IEs}},

...

}

SNodeReleaseRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionToBeReleasedList-RelRqd		CRITICALITY ignore		TYPE PDUSessionToBeReleasedList-RelRqd		PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-SN-to-MN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional },

...

}

PDUSessionToBeReleasedList-RelRqd ::= SEQUENCE {

pduSessionsToBeReleasedList-SNterminated		PDUSession-List-withDataForwardingRequest						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionToBeReleasedList-RelRqd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeReleasedList-RelRqd-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE RELEASE CONFIRM

--

-- **************************************************************

SNodeReleaseConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseConfirm-IEs}},

...

}

SNodeReleaseConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionReleasedList-RelConf			CRITICALITY ignore		TYPE PDUSessionReleasedList-RelConf			PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

PDUSessionReleasedList-RelConf ::= SEQUENCE {

pduSessionsReleasedList-SNterminated		PDUSession-List-withDataForwardingFromTarget					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionReleasedList-RelConf-ExtIEs} }	OPTIONAL,

...

}

PDUSessionReleasedList-RelConf-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE COUNTER CHECK REQUEST

--

-- **************************************************************

SNodeCounterCheckRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeCounterCheckRequest-IEs}},

...

}

SNodeCounterCheckRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-BearersSubjectToCounterCheck			CRITICALITY ignore		TYPE BearersSubjectToCounterCheck-List		PRESENCE mandatory},

...

}

BearersSubjectToCounterCheck-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF BearersSubjectToCounterCheck-Item

BearersSubjectToCounterCheck-Item ::= SEQUENCE {

drb-ID							DRB-ID,

ul-count						INTEGER (0.. 4294967295),

dl-count						INTEGER (0.. 4294967295),

iE-Extensions					ProtocolExtensionContainer { {BearersSubjectToCounterCheck-Item-ExtIEs} }	OPTIONAL,

...

}

BearersSubjectToCounterCheck-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE CHANGE REQUIRED

--

-- **************************************************************

SNodeChangeRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeRequired-IEs}},

...

}

SNodeChangeRequired-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-target-S-NG-RANnodeID					CRITICALITY reject		TYPE GlobalNG-RANNode-ID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDUSession-SNChangeRequired-List		CRITICALITY ignore		TYPE PDUSession-SNChangeRequired-List		PRESENCE optional }|

{ ID id-SN-to-MN-Container						CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory},

...

}

PDUSession-SNChangeRequired-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSession-SNChangeRequired-Item

PDUSession-SNChangeRequired-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sn-terminated			PDUSessionResourceChangeRequiredInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceChangeRequiredInfo-MNterminated	OPTIONAL,

-- NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present,

-- abnormal conditions as specified in clause 8.3.5.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSession-SNChangeRequired-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-SNChangeRequired-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE CHANGE CONFIRM

--

-- **************************************************************

SNodeChangeConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeConfirm-IEs}},

...

}

SNodeChangeConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSession-SNChangeConfirm-List			CRITICALITY ignore		TYPE PDUSession-SNChangeConfirm-List			PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

PDUSession-SNChangeConfirm-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSession-SNChangeConfirm-Item

PDUSession-SNChangeConfirm-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

sn-terminated			PDUSessionResourceChangeConfirmInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceChangeConfirmInfo-MNterminated	OPTIONAL,

-- NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present,

-- abnormal conditions as specified in clause 8.3.5.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSession-SNChangeConfirm-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-SNChangeConfirm-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- S-NODE CHANGE REFUSE

--

-- **************************************************************

SNodeChangeRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeRefuse-IEs}},

...

}

SNodeChangeRefuse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RRC TRANSFER

--

-- **************************************************************

RRCTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RRCTransfer-IEs}},

...

}

RRCTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-SplitSRB-RRCTransfer					CRITICALITY reject		TYPE SplitSRB-RRCTransfer						PRESENCE optional }|

{ ID id-UEReportRRCTransfer						CRITICALITY reject		TYPE UEReportRRCTransfer							PRESENCE optional }|

{ ID id-FastMCGRecoveryRRCTransfer-SN-to-MN		CRITICALITY ignore		TYPE FastMCGRecoveryRRCTransfer				PRESENCE optional}|

{ ID id-FastMCGRecoveryRRCTransfer-MN-to-SN		CRITICALITY ignore		TYPE FastMCGRecoveryRRCTransfer				PRESENCE optional},

...

}

SplitSRB-RRCTransfer ::= SEQUENCE {

rrcContainer					OCTET STRING						OPTIONAL,

srbType							ENUMERATED {srb1, srb2, ...},

deliveryStatus					DeliveryStatus						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {SplitSRB-RRCTransfer-ExtIEs} } OPTIONAL,

...

}

SplitSRB-RRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEReportRRCTransfer::= SEQUENCE {

rrcContainer					OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { {UEReportRRCTransfer-ExtIEs} } OPTIONAL,

...

}

UEReportRRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FastMCGRecoveryRRCTransfer::= SEQUENCE {

rrcContainer					OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { { FastMCGRecoveryRRCTransfer-ExtIEs} } OPTIONAL,

...

}

FastMCGRecoveryRRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- NOTIFICATION CONTROL INDICATION

--

-- **************************************************************

NotificationControlIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{NotificationControlIndication-IEs}},

...

}

NotificationControlIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionResourcesNotifyList		CRITICALITY reject		TYPE PDUSessionResourcesNotifyList			PRESENCE optional },

...

}

PDUSessionResourcesNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesNotify-Item

PDUSessionResourcesNotify-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

qosFlowsNotificationContrIndInfo	QoSFlowNotificationControlIndicationInfo,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourcesNotify-Item-ExtIEs} } OPTIONAL,

...

}

PDUSessionResourcesNotify-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- ACTIVITY NOTIFICATION

--

-- **************************************************************

ActivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ActivityNotification-IEs}},

...

}

ActivityNotification-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-UserPlaneTrafficActivityReport			CRITICALITY ignore		TYPE UserPlaneTrafficActivityReport			PRESENCE optional }|

{ ID id-PDUSessionResourcesActivityNotifyList	CRITICALITY ignore		TYPE PDUSessionResourcesActivityNotifyList	PRESENCE optional }|

{ ID id-RANPagingFailure						CRITICALITY ignore		TYPE RANPagingFailure								PRESENCE optional },

...

}

PDUSessionResourcesActivityNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesActivityNotify-Item

PDUSessionResourcesActivityNotify-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

pduSessionLevelUPactivityreport		UserPlaneTrafficActivityReport											OPTIONAL,

qosFlowsActivityNotifyList			QoSFlowsActivityNotifyList												OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourcesActivityNotify-Item-ExtIEs} } OPTIONAL,

...

}

PDUSessionResourcesActivityNotify-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsActivityNotifyList ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsActivityNotifyItem

QoSFlowsActivityNotifyItem ::= SEQUENCE {

qosFlowIdentifier					QoSFlowIdentifier,

pduSessionLevelUPactivityreport		UserPlaneTrafficActivityReport,

iE-Extensions						ProtocolExtensionContainer { {QoSFlowsActivityNotifyItem-ExtIEs} } OPTIONAL,

...

}

QoSFlowsActivityNotifyItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- XN SETUP REQUEST

--

-- **************************************************************

XnSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupRequest-IEs}},

...

}

XnSetupRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID					CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-TAISupport-list						CRITICALITY reject	TYPE TAISupport-List							PRESENCE mandatory}|

{ ID id-AMF-Region-Information				CRITICALITY reject	TYPE AMF-Region-Information						PRESENCE mandatory}|

{ ID id-List-of-served-cells-NR				CRITICALITY reject	TYPE ServedCells-NR								PRESENCE optional }|

{ ID id-List-of-served-cells-E-UTRA			CRITICALITY reject	TYPE ServedCells-E-UTRA							PRESENCE optional }|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject	TYPE InterfaceInstanceIndication				PRESENCE optional }|

{ ID id-TNLConfigurationInfo				CRITICALITY ignore	TYPE TNLConfigurationInfo						PRESENCE optional}|

{ ID id-PartialListIndicator-NR				CRITICALITY ignore	TYPE PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-NR 	CRITICALITY ignore 	TYPE CellAndCapacityAssistanceInfo-NR		PRESENCE optional }|

{ ID id-PartialListIndicator-EUTRA			CRITICALITY ignore	TYPE PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-EUTRA	CRITICALITY ignore 	TYPE CellAndCapacityAssistanceInfo-EUTRA	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN SETUP RESPONSE

--

-- **************************************************************

XnSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupResponse-IEs}},

...

}

XnSetupResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID				CRITICALITY reject	TYPE GlobalNG-RANNode-ID						PRESENCE mandatory}|

{ ID id-TAISupport-list						CRITICALITY reject	TYPE TAISupport-List							PRESENCE mandatory}|

{ ID id-List-of-served-cells-NR				CRITICALITY reject	TYPE ServedCells-NR								PRESENCE optional }|

{ ID id-List-of-served-cells-E-UTRA			CRITICALITY reject	TYPE ServedCells-E-UTRA							PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-AMF-Region-Information				CRITICALITY reject	TYPE AMF-Region-Information						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject	TYPE InterfaceInstanceIndication				PRESENCE optional }|

{ ID id-TNLConfigurationInfo				CRITICALITY ignore	TYPE TNLConfigurationInfo						PRESENCE optional	}|

{ ID id-PartialListIndicator-NR				CRITICALITY ignore	TYPE PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-NR 	CRITICALITY ignore 	TYPE CellAndCapacityAssistanceInfo-NR		PRESENCE optional }|

{ ID id-PartialListIndicator-EUTRA			CRITICALITY ignore	TYPE PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-EUTRA	CRITICALITY ignore 	TYPE CellAndCapacityAssistanceInfo-EUTRA	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN SETUP FAILURE

--

-- **************************************************************

XnSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupFailure-IEs}},

...

}

XnSetupFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait						PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional }|

{ ID id-MessageOversizeNotification	CRITICALITY ignore	TYPE MessageOversizeNotification	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE

--

-- **************************************************************

NGRANNodeConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdate-IEs}},

...

}

NGRANNodeConfigurationUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TAISupport-list							CRITICALITY reject	TYPE TAISupport-List									PRESENCE optional }|

{ ID id-ConfigurationUpdateInitiatingNodeChoice	CRITICALITY ignore	TYPE ConfigurationUpdateInitiatingNodeChoice	PRESENCE mandatory}|

{ ID id-TNLA-To-Add-List						CRITICALITY ignore	TYPE TNLA-To-Add-List									PRESENCE optional }|

{ ID id-TNLA-To-Remove-List						CRITICALITY ignore	TYPE TNLA-To-Remove-List								PRESENCE optional }|

{ ID id-TNLA-To-Update-List						CRITICALITY ignore	TYPE TNLA-To-Update-List								PRESENCE optional }|

{ ID id-GlobalNG-RAN-node-ID					CRITICALITY reject	TYPE GlobalNG-RANNode-ID								PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Add			CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Delete		CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication					PRESENCE optional }|

{ ID id-TNLConfigurationInfo					CRITICALITY ignore	TYPE TNLConfigurationInfo			   				PRESENCE optional	},

...

}

ConfigurationUpdateInitiatingNodeChoice ::= CHOICE {

gNB									ProtocolIE-Container	{ {ConfigurationUpdate-gNB} },

ng-eNB								ProtocolIE-Container	{ {ConfigurationUpdate-ng-eNB} },

choice-extension					ProtocolIE-Single-Container { {ServedCellsToUpdateInitiatingNodeChoice-ExtIEs} }

}

ServedCellsToUpdateInitiatingNodeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ConfigurationUpdate-gNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-NR			CRITICALITY ignore TYPE	ServedCellsToUpdate-NR					PRESENCE optional }|

{ ID id-cellAssistanceInfo-NR			CRITICALITY ignore TYPE	CellAssistanceInfo-NR					PRESENCE optional }|

{ ID id-cellAssistanceInfo-EUTRA			CRITICALITY ignore TYPE	CellAssistanceInfo-EUTRA					PRESENCE optional },

...

}

ConfigurationUpdate-ng-eNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-E-UTRA	CRITICALITY ignore TYPE	ServedCellsToUpdate-E-UTRA				PRESENCE optional }|

{ ID id-cellAssistanceInfo-NR		CRITICALITY ignore TYPE	CellAssistanceInfo-NR			PRESENCE optional }|

{ ID id-cellAssistanceInfo-EUTRA			CRITICALITY ignore TYPE	CellAssistanceInfo-EUTRA					PRESENCE optional },

...

}

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

NGRANNodeConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdateAcknowledge-IEs}},

...

}

NGRANNodeConfigurationUpdateAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeTypeConfigUpdateAck	CRITICALITY ignore	TYPE RespondingNodeTypeConfigUpdateAck		PRESENCE mandatory}|

{ ID id-TNLA-Setup-List						CRITICALITY ignore	TYPE TNLA-Setup-List							PRESENCE optional }|

{ ID id-TNLA-Failed-To-Setup-List			CRITICALITY ignore	TYPE TNLA-Failed-To-Setup-List				PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject	TYPE InterfaceInstanceIndication				PRESENCE optional }|

{ ID id-TNLConfigurationInfo				CRITICALITY ignore	TYPE TNLConfigurationInfo						PRESENCE optional	},

...

}

RespondingNodeTypeConfigUpdateAck ::= CHOICE {

ng-eNB					RespondingNodeTypeConfigUpdateAck-ng-eNB,

gNB						RespondingNodeTypeConfigUpdateAck-gNB,

choice-extension		ProtocolIE-Single-Container { {RespondingNodeTypeConfigUpdateAck-ExtIEs} }

}

RespondingNodeTypeConfigUpdateAck-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RespondingNodeTypeConfigUpdateAck-ng-eNB ::= SEQUENCE {

iE-Extension		ProtocolExtensionContainer { {RespondingNodeTypeConfigUpdateAck-ng-eNB-ExtIEs} }	OPTIONAL,

...

}

RespondingNodeTypeConfigUpdateAck-ng-eNB-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-List-of-served-cells-E-UTRA				CRITICALITY ignore	EXTENSION ServedCells-E-UTRA							PRESENCE optional }|

{ ID id-PartialListIndicator-EUTRA				CRITICALITY ignore	EXTENSION PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-EUTRA		CRITICALITY ignore	EXTENSION CellAndCapacityAssistanceInfo-EUTRA	PRESENCE optional },

...

}

RespondingNodeTypeConfigUpdateAck-gNB ::= SEQUENCE {

served-NR-Cells		ServedCells-NR																OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {RespondingNodeTypeConfigUpdateAck-gNB-ExtIEs} }	OPTIONAL,

...

}

RespondingNodeTypeConfigUpdateAck-gNB-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PartialListIndicator-NR				CRITICALITY ignore	EXTENSION PartialListIndicator						PRESENCE optional }|

{ ID id-CellAndCapacityAssistanceInfo-NR 	CRITICALITY ignore 	EXTENSION CellAndCapacityAssistanceInfo-NR		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

NGRANNodeConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{NGRANNodeConfigurationUpdateFailure-IEs}},

...

}

NGRANNodeConfigurationUpdateFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait						PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- E-UTRA NR CELL RESOURCE COORDINATION REQUEST

--

-- **************************************************************

E-UTRA-NR-CellResourceCoordinationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{E-UTRA-NR-CellResourceCoordinationRequest-IEs}},

...

}

E-UTRA-NR-CellResourceCoordinationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-initiatingNodeType-ResourceCoordRequest	CRITICALITY reject	TYPE InitiatingNodeType-ResourceCoordRequest		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication						PRESENCE optional },

...

}

InitiatingNodeType-ResourceCoordRequest ::= CHOICE {

ng-eNB								ResourceCoordRequest-ng-eNB-initiated,

gNB									ResourceCoordRequest-gNB-initiated,

choice-extension					ProtocolIE-Single-Container { {InitiatingNodeType-ResourceCoordRequest-ExtIEs} }

}

InitiatingNodeType-ResourceCoordRequest-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResourceCoordRequest-ng-eNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI							OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordRequest-ng-eNB-initiated-ExtIEs} }	OPTIONAL,

...

}

ResourceCoordRequest-ng-eNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordRequest-gNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI						OPTIONAL,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofNRCells						SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF NR-CGI								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordRequest-gNB-initiated-ExtIEs} }	OPTIONAL,

...

}

ResourceCoordRequest-gNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- E-UTRA NR CELL RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

E-UTRA-NR-CellResourceCoordinationResponse::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{E-UTRA-NR-CellResourceCoordinationResponse-IEs}},

...

}

E-UTRA-NR-CellResourceCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-respondingNodeType-ResourceCoordResponse  CRITICALITY reject  	TYPE RespondingNodeType-ResourceCoordResponse 	PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication						PRESENCE optional },

...

}

RespondingNodeType-ResourceCoordResponse ::= CHOICE {

ng-eNB								ResourceCoordResponse-ng-eNB-initiated,

gNB									ResourceCoordResponse-gNB-initiated,

choice-extension					ProtocolIE-Single-Container { {RespondingNodeType-ResourceCoordResponse-ExtIEs} }

}

RespondingNodeType-ResourceCoordResponse-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResourceCoordResponse-ng-eNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofE-UTRACells					SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordResponse-ng-eNB-initiated-ExtIEs} }		OPTIONAL,

...

}

ResourceCoordResponse-ng-eNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordResponse-gNB-initiated ::= SEQUENCE {

dataTrafficResourceIndication		DataTrafficResourceIndication,

spectrumSharingGroupID				SpectrumSharingGroupID,

listofNRCells						SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF NR-CGI									OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ResourceCoordResponse-gNB-initiated-ExtIEs} }		OPTIONAL,

...

}

ResourceCoordResponse-gNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SECONDARY RAT DATA USAGE REPORT

--

-- **************************************************************

SecondaryRATDataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SecondaryRATDataUsageReport-IEs}},

...

}

SecondaryRATDataUsageReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionResourceSecondaryRATUsageList	CRITICALITY reject		TYPE PDUSessionResourceSecondaryRATUsageList	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- XN REMOVAL REQUEST

--

-- **************************************************************

XnRemovalRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnRemovalRequest-IEs}},

...

}

XnRemovalRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-XnRemovalThreshold			CRITICALITY reject	TYPE XnBenefitValue					PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN REMOVAL RESPONSE

--

-- **************************************************************

XnRemovalResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnRemovalResponse-IEs}},

...

}

XnRemovalResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- XN REMOVAL FAILURE

--

-- **************************************************************

XnRemovalFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnRemovalFailure-IEs}},

...

}

XnRemovalFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION REQUEST

--

-- **************************************************************

CellActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ CellActivationRequest-IEs}},

...

}

CellActivationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ServedCellsToActivate					CRITICALITY reject		TYPE ServedCellsToActivate						PRESENCE mandatory}|

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

ServedCellsToActivate ::= CHOICE {

nr-cells							SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,

e-utra-cells						SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,

choice-extension					ProtocolIE-Single-Container { {ServedCellsToActivate-ExtIEs} }

}

ServedCellsToActivate-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION RESPONSE

--

-- **************************************************************

CellActivationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{CellActivationResponse-IEs}},

...

}

CellActivationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ActivatedServedCells					CRITICALITY reject		TYPE ActivatedServedCells						PRESENCE mandatory}|

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

ActivatedServedCells ::= CHOICE {

nr-cells							SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,

e-utra-cells						SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,

choice-extension					ProtocolIE-Single-Container { {ActivatedServedCells-ExtIEs} }

}

ActivatedServedCells-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION FAILURE

--

-- **************************************************************

CellActivationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{CellActivationFailure-IEs}},

...

}

CellActivationFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RESET REQUEST

--

-- **************************************************************

ResetRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResetRequest-IEs}},

...

}

ResetRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ResetRequestTypeInfo					CRITICALITY reject		TYPE ResetRequestTypeInfo						PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RESET RESPONSE

--

-- **************************************************************

ResetResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResetResponse-IEs}},

...

}

ResetResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ResetResponseTypeInfo					CRITICALITY reject		TYPE ResetResponseTypeInfo						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject		TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

-- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs XNAP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional },

...

}

-- **************************************************************

--

#### -- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-NG-RANTraceID							CRITICALITY ignore	TYPE NG-RANTraceID								PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- FAILURE INDICATION

--

-- **************************************************************

FailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{FailureIndication-IEs}},

...

}

FailureIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-InitiatingCondition-FailureIndication				CRITICALITY reject		TYPE InitiatingCondition-FailureIndication							PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- HANDOVER REPORT

--

-- **************************************************************

HandoverReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ HandoverReport-IEs}},

...

}

HandoverReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-HandoverReportType			CRITICALITY ignore		TYPE HandoverReportType				PRESENCE mandatory}|

{ ID id-HandoverCause				CRITICALITY ignore		TYPE Cause							PRESENCE mandatory}|

{ ID id-SourceCellCGI				CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE mandatory }|

{ ID id-TargetCellCGI            	CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE mandatory }|

{ ID id-ReEstablishmentCellCGI   	CRITICALITY ignore		TYPE GlobalCell-ID					PRESENCE conditional }|

-- This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell"

{ ID id-TargetCellinEUTRAN   		CRITICALITY ignore		TYPE TargetCellinEUTRAN				PRESENCE conditional }|

-- This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong"

{ ID id-SourceCellCRNTI   			CRITICALITY ignore		TYPE C-RNTI							PRESENCE optional }|

{ ID id-MobilityInformation   		CRITICALITY ignore		TYPE MobilityInformation			PRESENCE optional }|

{ ID id-UERLFReportContainer   		CRITICALITY ignore		TYPE UERLFReportContainer			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS REQUEST

--

-- **************************************************************

ResourceStatusRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusRequest-IEs}},

...

}

ResourceStatusRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID			CRITICALITY ignore	TYPE Measurement-ID					PRESENCE conditional}|

-- This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add".

{ ID id-RegistrationRequest					CRITICALITY reject	TYPE RegistrationRequest			PRESENCE mandatory}|

{ ID id-ReportCharacteristics				CRITICALITY reject	TYPE ReportCharacteristics			PRESENCE conditional}|

-- This IE shall be present if the Registration Request IE is set to the value "start".

{ ID id-CellToReport						CRITICALITY ignore	TYPE CellToReport					PRESENCE optional}|

{ ID id-ReportingPeriodicity				CRITICALITY ignore	TYPE ReportingPeriodicity			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS RESPONSE

--

-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusResponse-IEs}},

...

}

ResourceStatusResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID							PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS FAILURE

--

-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusFailure-IEs}},

...

}

ResourceStatusFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID				CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID				CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS UPDATE

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusUpdate-IEs}},

...

}

ResourceStatusUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-CellMeasurementResult			CRITICALITY ignore	TYPE CellMeasurementResult			PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE REQUEST

--

-- **************************************************************

MobilityChangeRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeRequest-IEs}},

...

}

MobilityChangeRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnode1CellID						CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode2CellID							CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode1MobilityParameters				CRITICALITY reject	TYPE MobilityParametersInformation				PRESENCE optional}|

{ ID id-NG-RANnode2ProposedMobilityParameters	CRITICALITY reject	TYPE MobilityParametersInformation				PRESENCE optional}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause											PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE ACKNOWLEDGE

--

-- **************************************************************

MobilityChangeAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeAcknowledge-IEs}},

...

}

MobilityChangeAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnode1CellID					CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode2CellID					CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE FAILURE

--

-- **************************************************************

MobilityChangeFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeFailure-IEs}},

...

}

MobilityChangeFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnode1CellID						CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-NG-RANnode2CellID							CRITICALITY reject	TYPE GlobalNG-RANCell-ID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-MobilityParametersModificationRange 	CRITICALITY reject	TYPE MobilityParametersModificationRange		PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ACCESS AND MOBILITY INDICATION

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ AccessAndMobilityIndication-IEs}},

...

}

AccessAndMobilityIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RACHReportInformation			CRITICALITY ignore		TYPE RACHReportInformation			PRESENCE optional},

...

}

END

-- ASN1STOP

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-IntendedTDD-DL-ULConfiguration-NR,

id-MaxIPrate-DL,

id-SecurityResult,

id-OldQoSFlowMap-ULendmarkerexpected,

id-PDUSessionCommonNetworkInstance,

id-BPLMN-ID-Info-EUTRA,

id-BPLMN-ID-Info-NR,

id-DRBsNotAdmittedSetupModifyList,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-ULForwardingProposal,

id-DRB-IDs-takenintouse,

id-SplitSessionIndicator,

id-NonGBRResources-Offered,

id-MDT-Configuration,

id-TraceCollectionEntityURI,

id-NPN-Broadcast-Information,

id-NPNPagingAssistanceInformation,

id-NPNMobilityInformation,

id-NPN-Support,

id-LTEUESidelinkAggregateMaximumBitRate,

id-NRUESidelinkAggregateMaximumBitRate,

id-ExtendedRATRestrictionInformation,

id-QoSMonitoringRequest,

id-QoSMonitoringDisabled,

id-QosMonitoringReportingFrequency,

id-DAPSRequestInfo,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-NBIoT-UL-DL-AlignmentOffset,

id-TDDULDLConfigurationCommonNR,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRCellPRACHConfig,

id-Redundant-UL-NG-U-TNLatUPF,

id-Redundant-DL-NG-U-TNLatNG-RAN,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-Additional-Redundant-UL-NG-U-TNLatUPF-List,

id-RedundantCommonNetworkInstance,

id-TSCTrafficCharacteristics,

id-RedundantQoSFlowIndicator,

id-Additional-PDCP-Duplication-TNL-List,

id-RedundantPDUSessionInformation,

id-UsedRSNInformation,

id-RLCDuplicationInformation,

id-CSI-RSTransmissionIndication,

id-UERadioCapabilityID,

id-secondary-SN-UL-PDCP-UP-TNLInfo,

id-pdcpDuplicationConfiguration,

id-duplicationActivation,

id-NPRACHConfiguration,

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-QoS-Mapping-Information,

id-AdditionLocationInformation,

id-dataForwardingInfoFromTargetE-UTRANnode,

id-Cause,

id-UESliceMaximumBitRateList,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEPLMNsplus1,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

maxnoofForbiddenTACs,

maxnoofMBSFNEUTRA,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighbours,

maxnoofNIDs,

maxnoofNRCellBands,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofProtectedResourcePatterns,

maxnoofQoSFlows,

maxnoofQoSParaSets,

maxnoofRANAreaCodes,

maxnoofRANAreasinRNA,

maxnoofSCellGroups,

maxnoofSCellGroupsplus1,

maxnoofSliceItems,

maxnoofExtSliceItems,

maxnoofSNPNIDs,

maxnoofsupportedTACs,

maxnoofsupportedPLMNs,

maxnoofTAI,

maxnoofTAIsinAoI,

maxnoofTNLAssociations,

maxnoofUEContexts,

maxNRARFCN,

maxNrOfErrors,

maxnoofRANNodesinAoI,

maxnooftimeperiods,

maxnoofslots,

maxnoofExtTLAs,

maxnoofGTPTLAs,

maxnoofCHOcells,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofRACHReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationstate,

maxnoofBluetoothName,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofTAforMDT,

maxnoofWLANName,

maxnoofSensorName,

maxnoofNeighPCIforMDT,

maxnoofFreqforMDT,

maxnoofNonAnchorCarrierFreqConfig,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofSMBR

FROM XnAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM XnAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES

FROM XnAP-Containers;

#### -- A

AdditionLocationInformation ::= ENUMERATED {

includePSCell,

...

}

Additional-PDCP-Duplication-TNL-List ::= SEQUENCE (SIZE(1..maxnoofAdditionalPDCPDuplicationTNL)) OF Additional-PDCP-Duplication-TNL-Item

Additional-PDCP-Duplication-TNL-Item ::= SEQUENCE {
	additional-PDCP-Duplication-UP-TNL-Information	UPTransportLayerInformation,
	iE-Extensions		ProtocolExtensionContainer { { Additional-PDCP-Duplication-TNL-ExtIEs} } 	OPTIONAL,
	...
}

Additional-PDCP-Duplication-TNL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
	...
}

Additional-UL-NG-U-TNLatUPF-Item ::= SEQUENCE {

additional-UL-NG-U-TNLatUPF				UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { { Additional-UL-NG-U-TNLatUPF-Item-ExtIEs} }	OPTIONAL,

...

}

Additional-UL-NG-U-TNLatUPF-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional},

...

}

Additional-UL-NG-U-TNLatUPF-List ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF Additional-UL-NG-U-TNLatUPF-Item

ActivationIDforCellActivation	::= INTEGER (0..255)

AllocationandRetentionPriority ::= SEQUENCE {

priorityLevel					INTEGER (0..15,...),

pre-emption-capability			ENUMERATED {shall-not-trigger-preemptdatDion, may-trigger-preemption, ...},

pre-emption-vulnerability		ENUMERATED {not-preemptable, preemptable, ...},

iE-Extensions					ProtocolExtensionContainer { {AllocationandRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationandRetentionPriority-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ActivationSFN ::= INTEGER (0..1023)

AllowedCAG-ID-List-perPLMN ::= SEQUENCE (SIZE(1..maxnoofCAGsperPLMN)) OF CAG-Identifier

AllowedPNI-NPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofEPLMNsplus1)) OF AllowedPNI-NPN-ID-Item

AllowedPNI-NPN-ID-Item ::= SEQUENCE {

plmn-id								PLMN-Identity,

pni-npn-restricted-information		PNI-NPN-Restricted-Information,

allowed-CAG-id-list-per-plmn		AllowedCAG-ID-List-perPLMN,

iE-Extensions						ProtocolExtensionContainer { {AllowedPNI-NPN-ID-Item-ExtIEs} } OPTIONAL,

...

}

AllowedPNI-NPN-ID-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			QoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-Region-Information ::= SEQUENCE (SIZE (1..maxnoofAMFRegions)) OF GlobalAMF-Region-Information

GlobalAMF-Region-Information ::= SEQUENCE {

plmn-ID				PLMN-Identity,

amf-region-id		BIT STRING (SIZE (8)),

iE-Extensions					ProtocolExtensionContainer { {GlobalAMF-Region-Information-ExtIEs} } OPTIONAL,

...

}

GlobalAMF-Region-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-UE-NGAP-ID ::= INTEGER (0..1099511627775)

AreaOfInterestInformation ::= SEQUENCE (SIZE(1..maxnoofAoIs)) OF AreaOfInterest-Item

AreaOfInterest-Item ::= SEQUENCE {

listOfTAIsinAoI					ListOfTAIsinAoI												OPTIONAL,

listOfCellsinAoI				ListOfCells													OPTIONAL,

listOfRANNodesinAoI				ListOfRANNodesinAoI											OPTIONAL,

requestReferenceID	RequestReferenceID,

iE-Extensions					ProtocolExtensionContainer { {AreaOfInterest-Item-ExtIEs} } OPTIONAL,

...

}

AreaOfInterest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

tAIBased					TAIBasedMDT,

...

}

AreaScopeOfMDT-EUTRA ::= CHOICE {

cellBased					CellBasedMDT-EUTRA,

tABased						TABasedMDT,

tAIBased					TAIBasedMDT,

...

}

AreaScopeOfNeighCellsList ::= SEQUENCE (SIZE(1..maxnoofFreqforMDT)) OF AreaScopeOfNeighCellsItem

AreaScopeOfNeighCellsItem ::= SEQUENCE {

nrFrequencyInfo				NRFrequencyInfo,

pciListForMDT				PCIListForMDT		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AreaScopeOfNeighCellsItem-ExtIEs} }	OPTIONAL,

...

}

AreaScopeOfNeighCellsItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AS-SecurityInformation ::= SEQUENCE {

key-NG-RAN-Star					BIT STRING (SIZE(256)),

ncc								INTEGER (0..7),

iE-Extensions					ProtocolExtensionContainer { {AS-SecurityInformation-ExtIEs} } OPTIONAL,

...

}

AS-SecurityInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForRANPaging ::= SEQUENCE {

ran-paging-attempt-info			RANPagingAttemptInfo	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {AssistanceDataForRANPaging-ExtIEs} } OPTIONAL,

...

}

AssistanceDataForRANPaging-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-NPNPagingAssistanceInformation	CRITICALITY ignore	EXTENSION NPNPagingAssistanceInformation	PRESENCE optional },

...

}

AvailableCapacity ::= INTEGER (1.. 100,...)

AvailableRRCConnectionCapacityValue ::= INTEGER (0..100)

AveragingWindow ::= INTEGER (0..4095, ...)

#### -- B

BluetoothMeasurementConfiguration ::= SEQUENCE {

bluetoothMeasConfig             BluetoothMeasConfig,

bluetoothMeasConfigNameList		BluetoothMeasConfigNameList     OPTIONAL,

bt-rssi                         ENUMERATED {true, ...}          OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { BluetoothMeasurementConfiguration-ExtIEs } } OPTIONAL,

...

}

BluetoothMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofBluetoothName)) OF BluetoothName

BluetoothMeasConfig::= ENUMERATED {setup,...}

BluetoothName ::= OCTET STRING (SIZE (1..248))

BPLMN-ID-Info-EUTRA ::= SEQUENCE (SIZE(1..maxnoofEUTRABPLMNs)) OF BPLMN-ID-Info-EUTRA-Item

BPLMN-ID-Info-EUTRA-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastEUTRAPLMNs,

tac								TAC,

e-utraCI						E-UTRA-Cell-Identity,

ranac							RANAC OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-EUTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BPLMN-ID-Info-NR ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BPLMN-ID-Info-NR-Item

BPLMN-ID-Info-NR-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastPLMNs,

tac								TAC,

nr-CI							NR-Cell-Identity,

ranac							RANAC OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-NR-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{ ID id-NPN-Broadcast-Information	CRITICALITY reject	EXTENSION NPN-Broadcast-Information		PRESENCE optional },

...

}

BitRate	::= INTEGER (0..4000000000000,...)

BroadcastCAG-Identifier-List ::= SEQUENCE (SIZE(1..maxnoofCAGs)) OF BroadcastCAG-Identifier-Item

BroadcastCAG-Identifier-Item ::= SEQUENCE {

cag-Identifier					CAG-Identifier,

iE-Extension					ProtocolExtensionContainer { {BroadcastCAG-Identifier-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastCAG-Identifier-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastNID-List ::= SEQUENCE (SIZE(1..maxnoofNIDs)) OF BroadcastNID-Item

BroadcastNID-Item ::= SEQUENCE {

nid							NID,

iE-Extension				ProtocolExtensionContainer { {BroadcastNID-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastNID-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastPLMNs ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF PLMN-Identity

BroadcastEUTRAPLMNs ::= SEQUENCE (SIZE(1..maxnoofEUTRABPLMNs)) OF PLMN-Identity

BroadcastPLMNinTAISupport-Item ::= SEQUENCE {

plmn-id							PLMN-Identity,

tAISliceSupport-List			SliceSupport-List,

iE-Extension					ProtocolExtensionContainer { {BroadcastPLMNinTAISupport-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastPLMNinTAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-NPN-Support						CRITICALITY reject	EXTENSION NPN-Support						PRESENCE optional}|

{ ID id-ExtendedTAISliceSupportList		CRITICALITY reject	EXTENSION ExtendedSliceSupportList		PRESENCE optional},

...

}

BroadcastPNI-NPN-ID-Information ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BroadcastPNI-NPN-ID-Information-Item

BroadcastPNI-NPN-ID-Information-Item ::= SEQUENCE {

plmn-id							PLMN-Identity,

broadcastCAG-Identifier-List	BroadcastCAG-Identifier-List,

iE-Extension					ProtocolExtensionContainer { {BroadcastPNI-NPN-ID-Information-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastPNI-NPN-ID-Information-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastSNPNID-List ::= SEQUENCE (SIZE(1..maxnoofSNPNIDs)) OF BroadcastSNPNID

BroadcastSNPNID ::= SEQUENCE {

plmn-id							PLMN-Identity,

broadcastNID-List								BroadcastNID-List,

iE-Extension					ProtocolExtensionContainer { {BroadcastSNPNID-ExtIEs} } OPTIONAL,

...

}

BroadcastSNPNID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- C

CAG-Identifier	::= BIT STRING (SIZE (32))

CapacityValue ::= INTEGER (0..100)

CapacityValueInfo ::= SEQUENCE {

capacityValue				CapacityValue,

ssbAreaCapacityValueList 	SSBAreaCapacityValue-List 	OPTIONAL,

iE-Extension 				ProtocolExtensionContainer { {CapacityValueInfo-ExtIEs} } OPTIONAL,

...

}

CapacityValueInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetworkLayer,

transport			CauseTransportLayer,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-Single-Container { {Cause-ExtIEs} }

}

Cause-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CauseRadioNetworkLayer ::= ENUMERATED {

cell-not-available,

handover-desirable-for-radio-reasons,

handover-target-not-allowed,

invalid-AMF-Set-ID,

no-radio-resources-available-in-target-cell,

partial-handover,

reduce-load-in-serving-cell,

resource-optimisation-handover,

time-critical-handover,

tXnRELOCoverall-expiry,

tXnRELOCprep-expiry,

unknown-GUAMI-ID,

unknown-local-NG-RAN-node-UE-XnAP-ID,

inconsistent-remote-NG-RAN-node-UE-XnAP-ID,

encryption-and-or-integrity-protection-algorithms-not-supported,

protection-algorithms-not-supported,

multiple-PDU-session-ID-instances,

unknown-PDU-session-ID,

unknown-QoS-Flow-ID,

multiple-QoS-Flow-ID-instances,

switch-off-ongoing,

not-supported-5QI-value,

tXnDCoverall-expiry,

tXnDCprep-expiry,

action-desirable-for-radio-reasons,

reduce-load,

resource-optimisation,

time-critical-action,

target-not-allowed,

no-radio-resources-available,

invalid-QoS-combination,

encryption-algorithms-not-supported,

procedure-cancelled,

rRM-purpose,

improve-user-bit-rate,

user-inactivity,

radio-connection-with-UE-lost,

failure-in-the-radio-interface-procedure,

bearer-option-not-supported,

up-integrity-protection-not-possible,

up-confidentiality-protection-not-possible,

resources-not-available-for-the-slice-s,

ue-max-IP-data-rate-reason,

cP-integrity-protection-failure,

uP-integrity-protection-failure,

slice-not-supported-by-NG-RAN,

mN-Mobility,

sN-Mobility,

count-reaches-max-value,

unknown-old-NG-RAN-node-UE-XnAP-ID,

pDCP-Overload,

drb-id-not-available,

unspecified,

...,

ue-context-id-not-known,

non-relocation-of-context,

cho-cpc-resources-tobechanged,

rSN-not-available-for-the-UP,

npn-access-denied,

report-characteristics-empty,

existing-measurement-ID,

measurement-temporarily-not-available,

measurement-not-supported-for-the-object,

ue-power-saving,

unknown-NG-RAN-node2-Measurement-ID,

insufficient-ue-capabilities,

normal-release

}

CauseTransportLayer ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

hardware-failure,

o-and-M-intervention,

not-enough-user-plane-processing-resources,

unspecified,

...

}

CellAssistanceInfo-NR	::= CHOICE {

limitedNR-List				SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,

full-List					ENUMERATED {all-served-cells-NR, ...},

choice-extension			ProtocolIE-Single-Container { {CellAssistanceInfo-NR-ExtIEs} }

}

CellAssistanceInfo-NR-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CellAndCapacityAssistanceInfo-NR	::= SEQUENCE {

maximumCellListSize					MaximumCellListSize								OPTIONAL,

cellAssistanceInfo-NR		CellAssistanceInfo-NR 				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellAndCapacityAssistanceInfo-NR-ExtIEs} }	OPTIONAL,

...

}

CellAndCapacityAssistanceInfo-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellAndCapacityAssistanceInfo-EUTRA	::= SEQUENCE {

maximumCellListSize					MaximumCellListSize								OPTIONAL,

cellAssistanceInfo-EUTRA			CellAssistanceInfo-EUTRA 				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellAndCapacityAssistanceInfo-EUTRA-ExtIEs} }	OPTIONAL,

...

}

CellAndCapacityAssistanceInfo-EUTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellAssistanceInfo-EUTRA	::= CHOICE {

limitedEUTRA-List			SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,

full-List					ENUMERATED {all-served-cells-E-UTRA, ...},

choice-extension			ProtocolIE-Single-Container { {CellAssistanceInfo-EUTRA-ExtIEs} }

}

CellAssistanceInfo-EUTRA-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CellBasedMDT-NR::= SEQUENCE {

cellIdListforMDT-NR	CellIdListforMDT-NR,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-NR-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforMDT-NR ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF NR-CGI

CellBasedMDT-EUTRA::= SEQUENCE {

cellIdListforMDT-EUTRA	CellIdListforMDT-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-EUTRA-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-EUTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforMDT-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF E-UTRA-CGI

CellCapacityClassValue ::= INTEGER (1..100,...)

CellGroupID ::= INTEGER (0..maxnoofSCellGroups)

CellMeasurementResult ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellMeasurementResult-Item

CellMeasurementResult-Item	::= SEQUENCE {

cell-ID								GlobalNG-RANCell-ID,

radioResourceStatus					RadioResourceStatus              OPTIONAL,

tNLCapacityIndicator					TNLCapacityIndicator          	 OPTIONAL,

compositeAvailableCapacityGroup  	CompositeAvailableCapacityGroup  OPTIONAL,

sliceAvailableCapacity          	 	SliceAvailableCapacity           OPTIONAL,

numberofActiveUEs                	NumberofActiveUEs                OPTIONAL,

rRCConnections                   	RRCConnections                   OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellMeasurementResult-Item-ExtIEs} }	OPTIONAL,

...

}

CellMeasurementResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CellToReport ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellToReport-Item

CellToReport-Item	::= SEQUENCE {

cell-ID									GlobalNG-RANCell-ID,

sSBToReport-List                        SSBToReport-List			OPTIONAL,

sliceToReport-List                      SliceToReport-List			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellToReport-Item-ExtIEs} }	OPTIONAL,

...

}

CellToReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Cell-Type-Choice ::= CHOICE {

ng-ran-e-utra			E-UTRA-Cell-Identity,

ng-ran-nr				NR-Cell-Identity,

e-utran					E-UTRA-Cell-Identity,

choice-extension		ProtocolIE-Single-Container { { Cell-Type-Choice-ExtIEs} }

}

Cell-Type-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

CompositeAvailableCapacityGroup ::= SEQUENCE {

compositeAvailableCapacityDownlink		CompositeAvailableCapacity,

compositeAvailableCapacityUplink	 	CompositeAvailableCapacity,

iE-Extensions				ProtocolExtensionContainer { { CompositeAvailableCapacityGroup-ExtIEs} }	OPTIONAL,

...

}

CompositeAvailableCapacityGroup-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue	CellCapacityClassValue             OPTIONAL,

capacityValueInfo	     	CapacityValueInfo, -- this IE represents the IE "CapacityValue" in 9.2.2.a, it’s used to distinguish the "CapacityValue"  in 9.2.2.c

iE-Extensions				ProtocolExtensionContainer { { CompositeAvailableCapacity-ExtIEs} }OPTIONAL,

...

}

CompositeAvailableCapacity-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CHO-MRDC-EarlyDataForwarding ::= ENUMERATED {stop, ...}

CHO-MRDC-Indicator ::= ENUMERATED {true, ...}

CHOtrigger ::= ENUMERATED {

cho-initiation,

cho-replace,

...

}

CHOinformation-Req ::= SEQUENCE {

cho-trigger						CHOtrigger,

targetNG-RANnodeUEXnAPID		NG-RANnodeUEXnAPID										OPTIONAL

-- This IE shall be present if the cho-trigger IE is present and set to "CHO-replace" --,

cHO-EstimatedArrivalProbability	CHO-Probability											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-Req-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-Req-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHOinformation-Ack ::= SEQUENCE {

requestedTargetCellGlobalID		Target-CGI,

maxCHOoperations				MaxCHOpreparations											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-Ack-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-Ack-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CHO-Probability ::= INTEGER (1..100)

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

Connectivity-Support		::= SEQUENCE {

eNDC-Support			ENUMERATED {supported, not-supported, ...},

iE-Extensions			ProtocolExtensionContainer { {Connectivity-Support-ExtIEs} }	OPTIONAL,

...

}

Connectivity-Support-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

COUNT-PDCP-SN12 ::= SEQUENCE {

pdcp-SN12						INTEGER (0..4095),

hfn-PDCP-SN12					INTEGER (0..1048575),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN12-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN12-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

COUNT-PDCP-SN18 ::= SEQUENCE {

pdcp-SN18						INTEGER (0..262143),

hfn-PDCP-SN18					INTEGER (0..16383),

iE-Extensions					ProtocolExtensionContainer { {COUNT-PDCP-SN18-ExtIEs} }	OPTIONAL,

...

}

COUNT-PDCP-SN18-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPTransportLayerInformation ::= CHOICE {

endpointIPAddress			TransportLayerAddress,

choice-extension			ProtocolIE-Single-Container { {CPTransportLayerInformation-ExtIEs} }

}

CPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

{ ID id-EndpointIPAddressAndPort		CRITICALITY reject	TYPE EndpointIPAddressAndPort		PRESENCE mandatory},

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

C-RNTI ::= BIT STRING (SIZE(16))

CyclicPrefix-E-UTRA-DL ::= ENUMERATED {

normal,

extended,

...

}

CyclicPrefix-E-UTRA-UL ::= ENUMERATED {

normal,

extended,

...

}

CSI-RSTransmissionIndication ::= ENUMERATED {

activated,

deactivated,

...

}

#### -- D

XnUAddressInfoperPDUSession-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF XnUAddressInfoperPDUSession-Item

XnUAddressInfoperPDUSession-Item ::= SEQUENCE {

pduSession-ID			PDUSession-ID,

dataForwardingInfoFromTargetNGRANnode		DataForwardingInfoFromTargetNGRANnode										OPTIONAL,

pduSessionResourceSetupCompleteInfo-SNterm			PDUSessionResourceBearerSetupCompleteInfo-SNterminated		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { XnUAddressInfoperPDUSession-Item-ExtIEs} }					OPTIONAL,

...

}

XnUAddressInfoperPDUSession-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondarydataForwardingInfoFromTarget-List	CRITICALITY ignore	EXTENSION SecondarydataForwardingInfoFromTarget-List	PRESENCE optional}|

{ ID id-DRB-IDs-takenintouse						CRITICALITY reject	EXTENSION DRB-List												PRESENCE optional}|

{ ID id-dataForwardingInfoFromTargetE-UTRANnode		CRITICALITY ignore	EXTENSION DataForwardingInfoFromTargetE-UTRANnode		PRESENCE optional},

...

}

DataForwardingAccepted	::= ENUMERATED {data-forwarding-accepted, ...}

DataForwardingInfoFromTargetE-UTRANnode ::= SEQUENCE {

dataForwardingInfoFromTargetE-UTRANnode-List			DataForwardingInfoFromTargetE-UTRANnode-List,

iE-Extension		ProtocolExtensionContainer { { DataForwardingInfoFromTargetE-UTRANnode-ExtIEs} }	OPTIONAL,

...

}

DataForwardingInfoFromTargetE-UTRANnode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataForwardingInfoFromTargetE-UTRANnode-List ::= SEQUENCE (SIZE(1.. maxnoofDataForwardingTunneltoE-UTRAN)) OF DataForwardingInfoFromTargetE-UTRANnode-Item

DataForwardingInfoFromTargetE-UTRANnode-Item ::= SEQUENCE {

dlForwardingUPTNLInformation	UPTransportLayerInformation,

qosFlowsToBeForwarded-List	QoSFlowsToBeForwarded-List,

iE-Extension		ProtocolExtensionContainer { { DataForwardingInfoFromTargetE-UTRANnode-Item-ExtIEs} }	OPTIONAL,

...

}

DataForwardingInfoFromTargetE-UTRANnode-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsToBeForwarded-List ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeForwarded-Item

QoSFlowsToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

iE-Extension		ProtocolExtensionContainer { { QoSFlowsToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataForwardingInfoFromTargetNGRANnode ::= SEQUENCE {

qosFlowsAcceptedForDataForwarding-List			QoSFLowsAcceptedToBeForwarded-List,

pduSessionLevelDLDataForwardingInfo				UPTransportLayerInformation								OPTIONAL,

pduSessionLevelULDataForwardingInfo				UPTransportLayerInformation								OPTIONAL,

dataForwardingResponseDRBItemList				DataForwardingResponseDRBItemList						OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataForwardingInfoFromTargetNGRANnode-ExtIEs} }	OPTIONAL,

...

}

DataForwardingInfoFromTargetNGRANnode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFLowsAcceptedToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsAcceptedToBeForwarded-Item

QoSFLowsAcceptedToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

iE-Extension				ProtocolExtensionContainer { {QoSFLowsAcceptedToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFLowsAcceptedToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataforwardingandOffloadingInfofromSource ::= SEQUENCE {

qosFlowsToBeForwarded			QoSFLowsToBeForwarded-List,

sourceDRBtoQoSFlowMapping		DRBToQoSFlowMapping-List													OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataforwardingandOffloadingInfofromSource-ExtIEs} }	OPTIONAL,

...

}

DataforwardingandOffloadingInfofromSource-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFLowsToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsToBeForwarded-Item

QoSFLowsToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

dl-dataforwarding			DLForwarding,

ul-dataforwarding			ULForwarding,

iE-Extension		ProtocolExtensionContainer { {QoSFLowsToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFLowsToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ULForwardingProposal	CRITICALITY ignore	EXTENSION ULForwardingProposal	PRESENCE optional },

...

}

DataForwardingResponseDRBItemList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DataForwardingResponseDRBItem

DataForwardingResponseDRBItem ::= SEQUENCE {

drb-ID				DRB-ID,

dlForwardingUPTNL	UPTransportLayerInformation												OPTIONAL,

ulForwardingUPTNL	UPTransportLayerInformation												OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataForwardingResponseDRBItem-ExtIEs} }	OPTIONAL,

...

}

DataForwardingResponseDRBItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataTrafficResources ::= BIT STRING (SIZE(6..17600))

DataTrafficResourceIndication ::= SEQUENCE {

activationSFN					ActivationSFN,

sharedResourceType				SharedResourceType,

reservedSubframePattern			ReservedSubframePattern										OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DataTrafficResourceIndication-ExtIEs} }	OPTIONAL,

...

}

DataTrafficResourceIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DAPSRequestInfo ::= SEQUENCE {

dapsIndicator				ENUMERATED {daps-HO-required, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSRequestInfo-ExtIEs} } OPTIONAL,

...

}

DAPSRequestInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DAPSResponseInfo-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DAPSResponseInfo-Item

DAPSResponseInfo-Item ::= SEQUENCE {

drbID						DRB-ID,

dapsResponseIndicator		ENUMERATED {daps-HO-accepted, daps-HO-not-accepted, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSResponseInfo-Item-ExtIEs} } OPTIONAL,

...

}

DAPSResponseInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DeliveryStatus	::= INTEGER (0..4095, ...)

DesiredActNotificationLevel	::= ENUMERATED {none, qos-flow, pdu-session, ue-level, ...}

DefaultDRB-Allowed ::= ENUMERATED {true, false, ...}

DLCountChoice ::= CHOICE {

count12bits				COUNT-PDCP-SN12,

count18bits				COUNT-PDCP-SN18,

choice-extension		ProtocolIE-Single-Container { {DLCountChoice-ExtIEs} }

}

DLCountChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

DLForwarding	::= ENUMERATED {dl-forwarding-proposed, ...}

DL-GBR-PRB-usage::= INTEGER (0..100)

DL-non-GBR-PRB-usage::= INTEGER (0..100)

DL-Total-PRB-usage::= INTEGER (0..100)

DRB-ID	::= INTEGER (1..32, ...)

DRB-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRB-ID

DRB-List-withCause ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRB-List-withCause-Item

DRB-List-withCause-Item ::= SEQUENCE {

drb-id		DRB-ID,

cause		Cause,

rLC-Mode	RLCMode								OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRB-List-withCause-Item-ExtIEs} }	OPTIONAL,

...

}

DRB-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Number ::= INTEGER (1..32, ...)

DRBsSubjectToDLDiscarding-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToDLDiscarding-Item

DRBsSubjectToDLDiscarding-Item ::= SEQUENCE {

drbID				DRB-ID,

dlCount				DLCountChoice,

iE-Extension		ProtocolExtensionContainer { { DRBsSubjectToDLDiscarding-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToDLDiscarding-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsSubjectToEarlyStatusTransfer-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToEarlyStatusTransfer-Item

DRBsSubjectToEarlyStatusTransfer-Item ::= SEQUENCE {

drbID				DRB-ID,

dlCount				DLCountChoice,

iE-Extension		ProtocolExtensionContainer { { DRBsSubjectToEarlyStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToEarlyStatusTransfer-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsSubjectToStatusTransfer-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToStatusTransfer-Item

DRBsSubjectToStatusTransfer-Item ::= SEQUENCE {

drbID				DRB-ID,

pdcpStatusTransfer-UL	DRBBStatusTransferChoice,

pdcpStatusTransfer-DL	DRBBStatusTransferChoice,

iE-Extension		ProtocolExtensionContainer { {DRBsSubjectToStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToStatusTransfer-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-OldQoSFlowMap-ULendmarkerexpected	CRITICALITY reject		EXTENSION QoSFlows-List					PRESENCE optional },

...

}

DRBBStatusTransferChoice ::= CHOICE {

pdcp-sn-12bits		DRBBStatusTransfer12bitsSN,

pdcp-sn-18bits		DRBBStatusTransfer18bitsSN,

choice-extension		ProtocolIE-Single-Container { {DRBBStatusTransferChoice-ExtIEs} }

}

DRBBStatusTransferChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

DRBBStatusTransfer12bitsSN ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..2048))											OPTIONAL,

cOUNTValue				COUNT-PDCP-SN12,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer12bitsSN-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer12bitsSN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBBStatusTransfer18bitsSN ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..131072))										OPTIONAL,

cOUNTValue				COUNT-PDCP-SN18,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer18bitsSN-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer18bitsSN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBToQoSFlowMapping-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBToQoSFlowMapping-Item

DRBToQoSFlowMapping-Item ::= SEQUENCE {

drb-ID							DRB-ID,

qosFlows-List					QoSFlows-List,

rLC-Mode						RLCMode								OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRBToQoSFlowMapping-Item-ExtIEs} }	OPTIONAL,

...

}

DRBToQoSFlowMapping-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DAPSRequestInfo		CRITICALITY ignore		EXTENSION DAPSRequestInfo		PRESENCE optional  },

...

}

DuplicationActivation ::= ENUMERATED {active, inactive, ...}

Dynamic5QIDescriptor ::= SEQUENCE {

priorityLevelQoS			PriorityLevelQoS,

packetDelayBudget			PacketDelayBudget,

packetErrorRate				PacketErrorRate,

fiveQI						FiveQI													OPTIONAL,

delayCritical				ENUMERATED {delay-critical, non-delay-critical, ...}	OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

averagingWindow				AveragingWindow											OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

maximumDataBurstVolume		MaximumDataBurstVolume									OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {Dynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

...

}

Dynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedPacketDelayBudget		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional}|

{ ID id-CNPacketDelayBudgetDownlink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional}|

{ ID id-CNPacketDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional},

...

}

#### -- E

E-RAB-ID		::= INTEGER (0..15, ...)

E-UTRAARFCN ::= INTEGER (0..maxEARFCN)

E-UTRA-Cell-Identity			::= BIT STRING (SIZE(28))

E-UTRA-CGI ::= SEQUENCE {

plmn-id				PLMN-Identity,

e-utra-CI			E-UTRA-Cell-Identity,

iE-Extension		ProtocolExtensionContainer { {E-UTRA-CGI-ExtIEs} } 	OPTIONAL,

...

}

E-UTRA-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRAFrequencyBandIndicator ::= INTEGER (1..256, ...)

E-UTRAMultibandInfoList ::= SEQUENCE (SIZE(1..maxnoofEUTRABands)) OF E-UTRAFrequencyBandIndicator

E-UTRAPCI ::= INTEGER (0..503, ...)

E-UTRAPRACHConfiguration ::= SEQUENCE {

rootSequenceIndex						INTEGER (0..837),

zeroCorrelationIndex					INTEGER (0..15),

highSpeedFlag							ENUMERATED {true, false, ...},

prach-FreqOffset						INTEGER (0..94),

prach-ConfigIndex						INTEGER (0..63)								OPTIONAL,

-- C-ifTDD: This IE shall be present if the EUTRA-Mode-Info IE in the Served Cell Information IE is set to the value "TDD" --

iE-Extensions							ProtocolExtensionContainer { {E-UTRAPRACHConfiguration-ExtIEs} }	OPTIONAL,

...

}

E-UTRAPRACHConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRATransmissionBandwidth ::= ENUMERATED {bw6, bw15, bw25, bw50, bw75, bw100, ..., bw1}

EndpointIPAddressAndPort ::=SEQUENCE {

endpointIPAddress 				TransportLayerAddress,

portNumber						PortNumber,

iE-Extensions					ProtocolExtensionContainer { { EndpointIPAddressAndPort-ExtIEs} } OPTIONAL

}

EndpointIPAddressAndPort-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

EventTriggered ::= SEQUENCE {

loggedEventTriggeredConfig			LoggedEventTriggeredConfig,

iE-Extensions		ProtocolExtensionContainer { { EventTriggered-ExtIEs} } OPTIONAL,

...

}

EventTriggered-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

EventType ::= ENUMERATED {

report-upon-change-of-serving-cell,

report-UE-moving-presence-into-or-out-of-the-Area-of-Interest,

...,

report-upon-change-of-serving-cell-and-Area-of-Interest

}

EventTypeTrigger ::= CHOICE {

outOfCoverage					ENUMERATED {true, ...},

eventL1		EventL1,

choice-Extensions		ProtocolIE-Single-Container { {EventTypeTrigger-ExtIEs} }

}

EventTypeTrigger-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

EventL1 ::= SEQUENCE {

l1Threshold					MeasurementThresholdL1LoggedMDT,

hysteresis					Hysteresis,

timeToTrigger				TimeToTrigger,

iE-Extensions		ProtocolExtensionContainer { { EventL1-ExtIEs} } OPTIONAL,

...

}

EventL1-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementThresholdL1LoggedMDT ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod						ExpectedActivityPeriod						OPTIONAL,

expectedIdlePeriod							ExpectedIdlePeriod							OPTIONAL,

sourceOfUEActivityBehaviourInformation		SourceOfUEActivityBehaviourInformation		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEActivityBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEBehaviour	::= SEQUENCE {

expectedUEActivityBehaviour		ExpectedUEActivityBehaviour 					OPTIONAL,

expectedHOInterval				ExpectedHOInterval		 						OPTIONAL,

expectedUEMobility				ExpectedUEMobility								OPTIONAL,

expectedUEMovingTrajectory		ExpectedUEMovingTrajectory						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEMobility ::= ENUMERATED {

stationary,

mobile,

...

}

ExpectedUEMovingTrajectory ::= SEQUENCE (SIZE(1..maxnoofCellsUEMovingTrajectory)) OF ExpectedUEMovingTrajectoryItem

ExpectedUEMovingTrajectoryItem ::= SEQUENCE {

nGRAN-CGI				GlobalNG-RANCell-ID,

timeStayedInCell		INTEGER (0..4095)													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEMovingTrajectoryItem-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEMovingTrajectoryItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

ExtendedRATRestrictionInformation ::= SEQUENCE {

primaryRATRestriction		BIT STRING (SIZE(8, ...)),

secondaryRATRestriction		BIT STRING (SIZE(8, ...)),

iE-Extensions		ProtocolExtensionContainer { {ExtendedRATRestrictionInformation-ExtIEs} }	OPTIONAL,

...

}

ExtendedRATRestrictionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedPacketDelayBudget ::= INTEGER (0..65535, ...)

ExtendedSliceSupportList	::= SEQUENCE (SIZE(1..maxnoofExtSliceItems)) OF S-NSSAI

ExtendedUEIdentityIndexValue ::= BIT STRING (SIZE(16))

ExtTLAs ::= SEQUENCE (SIZE(1..maxnoofExtTLAs)) OF ExtTLA-Item

ExtTLA-Item ::= SEQUENCE {

iPsecTLA							TransportLayerAddress		OPTIONAL,

gTPTransportLayerAddresses			GTPTLAs							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExtTLA-Item-ExtIEs} } OPTIONAL,

...

}

ExtTLA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GTPTLAs	::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF	GTPTLA-Item

GTPTLA-Item	::= SEQUENCE {

gTPTransportLayerAddresses				TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } }         OPTIONAL,

...

}

GTPTLA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- F

FiveGCMobilityRestrictionListContainer ::= OCTET STRING

-- This octets of the OCTET STRING contain the Mobility Restriction List IE as specified in TS 38.413 [5]. --

FiveQI ::= INTEGER (0..255, ...)

FrequencyShift7p5khz ::= ENUMERATED {false, true, ...}

#### -- G

GBRQoSFlowInfo ::= SEQUENCE {

maxFlowBitRateDL			BitRate,

maxFlowBitRateUL			BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			ENUMERATED {notification-requested, ...}				OPTIONAL,

maxPacketLossRateDL			PacketLossRate											OPTIONAL,

maxPacketLossRateUL			PacketLossRate											OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GBRQoSFlowInfo-ExtIEs} }	OPTIONAL,

...

}

GBRQoSFlowInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	},

...

}

GlobalgNB-ID	::= SEQUENCE {

plmn-id			PLMN-Identity,

gnb-id			GNB-ID-Choice,

iE-Extensions		ProtocolExtensionContainer { {GlobalgNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalgNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GNB-ID-Choice ::= CHOICE {

gnb-ID					BIT STRING (SIZE(22..32)),

choice-extension		ProtocolIE-Single-Container { {GNB-ID-Choice-ExtIEs} }

}

GNB-ID-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

GNB-RadioResourceStatus	::= SEQUENCE {

ssbAreaRadioResourceStatus-List				SSBAreaRadioResourceStatus-List,

iE-Extensions							ProtocolExtensionContainer { { GNB-RadioResourceStatus-ExtIEs} }	OPTIONAL,

...

}

GNB-RadioResourceStatus-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalCell-ID	::= SEQUENCE {

plmn-id				PLMN-Identity,

cell-type			Cell-Type-Choice,

iE-Extensions		ProtocolExtensionContainer { { GlobalCell-ID-ExtIEs} } OPTIONAL,

...

}

GlobalCell-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalngeNB-ID	::= SEQUENCE {

plmn-id			PLMN-Identity,

enb-id			ENB-ID-Choice,

iE-Extensions		ProtocolExtensionContainer { {GlobaleNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobaleNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID-Choice ::= CHOICE {

enb-ID-macro			BIT STRING (SIZE(20)),

enb-ID-shortmacro		BIT STRING (SIZE(18)),

enb-ID-longmacro		BIT STRING (SIZE(21)),

choice-extension		ProtocolIE-Single-Container { {ENB-ID-Choice-ExtIEs} }

}

ENB-ID-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

GlobalNG-RANCell-ID	::= SEQUENCE {

plmn-id					PLMN-Identity,

ng-RAN-Cell-id			NG-RAN-Cell-Identity,

iE-Extensions		ProtocolExtensionContainer { {GlobalNG-RANCell-ID-ExtIEs} } OPTIONAL,

...

}

GlobalNG-RANCell-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalNG-RANNode-ID ::= CHOICE {

gNB						GlobalgNB-ID,

ng-eNB					GlobalngeNB-ID,

choice-extension		ProtocolIE-Single-Container { {GlobalNG-RANNode-ID-ExtIEs} }

}

GlobalNG-RANNode-ID-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

GTP-TEID	::= OCTET STRING (SIZE(4))

GTPtunnelTransportLayerInformation ::= SEQUENCE {

tnl-address			TransportLayerAddress,

gtp-teid			GTP-TEID,

iE-Extensions		ProtocolExtensionContainer { {GTPtunnelTransportLayerInformation-ExtIEs} } OPTIONAL,

...

}

GTPtunnelTransportLayerInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GUAMI ::= SEQUENCE {

plmn-ID				PLMN-Identity,

amf-region-id		BIT STRING (SIZE (8)),

amf-set-id			BIT STRING (SIZE (10)),

amf-pointer			BIT STRING (SIZE (6)),

iE-Extensions		ProtocolExtensionContainer { {GUAMI-ExtIEs} } OPTIONAL,

...

}

GUAMI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- H

HandoverReportType ::= ENUMERATED {

hoTooEarly,

hoToWrongCell,

intersystempingpong,

...

}

Hysteresis ::=                       INTEGER (0..30)

#### -- I

IABNodeIndication ::= ENUMERATED {true,...}

ImmediateMDT-EUTRA ::= OCTET STRING

ImmediateMDT-NR ::= SEQUENCE {

measurementsToActivate		MeasurementsToActivate,

m1Configuration				M1Configuration				OPTIONAL,

m4Configuration				M4Configuration				OPTIONAL,

m5Configuration				M5Configuration				OPTIONAL,

mDT-Location-Info			MDT-Location-Info			OPTIONAL,

m6Configuration				M6Configuration				OPTIONAL,

m7Configuration				M7Configuration				OPTIONAL,

bluetoothMeasurementConfiguration				BluetoothMeasurementConfiguration				OPTIONAL,

wLANMeasurementConfiguration					WLANMeasurementConfiguration					OPTIONAL,

sensorMeasurementConfiguration					SensorMeasurementConfiguration					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ImmediateMDT-NR-ExtIEs} } OPTIONAL,

...

}

ImmediateMDT-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

InitiatingCondition-FailureIndication ::= CHOICE {

rRCReestab					RRCReestab-initiated,

rRCSetup					RRCSetup-initiated,

choice-extension				ProtocolIE-Single-Container { {InitiatingCondition-FailureIndication-ExtIEs} }

}

InitiatingCondition-FailureIndication-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

IntendedTDD-DL-ULConfiguration-NR ::= SEQUENCE {

nrscs							NRSCS,

nrCyclicPrefix					NRCyclicPrefix,

nrDL-ULTransmissionPeriodicity	NRDL-ULTransmissionPeriodicity,

slotConfiguration-List			SlotConfiguration-List,

iE-Extensions					ProtocolExtensionContainer { {IntendedTDD-DL-ULConfiguration-NR-ExtIEs} }	OPTIONAL,

...

}

IntendedTDD-DL-ULConfiguration-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

InterfaceInstanceIndication ::= INTEGER (0..255, ...)

InterfacesToTrace ::= BIT STRING { ng-c (0), x-nc (1), uu (2), f1-c (3), e1 (4)} (SIZE(8))

I-RNTI ::= CHOICE {

i-RNTI-full			BIT STRING (SIZE(40)),

i-RNTI-short		BIT STRING (SIZE(24)),

choice-extension	ProtocolIE-Single-Container { {I-RNTI-ExtIEs} }

}

I-RNTI-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

#### -- J

#### -- K

#### -- L

LastVisitedCell-Item ::= CHOICE {

nG-RAN-Cell						LastVisitedNGRANCellInformation,

e-UTRAN-Cell					LastVisitedEUTRANCellInformation,

uTRAN-Cell						LastVisitedUTRANCellInformation,

gERAN-Cell						LastVisitedGERANCellInformation,

choice-extension				ProtocolIE-Single-Container { { LastVisitedCell-Item-ExtIEs} }

}

LastVisitedCell-Item-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation	::= OCTET STRING

LastVisitedNGRANCellInformation	::= OCTET STRING

LastVisitedUTRANCellInformation	::= OCTET STRING

LCID ::= INTEGER (1..32, ...)

Links-to-log ::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

ListOfCells ::= SEQUENCE (SIZE(1..maxnoofCellsinAoI)) OF CellsinAoI-Item

CellsinAoI-Item ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

ng-ran-cell-id			NG-RAN-Cell-Identity,

iE-Extensions			ProtocolExtensionContainer { {CellsinAoI-Item-ExtIEs} } OPTIONAL,

...

}

CellsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfRANNodesinAoI ::= SEQUENCE (SIZE(1.. maxnoofRANNodesinAoI)) OF GlobalNG-RANNodesinAoI-Item

GlobalNG-RANNodesinAoI-Item ::= SEQUENCE {

global-NG-RAN-Node-ID		GlobalNG-RANNode-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalNG-RANNodesinAoI-Item-ExtIEs} } OPTIONAL,

...

}

GlobalNG-RANNodesinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfTAIsinAoI ::= SEQUENCE (SIZE(1..maxnoofTAIsinAoI)) OF TAIsinAoI-Item

TAIsinAoI-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

tAC					TAC,

iE-Extensions		ProtocolExtensionContainer { {TAIsinAoI-Item-ExtIEs} } OPTIONAL,

...

}

TAIsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LocationInformationSNReporting ::= ENUMERATED {

pSCell,

...

}

LocationReportingInformation ::= SEQUENCE {

eventType			EventType,

reportArea			ReportArea,

areaOfInterest		AreaOfInterestInformation			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LocationReportingInformation-ExtIEs} } OPTIONAL,

...

}

LocationReportingInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-AdditionLocationInformation	CRITICALITY ignore	EXTENSION AdditionLocationInformation	PRESENCE optional},

...

}

LoggedMDT-EUTRA ::= SEQUENCE {

loggingInterval								LoggingInterval,

loggingDuration								LoggingDuration,

bluetoothMeasurementConfiguration			BluetoothMeasurementConfiguration		OPTIONAL,

wLANMeasurementConfiguration				WLANMeasurementConfiguration			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {LoggedMDT-EUTRA-ExtIEs} } OPTIONAL,

...

}

LoggedMDT-EUTRA-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

LoggedEventTriggeredConfig ::= SEQUENCE {

eventTypeTrigger					EventTypeTrigger,

iE-Extensions		ProtocolExtensionContainer { { LoggedEventTriggeredConfig-ExtIEs} } OPTIONAL,

...

}

LoggedEventTriggeredConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LoggedMDT-NR ::= SEQUENCE {

loggingInterval						LoggingInterval,

loggingDuration						LoggingDuration,

reportType							ReportType,

bluetoothMeasurementConfiguration	BluetoothMeasurementConfiguration			OPTIONAL,

wLANMeasurementConfiguration		WLANMeasurementConfiguration				OPTIONAL,

sensorMeasurementConfiguration		SensorMeasurementConfiguration				OPTIONAL,

areaScopeOfNeighCellsList			AreaScopeOfNeighCellsList					OPTIONAL,	iE-Extensions						ProtocolExtensionContainer { {LoggedMDT-NR-ExtIEs} } OPTIONAL,

...

}

LoggedMDT-NR-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

LoggingInterval ::= ENUMERATED { ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960, ms61440}

LoggingDuration ::= ENUMERATED {m10, m20, m40, m60, m90, m120}

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

...,

suspend-lower-layers,

resume-lower-layers

}

LTEV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE											OPTIONAL,

pedestrianUE 		PedestrianUE										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTEV2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

LTEV2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTEUESidelinkAggregateMaximumBitRate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {LTEUESidelinkAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

LTEUESidelinkAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- M

M1Configuration ::= SEQUENCE {

m1reportingTrigger			M1ReportingTrigger,

m1thresholdeventA2			M1ThresholdEventA2				OPTIONAL,

-- Included in case of event-triggered, or event-triggered periodic reporting for measurement M1

m1periodicReporting			M1PeriodicReporting				OPTIONAL,

-- Included in case of periodic or event-triggered periodic reporting

iE-Extensions				ProtocolExtensionContainer { { M1Configuration-ExtIEs} } OPTIONAL,

...

}

M1Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M1PeriodicReporting ::= SEQUENCE {

reportInterval				ReportIntervalMDT,

reportAmount				ReportAmountMDT,

iE-Extensions				ProtocolExtensionContainer { { M1PeriodicReporting-ExtIEs} } OPTIONAL,

...

}

M1PeriodicReporting-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M1ReportingTrigger ::= ENUMERATED{

periodic,

a2eventtriggered,

a2eventtriggered-periodic,

...

}

M1ThresholdEventA2 ::= SEQUENCE {

measurementThreshold	MeasurementThresholdA2,

iE-Extensions			ProtocolExtensionContainer { { M1ThresholdEventA2-ExtIEs} } OPTIONAL,

...

}

M1ThresholdEventA2-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M4Configuration ::= SEQUENCE {

m4period			M4period,

m4-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M4Configuration-ExtIEs} } OPTIONAL,

...

}

M4Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M4period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M5period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M6report-Interval ::= ENUMERATED { ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30,... }

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

M7period ::= INTEGER(1..60, ...)

MAC-I ::= BIT STRING (SIZE(16))

MaskedIMEISV	::= BIT STRING (SIZE(64))

MaxCHOpreparations ::= INTEGER (1..8, ...)

MaximumDataBurstVolume ::= INTEGER (0..4095, ..., 4096.. 2000000)

MaximumIPdatarate ::= SEQUENCE {

maxIPrate-UL			MaxIPrate,

iE-Extensions		ProtocolExtensionContainer { {MaximumIPdatarate-ExtIEs} }	OPTIONAL,

...

}

MaximumIPdatarate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaxIPrate-DL	CRITICALITY ignore	EXTENSION MaxIPrate	PRESENCE optional},

...

}

MaxIPrate ::= ENUMERATED {

bitrate64kbs,

max-UErate,

...

}

MBSFNControlRegionLength ::= INTEGER (0..3)

MBSFNSubframeAllocation-E-UTRA ::= CHOICE {

oneframe				BIT STRING (SIZE(6)),

fourframes				BIT STRING (SIZE(24)),

choice-extension		ProtocolIE-Single-Container { {MBSFNSubframeAllocation-E-UTRA-ExtIEs} }

}

MBSFNSubframeAllocation-E-UTRA-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

MBSFNSubframeInfo-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofMBSFNEUTRA)) OF MBSFNSubframeInfo-E-UTRA-Item

MBSFNSubframeInfo-E-UTRA-Item ::= SEQUENCE {

radioframeAllocationPeriod		ENUMERATED{n1,n2,n4,n8,n16,n32,...},

radioframeAllocationOffset		INTEGER (0..7, ...),

subframeAllocation				MBSFNSubframeAllocation-E-UTRA,

iE-Extensions					ProtocolExtensionContainer { {MBSFNSubframeInfo-E-UTRA-Item-ExtIEs} } OPTIONAL,

...

}

MBSFNSubframeInfo-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

MDT-Activation 	::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

logged-MDT-only,

...

}

MDT-Configuration ::= SEQUENCE {

mDT-Configuration-NR		MDT-Configuration-NR		 OPTIONAL,

mDT-Configuration-EUTRA		MDT-Configuration-EUTRA		 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Configuration-NR ::= SEQUENCE {

mdt-Activation				MDT-Activation,

areaScopeOfMDT-NR			AreaScopeOfMDT-NR	OPTIONAL,

mDTMode-NR					MDTMode-NR,

signallingBasedMDTPLMNList	MDTPLMNList,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-NR-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Configuration-EUTRA ::= SEQUENCE {

mdt-Activation				MDT-Activation,

areaScopeOfMDT-EUTRA		AreaScopeOfMDT-EUTRA	OPTIONAL,

mDTMode-EUTRA				MDTMode-EUTRA,

signallingBasedMDTPLMNList	MDTPLMNList,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-EUTRA-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-EUTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Location-Info ::= BIT STRING (SIZE (8))

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMN-Identity

MDTMode-NR ::= CHOICE {

immediateMDT				ImmediateMDT-NR,

loggedMDT					LoggedMDT-NR,

...,

mDTMode-NR-Extension			MDTMode-NR-Extension

}

MDTMode-NR-Extension ::= ProtocolIE-Single-Container {{ MDTMode-NR-ExtensionIE }}

MDTMode-NR-ExtensionIE XNAP-PROTOCOL-IES ::= {

...

}

MDTMode-EUTRA ::= CHOICE {

immediateMDT				ImmediateMDT-EUTRA,

loggedMDT					LoggedMDT-EUTRA,

...,

mDTMode-EUTRA-Extension			MDTMode-EUTRA-Extension

}

MDTMode-EUTRA-Extension ::= ProtocolIE-Single-Container {{ MDTMode-EUTRA-ExtensionIE }}

MDTMode-EUTRA-ExtensionIE XNAP-PROTOCOL-IES ::= {

...

}

MeasurementsToActivate ::= BIT STRING (SIZE (8))

MeasurementThresholdA2 ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

threshold-SINR				Threshold-SINR,

choice-extension	ProtocolIE-Single-Container { { MeasurementThresholdA2-ExtIEs} }

}

MeasurementThresholdA2-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

Measurement-ID 	 ::= INTEGER (1..4095,...)

MobilityInformation	 ::= BIT STRING (SIZE(32))

MobilityParametersModificationRange ::= SEQUENCE {

handoverTriggerChangeLowerLimit		INTEGER (-20..20),

handoverTriggerChangeUpperLimit		INTEGER (-20..20),

...

}

MobilityParametersInformation ::= SEQUENCE {

handoverTriggerChange			INTEGER (-20..20),

...

}

MobilityRestrictionList ::= SEQUENCE {

serving-PLMN						PLMN-Identity,

equivalent-PLMNs					SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMN-Identity			OPTIONAL,

rat-Restrictions					RAT-RestrictionsList										OPTIONAL,

forbiddenAreaInformation			ForbiddenAreaList											OPTIONAL,

serviceAreaInformation				ServiceAreaList												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MobilityRestrictionList-ExtIEs} }				OPTIONAL,

...

}

MobilityRestrictionList-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-LastE-UTRANPLMNIdentity					CRITICALITY ignore	EXTENSION PLMN-Identity									PRESENCE optional		}|

{ ID id-CNTypeRestrictionsForServing			CRITICALITY ignore	EXTENSION CNTypeRestrictionsForServing				PRESENCE optional		}|

{ ID id-CNTypeRestrictionsForEquivalent			CRITICALITY ignore	EXTENSION CNTypeRestrictionsForEquivalent			PRESENCE optional		}|

{ ID id-NPNMobilityInformation					CRITICALITY reject	EXTENSION NPNMobilityInformation						PRESENCE optional		},

...

}

CNTypeRestrictionsForEquivalent ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF CNTypeRestrictionsForEquivalentItem

CNTypeRestrictionsForEquivalentItem ::= SEQUENCE {

plmn-Identity						PLMN-Identity,

cn-Type								ENUMERATED {epc-forbidden, fiveGC-forbidden, ...},

iE-Extensions						ProtocolExtensionContainer { {CNTypeRestrictionsForEquivalentItem-ExtIEs} }				OPTIONAL,

...

}

CNTypeRestrictionsForEquivalentItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

CNTypeRestrictionsForServing ::= ENUMERATED {

epc-forbidden,

...

}

RAT-RestrictionsList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF RAT-RestrictionsItem

RAT-RestrictionsItem ::= SEQUENCE {

plmn-Identity					PLMN-Identity,

rat-RestrictionInformation		RAT-RestrictionInformation,

iE-Extensions		ProtocolExtensionContainer { {RAT-RestrictionsItem-ExtIEs} } OPTIONAL,

...

}

RAT-RestrictionsItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-ExtendedRATRestrictionInformation	CRITICALITY ignore	EXTENSION ExtendedRATRestrictionInformation		PRESENCE optional},

...

}

RAT-RestrictionInformation ::= BIT STRING {e-UTRA (0),nR (1)} (SIZE(8, ...))

ForbiddenAreaList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF ForbiddenAreaItem

ForbiddenAreaItem ::= SEQUENCE {

plmn-Identity		PLMN-Identity,

forbidden-TACs		SEQUENCE (SIZE(1..maxnoofForbiddenTACs)) OF TAC,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenAreaItem-ExtIEs} } OPTIONAL,

...

}

ForbiddenAreaItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

ServiceAreaList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF ServiceAreaItem

ServiceAreaItem ::= SEQUENCE {

plmn-Identity						PLMN-Identity,

allowed-TACs-ServiceArea			SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC		OPTIONAL,

not-allowed-TACs-ServiceArea		SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServiceAreaItem-ExtIEs} }				OPTIONAL,

...

}

ServiceAreaItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

MR-DC-ResourceCoordinationInfo ::= SEQUENCE {

ng-RAN-Node-ResourceCoordinationInfo			NG-RAN-Node-ResourceCoordinationInfo,

iE-Extension											ProtocolExtensionContainer { {MR-DC-ResourceCoordinationInfo-ExtIEs}}	OPTIONAL,

...

}

MR-DC-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-Node-ResourceCoordinationInfo ::= CHOICE {

eutra-resource-coordination-info					E-UTRA-ResourceCoordinationInfo,

nr-resource-coordination-info						NR-ResourceCoordinationInfo

}

E-UTRA-ResourceCoordinationInfo ::= SEQUENCE {

e-utra-cell												E-UTRA-CGI,

ul-coordination-info									BIT STRING (SIZE (6..4400)),

dl-coordination-info									BIT STRING (SIZE (6..4400))	OPTIONAL,

nr-cell													NR-CGI	OPTIONAL,

e-utra-coordination-assistance-info				E-UTRA-CoordinationAssistanceInfo	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {E-UTRA-ResourceCoordinationInfo-ExtIEs} } 	OPTIONAL,

...

}

E-UTRA-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRA-CoordinationAssistanceInfo ::= ENUMERATED {coordination-not-required, ...}

NR-ResourceCoordinationInfo ::= SEQUENCE {

nr-cell													NR-CGI,

ul-coordination-info									BIT STRING (SIZE (6..4400)),

dl-coordination-info									BIT STRING (SIZE (6..4400))	OPTIONAL,

e-utra-cell												E-UTRA-CGI	OPTIONAL,

nr-coordination-assistance-info					NR-CoordinationAssistanceInfo		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {NR-ResourceCoordinationInfo-ExtIEs} } 	OPTIONAL,

...

}

NR-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NR-CoordinationAssistanceInfo ::= ENUMERATED {coordination-not-required, ...}

MessageOversizeNotification ::= SEQUENCE {

maximumCellListSize								MaximumCellListSize,

iE-Extension				ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs}}	OPTIONAL,

...

}

MessageOversizeNotification-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MaximumCellListSize ::= INTEGER(1..16384, ...)

#### -- N

NBIoT-UL-DL-AlignmentOffset ::= ENUMERATED {

khz-7dot5,

khz0,

khz7dot5,

...

}

NE-DC-TDM-Pattern ::= SEQUENCE {

subframeAssignment			ENUMERATED {sa0,sa1,sa2,sa3,sa4,sa5,sa6},

harqOffset					INTEGER (0..9),

iE-Extension				ProtocolExtensionContainer { {NE-DC-TDM-Pattern-ExtIEs}}	OPTIONAL,

...

}

NE-DC-TDM-Pattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourInformation-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-E-UTRA-Item

NeighbourInformation-E-UTRA-Item ::= SEQUENCE {

e-utra-PCI			E-UTRAPCI,

e-utra-cgi			E-UTRA-CGI,

earfcn				E-UTRAARFCN,

tac					TAC,

ranac				RANAC																		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NeighbourInformation-E-UTRA-Item-ExtIEs} } 	OPTIONAL,

...

}

NeighbourInformation-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

NeighbourInformation-NR ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-NR-Item

NeighbourInformation-NR-Item ::= SEQUENCE {

nr-PCI								NRPCI,

nr-cgi								NR-CGI,

tac									TAC,

ranac								RANAC														OPTIONAL,

nr-mode-info						NeighbourInformation-NR-ModeInfo,

connectivitySupport					Connectivity-Support,

measurementTimingConfiguration		OCTET STRING,

iE-Extensions			ProtocolExtensionContainer { {NeighbourInformation-NR-Item-ExtIEs} } 	OPTIONAL,

...

}

NeighbourInformation-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

NeighbourInformation-NR-ModeInfo ::= CHOICE {

fdd-info				NeighbourInformation-NR-ModeFDDInfo,

tdd-info				NeighbourInformation-NR-ModeTDDInfo,

choice-extension		ProtocolIE-Single-Container { {NeighbourInformation-NR-ModeInfo-ExtIEs} }

}

NeighbourInformation-NR-ModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NeighbourInformation-NR-ModeFDDInfo ::= SEQUENCE {

ul-NR-FreqInfo		NRFrequencyInfo,

dl-NR-FequInfo		NRFrequencyInfo,

ie-Extensions		ProtocolExtensionContainer { {NeighbourInformation-NR-ModeFDDInfo-ExtIEs} } OPTIONAL,

...

}

NeighbourInformation-NR-ModeFDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourInformation-NR-ModeTDDInfo ::= SEQUENCE {

nr-FreqInfo			NRFrequencyInfo,

ie-Extensions		ProtocolExtensionContainer { {NeighbourInformation-NR-ModeTDDInfo-ExtIEs} } OPTIONAL,

...

}

NeighbourInformation-NR-ModeTDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NID	::= BIT STRING (SIZE(44))

NRCarrierList ::= SEQUENCE (SIZE(1..maxnoofNRSCSs)) OF NRCarrierItem

NRCarrierItem ::= SEQUENCE {

carrierSCS					NRSCS,

offsetToCarrier				INTEGER (0..2199, ...),

carrierBandwidth			INTEGER (0..maxnoofPhysicalResourceBlocks, ...),

iE-Extension		ProtocolExtensionContainer { {NRCarrierItem-ExtIEs} } 		OPTIONAL,

...

}

NRCarrierItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRCellPRACHConfig ::= OCTET STRING

NG-RAN-Cell-Identity ::= CHOICE {

nr						NR-Cell-Identity,

e-utra					E-UTRA-Cell-Identity,

choice-extension		ProtocolIE-Single-Container { {NG-RAN-Cell-Identity-ExtIEs} }

}

NG-RAN-Cell-Identity-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NG-RAN-CellPCI ::= CHOICE {

nr					NRPCI,

e-utra				E-UTRAPCI,

choice-extension	ProtocolIE-Single-Container { {NG-RAN-CellPCI-ExtIEs} }

}

NG-RAN-CellPCI-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NG-RANnodeUEXnAPID ::= INTEGER (0.. 4294967295)

NumberofActiveUEs::= INTEGER(0..16777215, ...)

NoofRRCConnections ::= INTEGER (1..65536,...)

NonDynamic5QIDescriptor ::= SEQUENCE {

fiveQI						FiveQI,

priorityLevelQoS			PriorityLevelQoS													OPTIONAL,

averagingWindow				AveragingWindow														OPTIONAL,

maximumDataBurstVolume		MaximumDataBurstVolume 												OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {NonDynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

...

}

NonDynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDownlink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget	PRESENCE optional}|

{ ID id-CNPacketDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget	PRESENCE optional},

...

}

NRARFCN	::= INTEGER (0.. maxNRARFCN)

NG-eNB-RadioResourceStatus	::= SEQUENCE {

dL-GBR-PRB-usage							DL-GBR-PRB-usage,

uL-GBR-PRB-usage							UL-GBR-PRB-usage,

dL-non-GBR-PRB-usage						DL-non-GBR-PRB-usage,

uL-non-GBR-PRB-usage						UL-non-GBR-PRB-usage,

dL-Total-PRB-usage						DL-Total-PRB-usage,

uL-Total-PRB-usage						UL-Total-PRB-usage,

iE-Extensions							ProtocolExtensionContainer { { NG-eNB-RadioResourceStatus-ExtIEs} } OPTIONAL,

...

}

NG-eNB-RadioResourceStatus-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION DL-scheduling-PDCCH-CCE-usage	PRESENCE optional}|

{ ID id-UL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION UL-scheduling-PDCCH-CCE-usage	PRESENCE optional},

...

}

DL-scheduling-PDCCH-CCE-usage ::= INTEGER (0.. 100)

UL-scheduling-PDCCH-CCE-usage ::= INTEGER (0.. 100)

TNLCapacityIndicator ::= SEQUENCE {

dLTNLOfferedCapacity					OfferedCapacity,

dLTNLAvailableCapacity					AvailableCapacity,

uLTNLOfferedCapacity					OfferedCapacity,

uLTNLAvailableCapacity					AvailableCapacity,

iE-Extensions							ProtocolExtensionContainer { { TNLCapacityIndicator-ExtIEs} } OPTIONAL,

...

}

TNLCapacityIndicator-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPN-Broadcast-Information ::= CHOICE {

snpn-Information					NPN-Broadcast-Information-SNPN,

pni-npn-Information					NPN-Broadcast-Information-PNI-NPN,

choice-extension					ProtocolIE-Single-Container { {NPN-Broadcast-Information-ExtIEs} }

}

NPN-Broadcast-Information-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPN-Broadcast-Information-SNPN ::= SEQUENCE {

broadcastSNPNID-List		BroadcastSNPNID-List,

iE-Extension				ProtocolExtensionContainer { {NPN-Broadcast-Information-SNPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Broadcast-Information-SNPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPN-Broadcast-Information-PNI-NPN ::= SEQUENCE {

broadcastPNI-NPN-ID-Information		BroadcastPNI-NPN-ID-Information,

iE-Extension						ProtocolExtensionContainer { {NPN-Broadcast-Information-PNI-NPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Broadcast-Information-PNI-NPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPNMobilityInformation::= CHOICE {

snpn-mobility-information			NPNMobilityInformation-SNPN,

pni-npn-mobility-information		NPNMobilityInformation-PNI-NPN,

choice-extension					ProtocolIE-Single-Container { {NPNMobilityInformation-ExtIEs} }

}

NPNMobilityInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPNMobilityInformation-SNPN ::= SEQUENCE {

serving-NID					NID,

iE-Extension				ProtocolExtensionContainer { {NPNMobilityInformation-SNPN-ExtIEs} }	OPTIONAL,

...

}

NPNMobilityInformation-SNPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPNMobilityInformation-PNI-NPN ::= SEQUENCE {

allowedPNI-NPN-ID-List		AllowedPNI-NPN-ID-List,

iE-Extension				ProtocolExtensionContainer { {NPNMobilityInformation-PNI-NPN-ExtIEs} }	OPTIONAL,

...

}

NPNMobilityInformation-PNI-NPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPNPagingAssistanceInformation ::= CHOICE {

pni-npn-Information					NPNPagingAssistanceInformation-PNI-NPN,

choice-extension					ProtocolIE-Single-Container { {NPNPagingAssistanceInformation-ExtIEs} }

}

NPNPagingAssistanceInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPNPagingAssistanceInformation-PNI-NPN ::= SEQUENCE {

allowedPNI-NPN-ID-List			AllowedPNI-NPN-ID-List,

iE-Extension					ProtocolExtensionContainer { {NPNPagingAssistanceInformation-PNI-NPN-ExtIEs} }	OPTIONAL,

...

}

NPNPagingAssistanceInformation-PNI-NPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPN-Support ::= CHOICE {

sNPN					NPN-Support-SNPN,

choice-Extensions		ProtocolIE-Single-Container { {NPN-Support-ExtIEs} }

}

NPN-Support-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPN-Support-SNPN ::= SEQUENCE {

nid					NID,

ie-Extension		ProtocolExtensionContainer { {NPN-Support-SNPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Support-SNPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPRACHConfiguration::= SEQUENCE {

fdd-or-tdd				CHOICE {

fdd			NPRACHConfiguration-FDD,

tdd			NPRACHConfiguration-TDD,

choice-extension		ProtocolIE-Single-Container { { FDD-or-TDD-in-NPRACHConfiguration-Choice-ExtIEs} }

},

iE-Extensions						ProtocolExtensionContainer { { NPRACHConfiguration-ExtIEs} }	OPTIONAL,

...

}

NPRACHConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

FDD-or-TDD-in-NPRACHConfiguration-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NPRACHConfiguration-FDD::= SEQUENCE {

nprach-CP-length						NPRACH-CP-Length,

anchorCarrier-NPRACHConfig						OCTET STRING,

anchorCarrier-EDT-NPRACHConfig 					OCTET STRING 						OPTIONAL,

anchorCarrier-Format2-NPRACHConfig				OCTET STRING						OPTIONAL,

anchorCarrier-Format2-EDT-NPRACHConfig			OCTET STRING 						OPTIONAL,

non-anchorCarrier-NPRACHConfig					OCTET STRING 						OPTIONAL,

non-anchorCarrier-Format2-NPRACHConfig			OCTET STRING 						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { NPRACHConfiguration-FDD-ExtIEs} }	OPTIONAL,

...

}

NPRACHConfiguration-FDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPRACHConfiguration-TDD::= SEQUENCE {

nprach-preambleFormat							NPRACH-preambleFormat,

anchorCarrier-NPRACHConfigTDD					OCTET STRING,

non-anchorCarrierFequencyConfiglist				Non-AnchorCarrierFrequencylist 		OPTIONAL,

non-anchorCarrier-NPRACHConfigTDD				OCTET STRING 						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { NPRACHConfiguration-TDD-ExtIEs} }	OPTIONAL,

...

}

NPRACHConfiguration-TDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NPRACH-CP-Length::=		ENUMERATED {

us66dot7,

us266dot7,

...

}

NPRACH-preambleFormat::= 	ENUMERATED {fmt0,fmt1,fmt2,fmt0a,fmt1a,...}

Non-AnchorCarrierFrequencylist ::= SEQUENCE (SIZE(1..maxnoofNonAnchorCarrierFreqConfig)) OF

SEQUENCE {

non-anchorCarrierFrquency		OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { { Non-AnchorCarrierFrequencylist-ExtIEs} } OPTIONAL,

...

}

Non-AnchorCarrierFrequencylist-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NR-Cell-Identity		::= BIT STRING (SIZE (36))

NG-RAN-Cell-Identity-ListinRANPagingArea ::= SEQUENCE (SIZE (1..maxnoofCellsinRNA)) OF NG-RAN-Cell-Identity

NR-CGI ::= SEQUENCE {

plmn-id				PLMN-Identity,

nr-CI				NR-Cell-Identity,

iE-Extension		ProtocolExtensionContainer { {NR-CGI-ExtIEs} } 	OPTIONAL,

...

}

NR-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRCyclicPrefix ::= ENUMERATED {normal, extended, ...}

NRDL-ULTransmissionPeriodicity ::= ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms3, ms4, ms5, ms10, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, ...}

NRFrequencyBand ::= INTEGER (1..1024, ...)

NRFrequencyBand-List ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF NRFrequencyBandItem

NRFrequencyBandItem ::= SEQUENCE {

nr-frequency-band			NRFrequencyBand,

supported-SUL-Band-List		SupportedSULBandList											OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {NRFrequencyBandItem-ExtIEs} } 	OPTIONAL,

...

}

NRFrequencyBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRFrequencyInfo ::= SEQUENCE {

nrARFCN				NRARFCN,

sul-information		SUL-Information												OPTIONAL,

frequencyBand-List		NRFrequencyBand-List,

iE-Extension		ProtocolExtensionContainer { {NRFrequencyInfo-ExtIEs} }		OPTIONAL,

...

}

NRFrequencyInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-FrequencyShift7p5khz	CRITICALITY ignore	EXTENSION FrequencyShift7p5khz	PRESENCE optional },...

}

NRMobilityHistoryReport ::= OCTET STRING

NRModeInfo ::= CHOICE {

fdd							NRModeInfoFDD,

tdd							NRModeInfoTDD,

choice-extension			ProtocolIE-Single-Container { {NRModeInfo-ExtIEs} }

}

NRModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

NRModeInfoFDD ::= SEQUENCE {

ulNRFrequencyInfo			NRFrequencyInfo,

dlNRFrequencyInfo			NRFrequencyInfo,

ulNRTransmissonBandwidth	NRTransmissionBandwidth,

dlNRTransmissonBandwidth	NRTransmissionBandwidth,

iE-Extension		ProtocolExtensionContainer { {NRModeInfoFDD-ExtIEs} } 	OPTIONAL,

...

}

NRModeInfoFDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-ULCarrierList			CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional }|

{ ID id-DLCarrierList			CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional },

...

}

NRModeInfoTDD ::= SEQUENCE {

nrFrequencyInfo			NRFrequencyInfo,

nrTransmissonBandwidth	NRTransmissionBandwidth,

iE-Extension			ProtocolExtensionContainer { {NRModeInfoTDD-ExtIEs} } 	OPTIONAL,

...

}

NRModeInfoTDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-IntendedTDD-DL-ULConfiguration-NR	CRITICALITY ignore	EXTENSION IntendedTDD-DL-ULConfiguration-NR	PRESENCE optional }|

{ID id-TDDULDLConfigurationCommonNR			CRITICALITY ignore	EXTENSION TDDULDLConfigurationCommonNR			PRESENCE optional }|

{ ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional },

...

}

NRNRB ::= ENUMERATED { nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ...}

NRPCI ::= INTEGER (0..1007, ...)

NRSCS ::= ENUMERATED { scs15, scs30, scs60, scs120, ...}

NRTransmissionBandwidth	::= SEQUENCE {

nRSCS	NRSCS,

nRNRB	NRNRB,

iE-Extensions				ProtocolExtensionContainer { {NRTransmissionBandwidth-ExtIEs} } OPTIONAL,

...

}

NRTransmissionBandwidth-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NumberOfAntennaPorts-E-UTRA ::= ENUMERATED {an1, an2, an4, ...}

NG-RANTraceID 				::=OCTET STRING (SIZE (8))

NonGBRResources-Offered ::= ENUMERATED {true, ...}

NRV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE											OPTIONAL,

pedestrianUE 		PedestrianUE						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NRV2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

NRV2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRUESidelinkAggregateMaximumBitRate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {NRUESidelinkAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

NRUESidelinkAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- O

OfferedCapacity ::= INTEGER (1.. 16777216,...)

OffsetOfNbiotChannelNumberToEARFCN ::= ENUMERATED {

minusTen,

minusNine,

minusEightDotFive,

minusEight,

minusSeven,

minusSix,

minusFive,

minusFourDotFive,

minusFour,

minusThree,

minusTwo,

minusOne,

minusZeroDotFive,

zero,

one,

two,

three,

threeDotFive,

four,

five,

six,

seven,

sevenDotFive,

eight,

nine,

...

}

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PedestrianUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PacketLossRate ::= INTEGER (0..1000, ...)

PagingDRX	::= ENUMERATED {

v32,

v64,

v128,

v256,

... ,

v512,

v1024

}

PagingeDRXInformation ::= SEQUENCE {

paging-eDRX-Cycle		Paging-eDRX-Cycle,

paging-Time-Window		Paging-Time-Window											OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {PagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

PagingeDRXInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Paging-eDRX-Cycle ::= ENUMERATED {

hfhalf, hf1, hf2, hf4, hf6,

hf8, hf10, hf12, hf14, hf16,

hf32, hf64, hf128, hf256,

...

}

Paging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...

}

PagingPriority ::= ENUMERATED {

priolevel1,

priolevel2,

priolevel3,

priolevel4,

priolevel5,

priolevel6,

priolevel7,

priolevel8,

...

}

PartialListIndicator ::= ENUMERATED {partial, ...}

PC5QoSParameters ::= SEQUENCE {

pc5QoSFlowList				PC5QoSFlowList,

pc5LinkAggregateBitRates	BitRate				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PC5QoSParameters-ExtIEs} }	OPTIONAL,

...

}

PC5QoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PC5QoSFlowList ::= SEQUENCE (SIZE(1..maxnoofPC5QoSFlows)) OF PC5QoSFlowItem

PC5QoSFlowItem::= SEQUENCE {

pQI							FiveQI,

pc5FlowBitRates				PC5FlowBitRates				OPTIONAL,

range						Range						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PC5QoSFlowItem-ExtIEs} }	OPTIONAL,

...

}

PC5QoSFlowItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PC5FlowBitRates ::= SEQUENCE {

guaranteedFlowBitRate		BitRate,

maximumFlowBitRate			BitRate,

iE-Extensions		ProtocolExtensionContainer { { PC5FlowBitRates-ExtIEs} }	OPTIONAL,

...

}

PC5FlowBitRates-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDCPChangeIndication ::= CHOICE {

from-S-NG-RAN-node				ENUMERATED {s-ng-ran-node-key-update-required, pdcp-data-recovery-required, ...},

from-M-NG-RAN-node				ENUMERATED {pdcp-data-recovery-required, ...},

choice-extension				ProtocolIE-Single-Container { {PDCPChangeIndication-ExtIEs} }

}

PDCPChangeIndication-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

PDCPDuplicationConfiguration ::= ENUMERATED {

configured,

de-configured,

...

}

PDCPSNLength ::= SEQUENCE {

ulPDCPSNLength			ENUMERATED {v12bits, v18bits, ...},

dlPDCPSNLength			ENUMERATED {v12bits, v18bits, ...},

iE-Extension			ProtocolExtensionContainer { {PDCPSNLength-ExtIEs} }		OPTIONAL,

...

}

PDCPSNLength-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAggregateMaximumBitRate ::= SEQUENCE {

downlink-session-AMBR				BitRate,

uplink-session-AMBR					BitRate,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionAggregateMaximumBitRate-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSession-List ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-ID

PDUSession-List-withCause ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-List-withCause-Item

PDUSession-List-withCause-Item ::= SEQUENCE {

pduSessionId		PDUSession-ID,

cause				Cause				OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withCause-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSession-List-withDataForwardingFromTarget ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingFromTarget-Item

PDUSession-List-withDataForwardingFromTarget-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

dataforwardinginfoTarget			DataForwardingInfoFromTargetNGRANnode,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse		CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional},

...

}

PDUSession-List-withDataForwardingRequest ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingRequest-Item

PDUSession-List-withDataForwardingRequest-Item ::= SEQUENCE {

pduSessionId							PDUSession-ID,

dataforwardingInfofromSource			DataforwardingandOffloadingInfofromSource					OPTIONAL,

dRBtoBeReleasedList						DRBToQoSFlowMapping-List									OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-Cause				CRITICALITY ignore EXTENSION Cause				PRESENCE optional},

...

}

-- **************************************************************

--

##### -- PDU Session related message level IEs BEGIN

--

-- **************************************************************

-- **************************************************************

--

###### -- PDU Session Resources Admitted List

--

-- **************************************************************

PDUSessionResourcesAdmitted-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesAdmitted-Item

PDUSessionResourcesAdmitted-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

pduSessionResourceAdmittedInfo		PDUSessionResourceAdmittedInfo,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourcesAdmitted-Item-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourcesAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceAdmittedInfo ::= SEQUENCE {

dL-NG-U-TNL-Information-Unchanged		ENUMERATED {true, ...}														OPTIONAL,

qosFlowsAdmitted-List					QoSFlowsAdmitted-List,

qosFlowsNotAdmitted-List				QoSFlows-List-withCause														OPTIONAL,

dataForwardingInfoFromTarget			DataForwardingInfoFromTargetNGRANnode										OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionResourceAdmittedInfo-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceAdmittedInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondarydataForwardingInfoFromTarget-List	CRITICALITY ignore	EXTENSION SecondarydataForwardingInfoFromTarget-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resources Not Admitted List

--

-- **************************************************************

PDUSessionResourcesNotAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofPDUSessions)) OF PDUSessionResourcesNotAdmitted-Item

PDUSessionResourcesNotAdmitted-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

cause						Cause				OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resources To Be Setup List

--

-- **************************************************************

PDUSessionResourcesToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesToBeSetup-Item

PDUSessionResourcesToBeSetup-Item ::= SEQUENCE {

pduSessionId					PDUSession-ID,

s-NSSAI							S-NSSAI,

pduSessionAMBR					PDUSessionAggregateMaximumBitRate													OPTIONAL,

uL-NG-U-TNLatUPF				UPTransportLayerInformation,

source-DL-NG-U-TNL-Information  UPTransportLayerInformation															OPTIONAL,

securityIndication				SecurityIndication																	OPTIONAL,

pduSessionType					PDUSessionType,

pduSessionNetworkInstance		PDUSessionNetworkInstance															OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourcesToBeSetup-Item-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourcesToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-UL-NG-U-TNLatUPF-List			CRITICALITY ignore	EXTENSION Additional-UL-NG-U-TNLatUPF-List 		PRESENCE optional}|

{ ID id-PDUSessionCommonNetworkInstance				CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-Redundant-UL-NG-U-TNLatUPF					CRITICALITY	ignore	EXTENSION UPTransportLayerInformation				PRESENCE optional}|

{ ID id-Additional-Redundant-UL-NG-U-TNLatUPF-List	CRITICALITY ignore	EXTENSION Additional-UL-NG-U-TNLatUPF-List  	PRESENCE optional}|

{ ID id-RedundantCommonNetworkInstance				CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-RedundantPDUSessionInformation				CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation			PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Info - SN terminated

--

-- **************************************************************

PDUSessionResourceSetupInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF				UPTransportLayerInformation,

pduSessionType					PDUSessionType,

pduSessionNetworkInstance		PDUSessionNetworkInstance																	OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List-Setup-SNterminated,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource													OPTIONAL,

securityIndication				SecurityIndication																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-SNterminated-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecurityResult						CRITICALITY reject	EXTENSION SecurityResult							PRESENCE optional}|

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ID id-DefaultDRB-Allowed					CRITICALITY ignore	EXTENSION DefaultDRB-Allowed						PRESENCE optional}|

{ ID id-SplitSessionIndicator				CRITICALITY reject	EXTENSION SplitSessionIndicator					PRESENCE optional}|

{ID id-NonGBRResources-Offered				CRITICALITY ignore	EXTENSION NonGBRResources-Offered				PRESENCE optional}|

{ ID id-Redundant-UL-NG-U-TNLatUPF			CRITICALITY	ignore	EXTENSION UPTransportLayerInformation			PRESENCE optional}|

{ ID id-RedundantCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-RedundantPDUSessionInformation		CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation		PRESENCE optional},

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Setup-SNterminated-Item

QoSFlowsToBeSetup-List-Setup-SNterminated-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

offeredGBRQoSFlowInfo			GBRQoSFlowInfo																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional}|

{ ID id-RedundantQoSFlowIndicator		CRITICALITY ignore	EXTENSION RedundantQoSFlowIndicator	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Response Info - SN terminated

--

-- **************************************************************

PDUSessionResourceSetupResponseInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation,

dRBsToBeSetup					DRBsToBeSetupList-SetupResponse-SNterminated 	OPTIONAL,

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode			OPTIONAL,

qosFlowsNotAdmittedList			QoSFlows-List-withCause							OPTIONAL,

securityResult					SecurityResult									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse						CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional}|

{ ID id-Redundant-DL-NG-U-TNLatNG-RAN				CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional}|

{ ID id-UsedRSNInformation							CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation	PRESENCE optional},

...

}

DRBsToBeSetupList-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-SetupResponse-SNterminated-Item

DRBsToBeSetupList-SetupResponse-SNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

sN-UL-PDCP-UP-TNLInfo									UPTransportParameters,

dRB-QoS													QoSFlowLevelQoSParameters,

pDCP-SNLength											PDCPSNLength						OPTIONAL,

rLC-Mode												RLCMode,

uL-Configuration									ULConfiguration						OPTIONAL,

secondary-SN-UL-PDCP-UP-TNLInfo							UPTransportParameters				OPTIONAL,

duplicationActivation									DuplicationActivation				OPTIONAL,

qoSFlowsMappedtoDRB-SetupResponse-SNterminated			QoSFlowsMappedtoDRB-SetupResponse-SNterminated,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation	PRESENCE optional},

...

}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo		GBRQoSFlowInfo														OPTIONAL,

qosFlowMappingIndication		QoSFlowMappingIndication											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex		CRITICALITY ignore	EXTENSION QoSParaSetIndex	PRESENCE optional },

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Info - MN terminated

--

-- **************************************************************

PDUSessionResourceSetupInfo-MNterminated ::= SEQUENCE {

pduSessionType					PDUSessionType,

dRBsToBeSetup					DRBsToBeSetupList-Setup-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetupList-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-Setup-MNterminated-Item

DRBsToBeSetupList-Setup-MNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

mN-UL-PDCP-UP-TNLInfo									UPTransportParameters,

rLC-Mode												RLCMode,

uL-Configuration										ULConfiguration						OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters,

pDCP-SNLength											PDCPSNLength						OPTIONAL,

secondary-MN-UL-PDCP-UP-TNLInfo							UPTransportParameters				OPTIONAL,

duplicationActivation									DuplicationActivation				OPTIONAL,

qoSFlowsMappedtoDRB-Setup-MNterminated		QoSFlowsMappedtoDRB-Setup-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation	PRESENCE optional},

...

}

QoSFlowsMappedtoDRB-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsMappedtoDRB-Setup-MNterminated-Item

QoSFlowsMappedtoDRB-Setup-MNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

qosFlowMappingIndication		QoSFlowMappingIndication		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Response Info - MN terminated

--

-- **************************************************************

PDUSessionResourceSetupResponseInfo-MNterminated ::= SEQUENCE {

dRBsAdmittedList				DRBsAdmittedList-SetupResponse-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-DRBsNotAdmittedSetupModifyList	CRITICALITY ignore	EXTENSION DRB-List-withCause		PRESENCE optional},

...

}

DRBsAdmittedList-SetupResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-SetupResponse-MNterminated-Item

DRBsAdmittedList-SetupResponse-MNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

sN-DL-SCG-UP-TNLInfo					UPTransportParameters,

secondary-SN-DL-SCG-UP-TNLInfo			UPTransportParameters				OPTIONAL,

lCID									LCID								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated	CRITICALITY ignore	EXTENSION	QoSFlowsMappedtoDRB-SetupResponse-MNterminated	PRESENCE optional},

...

}

QoSFlowsMappedtoDRB-SetupResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item

QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

currentQoSParaSetIndex			QoSParaSetIndex,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModificationInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF				UPTransportLayerInformation						OPTIONAL,

pduSessionNetworkInstance		PDUSessionNetworkInstance						OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List-Setup-SNterminated		OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource		OPTIONAL,

qosFlowsToBeModified-List		QoSFlowsToBeSetup-List-Modified-SNterminated	OPTIONAL,

qoSFlowsToBeReleased-List		QoSFlows-List-withCause							OPTIONAL,

drbsToBeModifiedList			DRBsToBeModified-List-Modified-SNterminated		OPTIONAL,

dRBsToBeReleased				DRB-List-withCause								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ID id-DefaultDRB-Allowed					CRITICALITY ignore	EXTENSION DefaultDRB-Allowed						PRESENCE optional}|

{ID id-NonGBRResources-Offered				CRITICALITY ignore	EXTENSION NonGBRResources-Offered				PRESENCE optional}|

{ID id-Redundant-UL-NG-U-TNLatUPF			CRITICALITY ignore	EXTENSION UPTransportLayerInformation	PRESENCE optional}|

{ID id-RedundantCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance	PRESENCE optional},

...

}

QoSFlowsToBeSetup-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Modified-SNterminated-Item

QoSFlowsToBeSetup-List-Modified-SNterminated-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters							OPTIONAL,

offeredGBRQoSFlowInfo			GBRQoSFlowInfo										OPTIONAL,

qosFlowMappingIndication		QoSFlowMappingIndication 							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional}|

{ ID id-RedundantQoSFlowIndicator		CRITICALITY ignore	EXTENSION RedundantQoSFlowIndicator	PRESENCE optional},

...

}

DRBsToBeModified-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-Modified-SNterminated-Item

DRBsToBeModified-List-Modified-SNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

mN-DL-SCG-UP-TNLInfo					UPTransportParameters		OPTIONAL,

secondary-MN-DL-SCG-UP-TNLInfo			UPTransportParameters		OPTIONAL,

lCID									LCID						OPTIONAL,

rlc-status								RLC-Status					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Response Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModificationResponseInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation								OPTIONAL,

dRBsToBeSetup					DRBsToBeSetupList-SetupResponse-SNterminated			OPTIONAL,

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode					OPTIONAL,

dRBsToBeModified				DRBsToBeModifiedList-ModificationResponse-SNterminated	OPTIONAL,

dRBsToBeReleased				DRB-List-withCause										OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource				OPTIONAL,

qosFlowsNotAdmittedTBAdded		QoSFlows-List-withCause									OPTIONAL,

qosFlowsReleased				QoSFlows-List-withCause									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse		CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional}|

{ ID id-Redundant-DL-NG-U-TNLatNG-RAN	CRITICALITY ignore	EXTENSION UPTransportLayerInformation	PRESENCE optional},

...

}

DRBsToBeModifiedList-ModificationResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

sN-UL-PDCP-UP-TNLInfo									UPTransportParameters									OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters								OPTIONAL,

qoSFlowsMappedtoDRB-SetupResponse-SNterminated			QoSFlowsMappedtoDRB-SetupResponse-SNterminated		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation	PRESENCE optional}|

{ ID id-secondary-SN-UL-PDCP-UP-TNLInfo				CRITICALITY ignore	EXTENSION UPTransportParameters				PRESENCE optional}|

{ ID id-pdcpDuplicationConfiguration				CRITICALITY ignore	EXTENSION PDCPDuplicationConfiguration		PRESENCE optional}|

{ ID id-duplicationActivation						CRITICALITY ignore	EXTENSION DuplicationActivation				PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModificationInfo-MNterminated ::= SEQUENCE {

pduSessionType					PDUSessionType,

dRBsToBeSetup					DRBsToBeSetupList-Setup-MNterminated						OPTIONAL,

dRBsToBeModified				DRBsToBeModifiedList-Modification-MNterminated				OPTIONAL,

dRBsToBeReleased				DRB-List-withCause											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModifiedList-Modification-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsToBeModifiedList-Modification-MNterminated-Item

DRBsToBeModifiedList-Modification-MNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

mN-UL-PDCP-UP-TNLInfo									UPTransportParameters						OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters					OPTIONAL,

secondary-MN-UL-PDCP-UP-TNLInfo							UPTransportParameters						OPTIONAL,

uL-Configuration										ULConfiguration								OPTIONAL,

pdcpDuplicationConfiguration							PDCPDuplicationConfiguration 				OPTIONAL,

duplicationActivation									DuplicationActivation						OPTIONAL,

qoSFlowsMappedtoDRB-Setup-MNterminated					QoSFlowsMappedtoDRB-Setup-MNterminated	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Response Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModificationResponseInfo-MNterminated ::= SEQUENCE {

dRBsAdmittedList						DRBsAdmittedList-ModificationResponse-MNterminated,

dRBsReleasedList						DRB-List																		OPTIONAL,

dRBsNotAdmittedSetupModifyList			DRB-List-withCause																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsAdmittedList-ModificationResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-ModificationResponse-MNterminated-Item

DRBsAdmittedList-ModificationResponse-MNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

sN-DL-SCG-UP-TNLInfo					UPTransportParameters				OPTIONAL,

secondary-SN-DL-SCG-UP-TNLInfo			UPTransportParameters				OPTIONAL,

lCID									LCID								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated	CRITICALITY ignore	EXTENSION	QoSFlowsMappedtoDRB-SetupResponse-MNterminated	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Required Info - SN terminated

--

-- **************************************************************

PDUSessionResourceChangeRequiredInfo-SNterminated ::= SEQUENCE {

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeRequiredInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeRequiredInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Confirm Info - SN terminated

--

-- **************************************************************

PDUSessionResourceChangeConfirmInfo-SNterminated ::= SEQUENCE {

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeConfirmInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeConfirmInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse	CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Required Info - MN terminated

--

-- **************************************************************

PDUSessionResourceChangeRequiredInfo-MNterminated ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeRequiredInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeRequiredInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Change Confirm Info - MN terminated

--

-- **************************************************************

PDUSessionResourceChangeConfirmInfo-MNterminated ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceChangeConfirmInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceChangeConfirmInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Required Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModRqdInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation						OPTIONAL,

qoSFlowsToBeReleased-List		QoSFlows-List-withCause							OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource		OPTIONAL,

drbsToBeSetupList				DRBsToBeSetup-List-ModRqd-SNterminated			OPTIONAL,

drbsToBeModifiedList			DRBsToBeModified-List-ModRqd-SNterminated		OPTIONAL,

dRBsToBeReleased				DRB-List-withCause								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModRqdInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModRqdInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetup-List-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetup-List-ModRqd-SNterminated-Item

DRBsToBeSetup-List-ModRqd-SNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

pDCP-SNLength									PDCPSNLength											OPTIONAL,

sn-UL-PDCP-UPTNLinfo							UPTransportParameters,

dRB-QoS											QoSFlowLevelQoSParameters,

secondary-SN-UL-PDCP-UP-TNLInfo					UPTransportParameters								OPTIONAL,

duplicationActivation							DuplicationActivation								OPTIONAL,

uL-Configuration								ULConfiguration										OPTIONAL,

qoSFlowsMappedtoDRB-ModRqd-SNterminated			QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated,

rLC-Mode										RLCMode,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetup-List-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetup-List-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional},

...

}

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo		GBRQoSFlowInfo														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModified-List-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-ModRqd-SNterminated-Item

DRBsToBeModified-List-ModRqd-SNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

sN-UL-PDCP-UP-TNLInfo							UPTransportParameters									OPTIONAL,

dRB-QoS											QoSFlowLevelQoSParameters								OPTIONAL,

secondary-SN-UL-PDCP-UP-TNLInfo					UPTransportParameters									OPTIONAL,

uL-Configuration								ULConfiguration											OPTIONAL,

pdcpDuplicationConfiguration					PDCPDuplicationConfiguration							OPTIONAL,

duplicationActivation							DuplicationActivation									OPTIONAL,

qoSFlowsMappedtoDRB-ModRqd-SNterminated		QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModified-List-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional},

...

}

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier					QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo			GBRQoSFlowInfo														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Confirm Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModConfirmInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF						UPTransportLayerInformation								OPTIONAL,

dRBsAdmittedList						DRBsAdmittedList-ModConfirm-SNterminated,

dRBsNotAdmittedSetupModifyList			DRB-List-withCause										OPTIONAL,

dataforwardinginfoTarget				DataForwardingInfoFromTargetNGRANnode					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModConfirmInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModConfirmInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse		CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional},

...

}

DRBsAdmittedList-ModConfirm-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsAdmittedList-ModConfirm-SNterminated-Item

DRBsAdmittedList-ModConfirm-SNterminated-Item ::= SEQUENCE {

drb-ID											DRB-ID,

mN-DL-CG-UP-TNLInfo								UPTransportParameters							OPTIONAL,

secondary-MN-DL-CG-UP-TNLInfo					UPTransportParameters							OPTIONAL,

lCID											LCID											OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {DRBsAdmittedList-ModConfirm-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-ModConfirm-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Required Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModRqdInfo-MNterminated ::= SEQUENCE {

dRBsToBeModified				DRBsToBeModified-List-ModRqd-MNterminated				OPTIONAL,

dRBsToBeReleased				DRB-List-withCause											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModRqdInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModRqdInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModified-List-ModRqd-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-ModRqd-MNterminated-Item

DRBsToBeModified-List-ModRqd-MNterminated-Item ::= SEQUENCE {

drb-ID								DRB-ID,

sN-DL-SCG-UP-TNLInfo				UPTransportLayerInformation,

secondary-SN-DL-SCG-UP-TNLInfo		UPTransportLayerInformation		OPTIONAL,

lCID								LCID							OPTIONAL,

rlc-status							RLC-Status						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs} } OPTIONAL,

...

}

DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Confirm Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModConfirmInfo-MNterminated ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModConfirmInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModConfirmInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- PDU Session Resource Setup Complete Info - SN terminated

--

-- **************************************************************

PDUSessionResourceBearerSetupCompleteInfo-SNterminated ::= SEQUENCE {

dRBsToBeSetupList 			SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item,

iE-Extensions				ProtocolExtensionContainer { {PDUSessionResourceBearerSetupCompleteInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceBearerSetupCompleteInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item ::= SEQUENCE {

dRB-ID						DRB-ID,

mN-Xn-U-TNLInfoatM			UPTransportLayerInformation,

iE-Extensions				ProtocolExtensionContainer { {DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-Secondary-MN-Xn-U-TNLInfoatM	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional},

...

}

-- **************************************************************

--

##### -- PDU Session related message level IEs END

--

-- **************************************************************

PDUSessionResourceSecondaryRATUsageList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSecondaryRATUsageItem

PDUSessionResourceSecondaryRATUsageItem ::= SEQUENCE {

pDUSessionID								PDUSession-ID,

secondaryRATUsageInformation						SecondaryRATUsageInformation,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSecondaryRATUsageItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSecondaryRATUsageItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionUsageReport ::= SEQUENCE {

rATType								ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},

pDUSessionTimedReportList			VolumeTimedReportList,

iE-Extensions						ProtocolExtensionContainer { {PDUSessionUsageReport-ExtIEs} } OPTIONAL,

...

}

PDUSessionUsageReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionType ::= ENUMERATED {ipv4, ipv6, ipv4v6, ethernet, unstructured, ...}

PDUSession-ID	::= INTEGER (0..255)

PDUSessionNetworkInstance	::= INTEGER (1..256, ...)

PDUSessionCommonNetworkInstance	::= OCTET STRING

Periodical ::= SEQUENCE {

iE-Extensions		ProtocolExtensionContainer { { Periodical-ExtIEs} } OPTIONAL,

...

}

Periodical-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PLMN-Identity ::= OCTET STRING (SIZE(3))

PCIListForMDT ::= SEQUENCE (SIZE(1.. maxnoofNeighPCIforMDT)) OF NRPCI

PNI-NPN-Restricted-Information ::= ENUMERATED { restriced, not-restricted, ...}

PortNumber ::= BIT STRING (SIZE (16))

PriorityLevelQoS ::= INTEGER (1..127, ...)

ProtectedE-UTRAResourceIndication ::= SEQUENCE {

activationSFN					ActivationSFN,

protectedResourceList			ProtectedE-UTRAResourceList,

mbsfnControlRegionLength		MBSFNControlRegionLength					OPTIONAL,

pDCCHRegionLength				INTEGER (1..3),

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAResourceIndication-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAResourceIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ProtectedE-UTRAResourceList ::= SEQUENCE (SIZE (1.. maxnoofProtectedResourcePatterns)) OF ProtectedE-UTRAResource-Item

ProtectedE-UTRAResource-Item ::= SEQUENCE {

resourceType							ENUMERATED {downlinknonCRS, cRS, uplink, ...},

intra-PRBProtectedResourceFootprint		BIT STRING (SIZE(84, ...)),

protectedFootprintFrequencyPattern		BIT STRING (SIZE(6..110, ...)),

protectedFootprintTimePattern			ProtectedE-UTRAFootprintTimePattern,

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAResource-Item-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAResource-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ProtectedE-UTRAFootprintTimePattern ::= SEQUENCE {

protectedFootprintTimeperiodicity			INTEGER (1..320, ...),

protectedFootrpintStartTime					INTEGER (1..20, ...),

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAFootprintTimePattern-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAFootprintTimePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- Q

QoSCharacteristics ::= CHOICE {

non-dynamic						NonDynamic5QIDescriptor,

dynamic							Dynamic5QIDescriptor,

choice-extension				ProtocolIE-Single-Container { {QoSCharacteristics-ExtIEs} }

}

QoSCharacteristics-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

QoSFlowIdentifier	::= INTEGER (0..63, ...)

QoSFlowLevelQoSParameters ::= SEQUENCE {

qos-characteristics			QoSCharacteristics,

allocationAndRetentionPrio	AllocationandRetentionPriority,

gBRQoSFlowInfo				GBRQoSFlowInfo														OPTIONAL,

relectiveQoS				ReflectiveQoSAttribute												OPTIONAL,

additionalQoSflowInfo		ENUMERATED {more-likely, ...}										OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {QoSFlowLevelQoSParameters-ExtIEs} } OPTIONAL,

...

}

QoSFlowLevelQoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-QoSMonitoringRequest				CRITICALITY ignore	EXTENSION QosMonitoringRequest					PRESENCE optional}|

{ID id-QosMonitoringReportingFrequency	CRITICALITY ignore	EXTENSION QosMonitoringReportingFrequency	PRESENCE optional}|

{ID id-QoSMonitoringDisabled				CRITICALITY ignore	EXTENSION QoSMonitoringDisabled					PRESENCE optional},

...

}

QoSFlowMappingIndication ::= ENUMERATED {

ul,

dl,

...

}

QoSFlowNotificationControlIndicationInfo ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowNotify-Item

QoSFlowNotify-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

notificationInformation		ENUMERATED {fulfilled, not-fulfilled, ...},

iE-Extensions				ProtocolExtensionContainer { {QoSFlowNotificationControlIndicationInfo-ExtIEs} } OPTIONAL,

...

}

QoSFlowNotificationControlIndicationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{	ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION QoSParaSetNotifyIndex		PRESENCE optional },

...

}

QoSFlows-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlow-Item

QoSFlow-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowMappingIndication		QoSFlowMappingIndication					OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFlow-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlow-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlows-List-withCause ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowwithCause-Item

QoSFlowwithCause-Item ::= SEQUENCE {

qfi					QoSFlowIdentifier,

cause				Cause					OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFlowwithCause-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowwithCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Mapping-Information ::= SEQUENCE {

dscp							BIT STRING (SIZE(6))			OPTIONAL,

flow-label						BIT STRING (SIZE(20))		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoS-Mapping-Information-ExtIEs} }	OPTIONAL,

...

}

QoS-Mapping-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSParaSetIndex ::= INTEGER (1..8,...)

QoSParaSetNotifyIndex ::= INTEGER (0..8,...)

QoSFlowsAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowsAdmitted-Item

QoSFlowsAdmitted-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

iE-Extension		ProtocolExtensionContainer { {QoSFlowsAdmitted-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION QoSParaSetIndex	PRESENCE optional	},

...

}

QoSFlowsToBeSetup-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-Item

QoSFlowsToBeSetup-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

e-RAB-ID						E-RAB-ID											OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFlowsToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional}|

{ ID id-RedundantQoSFlowIndicator		CRITICALITY ignore	EXTENSION RedundantQoSFlowIndicator	PRESENCE optional},

...

}

QoSFlowsUsageReportList ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsUsageReport-Item

QoSFlowsUsageReport-Item ::= SEQUENCE {

qosFlowIdentifier					QoSFlowIdentifier,

rATType								ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},

qoSFlowsTimedReportList				VolumeTimedReportList,

iE-Extensions						ProtocolExtensionContainer { {QoSFlowsUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

QoSFlowsUsageReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both}

QoSMonitoringDisabled ::= ENUMERATED {true, ...}

QosMonitoringReportingFrequency ::= INTEGER (1..1800, ...)

#### -- R

RACHReportInformation	::= SEQUENCE (SIZE(1.. maxnoofRACHReports)) OF RACHReportList-Item

RACHReportList-Item	::= SEQUENCE {

rACHReport				RACHReportContainer,

iE-Extensions						ProtocolExtensionContainer { { RACHReportList-Item-ExtIEs} }	OPTIONAL,

...

}

RACHReportList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RACHReportContainer	::= OCTET STRING

RadioResourceStatus	::= CHOICE {

ng-eNB-RadioResourceStatus	NG-eNB-RadioResourceStatus,

gNB-RadioResourceStatus	        GNB-RadioResourceStatus,

choice-extension			ProtocolIE-Single-Container { { RadioResourceStatus-ExtIEs} }

}

RadioResourceStatus-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RANAC ::= INTEGER (0..255)

RANAreaID ::= SEQUENCE {

tAC					TAC,

rANAC				RANAC												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {RANAreaID-ExtIEs} } 	OPTIONAL,

...

}

RANAreaID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANAreaID-List ::= SEQUENCE (SIZE(1..maxnoofRANAreasinRNA)) OF RANAreaID

Range ::= ENUMERATED {m50, m80, m180, m200, m350, m400, m500, m700, m1000, ...}

RANPagingArea ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

rANPagingAreaChoice		RANPagingAreaChoice,

iE-Extensions			ProtocolExtensionContainer { {RANPagingArea-ExtIEs} } OPTIONAL,

...

}

RANPagingArea-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANPagingAreaChoice ::= CHOICE {

cell-List			NG-RAN-Cell-Identity-ListinRANPagingArea,

rANAreaID-List		RANAreaID-List,

choice-extension	ProtocolIE-Single-Container { {RANPagingAreaChoice-ExtIEs} }

}

RANPagingAreaChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RANPagingAttemptInfo ::= SEQUENCE {

pagingAttemptCount					INTEGER (1..16, ...),

intendedNumberOfPagingAttempts		INTEGER (1..16, ...),

nextPagingAreaScope					ENUMERATED {same, changed, ...}	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {RANPagingAttemptInfo-ExtIEs} } OPTIONAL,

...

}

RANPagingAttemptInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANPagingFailure		::= 	ENUMERATED {

true,

...

}

RedundantQoSFlowIndicator ::= ENUMERATED {true, false}

RedundantPDUSessionInformation ::= SEQUENCE {

rSN					RSN,

iE-Extensions		ProtocolExtensionContainer { {RedundantPDUSessionInformation-ExtIEs} }	OPTIONAL,

...

}

RedundantPDUSessionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RSN ::= ENUMERATED {v1, v2, ...}

ReferenceID ::= INTEGER (1..64, ...) -- This IE may need to be refined.

ReflectiveQoSAttribute ::= ENUMERATED {subject-to-reflective-QoS, ...}

ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

ReportArea ::= ENUMERATED {

cell,

...

}

ReportIntervalMDT ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60, ...}

ReportType ::= CHOICE {

periodical					Periodical,

eventTriggered				EventTriggered,

...

}

ReportCharacteristics ::=  BIT STRING(SIZE(32))

ReportingPeriodicity ::= ENUMERATED {

half-thousand-ms,

one-thousand-ms,

two-thousand-ms,

five-thousand-ms,

ten-thousand-ms,

...

}

RegistrationRequest ::= ENUMERATED {start, stop, add, ... }

RequestReferenceID ::= INTEGER (1..64, ...)

ReservedSubframePattern ::= SEQUENCE {

subframeType					ENUMERATED {mbsfn, non-mbsfn, ...},

reservedSubframePattern			BIT STRING (SIZE(10..160)),

mbsfnControlRegionLength		MBSFNControlRegionLength					OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {ReservedSubframePattern-ExtIEs} } OPTIONAL,

...

}

ReservedSubframePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetRequestTypeInfo ::= CHOICE {

fullReset			ResetRequestTypeInfo-Full,

partialReset		ResetRequestTypeInfo-Partial,

choice-extension	ProtocolIE-Single-Container { {ResetRequestTypeInfo-ExtIEs} }

}

ResetRequestTypeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResetRequestTypeInfo-Full ::= SEQUENCE {

iE-Extension					ProtocolExtensionContainer { {ResetRequestTypeInfo-Full-ExtIEs} } OPTIONAL,

...

}

ResetRequestTypeInfo-Full-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetRequestTypeInfo-Partial ::= SEQUENCE {

ue-contexts-ToBeReleasedList	ResetRequestPartialReleaseList,

iE-Extension					ProtocolExtensionContainer { {ResetRequestTypeInfo-Partial-ExtIEs} } OPTIONAL,

...

}

ResetRequestTypeInfo-Partial-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetRequestPartialReleaseList ::= SEQUENCE (SIZE(1..maxnoofUEContexts)) OF ResetRequestPartialReleaseItem

ResetRequestPartialReleaseItem ::= SEQUENCE {

ng-ran-node1UEXnAPID						NG-RANnodeUEXnAPID			OPTIONAL,

ng-ran-node2UEXnAPID						NG-RANnodeUEXnAPID			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {ResetRequestPartialReleaseItem-ExtIEs} } OPTIONAL,

...

}

ResetRequestPartialReleaseItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetResponseTypeInfo ::= CHOICE {

fullReset			ResetResponseTypeInfo-Full,

partialReset		ResetResponseTypeInfo-Partial,

choice-extension	ProtocolIE-Single-Container { {ResetResponseTypeInfo-ExtIEs} }

}

ResetResponseTypeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ResetResponseTypeInfo-Full ::= SEQUENCE {

iE-Extension					ProtocolExtensionContainer { {ResetResponseTypeInfo-Full-ExtIEs} } OPTIONAL,

...

}

ResetResponseTypeInfo-Full-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetResponseTypeInfo-Partial ::= SEQUENCE {

ue-contexts-AdmittedToBeReleasedList	ResetResponsePartialReleaseList,

iE-Extension					ProtocolExtensionContainer { {ResetResponseTypeInfo-Partial-ExtIEs} } OPTIONAL,

...

}

ResetResponseTypeInfo-Partial-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ResetResponsePartialReleaseList ::= SEQUENCE (SIZE(1..maxnoofUEContexts)) OF ResetResponsePartialReleaseItem

ResetResponsePartialReleaseItem ::= SEQUENCE {

ng-ran-node1UEXnAPID						NG-RANnodeUEXnAPID		OPTIONAL,

ng-ran-node2UEXnAPID						NG-RANnodeUEXnAPID		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {ResetResponsePartialReleaseItem-ExtIEs} } OPTIONAL,

...

}

ResetResponsePartialReleaseItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RLCMode	::= ENUMERATED {

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

RLC-Status ::= SEQUENCE {

reestablishment-Indication 	Reestablishment-Indication,

iE-Extensions					ProtocolExtensionContainer { {RLC-Status-ExtIEs} } OPTIONAL,

...

}

RLC-Status-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RLCDuplicationInformation ::= 		SEQUENCE {

rLCDuplicationStateList 		RLCDuplicationStateList,

rLC-PrimaryIndicator 		ENUMERATED {true, false}		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RLCDuplicationInformation-ItemExtIEs} }	OPTIONAL

}

RLCDuplicationInformation-ItemExtIEs 	XNAP-PROTOCOL-EXTENSION ::= {

...

}

RLCDuplicationStateList	::= 	SEQUENCE (SIZE(1..maxnoofRLCDuplicationstate)) OF RLCDuplicationState-Item

RLCDuplicationState-Item ::=	SEQUENCE {

duplicationState			ENUMERATED {active,inactive, ...},

iE-Extensions	ProtocolExtensionContainer { {RLCDuplicationState-ItemExtIEs } }	OPTIONAL,

...

}

RLCDuplicationState-ItemExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Reestablishment-Indication ::= ENUMERATED {

reestablished,

...

}

RFSP-Index ::= INTEGER (1..256)

RRCConfigIndication ::= ENUMERATED {

full-config,

delta-config,

...

}

RRCConnections::= SEQUENCE {

noofRRCConnections                   	NoofRRCConnections,

availableRRCConnectionCapacityValue	AvailableRRCConnectionCapacityValue,

iE-Extensions				ProtocolExtensionContainer { { RRCConnections-ExtIEs} }	OPTIONAL,

...

}

RRCConnections-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCReestab-initiated ::= SEQUENCE {

rRRCReestab-initiated-reporting	RRCReestab-Initiated-Reporting,

iE-Extensions			ProtocolExtensionContainer { { RRCReestab-initiated-ExtIEs} } OPTIONAL,

...

}

RRCReestab-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCReestab-Initiated-Reporting ::= CHOICE {

rRCReestab-reporting-wo-UERLFReport					RRCReestab-Initiated-Reporting-wo-UERLFReport,

rRCReestab-reporting-with-UERLFReport				RRCReestab-Initiated-Reporting-with-UERLFReport,

choice-extension				ProtocolIE-Single-Container { {RRCReestab-Initiated-Reporting-ExtIEs} }

}

RRCReestab-Initiated-Reporting-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RRCReestab-Initiated-Reporting-wo-UERLFReport ::= SEQUENCE {

failureCellPCI		NG-RAN-CellPCI,

reestabCellCGI		GlobalNG-RANCell-ID,

c-RNTI				C-RNTI,

shortMAC-I			MAC-I,

iE-Extensions		ProtocolExtensionContainer { { RRCReestab-Initiated-Reporting-wo-UERLFReport-ExtIEs} } OPTIONAL,

...

}

RRCReestab-Initiated-Reporting-wo-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCReestab-Initiated-Reporting-with-UERLFReport ::= SEQUENCE {

uERLFReportContainer	UERLFReportContainer,

iE-Extensions			ProtocolExtensionContainer { {RRCReestab-Initiated-Reporting-with-UERLFReport-ExtIEs} } OPTIONAL,

...

}

RRCReestab-Initiated-Reporting-with-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCSetup-initiated ::= SEQUENCE {

rRRCSetup-Initiated-Reporting	RRCSetup-Initiated-Reporting,

uERLFReportContainer    UERLFReportContainer			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RRCSetup-initiated-ExtIEs} } OPTIONAL,

...

}

RRCSetup-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCSetup-Initiated-Reporting ::= CHOICE {

rRCSetup-reporting-with-UERLFReport				RRCSetup-Initiated-Reporting-with-UERLFReport,

choice-extension				ProtocolIE-Single-Container { {RRCSetup-Initiated-Reporting-ExtIEs} }

}

RRCSetup-Initiated-Reporting-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RRCSetup-Initiated-Reporting-with-UERLFReport ::= SEQUENCE {

uERLFReportContainer	UERLFReportContainer,

iE-Extensions			ProtocolExtensionContainer { {RRCSetup-Initiated-Reporting-with-UERLFReport-ExtIEs} } OPTIONAL,

...

}

RRCSetup-Initiated-Reporting-with-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCResumeCause ::= ENUMERATED {

rna-Update,

...

}

#### -- S

SecondarydataForwardingInfoFromTarget-Item::= SEQUENCE {

secondarydataForwardingInfoFromTarget				DataForwardingInfoFromTargetNGRANnode,

iE-Extensions		ProtocolExtensionContainer { { SecondarydataForwardingInfoFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

SecondarydataForwardingInfoFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecondarydataForwardingInfoFromTarget-List ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF SecondarydataForwardingInfoFromTarget-Item

SCGConfigurationQuery	::= ENUMERATED {true, ...}

SCGIndicator	::=	ENUMERATED{released, ...}

SecondaryRATUsageInformation ::= SEQUENCE {

pDUSessionUsageReport		PDUSessionUsageReport				OPTIONAL,

qosFlowsUsageReportList		QoSFlowsUsageReportList				OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {SecondaryRATUsageInformation-ExtIEs} }	OPTIONAL,

...

}

SecondaryRATUsageInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityIndication ::= SEQUENCE {

integrityProtectionIndication			ENUMERATED {required, preferred, not-needed, ...},

confidentialityProtectionIndication		ENUMERATED {required, preferred, not-needed, ...},

maximumIPdatarate						MaximumIPdatarate												OPTIONAL,

-- This IE shall be present if the Integrity Protection IE within the Security Indication IE is present and set to "required" or "preferred". --

iE-Extensions							ProtocolExtensionContainer { {SecurityIndication-ExtIEs} } OPTIONAL,

...

}

SecurityIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityResult ::= SEQUENCE {

integrityProtectionResult				ENUMERATED {performed, not-performed, ...},

confidentialityProtectionResult			ENUMERATED {performed, not-performed, ...},

iE-Extensions							ProtocolExtensionContainer { {SecurityResult-ExtIEs} } OPTIONAL,

...

}

SecurityResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasurementConfiguration ::= SEQUENCE {

sensorMeasConfig 				SensorMeasConfig,

sensorMeasConfigNameList		SensorMeasConfigNameList            OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SensorMeasurementConfiguration-ExtIEs } } OPTIONAL,

...

}

SensorMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofSensorName)) OF SensorName

SensorMeasConfig::= ENUMERATED {setup,...}

SensorName ::= SEQUENCE {

uncompensatedBarometricConfig	ENUMERATED {true, ...}         OPTIONAL,

ueSpeedConfig					ENUMERATED {true, ...}         OPTIONAL,

ueOrientationConfig				ENUMERATED {true, ...}         OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SensorNameConfig-ExtIEs} } OPTIONAL,

...

}

SensorNameConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

##### -- Served Cells E-UTRA IEs

ServedCellInformation-E-UTRA ::= SEQUENCE {

e-utra-pci								E-UTRAPCI,

e-utra-cgi								E-UTRA-CGI,

tac										TAC,

ranac									RANAC																					OPTIONAL,

broadcastPLMNs							SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedCellInformation-E-UTRA-perBPLMN,

e-utra-mode-info						ServedCellInformation-E-UTRA-ModeInfo,

numberofAntennaPorts					NumberOfAntennaPorts-E-UTRA																OPTIONAL,

prach-configuration						E-UTRAPRACHConfiguration																OPTIONAL,

mBSFNsubframeInfo						MBSFNSubframeInfo-E-UTRA																OPTIONAL,

multibandInfo							E-UTRAMultibandInfoList																	OPTIONAL,

freqBandIndicatorPriority				ENUMERATED {not-broadcast, broadcast, ...} 												OPTIONAL,

bandwidthReducedSI						ENUMERATED {scheduled, ...}																OPTIONAL,

protectedE-UTRAResourceIndication		ProtectedE-UTRAResourceIndication														OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-ExtIEs} }					OPTIONAL,

...

}

ServedCellInformation-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-EUTRA		CRITICALITY ignore	EXTENSION BPLMN-ID-Info-EUTRA		PRESENCE optional }|

{ ID id-NPRACHConfiguration		CRITICALITY ignore	EXTENSION	NPRACHConfiguration		PRESENCE optional},

...

}

ServedCellInformation-E-UTRA-perBPLMN ::= SEQUENCE {

plmn-id					PLMN-Identity,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-perBPLMN-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-perBPLMN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellInformation-E-UTRA-ModeInfo ::= CHOICE {

fdd					ServedCellInformation-E-UTRA-FDDInfo,

tdd					ServedCellInformation-E-UTRA-TDDInfo,

choice-extension	ProtocolIE-Single-Container{ {ServedCellInformation-E-UTRA-ModeInfo-ExtIEs} }

}

ServedCellInformation-E-UTRA-ModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ServedCellInformation-E-UTRA-FDDInfo ::= SEQUENCE {

ul-earfcn			E-UTRAARFCN,

dl-earfcn			E-UTRAARFCN,

ul-e-utraTxBW		E-UTRATransmissionBandwidth,

dl-e-utraTxBW		E-UTRATransmissionBandwidth,

iE-Extensions		ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-FDDInfo-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-FDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-OffsetOfNbiotChannelNumberToUL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional},

...

}

ServedCellInformation-E-UTRA-TDDInfo ::= SEQUENCE {

earfcn					E-UTRAARFCN,

e-utraTxBW				E-UTRATransmissionBandwidth,

subframeAssignmnet		ENUMERATED {sa0,sa1,sa2,sa3,sa4,sa5,sa6,...},

specialSubframeInfo		SpecialSubframeInfo-E-UTRA,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-TDDInfo-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-TDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-NBIoT-UL-DL-AlignmentOffset				CRITICALITY reject	EXTENSION NBIoT-UL-DL-AlignmentOffset					PRESENCE optional},

...

}

ServedCells-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-E-UTRA-Item

ServedCells-E-UTRA-Item ::= SEQUENCE {

served-cell-info-E-UTRA		ServedCellInformation-E-UTRA,

neighbour-info-NR			NeighbourInformation-NR									OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCells-E-UTRA-Item-ExtIEs} } OPTIONAL,

...

}

ServedCells-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SFN-Offset				CRITICALITY ignore EXTENSION SFN-Offset					PRESENCE optional },

...

}

ServedCellsToUpdate-E-UTRA ::= SEQUENCE {

served-Cells-ToAdd-E-UTRA		ServedCells-E-UTRA																OPTIONAL,

served-Cells-ToModify-E-UTRA	ServedCells-ToModify-E-UTRA														OPTIONAL,

served-Cells-ToDelete-E-UTRA	SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI 					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedCellsToUpdate-E-UTRA-ExtIEs} }	OPTIONAL,

...

}

ServedCellsToUpdate-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCells-ToModify-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-ToModify-E-UTRA-Item

ServedCells-ToModify-E-UTRA-Item ::= SEQUENCE {

old-ECGI					E-UTRA-CGI,

served-cell-info-E-UTRA		ServedCellInformation-E-UTRA,

neighbour-info-NR			NeighbourInformation-NR											OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA										OPTIONAL,

deactivation-indication		ENUMERATED {deactivated, ...}									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Served-cells-ToModify-E-UTRA-Item-ExtIEs} }	OPTIONAL,

...

}

Served-cells-ToModify-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

##### -- Served Cells NR IEs

ServedCellInformation-NR ::= SEQUENCE {

nrPCI								NRPCI,

cellID								NR-CGI,

tac									TAC,

ranac								RANAC						OPTIONAL,

broadcastPLMN						BroadcastPLMNs,

nrModeInfo							NRModeInfo,

measurementTimingConfiguration		OCTET STRING,

connectivitySupport					Connectivity-Support,

iE-Extensions						ProtocolExtensionContainer { {ServedCellInformation-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-NR				CRITICALITY ignore	EXTENSION BPLMN-ID-Info-NR						PRESENCE optional }|

{ ID id-ConfiguredTACIndication			CRITICALITY ignore	EXTENSION ConfiguredTACIndication				PRESENCE optional }|

{ ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst					PRESENCE optional }|

{ ID id-NRCellPRACHConfig				CRITICALITY ignore	EXTENSION NRCellPRACHConfig						PRESENCE optional }|

{ ID id-NPN-Broadcast-Information		CRITICALITY reject	EXTENSION NPN-Broadcast-Information			PRESENCE optional }|

{ ID id-CSI-RSTransmissionIndication	CRITICALITY ignore	EXTENSION CSI-RSTransmissionIndication		PRESENCE optional } |

{ ID id-SFN-Offset						CRITICALITY ignore	EXTENSION SFN-Offset							PRESENCE optional },

...

}

SFN-Offset ::= SEQUENCE {

sFN-Time-Offset					BIT STRING (SIZE(24)),

iE-Extensions		ProtocolExtensionContainer { {SFN-Offset-ExtIEs} } OPTIONAL,

...

}

SFN-Offset-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCells-NR ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-NR-Item

ServedCells-NR-Item ::= SEQUENCE {

served-cell-info-NR			ServedCellInformation-NR,

neighbour-info-NR			NeighbourInformation-NR				OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCells-NR-Item-ExtIEs} } OPTIONAL,

...

}

ServedCells-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCells-ToModify-NR ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-ToModify-NR-Item

ServedCells-ToModify-NR-Item ::= SEQUENCE {

old-NR-CGI					NR-CGI,

served-cell-info-NR			ServedCellInformation-NR,

neighbour-info-NR			NeighbourInformation-NR				 							OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA										OPTIONAL,

deactivation-indication		ENUMERATED {deactivated, ...}									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Served-cells-ToModify-NR-Item-ExtIEs} } 	OPTIONAL,

...

}

Served-cells-ToModify-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellsToUpdate-NR ::= SEQUENCE {

served-Cells-ToAdd-NR		ServedCells-NR															OPTIONAL,

served-Cells-ToModify-NR	ServedCells-ToModify-NR													OPTIONAL,

served-Cells-ToDelete-NR	SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF NR-CGI 				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCellsToUpdate-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellsToUpdate-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SharedResourceType ::= CHOICE {

ul-onlySharing				SharedResourceType-UL-OnlySharing,

ul-and-dl-Sharing			SharedResourceType-ULDL-Sharing,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ExtIEs} }

}

SharedResourceType-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-UL-OnlySharing ::= SEQUENCE {

ul-resourceBitmap			DataTrafficResources,

iE-Extensions			ProtocolExtensionContainer { {SharedResourceType-UL-OnlySharing-ExtIEs} } OPTIONAL,

...

}

SharedResourceType-UL-OnlySharing-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SharedResourceType-ULDL-Sharing ::= CHOICE {

ul-resources				SharedResourceType-ULDL-Sharing-UL-Resources,

dl-resources				SharedResourceType-ULDL-Sharing-DL-Resources,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-ExtIEs} }

}

SharedResourceType-ULDL-Sharing-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-ULDL-Sharing-UL-Resources ::= CHOICE {

unchanged					NULL,

changed						SharedResourceType-ULDL-Sharing-UL-ResourcesChanged,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-UL-Resources-ExtIEs} }

}

SharedResourceType-ULDL-Sharing-UL-Resources-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-ULDL-Sharing-UL-ResourcesChanged ::= SEQUENCE {

ul-resourceBitmap			DataTrafficResources,

iE-Extensions			ProtocolExtensionContainer { {SharedResourceType-ULDL-Sharing-UL-ResourcesChanged-ExtIEs} } OPTIONAL,

...

}

SharedResourceType-ULDL-Sharing-UL-ResourcesChanged-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SharedResourceType-ULDL-Sharing-DL-Resources ::= CHOICE {

unchanged					NULL,

changed						SharedResourceType-ULDL-Sharing-DL-ResourcesChanged,

choice-extension			ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-DL-Resources-ExtIEs} }

}

SharedResourceType-ULDL-Sharing-DL-Resources-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SharedResourceType-ULDL-Sharing-DL-ResourcesChanged ::= SEQUENCE {

dl-resourceBitmap			DataTrafficResources,

iE-Extensions			ProtocolExtensionContainer { {SharedResourceType-ULDL-Sharing-DL-ResourcesChanged-ExtIEs} } OPTIONAL,

...

}

SharedResourceType-ULDL-Sharing-DL-ResourcesChanged-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceAvailableCapacity ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceAvailableCapacity-Item

SliceAvailableCapacity-Item	::= SEQUENCE {

pLMNIdentity						PLMN-Identity,

sNSSAIAvailableCapacity-List		SNSSAIAvailableCapacity-List,

iE-Extensions						ProtocolExtensionContainer { { SliceAvailableCapacity-Item-ExtIEs} }	OPTIONAL,

...

}

SliceAvailableCapacity-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIAvailableCapacity-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAIAvailableCapacity-Item

SNSSAIAvailableCapacity-Item ::= SEQUENCE {

sNSSAI		S-NSSAI,

sliceAvailableCapacityValueDownlink	INTEGER (0..100),

sliceAvailableCapacityValueUplink	INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { SNSSAIAvailableCapacity-Item-ExtIEs } }	OPTIONAL

}

SNSSAIAvailableCapacity-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceSupport-List	::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF S-NSSAI

SliceToReport-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceToReport-List-Item

SliceToReport-List-Item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions						ProtocolExtensionContainer { { SliceToReport-List-Item-ExtIEs} }	OPTIONAL,

...

}

SliceToReport-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI-list ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAI-Item

SNSSAI-Item ::= SEQUENCE {

sNSSAI		S-NSSAI,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-Item-ExtIEs } }	OPTIONAL

}

SNSSAI-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

SlotConfiguration-List ::= SEQUENCE (SIZE (1..maxnoofslots)) OF SlotConfiguration-List-Item

SlotConfiguration-List-Item ::= SEQUENCE {

slotIndex						INTEGER (0..5119),

symbolAllocation-in-Slot		SymbolAllocation-in-Slot,

iE-Extensions				ProtocolExtensionContainer { {SlotConfiguration-List-Item-ExtIEs} }	OPTIONAL,

...

}

SlotConfiguration-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

S-NG-RANnode-SecurityKey ::= BIT STRING (SIZE(256))

S-NG-RANnode-Addition-Trigger-Ind ::= ENUMERATED {

sn-change,

inter-MN-HO,

intra-MN-HO,

...

}

S-NSSAI ::= SEQUENCE {

sst						OCTET STRING (SIZE(1)),

sd						OCTET STRING (SIZE(3))							OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {S-NSSAI-ExtIEs} } OPTIONAL,

...

}

S-NSSAI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNTriggered ::=ENUMERATED{

true,

...

}

SpecialSubframeInfo-E-UTRA ::= SEQUENCE {

specialSubframePattern	SpecialSubframePatterns-E-UTRA,

cyclicPrefixDL			CyclicPrefix-E-UTRA-DL,

cyclicPrefixUL			CyclicPrefix-E-UTRA-UL,

iE-Extensions			ProtocolExtensionContainer { {SpecialSubframeInfo-E-UTRA-ExtIEs} } OPTIONAL,

...

}

SpecialSubframeInfo-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SpecialSubframePatterns-E-UTRA ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

ssp9,

ssp10,

...

}

SpectrumSharingGroupID ::= INTEGER (1..maxnoofCellsinNG-RANnode)

SplitSessionIndicator ::= ENUMERATED {

split,

...

}

SplitSRBsTypes ::= ENUMERATED {srb1, srb2, srb1and2, ...}

SSBAreaCapacityValue-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaCapacityValue-List-Item

SSBAreaCapacityValue-List-Item	::= SEQUENCE {

sSBIndex				INTEGER(0..63),

ssbAreaCapacityValue	INTEGER (0..100),

iE-Extensions						ProtocolExtensionContainer { { SSBAreaCapacityValue-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaCapacityValue-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SSBAreaRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaRadioResourceStatus-List-Item

SSBAreaRadioResourceStatus-List-Item	::= SEQUENCE {

sSBIndex								INTEGER(0..63),

ssb-Area-DL-GBR-PRB-usage	DL-GBR-PRB-usage,

ssb-Area-UL-GBR-PRB-usage	UL-GBR-PRB-usage,

ssb-Area-dL-non-GBR-PRB-usage		DL-non-GBR-PRB-usage,

ssb-Area-uL-non-GBR-PRB-usage		UL-non-GBR-PRB-usage,

ssb-Area-dL-Total-PRB-usage			DL-Total-PRB-usage,

ssb-Area-uL-Total-PRB-usage			UL-Total-PRB-usage,

iE-Extensions						ProtocolExtensionContainer { { SSBAreaRadioResourceStatus-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBAreaRadioResourceStatus-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION DL-scheduling-PDCCH-CCE-usage	PRESENCE optional}|

{ ID id-UL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION UL-scheduling-PDCCH-CCE-usage	PRESENCE optional},

...

}

SSB-PositionsInBurst ::= CHOICE {

shortBitmap						BIT STRING (SIZE (4)),

mediumBitmap					BIT STRING (SIZE (8)),

longBitmap						BIT STRING (SIZE (64)),

choice-extension				ProtocolIE-Single-Container { {SSB-PositionsInBurst-ExtIEs} }

}

SSB-PositionsInBurst-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SSBToReport-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBToReport-List-Item

SSBToReport-List-Item	::= SEQUENCE {

sSBIndex				INTEGER(0..63),

iE-Extensions						ProtocolExtensionContainer { { SSBToReport-List-Item-ExtIEs} }	OPTIONAL,

...

}

SSBToReport-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SUL-FrequencyBand ::= INTEGER (1..1024)

SUL-Information ::= SEQUENCE {

sulFrequencyInfo			NRARFCN,

sulTransmissionBandwidth	NRTransmissionBandwidth,

iE-Extensions				ProtocolExtensionContainer { {SUL-Information-ExtIEs} } OPTIONAL,

...

}

SUL-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional }|

{ ID id-FrequencyShift7p5khz	CRITICALITY ignore	EXTENSION FrequencyShift7p5khz	PRESENCE optional },

...

}

SupportedSULBandList ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF SupportedSULBandItem

SupportedSULBandItem ::= SEQUENCE {

sulBandItem					SUL-FrequencyBand,

iE-Extensions				ProtocolExtensionContainer { {SupportedSULBandItem-ExtIEs} } OPTIONAL,

...

}

SupportedSULBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SymbolAllocation-in-Slot ::= CHOICE {

allDL				SymbolAllocation-in-Slot-AllDL,

allUL				SymbolAllocation-in-Slot-AllUL,

bothDLandUL			SymbolAllocation-in-Slot-BothDLandUL,

choice-extension	ProtocolIE-Single-Container { {SymbolAllocation-in-Slot-ExtIEs} }

}

SymbolAllocation-in-Slot-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SymbolAllocation-in-Slot-AllDL ::= SEQUENCE {

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-AllDL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-AllDL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SymbolAllocation-in-Slot-AllUL ::= SEQUENCE {

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-AllUL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-AllUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SymbolAllocation-in-Slot-BothDLandUL ::= SEQUENCE {

numberofDLSymbols	INTEGER (0..13),

numberofULSymbols	INTEGER (0..13),

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-BothDLandUL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-BothDLandUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

TAIBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAIforMDT-Item

TAIforMDT-Item ::= SEQUENCE {

plmn-ID				PLMN-Identity,

tAC						TAC,

iE-Extensions			ProtocolExtensionContainer { {TAIforMDT-Item-ExtIEs} } OPTIONAL,

...

}

TAIforMDT-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAC ::= OCTET STRING (SIZE (3))

TAISupport-List	::= SEQUENCE (SIZE(1..maxnoofsupportedTACs)) OF TAISupport-Item

TAISupport-Item ::= SEQUENCE {

tac								TAC,

broadcastPLMNs					SEQUENCE (SIZE(1..maxnoofsupportedPLMNs)) OF BroadcastPLMNinTAISupport-Item,

iE-Extensions					ProtocolExtensionContainer { {TAISupport-Item-ExtIEs} } OPTIONAL,

...

}

TAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC

TargetCellinEUTRAN  ::= OCTET STRING -- This IE is to be encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in in TS 36.413 [31]

Target-CGI ::= CHOICE {

nr							NR-CGI,

e-utra						E-UTRA-CGI,

choice-extension			ProtocolIE-Single-Container { {TargetCGI-ExtIEs} }

}

TargetCGI-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

TDDULDLConfigurationCommonNR ::= OCTET STRING

TargetCellList ::= SEQUENCE (SIZE(1..maxnoofCHOcells)) OF TargetCellList-Item

TargetCellList-Item ::= SEQUENCE {

target-cell								Target-CGI,

iE-Extensions							ProtocolExtensionContainer { { TargetCellList-Item-ExtIEs} } OPTIONAL

}

TargetCellList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Threshold-RSRQ ::= INTEGER(0..34)

Threshold-RSRP ::= INTEGER(0..97)

Threshold-SINR ::= INTEGER(0..127)

TimeToTrigger ::= ENUMERATED {ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120}

TimeToWait ::= ENUMERATED {

v1s,

v2s,

v5s,

v10s,

v20s,

v60s,

...

}

TNLConfigurationInfo ::= SEQUENCE {

extendedUPTransportLayerAddressesToAdd			ExtTLAs										OPTIONAL,

extendedUPTransportLayerAddressesToRemove		ExtTLAs										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TNLConfigurationInfo-ExtIEs} }	OPTIONAL,

...

}

TNLConfigurationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Add-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Add-Item

TNLA-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Add-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Add-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Update-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Update-Item

TNLA-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

tNLAssociationUsage						TNLAssociationUsage 	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Update-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Update-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-To-Remove-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Remove-Item

TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

iE-Extensions							ProtocolExtensionContainer { { TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

TNLA-To-Remove-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Setup-Item

TNLA-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

iE-Extensions							ProtocolExtensionContainer { { TNLA-Setup-Item-ExtIEs} } OPTIONAL,

...

}

TNLA-Setup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLA-Failed-To-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Failed-To-Setup-Item

TNLA-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { TNLA-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

TNLA-Failed-To-Setup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TransportLayerAddress ::= BIT STRING (SIZE(1..160, ...))

TraceActivation ::= SEQUENCE {

ng-ran-TraceID			NG-RANTraceID,

interfaces-to-trace 	BIT STRING { ng-c (0), x-nc (1), uu (2), f1-c (3), e1 (4)} (SIZE(8)),

trace-depth 			Trace-Depth,

trace-coll-address		TransportLayerAddress,

ie-Extension 			ProtocolExtensionContainer { {TraceActivation-ExtIEs} } OPTIONAL,

...

}

TraceActivation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

-- Extension to support MDT –

{ ID id-TraceCollectionEntityURI	CRITICALITY ignore	EXTENSION URIaddress				PRESENCE optional}|

{ ID id-MDT-Configuration			CRITICALITY ignore	EXTENSION MDT-Configuration			PRESENCE optional},

...

}

Trace-Depth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TSCTrafficCharacteristics ::= SEQUENCE {

tSCAssistanceInformationDownlink	TSCAssistanceInformation OPTIONAL,

tSCAssistanceInformationUplink		TSCAssistanceInformation OPTIONAL,

ie-Extension 			ProtocolExtensionContainer { {TSCTrafficCharacteristics-ExtIEs} } OPTIONAL,

...

}

TSCTrafficCharacteristics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TSCAssistanceInformation ::= SEQUENCE {

periodicity			INTEGER (0.. 640000, ...),

burstArrivalTime	OCTET STRING				OPTIONAL,

ie-Extension 			ProtocolExtensionContainer { { TSCAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

UEAggregateMaximumBitRate ::= SEQUENCE {

dl-UE-AMBR				BitRate,

ul-UE-AMBR				BitRate,

iE-Extension			ProtocolExtensionContainer { {UEAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

UEAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextKeptIndicator ::= ENUMERATED {true, ...}

UEContextID ::= CHOICE {

rRCResume				UEContextIDforRRCResume,

rRRCReestablishment		UEContextIDforRRCReestablishment,

choice-extension		ProtocolIE-Single-Container { {UEContextID-ExtIEs} }

}

UEContextID-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UEContextIDforRRCResume ::= SEQUENCE {

i-rnti					I-RNTI,

allocated-c-rnti			C-RNTI,

accessPCI				NG-RAN-CellPCI,

iE-Extension			ProtocolExtensionContainer { {UEContextIDforRRCResume-ExtIEs} } OPTIONAL,

...

}

UEContextIDforRRCResume-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextIDforRRCReestablishment ::= SEQUENCE {

c-rnti					C-RNTI,

failureCellPCI			NG-RAN-CellPCI,

iE-Extension			ProtocolExtensionContainer { {UEContextIDforRRCReestablishment-ExtIEs} } OPTIONAL,

...

}

UEContextIDforRRCReestablishment-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextInfoRetrUECtxtResp ::= SEQUENCE {

ng-c-UE-signalling-ref					AMF-UE-NGAP-ID,

signalling-TNL-at-source				CPTransportLayerInformation,

ueSecurityCapabilities					UESecurityCapabilities,

securityInformation						AS-SecurityInformation,

ue-AMBR									UEAggregateMaximumBitRate,

pduSessionResourcesToBeSetup-List		PDUSessionResourcesToBeSetup-List,

rrc-Context								OCTET STRING,

mobilityRestrictionList					MobilityRestrictionList									OPTIONAL,

indexToRatFrequencySelectionPriority	RFSP-Index												OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {UEContextInfoRetrUECtxtResp-ExtIEs} } 	OPTIONAL,

...

}

UEContextInfoRetrUECtxtResp-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-FiveGCMobilityRestrictionListContainer 	CRITICALITY ignore	EXTENSION FiveGCMobilityRestrictionListContainer		PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitRate		CRITICALITY ignore	EXTENSION NRUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION LTEUESidelinkAggregateMaximumBitRate			PRESENCE optional }|

{ ID id-UERadioCapabilityID						CRITICALITY reject	EXTENSION UERadioCapabilityID								PRESENCE optional }|

{ ID id-UESliceMaximumBitRateList				CRITICALITY reject	EXTENSION UESliceMaximumBitRateList							PRESENCE optional },

...

}

UEHistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCellsinUEHistoryInfo)) OF LastVisitedCell-Item

UEHistoryInformationFromTheUE ::= CHOICE {

nR						NRMobilityHistoryReport,

choice-extension			ProtocolIE-Single-Container { {UEHistoryInformationFromTheUE-ExtIEs} }

}

UEHistoryInformationFromTheUE-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UEIdentityIndexValue ::= CHOICE {

indexLength10				BIT STRING (SIZE(10)),

choice-extension			ProtocolIE-Single-Container { {UEIdentityIndexValue-ExtIEs} }

}

UEIdentityIndexValue-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UERadioCapabilityForPaging ::= SEQUENCE {

uERadioCapabilityForPagingOfNR			UERadioCapabilityForPagingOfNR			OPTIONAL,

uERadioCapabilityForPagingOfEUTRA		UERadioCapabilityForPagingOfEUTRA		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UERadioCapabilityForPaging-ExtIEs} }	OPTIONAL,

...

}

UERadioCapabilityForPaging-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UERadioCapabilityForPagingOfNR ::= OCTET STRING

UERadioCapabilityForPagingOfEUTRA ::= OCTET STRING

UERadioCapabilityID ::= OCTET STRING

UERANPagingIdentity ::= CHOICE {

i-RNTI-full			BIT STRING ( SIZE (40)),

choice-extension	ProtocolIE-Single-Container { {UERANPagingIdentity-ExtIEs} }

}

UERANPagingIdentity-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UERLFReportContainer ::= CHOICE {

nR-UERLFReportContainer			UERLFReportContainerNR,

lTE-UERLFReportContainer		UERLFReportContainerLTE,

choice-Extension		ProtocolIE-Single-Container { {UERLFReportContainer-ExtIEs} }

}

UERLFReportContainer-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UERLFReportContainerLTE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-r9 IE contained in the UEInformationResponse message (TS 36.331 [14])

UERLFReportContainerNR ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the nr-RLF-Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10])

UESliceMaximumBitRateList ::= SEQUENCE (SIZE(1.. maxnoofSMBR)) OF UESliceMaximumBitRate-Item

UESliceMaximumBitRate-Item ::= SEQUENCE {

s-NSSAI						S-NSSAI,

dl-UE-Slice-MBR				BitRate,

ul-UE-Slice-MBR				BitRate,

iE-Extensions						ProtocolExtensionContainer { { UESliceMaximumBitRate-Item-ExtIEs} } OPTIONAL,

...

}

UESliceMaximumBitRate-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UESecurityCapabilities ::= SEQUENCE {

nr-EncyptionAlgorithms					BIT STRING {nea1-128(1),

nea2-128(2),

nea3-128(3)} (SIZE(16, ...)),

nr-IntegrityProtectionAlgorithms		BIT STRING {nia1-128(1),

nia2-128(2),

nia3-128(3)} (SIZE(16, ...)),

e-utra-EncyptionAlgorithms				BIT STRING {eea1-128(1),

eea2-128(2),

eea3-128(3)} (SIZE(16, ...)),

e-utra-IntegrityProtectionAlgorithms	BIT STRING {eia1-128(1),

eia2-128(2),

eia3-128(3)} (SIZE(16, ...)),

iE-Extension			ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} } OPTIONAL,

...

}

UESecurityCapabilities-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UESpecificDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

ULConfiguration::= SEQUENCE {

uL-PDCP							UL-UE-Configuration,

iE-Extensions					ProtocolExtensionContainer { {ULConfiguration-ExtIEs} } OPTIONAL,

...

}

ULConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UL-UE-Configuration::= ENUMERATED {no-data, shared, only, ...}

ULForwarding	::= ENUMERATED {ul-forwarding-proposed, ...}

ULForwardingProposal	::= ENUMERATED {ul-forwarding-proposed, ...}

UL-GBR-PRB-usage::= INTEGER (0..100)

UL-non-GBR-PRB-usage::= INTEGER (0..100)

UL-Total-PRB-usage::= INTEGER (0..100)

UPTransportLayerInformation ::= CHOICE {

gtpTunnel					GTPtunnelTransportLayerInformation,

choice-extension			ProtocolIE-Single-Container { {UPTransportLayerInformation-ExtIEs} }

}

UPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UPTransportParameters ::= SEQUENCE (SIZE(1..maxnoofSCellGroupsplus1)) OF UPTransportParametersItem

UPTransportParametersItem ::= SEQUENCE {

upTNLInfo		UPTransportLayerInformation,

cellGroupID		CellGroupID,

iE-Extension	ProtocolExtensionContainer { {UPTransportParametersItem-ExtIEs} } OPTIONAL,

...

}

UPTransportParametersItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-QoS-Mapping-Information	CRITICALITY reject	EXTENSION QoS-Mapping-Information	PRESENCE optional},

...

}

UserPlaneTrafficActivityReport ::= ENUMERATED {inactive, re-activated, ...}

URIaddress ::= VisibleString

#### -- V

VehicleUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

VolumeTimedReportList ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF VolumeTimedReport-Item

VolumeTimedReport-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp					OCTET STRING (SIZE(4)),

usageCountUL					INTEGER (0..18446744073709551615),

usageCountDL					INTEGER (0..18446744073709551615),

iE-Extensions				ProtocolExtensionContainer { {VolumeTimedReport-Item-ExtIEs} } OPTIONAL,

...

}

VolumeTimedReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- W

WLANMeasurementConfiguration ::= SEQUENCE {

wlanMeasConfig             WLANMeasConfig,

wlanMeasConfigNameList		WLANMeasConfigNameList            OPTIONAL,

wlan-rssi                  ENUMERATED {true, ...}            OPTIONAL,

wlan-rtt                   ENUMERATED {true, ...}            OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { WLANMeasurementConfiguration-ExtIEs } } OPTIONAL,

...

}

WLANMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofWLANName)) OF WLANName

WLANMeasConfig::= ENUMERATED {setup,...}

WLANName ::= OCTET STRING (SIZE (1..32))

#### -- X

XnBenefitValue ::= INTEGER (1..8, ...)

#### -- Y

#### -- Z

END

-- ASN1STOP

### 9.3.6	Common definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

XnAP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

#### -- Common Data Types

--

-- **************************************************************

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0.. maxPrivateIEs),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs						INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI 							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRACHReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN    	INTEGER ::= 256

maxnoofSMBR									INTEGER ::= 8

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold 																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs 																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 																				ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse																				ProtocolIE-ID ::= 133

id-SplitSessionIndicator																			ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent																	ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing																		ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= 137

id-ULForwardingProposal																				ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort 																		ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR																ProtocolIE-ID ::= 140

id-TNLConfigurationInfo																				ProtocolIE-ID ::= 141

id-PartialListIndicator-NR																			ProtocolIE-ID ::= 142

id-MessageOversizeNotification																		ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR																	ProtocolIE-ID ::= 144

id-NG-RANTraceID																					ProtocolIE-ID ::= 145

id-NonGBRResources-Offered																			ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN																ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																				ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= 158

id-CHOinformation-Ack																				ProtocolIE-ID ::= 159

id-targetCellsToCancel																				ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= 161

id-procedureStage																					ProtocolIE-ID ::= 162

id-DAPSRequestInfo																					ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List																			ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator																				ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN															ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN															ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset																		ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized																			ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized																			ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 172

id-PC5QoSParameters																					ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList																		ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex																			ProtocolIE-ID ::= 175

id-MobilityInformation 																				ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication															ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE																	ProtocolIE-ID ::= 178

id-HandoverReportType																				ProtocolIE-ID ::= 179

id-HandoverCause																					ProtocolIE-ID ::= 180

id-SourceCellCGI																					ProtocolIE-ID ::= 181

id-TargetCellCGI																					ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI																			ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN																				ProtocolIE-ID ::= 184

id-SourceCellCRNTI																					ProtocolIE-ID ::= 185

id-UERLFReportContainer																				ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID																		ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID																		ProtocolIE-ID ::= 188

id-RegistrationRequest																				ProtocolIE-ID ::= 189

id-ReportCharacteristics																				ProtocolIE-ID ::= 190

id-CellToReport																								ProtocolIE-ID ::= 191

id-ReportingPeriodicity																					ProtocolIE-ID ::= 192

id-CellMeasurementResult																				ProtocolIE-ID ::= 193

id-NG-RANnode1CellID																						ProtocolIE-ID ::= 194

id-NG-RANnode2CellID																				ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters																	ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters															ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange																ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR																		ProtocolIE-ID ::= 199

id-CarrierList																						ProtocolIE-ID ::= 200

id-ULCarrierList																					ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz																				ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst																				ProtocolIE-ID ::= 203

id-NRCellPRACHConfig																				ProtocolIE-ID ::= 204

id-RACHReportInformation																			ProtocolIE-ID ::= 205

id-IABNodeIndication																				ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF																		ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink																		ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink																		ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List														ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance																	ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics																		ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator																		ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN																	ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget																		ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List																ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation																	ProtocolIE-ID ::= 217

id-UsedRSNInformation																				ProtocolIE-ID ::= 218

id-RLCDuplicationInformation																		ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information																		ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation																	ProtocolIE-ID ::= 221

id-NPNMobilityInformation																			ProtocolIE-ID ::= 222

id-NPN-Support																						ProtocolIE-ID ::= 223

id-MDT-Configuration																				ProtocolIE-ID ::= 224

id-MDTPLMNList																						ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI																			ProtocolIE-ID ::= 226

id-UERadioCapabilityID																				ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication																		ProtocolIE-ID ::= 228

id-SNTriggered  																					ProtocolIE-ID ::= 229

id-DLCarrierList																					ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList																		ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA																			ProtocolIE-ID ::= 232

id-ConfiguredTACIndication																			ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo																	ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated													ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-PagingeDRXInformation																			ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-UESliceMaximumBitRateList																	ProtocolIE-ID ::= xxx

id-S-NG-RANnodeUE-Slice-MBR																		ProtocolIE-ID ::= yyy

END

-- ASN1STOP

### 9.3.8	Container definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

XnAP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

XNAP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID 			UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol IE pairs

--

-- **************************************************************

XNAP-PROTOCOL-IES-PAIR ::= CLASS {

&id						ProtocolIE-ID 		UNIQUE,

&firstCriticality		Criticality,

&FirstValue,

&secondCriticality		Criticality,

&SecondValue,

&presence				Presence

}

WITH SYNTAX {

ID						&id

FIRST CRITICALITY 		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY 		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

XNAP-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID 		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

XNAP-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::= 	ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {XNAP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				XNAP-PROTOCOL-IES.&id					({IEsSetParam}),

criticality		XNAP-PROTOCOL-IES.&criticality			({IEsSetParam}{@id}),

value			XNAP-PROTOCOL-IES.&Value				({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					XNAP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	XNAP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			XNAP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	XNAP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			XNAP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=	SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					XNAP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			XNAP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		XNAP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {XNAP-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {XNAP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				XNAP-PRIVATE-IES.&id			({IEsSetParam}),

criticality		XNAP-PRIVATE-IES.&criticality	({IEsSetParam}{@id}),

value			XNAP-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

<End of change>