3GPP TSG-RAN WG3 Meeting #114bis-e	R3-221354

Online, 17th -26th  January 2022

Agenda item:	19.2.1

Source: 	Ericsson, Huawei, CATT

Title: 	Discussion on signalling UE TX TEG association

Document for:	Other

# 1. Introduction

This TP proposes the addition of Tx TEG agreements by RAN1 with the references to the RAN1 list of agreements in R3-2112976 [1] and LS R3-220093 [2]:

Proposal 1:   	Add a UE Tx TEG Association IE in the POSITONING INFORMATION UPDATE message. Maps to row 2 in [1] and LS in [2] (FFS)

Proposal 2: 	The UE Tx TEG Association IE has an ID that can be associated with one or more UL positioning SRS resource IDs. Maps to row 3 and 4 in [1]

Proposal 3: 	Add a TRP Tx TEG Association IE the TRP Information IE in the TRP INFORMATION RESPONSE message and corresponding codepoint in the TRP INFORMATION REQUEST message. Maps to row 44 in [1]

Proposal 4: 	The TRP Tx TEG Association IE has an ID that can be associated with one or more DL PRS resources. Row 27 in [1]

Proposal 5: 	Allow the LMF to request the number of TRP Rx TEGs and number of TRP RxTx TEGs and in the MEASUREMENT REQUEST message to gNB. Maps to row 41 in [1]

Proposal 6:	(not captured in this TP) Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF, Port ID (FFS, if not signalled already in 9.2.37 as captured in R3-221200). Row 34, 35 in [1]

Proposal 7: 	Adding the TRP Rx ID with RTOA measurements from gNB to LMF. Maps to row 25 in [1]

Proposal 8.1: Adding a choice for the TEG ID reported with gNB-Rx-Tx measurements from gNB to LMF

- Reporting of a TRP RxTx TEG ID, and optionally a TRP Tx TEG ID

- Reporting of a TRP Rx TEG ID and a TRP Tx TEG ID

Proposal 8.2: Adding the TRP Tx TEG Association IE (FFS) when the TRP Tx TEG ID is reported with a gNB Rx Tx time difference measurement. Maps to row 30 in [1]

Editor’s note 1: The transmission of UE Tx TEG in the POSITONING INFORMATION UPDATE needs further checking

Editor’s note 2: FFS if an explicit indication is required in the POSITIONING INFORMATION REQUEST message to trigger gNB to report UE Tx TEG

Editor’s note 3: Further checking of the RAN1 agreements implementation to be done based on RAN1 and RAN2 progress

# Annex: TP to NRPPa BL CR

<<START OF CHANGES>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

CID	Cell-ID (positioning method)

DL-PRS	Downlink Positioning Reference Signal

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

LMF	Location Management Function

LPP	LTE Positioning Protocol

MSAS	Multi-functional Satellite Augmentation System

NavIC	NAVigation with Indian Constellation

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference of Arrival

posSIB	Positioning SIB

PRS	Positioning Reference Signal (for E-UTRA)

QZSS	Quasi-Zenith Satellite System

RSRP	Reference Signal Received Power

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SBAS	Space Based Augmentation System

SRS	Sounding Reference Signal

TEG	Timing Error Group

TRP	Transmission-Reception Point

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

WAAS	Wide Area Augmentation System

Z-AoA	Zenith Angles of Arrival

<<NEXT CHANGE>>

Editor’s note 1: The transmission of UE Tx TEG in the POSITONING INFORMATION UPDATE needs further checking

### 8.2.7	Positioning Information Update

#### 8.2.7.1	General

The Positioning Information Update procedure is initiated by the NG-RAN node to indicate to the LMF that a change has occurred in the SRS configuration, or to report UE Tx TEG information. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: Positioning Information Update procedure, successful operation

The NG-RAN node initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the LMF. If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

If the UE Tx TEG Association IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall, if supported, consider it as the UE TEG association for the SRS resources that have changed their TEG association since the last update.

Editor’s note 2: FFS if an explicit indication is required in the POSITIONING INFORMATION REQUEST message to trigger gNB to report UE Tx TEG

<<NEXT CHANGE>>

#### 9.1.1.13	POSITIONING INFORMATION UPDATE

This message is sent by NG-RAN node to indicate that a change in the SRS configuration has occurred.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| UE Tx TEG Association | O |  | 9.2.X |  | YES | ignore |

#### 9.1.1.14	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  | YES | ignore |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type List |  | 1 |  |  |  |  |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, trp Tx teg) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

<<NEXT CHANGE>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms) | The codepoint 60min is not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

<<NEXT CHANGE>>

### 9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | - |  |
| >>NR CGI | M |  | 9.2.9 |  | - |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>PRS Configuration | M |  | 9.2.44 |  | - |  |
| >>SSB Information | M |  | 9.2.54 |  | - |  |
| >>SFN Initialisation Time | M |  | Relative Time 1900 9.2.36 |  | - |  |
| >>Spatial Direction Information | M |  | 9.2.45 |  | - |  |
| >>Geographical Coordinates | M |  | 9.2.46 |  | - |  |
| >>TRP type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp…) | TS 38.305 [18] | YES | reject |
| >>TRP Tx TEG Association | M |  | 9.2.Y |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

<<NEXT CHANGE>>

### 9.2.39	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  |  |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] |  |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] |  |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] |  |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] |  |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] |  |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] |  |  |
| Additional Path List | O |  | 9.2.41 |  |  |  |
| TRP Rx TEG ID | M |  | INTEGER (1..FFS) |  |  |  |

### 9.2.40	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] | - |  |
| Additional Path List | O |  | 9.2.41 |  | - |  |
| CHOICE TEG ID | O |  |  |  |  |  |
| >RxTx TEG |  |  |  |  |  |  |
| >>TRP RxTx TEG ID | M |  | INTEGER (1..FFS) |  |  |  |
| >> TRP Tx TEG ID | O |  | INTEGER (1..FFS) |  |  |  |
| >Rx TEG |  |  |  |  |  |  |
| >>TRP Rx TEG ID | M |  | INTEGER (1..FFS) |  |  |  |
| >>TRP Tx TEG ID | M |  | INTEGER (1..FFS) |  |  |  |
| TRP Tx TEG Association (FFS) | O |  | 9.2.Y | This IE is included when a TRP Tx TEG ID is present |  |  |

<<NEXT CHANGE>>

### 9.2.X UE Tx TEG Association

This information element contains the UE Tx TEG association.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Tx TEG Association item | 1 .. <maxnoUETEGs> |  |  |  |
| >UE Tx TEG ID | M |  | INTEGER (1..FFS) |  |
| >SRS Resource Set ID | M |  | INTEGER (0..15) |  |
| >SRS Resource ID List |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >>SRS Resource ID | M |  | INTEGER(0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoUETEGs | Maximum no of reported UE Tx TEG association. Value is FFS. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

### 9.2.Y TRP Tx TEG Association

This information element contains the TRP Tx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP TEG item | 1 .. <maxnoTRPTEGs> |  |  |  |
| >TRP Tx TEG ID | M |  | INTEGER (1..8, …FFS) |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >DL-PRS Resource ID List |  | 1..<maxPRS-ResourcesPerSet> |  |  |
| >>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPTEGs | Maximum no of reported TRP Tx TEG association. Value is FFS. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

<<End of CHANGES>>