3GPP TSG-RAN WG3 Meeting #114bis-e	R3-221464

Online, January 17th – 26th 2022

Title:	LS on Support for Configuration and Reporting of RAN Visible QoE Measurements

Response to:	-

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	RAN3

To:	SA4

Cc:	RAN2

Contact Person:

Name: 		Filip Barać

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description:

RAN3 has agreed to support the following RAN visible QoE metrics generated by the UE Application layer for DASH and VR service types:

- Buffer Level (as defined in TS 26.247)

- Playout Delay for Media Startup (as defined in TS 26.247)

The RAN visible QoE metrics are supposed to be readable by the NG-RAN, which means that the RAN visible QoE report should be sent to the NG-RAN as explicit IEs outside the QoE report container.

Moreover, RAN3 also agreed the following:

- Include PDU session ID in RAN Visible QoE report, FFS on Slice information.

- RAN visible QoE reports and legacy QoE reports can use different periodicity, the reporting periodicity can be ms120, ms240, ms480, ms640, ms1024, FFS for (ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, min60).

The PDU session ID information in the first agreement includes the PDU session ID(s) corresponding to the service that is measured. The format of PDU Session ID is INTEGER (0..255), and it is provided per measConfigAppLayerId. The measConfigAppLayerId is a part of the legacy QoE configuration sent to the UE Application layer, as per RAN2 agreement.

The latter agreement implies that legacy QoE and RAN visible QoE can be reported together, at the periodicity of legacy QoE, but the RAN visible QoE can also be reported separately at additional periodicities as listed above.

2. Actions:

To 3GPP SA4

ACTION:

RAN2 will provide the detailed content of the RAN visible QoE configuration and report, once they complete their work. RAN3 respectfully asks SA4 to take the RAN3 agreements into account and provide the necessary specification support accordingly.

3. Date of next TSG RAN WG3 meeting:

RAN3#115-e                         February 21st - March 3rd 2022		Online