3GPP TSG-RAN WG3 #115-e                                                                              R3-221525

Online, 21 Feb – 3 Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.420 | CR | 0022 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BL CR to TS38.420 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduce a new class 2 procedure for Multicast Group Paging over Xn. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR multicast group paging is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR multicast group paging is not supported over Xn. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-0: capture the agreed TP in R3-214378 Rev-1: Resubmission to RAN3 #114e Rev-2: Resubmission to RAN3 #114bis-e Rev-3: Resubmission to RAN3 #115e |  |  |  |  |  |  |  |  |

-------------------------------------------------Start of the change--------------------------------------------------

### 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SCTP	Stream Control Transmission Protocol

Xn-C	Xn Control plane

Xn-U	Xn User plane

MBS	Multicast Broadcast Service

-----------------------------------------------------Next Change-----------------------------------------------------------

### 6.2.X	MBS Management procedures

The MBS management procedures are used to manage the MBS Session:

-	RAN Multicast Group Paging procedure

--------------------------------------------------End of the change-------------------------------------------------------