3GPP TSG-RAN WG3 Meeting #115-e	R3-221686

# Online, February 21st – March 3rd 2022

Agenda Item:	13.4.1

Source:	Ericsson

Title:	(TP for IAB BL CR for TS 38.423): Inter-Donor Resource Coordination in IAB Networks

Document for:	Agreement

# Introduction

At the RAN3#114bis-e meeting, the following was agreed:

FFS on cell-specific/common configurations of the neighboring IAB-node(s)/donor(s), or peer parent nodes. This information may be added in an IE other than the gNB-DU Cell Resource Configuration IE.

Revert previous agreement and only use a new XnAP procedure for resource coordination.

Further discussion on the XnAP IE(s). R3-220590 is taken as baseline.

This paper discusses the way forward with respect to the above agreements. A TP for IAB BL CR for TS 38.423 is provided in the Annex.

# XnAP procedure for boundary node resource co-ordination

At the RAN3#114bis-e meeting it was agreed to revert previous agreement on Xn enhancements and use a new XnAP procedure for inter-resource coordination. In our view, this should be a dedicated procedure, separate from the newly defined one for LS/QoS info exchange.

Proposal 1: A new dedicated XnAP procedure is defined for inter-donor resource coordination (i.e., the new procedure for QoS/L2 info exchange is not reused).

The new procedure described in Annex presents signalling for this dedicated procedure. The procedure includes the following XnAP enhancements described in R1-2112966, mapped with the relevant “P’s” agreed by RAN1.

#### P05, P14 – RAN1 input:

| Param. ID | Sub-feature group | Parameter name in specification | Description | Value range | Default value |  |
| --- | --- | --- | --- | --- | --- | --- |
| P05 |  | Resource multiplexing | New | Peer Parent Common Resource Configuration | Indicates the semi-static and/or cell-common higher layer configuration (e.g. SSB, CORESET 0, and RACH and configurations) from/for different parent nodes. | FFS (at least cell-common higher layer configuration (e.g. SSB, CORESET 0, and RACH and configurations) |
| P14 | FFS: Resource multiplexing or Interference management | Peer DU Resource Configuration | Indicates the DU resource configuration (UL/DL/FL, H/S/NA) of the peer IAB-node or donor DU that can be used for resource coordination in case of DC, for interference management, and/or for resource coordination. | (Rel-16) gNB-DU Cell Resource Configuration (which includes SCS, DUF TX periodicity, DUF config, HSNA periodicity and HSNA config) + (Rel-17 frequency-domain) gNB-DU Cell Resource Configuration (which includes “Rel-17 IAB-DU-Resource-Configuration-H/S/NA-Config”, “RB Set Configuration”, and “Frequency Domain H/S/NA Configuration Reference SCS”) |  |  |

#### P05, P14 - Proposed signalling:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Activated Cells List |  | 0.. 1 |  | List of cells configured in the boundary gNB-DU | YES | reject |
| >Activated Cells Item |  | 1.. <maxnoofServedCellsIAB> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.2.2.7 |  | - |  |
| >>Multiplexing Info | O |  | 9.2.2.b | Contains information on multiplexing with cells configured for collocated IAB-MT. Defined in TS 38.473 [x], section 9.3.1.108. | - |  |
| >>HSNA resource Configurations | M |  | 9.2.2.c | Contains Hard, Soft, N/A information and DL/UL configurations for either TDD or FDD configurations. | - |  |
| >>IAB STC Info | O |  | 9.2.2.d | STC configuration of boundary-node IAB-DU’s cell. Defined in TS 38.473 [x], section 9.3.1.109. | - |  |
| >>RACH Config Common | O |  | OCTET STRING | Corresponds to the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>RACH Config Common IAB | O |  | OCTET STRING | Corresponds to the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>CSI-RS Configuration | O |  | OCTET STRING | Corresponds to the NZP-CSI-RS-Resource as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SR Configuration | O |  | OCTET STRING | Corresponds to the SchedulingRequestResourceConfig as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>PDCCH Configuration SIB1 | O |  | OCTET STRING | Corresponds to the PDCCH-ConfigSIB1 as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SCS Common | O |  | OCTET STRING | Corresponds to the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [8]. | - |  |
| Parent gNB-DU Served Cells List |  | 0.. 1 |  | This information is present when a secondary leg for a DC-capable boundary IAB-node is set up. The information pertains to the resources allocated to the boundary IAB-MT on the master leg. | YES | reject |
| >Parent gNB-DU Served Cells Item |  | 1.. <maxnoofServedCellsIAB> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.2.2.7 |  | - |  |
| >>Multiplexing Info | O |  | 9.2.2.b | Contains information on multiplexing with cells configured for collocated IAB-MT. Defined in TS 38.473 [x], section 9.3.1.108. | - |  |
| >>HSNA resource Configurations | M |  | 9.2.2.c |  | - |  |
| >>IAB STC Info | O |  | 9.2.2.d | STC configuration of boundary-node IAB-DU’s cell. Defined in TS 38.473 [x], section 9.3.1.109. | - |  |
| >>RACH Config Common | O |  | OCTET STRING | Corresponds to the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>RACH Config Common IAB | O |  | OCTET STRING | Corresponds to the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>CSI-RS Configuration | O |  | OCTET STRING | Corresponds to the NZP-CSI-RS-Resource as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SR Configuration | O |  | OCTET STRING | Corresponds to the SchedulingRequestResourceConfig as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>PDCCH Configuration SIB1 | O |  | OCTET STRING | Corresponds to the PDCCH-ConfigSIB1 as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SCS Common | O |  | OCTET STRING | Corresponds to the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsIAB | Maximum number of cells served by an IAB-DU. Value is 512. |

#### P13 – RAN1 input:

| Param. ID | Sub-feature group | Parameter name in specification | Description | Value range | Default value |
| --- | --- | --- | --- | --- | --- |
| P13 | Interference management | Intended TDD DL-UL Configuration | Rel-16 Intended TDD DL-UL Configuration is extended to support IAB-specific UFD patterns. | Permutation: ENUMERATED (DFU, UFD, …) | DFU |

#### P13 - Proposed signalling:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …) | The values scs15, scs30, scs60 and scs120 corresponds to the sub carrier spacing in TS 38.104 [24]. |
| NR Cyclic Prefix | M |  | ENUMERATED (Normal, Extended, …) | The type of cyclic prefix, which determines the number of symbols in a slot. |
| NR DL-UL Transmission Periodicity | M |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms3, ms4, ms5, ms10, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, …) | The periodicity is expressed in the format msXpYZ, and equals X.YZ milliseconds. |
| Slot Configuration List |  | 1 |  |  |
| >Slot Configuration List Item |  | 1..<maxnoofslots> |  |  |
| >>Slot Index |  |  | INTEGER (0.. 5119) |  |
| >>CHOICE Symbol Allocation in Slot | M |  |  |  |
| >>>All DL |  |  |  |  |
| >>>All UL |  |  |  |  |
| >>>Both DL and UL |  |  |  |  |
| >>>>Number of DL Symbols | M |  | INTEGER (0..13) | Number of consecutive DL symbols inthe slot identified by Slot Index. If extended cyclic prefix is used, the maximum value is 11. The permutation IE indicates the location of DL symbols |
| >>>>Number of UL Symbols | M |  | INTEGER (0..13) | Number of consecutive UL symbols in the slot identified by Slot Index. If extended cyclic prefix is used, the maximum value is 11. The permutation IE indicates the location of UL symbols. |
| >>>>Permutation | O |  | ENUMERATED (DFU, UFD, …) | If not present, the default value is DFU |

| Range bound | Explanation |
| --- | --- |
| maxnoofslots | Maximum length of number of slots in a 160-ms period. Value is 5120. |

Proposal 2: RAN3 to agree to include Boundary IAB-node Resource Indication IE defined in Annex in the newly defined messages for boundary IAB-node’s resource coordination.

# Conclusion

In this paper we discuss resource multiplexing between child and parent links in IAB networks with required Xn enhancements.

The following is proposed:

Proposal 1: A new dedicated XnAP procedure is defined for inter-donor resource coordination (i.e., the new procedure for QoS/L2 info exchange is not reused).

Proposal 2: RAN3 to agree to include Boundary IAB-node Resource Indication IE and Xn enhancements defined in Annex  in the newly defined messages for boundary IAB-node’s resource coordination.

# Annex: TP for IAB BL CR for TS 38.423

-------------------------------------------Start of changes-------------------------------------------

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

----------------------------------------------<unchanged part is omitted>-----------------------------------------------

If the IAB Node Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the handover is for an IAB node.

If the UE Radio Capability ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

-------------------------------------------Next change-------------------------------------------

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

----------------------------------------------<unchanged part is omitted>-----------------------------------------------

If the UE Radio Capability ID IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG- RAN node shall, if supported store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

-------------------------------------------Next change-------------------------------------------

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

- Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

----------------------------------------------<unchanged part is omitted>-----------------------------------------------

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted List IE in the PDU Session Resource Setup Response Info – MN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

-------------------------------------------Next change-------------------------------------------

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

----------------------------------------------<unchanged part is omitted>-----------------------------------------------

For each DRB configured as SN-terminated split bearer/MCG bearer, if the QoS Mapping Information IE is included in the DRBs To Be Modified List IE in the PDU Session Resource Modification Info – SN terminated IE of the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from S-NG-RAN node to M-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

-------------------------------------------Next change-------------------------------------------

### 8.3.4	S-NG-RAN node initiated S-NG-RAN node Modification

#### 8.3.4.1	General

This procedure is used by the S-NG-RAN node to modify the UE context in the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

- Figure 8.3.4.2-1: S-NG-RAN node initiated S-NG-RAN node Modification, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUIRED message to the M-NG-RAN node.

----------------------------------------------<unchanged part is omitted>-----------------------------------------------

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs To Be Modified List IE in the PDU Session Resource Modification Required Info – MN terminated IE of the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

For each DRB configured as SN-terminated split bearer/MCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted to be Setup or Modified List IE in the PDU Session Resource Modification Confirm Info – SN terminated IE of the S-NODE MODIFICATION CONFIRM message, the S-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from S-NG-RAN node to M-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

-------------------------------------------Next change-------------------------------------------

## 8.y IAB Procedures

### 8.y.1 IAB-DU Cell Resource Coordination

#### 8.y.1.1	General

The purpose of the IAB-DU Cell Resource Coordination procedure is to enable coordination of semi-static radio resource configuration for the IAB-DU of a single- or dual-connected boundary IAB-node. During the procedure, the F1-terminating and non-F1-terminating donors of the boundary IAB-node exchange their intended radio resource allocations for the IAB-DU of the boundary IAB-node.

#### 8.y.1.2	Successful Operation

Figure 8.y.1.2-1: IAB-DU Cell Resource Coordination procedure: Successful Operation

The NG-RAN node1 initiates the procedure by sending the IAB-DU CELL RESOURCE COORDINATION REQUEST message to the NG-RAN node2, containing the semi-static resource configuration information for the IAB-DU collocated with the boundary IAB-MT designated by the Source NG-RAN Node UE XnAP ID IE therein. The NG-RAN node2 responds with the IAB-DU CELL RESOURCE COORDINATION RESPONSE message, containing the accepted semi-static resource allocation or an alternative semi-static radio resource allocation.

The exchanged information shall be stored in the respective nodes until the partial IAB-node migration for the IAB-node is revoked.

#### 8.y.1.3	Unsuccessful Operation

Figure 8.y.1.3-1: IAB-DU Cell Resource Coordination procedure: Unsuccessful Operation

If the NG-RAN node2 cannot accept the semi-static resource allocation request, it shall respond with an IAB-DU CELL RESOURCE COORDINATION FAILURE and an appropriate cause value.

#### 8.y.1.4	Abnormal Conditions

Not applicable.

-------------------------------------------Next change-------------------------------------------

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| <unchanged part is omitted> |  |  |  |  |  |  |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

-------------------------------------------Next change-------------------------------------------

#### 9.1.1.9	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information – Retrieve UE Context Response | M |  | 9.2.1.13 |  | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

-------------------------------------------Next change-------------------------------------------

### 9.1.2	Messages for Dual Connectivity Procedures

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| <unchanged part is omitted> |  |  |  |  |  |  |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

-------------------------------------------Next change-------------------------------------------

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node |  | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. |  | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  |  | YES | ignore |
| <unchanged part is omitted> |  |  |  |  |  |  |  |
| SN triggered | O |  | ENUMERATED (TRUE ...) |  |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

-------------------------------------------Next change-------------------------------------------

#### 9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  | YES | ignore |
| <unchanged part is omitted> |  |  |  |  |  |  |
| SCG Indicator | O |  | ENUMERATED(released,...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

-------------------------------------------Next change-------------------------------------------

### 9.1.y	Messages for IAB-related Procedures

#### 9.1.y.1	IAB-DU CELL RESOURCE COORDINATION REQUEST

This message is sent by a NG-RAN node1 to NG-RAN node2 to indicate the intended cell resource allocation for a boundary IAB-DU.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN Node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | reject |
| Boundary IAB-node Resource Indication | M |  | 9.2.2.a |  | YES | reject |

#### 9.1.y.2	IAB-DU CELL RESOURCE COORDINATION RESPONSE

This message is sent by a NG-RAN node2 to a peer NG-RAN node1 in response to an IAB-DU CELL RESOURCE COORDINATION REQUEST.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Target NG-RAN Node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | reject |
| Boundary IAB-node Resource Indication | M |  | 9.2.2.a |  | YES | reject |

#### 9.1.y.3	IAB-DU CELL RESOURCE COORDINATION FAILURE

This message is sent by a NG-RAN node2 to a peer NG-RAN node1 in response to IAB-DU CELL RESOURCE COORDINATION REQUEST, to indicate a failure of radio resource coordination for a boundary IAB-DU.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | reject |
| Cause | M |  | 9.2.3.3 |  | YES | reject |

-------------------------------------------Next change-------------------------------------------

### 9.2.2	NG-RAN Node and Cell Configuration related IE definitions

#### 9.2.2.40	Intended TDD DL-UL Configuration NR

This IE contains the subcarrier spacing, cyclic prefix and TDD DL-UL slot configuration of an NR cell that a neighbour NG-RAN node needs to take into account for cross-link interference mitigation, and/or for NR-DC power coordination, when operating its own cells.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …) | The values scs15, scs30, scs60 and scs120 corresponds to the sub carrier spacing in TS 38.104 [24]. |
| NR Cyclic Prefix | M |  | ENUMERATED (Normal, Extended, …) | The type of cyclic prefix, which determines the number of symbols in a slot. |
| NR DL-UL Transmission Periodicity | M |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms3, ms4, ms5, ms10, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, …) | The periodicity is expressed in the format msXpYZ, and equals X.YZ milliseconds. |
| Slot Configuration List |  | 1 |  |  |
| >Slot Configuration List Item |  | 1..<maxnoofslots> |  |  |
| >>Slot Index |  |  | INTEGER (0.. 5119) |  |
| >>CHOICE Symbol Allocation in Slot | M |  |  |  |
| >>>All DL |  |  |  |  |
| >>>All UL |  |  |  |  |
| >>>Both DL and UL |  |  |  |  |
| >>>>Number of DL Symbols | M |  | INTEGER (0..13) | Number of consecutive DL symbols inthe slot identified by Slot Index. If extended cyclic prefix is used, the maximum value is 11. The permutation IE indicates the location of DL symbols |
| >>>>Number of UL Symbols | M |  | INTEGER (0..13) | Number of consecutive UL symbols in the slot identified by Slot Index. If extended cyclic prefix is used, the maximum value is 11. The permutation IE indicates the location of UL symbols. |
| >>>>Permutation | O |  | ENUMERATED (DFU, UFD, …) | If not present, the default value is DFU. |

| Range bound | Explanation |
| --- | --- |
| maxnoofslots | Maximum length of number of slots in a 160-ms period. Value is 5120. |

-------------------------------------------Next change-------------------------------------------

#### 9.2.2.a Boundary IAB-node Resource Indication

This IE contains the information about the radio resources configured in an IAB-DU and, optionally, its parent IAB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Activated Cells List |  | 0.. 1 |  | List of cells configured in the boundary gNB-DU | YES | reject |
| >Activated Cells Item |  | 1.. <maxnoofServedCellsIAB> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.2.2.7 |  | - |  |
| >>Multiplexing Info | O |  | 9.2.2.b | Contains information on multiplexing with cells configured for collocated IAB-MT. Defined in TS 38.473 [x], section 9.3.1.108. | - |  |
| >>HSNA resource Configurations | M |  | 9.2.2.c | Contains Hard, Soft, N/A information and DL/UL configurations for either TDD or FDD configurations. | - |  |
| >>IAB STC Info | O |  | 9.2.2.d | STC configuration of boundary-node IAB-DU’s cell. Defined in TS 38.473 [x], section 9.3.1.109. | - |  |
| >>RACH Config Common | O |  | OCTET STRING | Corresponds to the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>RACH Config Common IAB | O |  | OCTET STRING | Corresponds to the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>CSI-RS Configuration | O |  | OCTET STRING | Corresponds to the NZP-CSI-RS-Resource as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SR Configuration | O |  | OCTET STRING | Corresponds to the SchedulingRequestResourceConfig as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>PDCCH Configuration SIB1 | O |  | OCTET STRING | Corresponds to the PDCCH-ConfigSIB1 as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SCS Common | O |  | OCTET STRING | Corresponds to the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [8]. | - |  |
| Peer Parent gNB-DU Served Cells List |  | 0.. 1 |  | This information is present when a secondary leg for a DC-capable boundary IAB-node is set up. The information pertains to the resources allocated to the boundary IAB-MT on the master leg. | YES | reject |
| >Peer Parent gNB-DU Served Cells Item |  | 1.. <maxnoofServedCellsIAB> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.2.2.7 |  | - |  |
| >>Multiplexing Info | O |  | 9.2.2.b | Contains information on multiplexing with cells configured for collocated IAB-MT. Defined in TS 38.473 [x], section 9.3.1.108. | - |  |
| >>HSNA resource Configurations | M |  | 9.2.2.c |  | - |  |
| >>IAB STC Info | O |  | 9.2.2.d | STC configuration of boundary-node IAB-DU’s cell. Defined in TS 38.473 [x], section 9.3.1.109. | - |  |
| >>RACH Config Common | O |  | OCTET STRING | Corresponds to the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>RACH Config Common IAB | O |  | OCTET STRING | Corresponds to the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>CSI-RS Configuration | O |  | OCTET STRING | Corresponds to the NZP-CSI-RS-Resource as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SR Configuration | O |  | OCTET STRING | Corresponds to the SchedulingRequestResourceConfig as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>PDCCH Configuration SIB1 | O |  | OCTET STRING | Corresponds to the PDCCH-ConfigSIB1 as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SCS Common | O |  | OCTET STRING | Corresponds to the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [8]. | - |  |
| Peer gNB-DU Resource Configuration List |  | 0.. 1 |  | This information is present when a secondary leg for a DC-capable boundary IAB-node is set up. The information pertains to the resources allocated to the boundary IAB-MT on the master leg. | YES | reject |
| >Peer gNB-DU Resource Configuration Item |  | 1.. <maxnoofServedCellsIAB> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.2.2.7 |  | - |  |
| >>gNB-DU Cell Resource Configuration | M |  | 9.2.2.e |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsIAB | Maximum number of cells served by an IAB-DU. Value is 512. |

#### 9.2.2.b	Multiplexing Info

This IE contains information about the multiplexing capabilities between the IAB-DU’s cell and the cells configured on the collocated IAB-MT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB-MT Cell List |  | 1 |  |  |
| >IAB-MT Cell Item |  | 1 .. <maxnoofServingCells> |  |  |
| >>NR Cell Identity | M |  | BIT STRING (SIZE(36)) | Cell identity of a serving cell configured for a collocated IAB-MT. |
| >>DU_RX/MT_RX | M |  | ENUMERATED (supported, not supported, supported-FDM-required) | An indication of whether the IAB-node supports simultaneous reception at its DU and MT side. |
| >>DU_TX/MT_TX | M |  | ENUMERATED (supported, not supported, supported-FDM-required) | An indication of whether the IAB-node supports simultaneous transmission at its DU and MT side. |
| >>DU_TX/MT_RX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous transmission at its DU and reception at its MT side. |
| >>DU_RX/MT_TX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous reception at its DU and transmission at its MT side. |

| Range bound | Explanation |
| --- | --- |
| maxnoofServingCells | Maximum no. of serving cells for IAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

#### 9.2.2.c HSNA Resource Configurations

This IE contains the H/S/NA resource configurations of the cells served by a gNB-DU, i.e. the TDD/FDD resource parameters

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE IAB-DU Cell Resource Configuration-Mode-Info | M |  |  |  | YES | reject |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  |  |  |
| >>>gNB-DU Cell Resource Configuration-TDD | M |  | 9.2.2.e | Contains TDD resource configuration of the gNB-DU’s cell defined in 9.2.2.e under gNB-DU Cell Resource Configuration | - |  |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>gNB-DU Cell Resource Configuration-FDD-UL | M |  | 9.2.2.e | Contains FDD UL resource configuration of the gNB-DU’s cell defined in 9.2.2.e under gNB-DU Cell Resource Configuration | - |  |
| >>>gNB-DU Cell Resource Configuration-FDD-DL | M |  | 9.2.2.e | Contains FDD DL resource configuration of the gNB-DU’s cell defined in 9.2.2.e under gNB-DU Cell Resource Configuration | - |  |

#### 9.2.2.d	IAB STC Info

This IE contains cell SSB Transmission Configuration (STC) information of an IAB-DU or IAB-donor-DU. The information is used by neighbour IAB-MTs for discovery and measurements of this IAB-DU or IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB STC-Info List |  | 1 |  |  |
| >IAB STC-Info Item |  | 1 ..<maxnoofIABSTCInfo> |  |  |
| >>SSB Frequency Info | M |  | INTEGER (0.. maxNRARFCN) | The SSB central frequency. |
| >>SSB Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, …) | The SSB subcarrier spacing. |
| >>SSB Transmission Periodicity | M |  | ENUMERATED (sf5, sf10, sf20, sf40, sf80, sf160, sf320, sf640, ,,,) |  |
| >>SSB Transmission Timing Offset | M |  | INTEGER (0.. 127, …) | SSB transmission timing offset in number of half-frames. |
| >>CHOICE SSB Transmission Bitmap | M |  |  | The SSB-ToMeasure IE defined in TS 38.331 [8]. |
| >>>Short Bitmap | M |  | BIT STRING (SIZE (4)) |  |
| >>>Medium Bitmap | M |  | BIT STRING (SIZE (8)) |  |
| >>>Long Bitmap | M |  | BIT STRING (SIZE (64)) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIABSTCInfo | Maximum no. of STC configurations. Value is 5. This includes 1 STC configuration for access and 4 STC configurations for backhaul. |
| maxNRARFCN | Maximum value of NR ARFCNs. Value is 3279165. |

#### 9.2.2.e	gNB-DU Cell Resource Configuration

This IE contains the resource configuration of the cells served by a gNB-DU, i.e., the TDD/FDD resource parameters for each activated cell TSS 38.213 [31], clause 11.1.1).

| IE/Group Name | Presence |  | Range | IE type and reference |  | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) |  | Subcarrier spacing used as reference for the TDD/FDD slot configuration. | YES | reject |
| DUF Transmission Periodicity | O |  |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, …) |  |  | YES | reject |
| DUF Slot Configuration List |  |  | 0..1 |  |  |  |  |  |
| >DUF Slot Configuration Item |  |  | 1..<maxnoofDUFSlots> |  |  | The maxNrofSlots in TS 38.331 [8]. | - |  |
| >>CHOICE DUF Slot Configuration | M |  |  |  |  |  | - |  |
| >>>Explicit Format |  |  |  |  |  |  | - |  |
| >>>>Permutation | M |  |  | ENUMERATED (DFU, UFD, …) |  |  | - |  |
| >>>>Number of Downlink Symbols | O |  |  | INTEGER (0..14) |  |  | - |  |
| >>>>Number of Uplink Symbols | O |  |  | INTEGER (0..14) |  |  | - |  |
| >>>Implicit Format |  |  |  |  |  |  |  |  |
| >>>>DUF Slot Format Index | M |  |  | INTEGER (0..254) |  | Index into Table 11.1.1-1 and Table 14-2 in TS 38.213 [31], excluding the last row in Table 14-2. | - |  |
| HSNA Transmission Periodicity | M |  |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, …) |  |  | YES | reject |
| HSNA Slot Configuration List |  |  | 0..1 |  |  |  |  |  |
| >HSNA Slot Configuration Item |  |  | 1..<maxnoofHSNASlots> |  |  |  |  |  |
| >>HSNA Downlink | O |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) |  | HSNA value for downlink symbols in a slot. | - |  |
| >>HSNA Uplink | O |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) |  | HSNA value for uplink symbols in a slot. | - |  |
| >>HSNA Flexible | O |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) |  | HSNA value for flexible symbols in a slot. | - |  |
| RB Set Configuration | O |  |  |  | 9.2.2.f |  |  |  |
| Frequency-domain HSNA Configuration List |  | 0..1 |  |  |  |  |  |  |
| >Frequency-domain HSNA Configuration Item |  | 1..<maxnoofRBsetsPerCell> |  |  |  |  |  |  |
| >>RB set Index | M |  |  |  | INTEGER (0.. maxnoofRBsetsPerCell-1) |  |  |  |
| >>Frequency-domain HSNA Slot Configuration List |  | 1 |  |  |  |  |  |  |
| >>>Frequency-domain HSNA Slot Configuration item |  | 1..<maxnoofHSNASlots> |  |  |  |  |  |  |
| >>>>Slot Index |  |  |  |  | INTEGER (0.. maxnoofHSNASlots-1) | Index to a slot within the HSNA Transmission Periodicity. * |  |  |
| >>>>HSNA Downlink | O |  |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot, for an RB set. |  |  |
| >>>>HSNA Uplink | O |  |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot, for an RB set. |  |  |
| >>>>HSNA Flexible | O |  |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot, for an RB set. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDUFSlots | Maximum no. of slots in 10ms. Value is 320. |
| maxnoofSymbols | Maximum no. of symbols in a slot. Value is 14. |
| maxnoofHSNASlots | Maximum no of "Hard", "Soft" or "Not available" slots in 160ms. Value is 5120. |
| maxnoofRBSetsPerCell | Maximum number of contiguous and non-overlapping resource block sets. Value is 8. |
| maxnoofChildIABNodes | Maximum number of child nodes served by an IAB-DU or IAB-donor-DU. Value is 1024. |

#### 9.2.2.f RB Set Configuration

This IE contains resource block set configuration of a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing | O |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Indicates the reference SCS of the cell HSNA configuration |  |  |
| RB Set Size | O |  | ENUMERATED (2, 4, 8, 16, 32, 64) | Number of PRBs in each RB set. The value is atleast the number of PRBs corresponding to the number of configured MT’s PRBs. |  |  |
| RB Set List |  | 0..1 |  |  |  |  |
| >RB Set Item |  | 1..<maxnoofRBsetsPerCell> |  |  |  |  |
| >>RB Set Index | M |  | INTEGER (0.. maxnoofRBsetsPerCell-1) |  |  |  |
| >>Initial RB Index | M |  | INTEGER (0.. [maxnoofPhysicalResourceBlocks]-1) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per DU cell. Value is 8. |
| maxnoofPhysicalResourceBlocks | Maximum no. of Physical Resource Blocks. Value is [FFS]. |

-------------------------------------------End of changes-------------------------------------------