3GPP TSG-RAN WG3 Meeting #115-e	R3-221687

# Online, February 21st – March 3rd 2022

Agenda Item:	13.4.1

Source:	Ericsson

Title:	(TP for IAB BL CR for TS 38.473): Inter-Donor Resource Coordination in IAB Networks

Document for:	Agreement

# Introduction

At the RAN3#114bis-e meeting, the following was agreed:

Enhance the Intended TDD DL-UL Configuration IE and the gNB-DU Cell Resource Configuration IE.

gNB-DU UE F1AP ID is used to identify the child IAB node.

Agree “Extension to Intended TDD DL-UL Configuration IE in Contribution R3-220590”.

Extend GNB-DU RESOURCE CONFIGURATION message to capture the following new elements (as agreed by RAN1): The new info is added in the gNB-DU Cell Resource Configuration IE.

- Child-node’s frequency configuration,

- Parent-node’s DU cell resource configurations

- DU cell resource configurations of the neighboring IAB-node(s)/donor(s)

- FFS on cell-specific/common configurations of the neighboring IAB-node(s)/donor(s), or peer parent nodes. This information may be added in an IE other than the gNB-DU Cell Resource Configuration IE.

Further discussion on the F1-AP IE(s). R3-220590 is taken as baseline.

Enhancement on Time alignment of resource configurations across nodes will not be discussed in Rel-17.

This paper discusses the way forward with respect to the above agreements. A TP for IAB CL CR for TS 38.473 is provided in the Annex.

# F1AP enhancements for resource coordination

This section discusses the different Physical layer parameters agreed by RAN1 in R1-2112966. First, we explain the different parameters that are agreed in RAN1 and then propose enhancements to F1AP. The Annex contains a TP for IAB BL CR for TS 38.473 that incorporates all the proposed enhancements.

#### P01, P02, P03 – RAN1 input:

| Param. ID | Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range |
| --- | --- | --- | --- | --- | --- |
| P01 | Resource multiplexing | New | Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config (final name in specification to be determined by RAN2/3) | Indicates H/S/NA attributes per RB set, per D/U/F resource type within a slot, for multiple slots and/or over a subset of slots. | {Hard, Soft, Not Available} per RB set, per resource type in a slot [TBD relative to IAB-DU-Resource-Configuration-TDD-Config] |
| P02 | Resource multiplexing | New | RB Set Configuration | Indicates the configuration for up to M non-overlapping RB sets for a given DU cell, used for frequency domain resource allocation via [Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config]. For a given DU cell, the RB set size, in terms of number of PRBs, is N. | - List of values for N = {2, 4, 8, 16, 32, 64} - [N is at least the # PRBs corresponding to the MT’s configured #PRB of an RBG] - M = 8. |
| P03 | Resource multiplexing | New | Frequency Domain H/S/NA Configuration and Reference SCS | Indicates reference SCS to be applied to Rel-17 IAB-DU-Resource-Configuration-H/S/NA-Config at a given IAB-DU's cell. | FR1: {15kHz, 30kHz, 60kHz} FR2: {60kHz, 120kHz} |

#### P01, P02, P03 - Proposed signalling:

The following new IE is introduced.

9.3.1.x RB Set Configuration

This IE contains resource block set configuration of a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing | O |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Indicates the reference SCS of the cell HSNA configuration |  |  |
| RB Set Size | O |  | ENUMERATED (2, 4, 8, 16, 32, 64) | Number of PRBs in each RB set. The value is atleast the number of PRBs corresponding to the number of configured MT’s PRBs. |  |  |
| RB Set List |  | 0..1 |  |  |  |  |
| >RB Set Item |  | 1..<maxnoofRBsetsPerCell> |  |  |  |  |
| >>RB Set Index | M |  | INTEGER (0.. maxnoofRBsetsPerCell-1) |  |  |  |
| >>Initial RB Index | M |  | INTEGER (0.. maxnoofPhysicalResourceBlocks-1) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per DU cell. Value is 8. |
| maxnoofPhysicalResourceBlocks | Maximum no. of Physical Resource Blocks. Value is FFS. |

The proposed IE is then inserted into GNB-DU Cell Resource Configuration IE (9.3.1.107).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … | … |
| RB Set Configuration | O |  | 9.3.1.x |  |  |  |
| Frequency-domain HSNA Configuration List |  | 0..1 |  |  |  |  |
| >Frequency-domain HSNA Configuration Item |  | 1..<maxnoofRBsetsPerCell> |  |  |  |  |
| >>RB Set Index | M |  | INTEGER (0.. maxnoofRBsetsPerCell-1) |  |  |  |
| >>Frequency-domain HSNA Slot Configuration List |  | 1 |  |  |  |  |
| >>>Frequency-domain HSNA Slot Configuration item |  | 1..<maxnoofHSNASlots> |  |  |  |  |
| >>>>Slot Index |  |  | INTEGER (0.. maxnoofHSNASlots-1) | Index to a slot within the HSNA Transmission Periodicity. * |  |  |
| >>>>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot, for an RB set. |  |  |
| >>>>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot, for an RB set. |  |  |
| >>>>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot, for an RB set. |  |  |

| Range bound | Explanation |
| --- | --- |
| … | … |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per DU cell. Value is 8. |

#### P05, P14 – RAN1 input:

| Param. ID | Sub-feature group | Parameter name in specification | Description | Value range | Default value |  |
| --- | --- | --- | --- | --- | --- | --- |
| P05 |  | Resource multiplexing | New | Peer Parent Common Resource Configuration | Indicates the semi-static and/or cell-common higher layer configuration (e.g. SSB, CORESET 0, and RACH and configurations) from/for different parent nodes. | FFS (at least cell-common higher layer configuration (e.g. SSB, CORESET 0, and RACH and configurations) |
| P14 | FFS: Resource multiplexing or Interference management | Peer DU Resource Configuration | Indicates the DU resource configuration (UL/DL/FL, H/S/NA) of the peer IAB-node or donor DU that can be used for resource coordination in case of DC, for interference management, and/or for resource coordination. | (Rel-16) gNB-DU Cell Resource Configuration (which includes SCS, DUF TX periodicity, DUF config, HSNA periodicity and HSNA config) + (Rel-17 frequency-domain) gNB-DU Cell Resource Configuration (which includes “Rel-17 IAB-DU-Resource-Configuration-H/S/NA-Config”, “RB Set Configuration”, and “Frequency Domain H/S/NA Configuration Reference SCS”) |  |  |

#### P05, P14 – Proposed signalling

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … | … |  |
| Child Boundary IAB-DU Resource Indicator | O |  |  | If present, this IE indicates that the resources indicated in this message pertain to a child boundary IAB-DU. |  |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 | Identifies the IAB-MT collocated with the child boundary IAB-DU. |  |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 | Identifies the IAB-MT collocated with the child boundary IAB-DU. |  |  |
| Peer gNB-DU Resource Configuration List |  | 0..1 |  |  | YES | reject |
| >Peer gNB-DU Resource Configuration Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>Peer Parent indicator | O |  | ENUMERATED (true, ...) | The value of this indicator is set to ”true” if the resources indicated in this message pertain to a peer parent IAB-DU. | - | - |
| >>Peer gNB-DU Cells List |  | 0..1 |  | List of cells served by the child-node IAB-DU whose resource configuration is updated. | YES | reject |
| >>>Peer gNB-DU Cells List Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>>>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>>>CHOICE IAB-DU Cell Resource Configuration-Mode-Info | O |  |  |  | - |  |
| >>>>>TDD |  |  |  |  | - |  |
| >>>>>>TDD Info |  | 1 |  |  | - |  |
| >>>>>>>gNB-DU Cell Resource Configuration-TDD | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains TDD resource configuration of gNB-DU’s cell. | - |  |
| >>>>>>>NR Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>>FDD |  |  |  |  | - |  |
| >>>>>>FDD Info |  | 1 |  |  | - |  |
| >>>>>>>gNB-DU Cell Resource Configuration-FDD-UL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD UL resource configuration of gNB-DU’s cell. | - |  |
| >>>>>>>gNB-DU Cell Resource Configuration-FDD-DL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD DL resource configuration of gNB-DU’s cell. | - |  |
| >>>>>>>UL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>DL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>UL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>DL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>>>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>IAB STC Info | O |  | 9.3.1.109 | STC configuration of child-node IAB-DU’s cell. |  |  |
| >>>>RACH Config Common | O |  | OCTET STRING | Corresponds to the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>RACH Config Common IAB | O |  | OCTET STRING | Corresponds to the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>CSI-RS Configuration | O |  | OCTET STRING | Corresponds to the NZP-CSI-RS-Resource as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>SR Configuration | O |  | OCTET STRING | Corresponds to the SchedulingRequestResourceConfig as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>PDCCH Configuration SIB1 | O |  | OCTET STRING | Corresponds to the PDCCH-ConfigSIB1 as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>SCS Common | O |  | OCTET STRING | Corresponds to the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [8]. |  |  |
| >>>>Multiplexing Info | O |  | 9.3.1.108 | Contains information on multiplexing with cells configured for collocated IAB-MT. |  |  |

#### P13 – RAN1 input:

| Param. ID | Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default Value |
| --- | --- | --- | --- | --- | --- | --- |
| P13 | Interference management | Existing parameter | Intended TDD DL-UL Configuration | Rel-16 Intended TDD DL-UL Configuration is extended to support IAB-specific UFD patterns. | Permutation: ENUMERATED (DFU, UFD, …) | DFU |

#### P13 - Proposed signalling:

The following changes are made to the Intended TDD DL-UL Configuration IE (9.3.1.89)

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| …. | ….. | …. | ….. | …… |
| Slot Configuration List |  | 1 |  |  |
| >Slot Configuration List Item |  | 1..<maxnoofslots> |  |  |
| >>Slot Index | M |  | INTEGER (0..5119) |  |
| >>CHOICE Symbol Allocation in Slot | M |  |  |  |
| >>>All DL |  |  | NULL | This choice implies that all symbols in the slot are DL symbols. |
| >>>All UL |  |  | NULL | This choice implies that all symbols in the slot are UL symbols. |
| >>>Both DL and UL |  |  |  |  |
| >>>>Number of DL Symbols | M |  | INTEGER (0..13) | Number of consecutive DL symbols in the slot identified by Slot Index. If extended cyclic prefix is used, the maximum value is 11. The permutation IE indicates the location of DL symbols. |
| >>>>Number of UL Symbols | M |  | INTEGER (0..13) | Number of consecutive UL symbols in the slot identified by Slot Index. If extended cyclic prefix is used, the maximum value is 11. The permutation IE indicates the location of UL symbols. |
| >>>>Permutation | O |  | ENUMERATED (DFU, UFD, …) | If not present, the default value is DFU. |

| Range bound | Explanation |
| --- | --- |
| maxnoofslots | Maximum length of number of slots in a 160-ms period. Value is 5120. |

#### P21 – RAN1 input:

| Param. ID | Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range |
| --- | --- | --- | --- | --- | --- |
| P21 | Resource multiplexing | New | Child IAB-MT Link NA Resource Configuration (final name in specification to be determined by RAN2/3) | IAB-donor CU indicates, to an IAB-node/donor DU, NA attribute per D/U/F resource type within a slot, for a child IAB-MT. | {NA Downlink: ENUMERATED (true, false), NA Uplink: ENUMERATED (true, false) NA Flexible: ENUMERATED (true, false)} per slot, per child IAB-MT |

#### P21 - Proposed signalling:

The following changes are made to the GNB-DU Cell Resource Configuration IE (9.3.1.107).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … | … |
| Child-Nodes List |  | 0..1 |  | List of child IAB-nodes served by the IAB-DU or IAB-donor-DU. This list is only applicable in dual-connectivity. | YES |  |
| >Child-Nodes List Item |  | 1 .. <maxnoofChildIABNodes> |  |  | EACH |  |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 | Identifier of a descendant node IAB-MT at the IAB-donor-CU. | YES |  |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 | Identifier of a child-node IAB-MT at an IAB-DU or IAB-donor-DU. | YES |  |
| >>Child NA Resource Configuration List |  | 0..1 |  |  |  |  |
| >>>Child NA Resource Configuration Item |  | 1..<maxnoofHSNASlots> |  |  |  |  |
| >>>>NA Downlink | O |  | ENUMERATED (true, false, …) | Indicates whether downlink symbols, in a slot, are not available to serve the child-node. |  |  |
| >>>>NA Uplink | O |  | ENUMERATED (true, false, …) | Indicates whether uplink symbols, in a slot, are not available to serve the child-node. |  |  |
| >>>>NA Flexible | O |  | ENUMERATED (true, false, …) | Indicates whether flexible symbols, in a slot, are not available to serve the child-node. |  |  |

| Range bound | Explanation |
| --- | --- |
| … | … |
| maxnoofChildIABNodes | Maximum number of child nodes served by an IAB-DU or IAB-donor-DU. Value is 1024. |

#### P22 – RAN1 input:

| Param. ID | Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default Value |
| --- | --- | --- | --- | --- | --- | --- |
| P22 | Resource multiplexing | New | FDMrequired (final name in specification to be determined by RAN3) | The IAB-node indicates to Donor CU whether FDM is required or not for an enhanced multiplexing operation [for a given (MT CC, DU cell) pair]. | {FDM required, FDM not required} per multiplexing mode (DU_RX/MT_RX, DU_TX/MT_TX, DU_TX/MT_RX, DU_RX/MT_TX) per IAB-MT cell and DU cell pair |  |

#### P22 - Proposed signalling:

The following changes are made to the Multiplexing Info IE (9.3.1.108).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB-MT Cell List |  | 1 |  |  |
| >IAB-MT Cell Item |  | 1 .. <maxnoofServingCells> |  |  |
| >>NR Cell Identity | M |  | BIT STRING (SIZE(36)) | Cell identity of a serving cell configured for a collocated IAB-MT. |
| >>DU_RX/MT_RX | M |  | ENUMERATED (supported, not supported, supported-FDM-required) | An indication of whether the IAB-node supports simultaneous reception at its DU and MT side. |
| >>DU_TX/MT_TX | M |  | ENUMERATED (supported, not supported, supported-FDM-required) | An indication of whether the IAB-node supports simultaneous transmission at its DU and MT side. |
| >>DU_TX/MT_RX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous transmission at its DU and reception at its MT side. |
| >>DU_RX/MT_TX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous reception at its DU and transmission at its MT side. |

#### P26 – RAN1 input:

| Param. ID | Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default Value |
| --- | --- | --- | --- | --- | --- | --- |
| P26 | Resource multiplexing | New | Child DU cell frequency configuration | Indicates the DU cell frequency configuration to the parent-node. This information comprises - [NR FreqInfo (38.473, 9.3.1.17)] - [Carrier List (38.473, 9.3.1.137)] | FFS |  |

#### P26 - Proposed signalling:

The following changes are made to the GNB-DU RESOURCE CONFIGURATION message (9.2.9.3).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … | … |
| Child-Nodes List |  | 0..1 |  | List of child IAB-nodes served by the IAB-DU or IAB-donor-DU. | YES | reject |
| >Child-Nodes List Item |  | 1 .. <maxnoofChildIABNodes> |  |  | EACH | reject |
| … | … | … | … | … | … | … |
| >>Child-Node Cells List |  | 0..1 |  | List of cells served by the child-node IAB-DU whose resource configuration is updated. | YES | reject |
| >>>Child-Node Cells List Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>>>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>>>CHOICE IAB-DU Cell Resource Configuration-Mode-Info | O |  |  |  | - |  |
| >>>>>TDD |  |  |  |  | - |  |
| >>>>>>TDD Info |  | 1 |  |  | - |  |
| >>>>>>>NR Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. |  |  |
| … | … | … | … | … | … | … |
| >>>>>FDD |  |  |  |  | - |  |
| >>>>>>FDD Info |  | 1 |  |  | - |  |
| >>>>>>>UL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>DL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>UL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>DL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>>>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. |  |  |

Proposal 1: Agree the TP for IAB BL CR for TS 38.473, presented in the Annex.

# F1AP signalling for resource coordination of boundary node and its descendants

At the RAN3#114-e meeting it was agreed to reuse the existing F1AP GNB-DU RESOURCE CONFIGURATION message to inform the parent node about the boundary IAB-DU cell configurations.

Based on the boundary node’s resource configuration negotiated between CU1 and CU2, CU1 may use the Rel-16 F1AP signalling to reconfigure the descendants of the boundary node. In other words, there is no specification impact.

Proposal 2: There is no specification impact for resource coordination of descendants of the boundary node once the boundary node has completed its resource coordination with the parent nodes.

# Conclusion

In this paper we discuss resource multiplexing between child and parent links in IAB networks. The following is observed and proposed:

Proposal 1: Agree the TP for IAB BL CR for TS 38.473, presented in the Annex.

Proposal 2: There is no specification impact for resource coordination of descendants of the boundary node once the boundary node has completed its resource coordination with the parent nodes.

# Annex: TP for IAB BL CR for TS 38.473

-------------------------------------------Start of changes-------------------------------------------

### 8.10.2	gNB-DU Resource Configuration

#### 8.10.2.1	General

The gNB-DU Resource Configuration procedure is initiated by the gNB-CU in order to configure the resource usage for a gNB-DU. The procedure uses non-UE associated signalling.

In this version of the specification, this procedure is used to configure IAB resources.

#### 8.10.2.2	Successful Operation

Figure 8.10.2.2-1: gNB-DU Resource Configuration procedure: Successful Operation

The gNB-CU initiates the procedure by sending the GNB-DU RESOURCE CONFIGURATION message to gNB-DU. The gNB-DU replies to the gNB-CU with the GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE message.

For each cell in the Activated Cells to Be Updated List IE of the GNB-DU RESOURCE CONFIGURATION message, the gNB-DU shall store the resource configuration contained in the IAB-DU Cell Resource Configuration-Mode-Info IE and use it when performing scheduling in compliance with TS 38.213 [31].

If the Child-Node List IE is included in the GNB-DU RESOURCE CONFIGURATION message, for each child-node indicated by the gNB-CU UE F1AP ID IE and gNB-DU UE F1AP ID IE, and for each cell served by this child node indicated by the NR CGI IE in the Child-Node Cells List IE, the gNB-DU shall store the received information and use this information for scheduling, in compliance with TS 38.213 [31], clause 14.

#### 8.10.2.B	Unsuccessful Operation

Figure 8.10.2.3-1: gNB-DU Resource Configuration procedure: Unsuccessful Operation

If the gNB-DU cannot accept the configuration, it shall respond with a GNB-DU RESOURCE CONFIGURATION FAILURE and appropriate cause value.

If the GNB-DU RESOURCE CONFIGURATION FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the GNB-DU RESOURCE CONFIGURATION message towards the same gNB-DU.

#### 8.10.2.3	Abnormal Conditions

Not applicable.

-------------------------------------------Next change-------------------------------------------

#### GNB-DU RESOURCE CONFIGURATION

This message is sent by the gNB-CU to provide the resource configuration for an gNB-DU.

Direction: gNB-CU ® gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Activated Cells to Be Updated List |  | 0..1 |  | List of activated cells served by the IAB-DU or the IAB-donor-DU whose resource configuration is updated | YES | reject |
| >Activated Cells To Be Updated List Item |  | 1 .. <maxnoofServedCellsIAB> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>CHOICE IAB-DU Cell Resource Configuration-Mode-Info | M |  |  |  | - |  |
| >>>TDD |  |  |  |  |  |  |
| >>>>TDD Info |  | 1 |  |  |  |  |
| >>>>>gNB-DU Cell Resource Configuration-TDD | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains TDD resource configuration of the gNB-DU’s cell. | - |  |
| >>>FDD |  |  |  |  |  |  |
| >>>>FDD Info |  | 1 |  |  | - |  |
| >>>>>gNB-DU Cell Resource Configuration-FDD-UL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD UL resource configuration of the gNB-DU’s cell. | - |  |
| >>>>>gNB-DU Cell Resource Configuration-FDD-DL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD DL resource configuration of the gNB-DU’s cell. | - |  |
| Child-Nodes List |  | 0..1 |  | List of child IAB-nodes served by the IAB-DU or IAB-donor-DU. | YES | reject |
| >Child-Nodes List Item |  | 1 .. <maxnoofChildIABNodes> |  |  | EACH | reject |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 | Identifier of a descendant node IAB-MT at the IAB-donor-CU. | YES | reject |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 | Identifier of a child-node IAB-MT at an IAB-DU or IAB-donor-DU. | YES | reject |
| >>Child-Node Cells List |  | 0..1 |  | List of cells served by the child-node IAB-DU whose resource configuration is updated. | YES | reject |
| >>>Child-Node Cells List Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>>>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>>>CHOICE IAB-DU Cell Resource Configuration-Mode-Info | O |  |  |  | - |  |
| >>>>>TDD |  |  |  |  | - |  |
| >>>>>>TDD Info |  | 1 |  |  | - |  |
| >>>>>>>gNB-DU Cell Resource Configuration-TDD | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains TDD resource configuration of gNB-DU’s cell. | - |  |
| >>>>>>>NR Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>>FDD |  |  |  |  | - |  |
| >>>>>>FDD Info |  | 1 |  |  | - |  |
| >>>>>>>gNB-DU Cell Resource Configuration-FDD-UL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD UL resource configuration of gNB-DU’s cell. | - |  |
| >>>>>>> gNB-DU Cell Resource Configuration-FDD-DL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD DL resource configuration of gNB-DU’s cell. | - |  |
| >>>>>>>UL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>DL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>UL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>DL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>>>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>IAB STC Info | O |  | 9.3.1.109 | STC configuration of child-node IAB-DU’s cell. |  |  |
| >>>>RACH Config Common | O |  | OCTET STRING | Corresponds to the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>RACH Config Common IAB | O |  | OCTET STRING | Corresponds to the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>CSI-RS Configuration | O |  | OCTET STRING | Corresponds to the NZP-CSI-RS-Resource as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>SR Configuration | O |  | OCTET STRING | Corresponds to the SchedulingRequestResourceConfig as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>PDCCH Configuration SIB1 | O |  | OCTET STRING | Corresponds to the PDCCH-ConfigSIB1 as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>SCS Common | O |  | OCTET STRING | Corresponds to the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [8]. |  |  |
| >>>>Multiplexing Info | O |  | 9.3.1.108 | Contains information on multiplexing with cells configured for collocated IAB-MT. |  |  |
| Child Boundary IAB-DU Resource Indicator | O |  |  | If present, this IE indicates that the resources indicated in this message pertain to a child boundary IAB-DU. |  |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 | Identifies the IAB-MT collocated with the child boundary IAB-DU. |  |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 | Identifies the IAB-MT collocated with the child boundary IAB-DU. |  |  |
| Peer gNB-DU Resource Configuration List |  | 0..1 |  |  | YES | reject |
| >Peer gNB-DU Resource Configuration Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>Peer Parent indicator | O |  | ENUMERATED (true, ...) | The value of this indicator is set to ”true” if the resources indicated in this message pertain to a peer parent IAB-DU. | - | - |
| >>Peer gNB-DU Cells List |  | 0..1 |  | List of cells served by the child-node IAB-DU whose resource configuration is updated. | YES | reject |
| >>>Peer gNB-DU Cells List Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>>>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>>>CHOICE IAB-DU Cell Resource Configuration-Mode-Info | O |  |  |  | - |  |
| >>>>>TDD |  |  |  |  | - |  |
| >>>>>>TDD Info |  | 1 |  |  | - |  |
| >>>>>>>gNB-DU Cell Resource Configuration-TDD | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains TDD resource configuration of gNB-DU’s cell. | - |  |
| >>>>>>>NR Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>>FDD |  |  |  |  | - |  |
| >>>>>>FDD Info |  | 1 |  |  | - |  |
| >>>>>>>gNB-DU Cell Resource Configuration-FDD-UL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD UL resource configuration of gNB-DU’s cell. | - |  |
| >>>>>>>gNB-DU Cell Resource Configuration-FDD-DL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD DL resource configuration of gNB-DU’s cell. | - |  |
| >>>>>>>UL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>DL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  |  |  |
| >>>>>>>UL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>DL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  |  |  |
| >>>>>>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>>>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. |  |  |
| >>>>IAB STC Info | O |  | 9.3.1.109 | STC configuration of child-node IAB-DU’s cell. |  |  |
| >>>>RACH Config Common | O |  | OCTET STRING | Corresponds to the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>RACH Config Common IAB | O |  | OCTET STRING | Corresponds to the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>CSI-RS Configuration | O |  | OCTET STRING | Corresponds to the NZP-CSI-RS-Resource as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>SR Configuration | O |  | OCTET STRING | Corresponds to the SchedulingRequestResourceConfig as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>PDCCH Configuration SIB1 | O |  | OCTET STRING | Corresponds to the PDCCH-ConfigSIB1 as defined in subclause 6.3.2 of TS 38.331 [8]. |  |  |
| >>>>SCS Common | O |  | OCTET STRING | Corresponds to the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [8]. |  |  |
| >>>>Multiplexing Info | O |  | 9.3.1.108 | Contains information on multiplexing with cells configured for collocated IAB-MT. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofChildIABNodes | Maximum number of child nodes served by an IAB-DU or IAB-donor-DU. Value is 1024. |
| maxnoofServedCellsIAB | Maximum number of cells served by an IAB-DU or IAB-donor-DU. Value is 512. |

-------------------------------------------Next change-------------------------------------------

#### 9.3.1.89	Intended TDD DL-UL Configuration

This IE contains the subcarrier spacing, cyclic prefix and TDD DL-UL slot configuration of an NR cell that the receiving NG-RAN node needs to take into account for cross-link interference mitigation, and/or for NR-DC power coordination, when operating its own cells.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …) | The values scs15, scs30, scs60 and scs120 corresponds to the sub carrier spacing in TS 38.104 [17]. |
| NR Cyclic Prefix | M |  | ENUMERATED (Normal, Extended, …) | The type of cyclic prefix, which determines the number of symbols in a slot. |
| NR DL-UL Transmission Periodicity | M |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms3, ms4, ms5, ms10, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, …) | The periodicity is expressed in the format msXpYZ, and equals X.YZ milliseconds. |
| Slot Configuration List |  | 1 |  |  |
| >Slot Configuration List Item |  | 1..<maxnoofslots> |  |  |
| >>Slot Index | M |  | INTEGER (0..5119) |  |
| >>CHOICE Symbol Allocation in Slot | M |  |  |  |
| >>>All DL |  |  | NULL | This choice implies that all symbols in the slot are DL symbols. |
| >>>All UL |  |  | NULL | This choice implies that all symbols in the slot are UL symbols. |
| >>>Both DL and UL |  |  |  |  |
| >>>>Number of DL Symbols | M |  | INTEGER (0..13) | Number of consecutive DL symbols inthe slot identified by Slot Index. If extended cyclic prefix is used, the maximum value is 11. The permutation IE indicates the location of DL symbols |
| >>>>Number of UL Symbols | M |  | INTEGER (0..13) | Number of consecutive UL symbols in the slot identified by Slot Index. If extended cyclic prefix is used, the maximum value is 11. The permutation IE indicates the location of UL symbols. |
| >>>>Permutation | O |  | ENUMERATED (DFU, UFD, …) | If not present, the default value is DFU |

| Range bound | Explanation |
| --- | --- |
| maxnoofslots | Maximum length of number of slots in a 160-ms period. Value is 5120. |

-------------------------------------------Next change-------------------------------------------

#### 9.3.1.107	gNB-DU Cell Resource Configuration

This IE contains the resource configuration of the cells served by a gNB-DU, i.e. the TDD/FDD resource parameters for each activated cell (TS 38.213 [31], clause 11.1.1).

| IE/Group Name | Presence |  | Range | IE type and reference |  | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) |  | Subcarrier spacing used as reference for the TDD/FDD slot configuration. | YES | reject |
| DUF Transmission Periodicity | O |  |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, …) |  |  | YES | reject |
| DUF Slot Configuration List |  |  | 0..1 |  |  |  |  |  |
| >DUF Slot Configuration Item |  |  | 1..<maxnoofDUFSlots> |  |  | The maxNrofSlots in TS 38.331 [8]. | - |  |
| >>CHOICE DUF Slot Configuration | M |  |  |  |  |  | - |  |
| >>>Explicit Format |  |  |  |  |  |  | - |  |
| >>>>Permutation | M |  |  | ENUMERATED (DFU, UFD, …) |  |  | - |  |
| >>>>Number of Downlink Symbols | O |  |  | INTEGER (0..14) |  |  | - |  |
| >>>>Number of Uplink Symbols | O |  |  | INTEGER (0..14) |  |  | - |  |
| >>>Implicit Format |  |  |  |  |  |  |  |  |
| >>>>DUF Slot Format Index | M |  |  | INTEGER (0..254) |  | Index into Table 11.1.1-1 and Table 14-2 in TS 38.213 [31], excluding the last row in Table 14-2. | - |  |
| HSNA Transmission Periodicity | M |  |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, …) |  |  | YES | reject |
| HSNA Slot Configuration List |  |  | 0..1 |  |  |  |  |  |
| >HSNA Slot Configuration Item |  |  | 1..<maxnoofHSNASlots> |  |  |  |  |  |
| >>HSNA Downlink | O |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) |  | HSNA value for downlink symbols in a slot. | - |  |
| >>HSNA Uplink | O |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) |  | HSNA value for uplink symbols in a slot. | - |  |
| >>HSNA Flexible | O |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) |  | HSNA value for flexible symbols in a slot. | - |  |
| RB Set Configuration | O |  |  |  | 9.3.1.x |  |  |  |
| Frequency-domain HSNA Configuration List |  | 0..1 |  |  |  |  |  |  |
| >Frequency-domain HSNA Configuration Item |  | 1..<maxnoofRBsetsPerCell> |  |  |  |  |  |  |
| >>RB set Index | M |  |  |  | INTEGER (0.. maxnoofRBsetsPerCell-1) |  |  |  |
| >>Frequency-domain HSNA Slot Configuration List |  | 1 |  |  |  |  |  |  |
| >>>Frequency-domain HSNA Slot Configuration item |  | 1..<maxnoofHSNASlots> |  |  |  |  |  |  |
| >>>>Slot Index |  |  |  |  | INTEGER (0.. maxnoofHSNASlots-1) | Index to a slot within the HSNA Transmission Periodicity. * |  |  |
| >>>>HSNA Downlink | O |  |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot, for an RB set. |  |  |
| >>>>HSNA Uplink | O |  |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot, for an RB set. |  |  |
| >>>>HSNA Flexible | O |  |  |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot, for an RB set. |  |  |
| Child-Nodes List |  | 0..1 |  |  |  | List of child IAB-nodes served by the IAB-DU or IAB-donor-DU. This list is only applicable in dual-connectivity. | YES |  |
| >Child-Nodes List Item |  | 1 .. <maxnoofChildIABNodes> |  |  |  |  | EACH |  |
| >>gNB-CU UE F1AP ID | M |  |  |  | 9.3.1.4 | Identifier of a descendant node IAB-MT at the IAB-donor-CU. | YES |  |
| >>gNB-DU UE F1AP ID | M |  |  |  | 9.3.1.5 | Identifier of a child-node IAB-MT at an IAB-DU or IAB-donor-DU. | YES |  |
| >>Child NA Resource Configuration List |  | 0..1 |  |  |  |  |  |  |
| >>>Child NA Resource Configuration Item |  | 1..<maxnoofHSNASlots> |  |  |  |  |  |  |
| >>>>NA Downlink | O |  |  |  | ENUMERATED (true, false, …) | Indicates whether downlink symbols, in a slot, are not available to serve the child-node. |  |  |
| >>>>NA Uplink | O |  |  |  | ENUMERATED (true, false, …) | Indicates whether uplink symbols, in a slot, are not available to serve the child-node. |  |  |
| >>>>NA Flexible | O |  |  |  | ENUMERATED (true, false, …) | Indicates whether flexible symbols, in a slot, are not available to serve the child-node. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDUFSlots | Maximum no. of slots in 10ms. Value is 320. |
| maxnoofSymbols | Maximum no. of symbols in a slot. Value is 14. |
| maxnoofHSNASlots | Maximum no of "Hard", "Soft" or "Not available" slots in 160ms. Value is 5120. |
| maxnoofRBSetsPerCell | Maximum number of contiguous and non-overlapping resource block sets. Value is 8. |
| maxnoofChildIABNodes | Maximum number of child nodes served by an IAB-DU or IAB-donor-DU. Value is 1024. |

-------------------------------------------Next change-------------------------------------------

#### 9.3.1.108	Multiplexing Info

This IE contains information about the multiplexing capabilities between the gNB-DU’s cell and the cells configured on the collocated IAB-MT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB-MT Cell List |  | 1 |  |  |
| >IAB-MT Cell Item |  | 1 .. <maxnoofServingCells> |  |  |
| >>NR Cell Identity | M |  | BIT STRING (SIZE(36)) | Cell identity of a serving cell configured for a collocated IAB-MT. |
| >>DU_RX/MT_RX | M |  | ENUMERATED (supported, not supported, supported-FDM-required) | An indication of whether the IAB-node supports simultaneous reception at its DU and MT side. |
| >>DU_TX/MT_TX | M |  | ENUMERATED (supported, not supported, supported-FDM-required) | An indication of whether the IAB-node supports simultaneous transmission at its DU and MT side. |
| >>DU_TX/MT_RX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous transmission at its DU and reception at its MT side. |
| >>DU_RX/MT_TX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous reception at its DU and transmission at its MT side. |

| Range bound | Explanation |
| --- | --- |
| maxnoofServingCells | Maximum no. of serving cells for IAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [8]. |

-------------------------------------------Next change-------------------------------------------

#### 9.3.1.x	RB Set Configuration

This IE contains resource block set configuration of a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing | O |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Indicates the reference SCS of the cell HSNA configuration |  |  |
| RB Set Size | O |  | ENUMERATED (2, 4, 8, 16, 32, 64) | Number of PRBs in each RB set. The value is atleast the number of PRBs corresponding to the number of configured MT’s PRBs. |  |  |
| RB Set List |  | 0..1 |  |  |  |  |
| >RB Set Item |  | 1..<maxnoofRBsetsPerCell> |  |  |  |  |
| >>RB Set Index | M |  | INTEGER (0.. maxnoofRBsetsPerCell-1) |  |  |  |
| >>Initial RB Index | M |  | INTEGER (0.. [maxnoofPhysicalResourceBlocks]-1) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per DU cell. Value is 8. |
| maxnoofPhysicalResourceBlocks | Maximum no. of Physical Resource Blocks. Value is [FFS]. |

-------------------------------------------End of changes-------------------------------------------