3GPP TSG-RAN WG3 Meeting #115	R3-221725

E-meeting, 21 Feb – 03 Mar 2022

Title: 	Further discussion on Local NG-RAN Node Identifier Update

Source: 	Huawei

Agenda item:	31.2.1

Document Type:	Discussion

1. Introduction

The solution for local NG-RAN node identifier was agreed at last meeting. And the corresponding stage 2 CR and stage 3 CR were endorsed in [1] and [2]. However, a left issue was captured in chair notes as below.

Solution 3 will be continued next meeting.

To be continued...

The critical point of solution 3 is to support local NG-RAN node identifier update smoothly, namely the local NG-RAN node identifier of a NG-RAN node can be changed without any negative impact on UEs using the old identifier.

This contribution continues the discussion and proposes an amendment to the endorsed XnAP CR of last meeting in order to support smooth local NG-RAN node identifier update.

2. Discussion

During the discussion in the past, the local NG-RAN node identifier update is a critical requirement and was captured in the endorsed stage 2 CR s below:

The I-RNTI provides an NG-RAN node with a reference to the UE context and a reference to the NG-RAN node hosting the UE context.

To support an NG-RAN node in resolving the Local gNB ID of the NG-RAN node hosting the UE context, the I-RNTI structure is as follows: a first part indicating an I-RNTI profile, a second part indicating a Local NG-RAN Node ID, and a third part to reference the UE context. The exact encoding of the I-RNTI is specified in TS 38.423 [xx]. In case of I-RNTI profile update, the NG-RAN node informs the neighbor NG-RAN nodes about the I-RNTI profile in use and the removal of the old I-RNTI profile.

In line with the spirit of stage 2, the NG-RAN node needs to inform its neighbours about the invalidity of the old local NG-RAN node identifier when there is no remaining UEs using the old identifier. This will lead to following addition to the endorsed stage 3 CR:

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name |  | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List |  | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType |  | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  |  |  |  |
| >>Served Cells To Update NR |  | O |  | 9.2.2.15 |  | YES | ignore |
| >>Cell Assistance Information NR |  | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA |  | O |  | 9.2.2.43 |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA |  | O |  | 9.2.2.16 |  | YES | ignore |
| >>Cell Assistance Information NR |  | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA |  | O |  | 9.2.2.43 |  | YES | ignore |
| TNLA To Add List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | M |  |  | 9.2.3.84 |  | – |  |
| TNLA To Update List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | O |  |  | 9.2.3.84 |  | – |  |
| TNLA To Remove List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| Global NG-RAN Node ID | O |  |  | 9.2.2.3 |  | YES | reject |
| AMF Region Information To Add | O |  |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES | reject |
| AMF Region Information To Delete | O |  |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  |  | 9.2.3.96 |  | YES | ignore |
| Local NG-RAN Node Identifier | O |  |  | 9.2.2.x |  | YES | ignore |
| Neighbour NG-RAN Node List |  |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | – |  |
| > Global NG-RAN Node ID | M |  |  | 9.2.2.3 |  | – |  |
| > Local NG-RAN Node Identifier | M |  |  | 9.2.2.x |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  |  | 9.2.2.x |  | YES | ignore |

When receiving the Local NG-RAN Node Identifier Removal IE in NG-RAN NODE CONFIGURATION UPDATE message, the receiving node will learn that the old identifier identified by the Local NG-RAN Node Identifier Removal IE is not used any more by the sending node. And the receiving node will remove it from its context and not use it for further NG-RAN node ID retrieval.

Without such new IE, it’s ambiguous that when the receiving node will discard the neighbours’ old local NG-RAN node identifier. One implementation could be at the time when receiving a new one in NG-RAN NODE CONFIGURATION UPDATE message. Then the Retrieve UE Context Requests for remaining UEs using old identifier cannot be routed correctly. The new serving node will fail the RRCResume procedure and try to establish a new RRC connection which brings bad user experience.

Proposal: To add a Local NG-RAN Node Identifier Removal IE in NG-RAN NODE CONFIGURATION UPDATE message to inform the removal of the old I-RNTI profile to enable smooth local NG-RAN node identifier update.

3. Conclusion

In this contribution, we further discuss the smooth local NG-RAN node identifier update issue and propose:

Proposal: To add a Local NG-RAN Node Identifier Removal IE in NG-RAN NODE CONFIGURATION UPDATE message to inform the removal of the old I-RNTI profile to enable smooth local NG-RAN node identifier update.

The corresponding CR is provided in [3].

4. Reference

[1]	R3-216079 Local NG-RAN Node Identifier Stage 2 CR

[2] R3-216187 Local NG-RAN Node Identifier XnAP CR

[3] R3-221726 Support flexible I-RNTI partitioning [RRCInactive]