3GPP TSG-RAN WG3 Meeting #115e                                                      R3-221752

E-Meeting: Feb 21st – Mar 3rd, 2022

Agenda item:	15.2.2

Source:	Qualcomm Incorporated

Title:	QoE measurement collection and reporting continuity in mobility scenarios

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss mobility related QoE issues based on some agreements and open issues identified from previous meetings.

# Discussion

After receiving NR QoE configuration from OAM/AMF, NG-RAN may fail to configure NR QMC to the UE in the following cases:

- NG-RAN is overloaded e.g., if there are insufficient resources available for QMC activation

- ng-eNB case (as ng-eNB is LTE RAT)

- UE is out of area scope

- UE is in inactive or idle state

- Ongoing handover

Observation 1: There can be many cases where an NG-RAN node may fail to configure NR QMC to the UE after receiving the NR QoE configuration from OAM/AMF (e.g., NG-RAN is overloaded, NG-RAN is ng-eNB, UE is out of area scope, UE is in inactive or idle state, Ongoing handover) and it is not a corner case.

## Presence of s-based QoE configuration container (XML file) in XnAP

WA: for s-based QoE, the s-based QoE configuration container (XML file) is included in XnAP HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages

The presence of s-based QoE configuration container (XML file) in XnAP HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages can take either Option 1 or Option 2

| Option 1: Mandatory | Option 2: Conditional that QoE configuration is received at NG-RAN but not configured at the UE |
| --- | --- |
| Simply the presence of the container means it will be s-based QoE, otherwise it will be m-based QoE | Need to introduce QoE Measurement Type ={s-based, m-based} IE to help target NG-RAN be aware whether the propagated QoE config is s-based or m-based |
| No need to introduce codepoints = {configured, not configured} in QoE Measurement Status | Need to introduce codepoints = {configured, not configured} in QoE Measurement Status |
| In case of full config HO, the QoE configurations are released in the UE AS and UE APP. It would be easy for the target NG-RAN node to configure a new QoE configuration if the container is propagated over Xn without waiting for a new configuration from AMF/OAM. | Need to wait for a new configuration from AMF/OAM in case of a full config HO. |
| NG-RAN will be required to store the container (XML file) even when QoE is configured to the UE | NG-RAN is not required to store the container (XML file) when QoE is configured to the UE |

Proposal 1: We slightly prefer the presence of s-based QoE configuration container (XML file) to be mandatory in XnAP HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages to make the Xn signaling and handling of QoE configurations at NG-RAN node simpler

## QoE Measurement Status

Clarify the codepoints of the QoE Measurement Status IE

Proposal 2:  If the presence of s-based QoE configuration (XML file) is mandatory, a QoE Measurement Status = {ongoing} IE can be included while propagating the s-based QoE configuration over Xn

- “ongoing” means QMC for the session is ongoing based on UE assistance of session/start end

Proposal 2bis:  If the presence of s-based QoE configuration (XML file) is conditional, a QoE Measurement Status = {ongoing, configured, not-configured} IE can be included while propagating the s-based QoE configuration over Xn

- “ongoing” means QMC for the session is ongoing based on UE assistance of session/start end

- “configured” means QMC is configured but no ongoing session

- “not-configured” means QMC is received at NG-RAN but not configured at the UE

## QoE Measurement Type

FFS whether to include QoE Measurement Type

Proposal 3: If the presence of s-based QoE configuration (XML file) is mandatory, there is no neeed to include a QoE Measurement Type IE.

Proposal 3bis: If the presence of s-based QoE configuration container is conditional in XnAP, include a QoE Measurement Type = {s-based, m-based} IE to help target NG-RAN be aware whether the propagated QoE config is s-based or m-based.

## Presence of s-based QoE configuration container (XML file) in NGAP: HANDOVER REQUIRED

FFS on the presence of s-based configuration container in NGAP HANDOVER REQUIRED message

- Does this create a requirement on an AMF to store all s-based configurations?

- Does the AMF know for how long it should store the container?

It is true that if we include the s-based configuration container in NGAP HANDOVER REQUIRED, this means AMF has to store the XML file, but this should not be for too long as it needs to store it till only the XML file is sent in NGAP HANDOVER REQUEST from AMF to target NG-RAN node.

Proposal 4: The s-based QoE configuration container (XML file) should be included in the NGAP: HANDOVER REQUIRED message and the presence of this container should be same as what is agreed over Xn

Proposal 5: If the s-based configuration container (XML file) is included in NGAP HANDOVER REQUIRED, the AMF has to store the XML file till the NGAP HANDOVER REQUEST is sent from AMF to the target NG-RAN node. There is no need to define any timer in AMF on how long to store the XML file

## MDT Alignment Information structure during m-based QoE mobility

This is the structure from the current running BL CR of NGAP and XnAP

| CHOICE MDT Alignment Information | O |  |  | Indicates the MDT measurements with which alignment is required. |
| --- | --- | --- | --- | --- |
| >S-based MDT |  |  |  |  |
| >>NG-RAN Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [11] (leftmost 6 octets, with PLMN information encoded as in 9.3.3.1), and Trace Recording Session Reference defined in TS 32.422 [11] (last 2 octets). |
| >M-based MDT |  |  |  |  |
| >>Trace Reference | M |  | OCTET STRING (SIZE(6)) | Trace Reference defined in TS 32.422 [11]. |

Observation 2: m-based MDT configuration in not propagated during handover. Hence there is no benefit of passing the MDT Alignment Info for the m-based MDT to the target node during mobility

Proposal 6: There is no need to include m-based MDT IE in MDT Alignment Information over Xn and during NG handovers

Proposal 7: Include the m-based MDT IE in MDT Alignment Information only during Initial context setup in NG

# Conclusion

Observation 1: There can be many cases where an NG-RAN node may fail to configure NR QMC to the UE after receiving the NR QoE configuration from OAM/AMF (e.g., NG-RAN is overloaded, NG-RAN is ng-eNB, UE is out of area scope, UE is in inactive or idle state, Ongoing handover) and it is not a corner case.

Proposal 1: We slightly prefer the presence of s-based QoE configuration container (XML file) to be mandatory in XnAP HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages to make the Xn signaling and handling of QoE configurations at NG-RAN node simpler

Proposal 2:  If the presence of s-based QoE configuration (XML file) is mandatory in XnAP, a QoE Measurement Status = {ongoing} IE can be included while propagating the s-based QoE configuration over Xn

- “ongoing” means QMC for the session is ongoing based on UE assistance of session/start end

Proposal 2bis:  If the presence of s-based QoE configuration (XML file) is conditional in XnAP, a QoE Measurement Status = {ongoing, configured, not-configured} IE can be included while propagating the s-based QoE configuration over Xn

- “ongoing” means QMC for the session is ongoing based on UE assistance of session/start end

- “configured” means QMC is configured but no ongoing session

- “not-configured” means QMC is received at NG-RAN but not configured at the UE

Proposal 3: If the presence of s-based QoE configuration (XML file) is mandatory in XnAP, there is no need to include a QoE Measurement Type IE.

Proposal 3bis: If the presence of s-based QoE configuration container is conditional in XnAP, include a QoE Measurement Type = {s-based, m-based} IE to help target NG-RAN be aware whether the propagated QoE config is s-based or m-based.

Proposal 4: The s-based QoE configuration container (XML file) should be included in the NGAP: HANDOVER REQUIRED message and the presence of this container should be same as what is agreed over Xn

Proposal 5: If the s-based configuration container (XML file) is included in NGAP HANDOVER REQUIRED, the AMF has to store the XML file till the NGAP HANDOVER REQUEST is sent from AMF to the target NG-RAN node. There is no need to define any timer in AMF on how long to store the XML file

Observation 2: m-based MDT configuration in not propagated during handover. Hence there is no benefit of passing the MDT Alignment Info for the m-based MDT to the target node during mobility

Proposal 6: There is no need to include m-based MDT IE in MDT Alignment Information over Xn and during NG handovers

Proposal 7: Include the m-based MDT IE in MDT Alignment Information only during Initial context setup in NG

# 4. References