3GPP TSG-RAN WG3 Meeting #115-e                                                                  R3-221755

E-Meeting: Feb 21st – Mar 3rd, 2022

Agenda item:	23.3

Source:	Qualcomm Incorporated

Title:	Control Plane procedures and Adaptation layer design for U2N relays

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the open issues identified in last meeting related to control plane procedures of L2 U2N sidelink relay in a split gNB architecture.

# Discussion

## F1 related open issues

### Whether to support bearer mapping configuration via the F1AP signalling of relay UE

RAN3 already agreed that “Over F1, the remote UE is configured via the UE-associated F1AP messages for itself”

Although it is true that relay UE associated F1AP signaling can support the configuration update of multiple remote UE's Uu E2E bearer mapping and would be better from an overhead point of view, it seems unnecessary to introduce bearer mapping configuration via both remote UE associated F1AP signaling and relay UE associated F1AP signaling.

Proposal 1: Bearer mapping configuration via F1AP signaling of remote UE is sufficient and there is no need to support bearer mapping configuration via the F1AP signaling of relay UE as well

### Whether the bearer mapping between PC5 RLC channel and remote UE's E2E bearer should be included in F1AP signaling

gNB-CU anyway needs to send the bearer mapping between PC5 RLC channel and remote UE's E2E bearer to remote/relay UE via RRC signaling, hence it is not necessary to send it to gNB-DU via F1AP signaling.

Proposal 2: There is no need to provide the bearer mapping between PC5 RLC channel and remote UE’s E2E bearer to gNB-DU via F1AP signaling

### Bearer mapping configuration format

The following two options are possible for the bearer mapping configuration format:

Option 1: Add+remove list

Uu RLC CH ID

>> Remote UE E2E Bearer To Add

>> Remote UE E2E Bearer To Remove

Option 2: One-to-one

Uu RLC CH ID à Remote UE E2E Bearer ID

Proposal 3: For the bearer mapping configuration, use an add/remove list in order to reduce the need to send the full list all the time during UE Context setup/modification to establish the PC5/Uu RLC channel

### Representation of UL tunnel in bearer mapping configuration

The mapping between DRB and Uu RLC Channel is configured at the granularity of GTP-U tunnel

- Option 1: Uu RLC CH ID  DL tunnel corresponding to an UL tunnel

- Option 2: Uu RLC CH ID  DL tunnel directly

Since no IE exists for the DL UP tunnel information in F1AP, it is simple to use Option 1 i.e., refer to a DL tunnel corresponding to an UL tunnel.

Proposal 4: There is no need to introduce a new IE for DL UP tunnel in UE context setup procedures and the DRB mapping information for sidelink L2 relays can contain the mapping to the DL tunnel corresponding to an existing UL tunnel.

## Relay and Remote UE association

Observation 1: RAN2 agreed that local ID of remote UE is 8 bits. This means that there can be 2^8 = 256 unique remote UE IDs

Example:

(

relay UE = 1, local ID of remote UE1 = 1,

…

relay UE = 1, local ID of remote UE1 = 256,

relay UE = 2, local ID of remote UE1 = 1

…

relay UE = 2, local ID of remote UE1 = 256,

)

Proposal 5: RAN3 should confirm that local ID of remote UE is unique per relay UE. This means that there can be 256 unique local ID of remote UEs per relay UE but can be shared across relay UEs.

Observation 2: Simply indicating the local ID of remote UE is not sufficient to uniquely identify a remote UE, hence a relay UE identifier is needed.

Proposal 6: Convert the following WA into agreement:

WA: the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE

## Local ID

### Whether the local ID of remote UE can be notified to gNB-DU before initial access of remote UE

The gNB-DU can simply decode the local ID of remote UE via the SRAP header in RRCSetupRequest during remote UE initial access. Hence there is no need for gNB-CU to explicitly notify the local ID of remote UE to gNB-DU via UE CONTEXT MODIFICATION REQUEST.

Proposal 7: The local ID of remote UE need not be notified to gNB-DU before initial access of remote UE via UE CONTEXT MODIFICATION REQUEST

### Local ID update

The gNB-CU can send the updated local ID of remote UE to gNB-DU after initial access of remote UE

- Option 1: new local ID + old local ID

- Option 2: new local ID + L2 ID

L2 ID is not known to the gNB-DU so far and there is no need to introduce it now. Hence Option 1 is better.

Proposal 8: The gNB-CU can send the updated local ID of remote UE to gNB-DU after initial access of remote UE along with its old local ID

## Lower layer configurations in INITIAL UL RRC MESSAGE TRANSFER

It is seen that the DU to gNB-CU RRC Container just includes the CellGroupConfig IE (this is a container under RRCReconfiguration) which configures an MCG or SCG with PHY/MAC/RLC configuration and does not include the PC5 PHY/MAC/RLC (SL-PHY-MAC-RLC-Config) which is signaled in RRCReconfiguration> sl-ConfigDedicatedNR-r16. Following options are possible to signal the lower layer configurations in INITIAL UL RRC MESSAGE TRANSFER:

- Option 1: Change semantics description of DU to gNB-CU RRC Container to also include SL-PHY-MAC-RLC-Config

- Option 2: Use a separate Sidelink Configuration Container

Option 1 is not possible as SL-PHY-MAC-RLC-Config is not a container in RRC. Hence, we propose Option 2.

Proposal 9: Define a separate Sidelink Configuration Container in INITIAL UL RRC MESSAGE TRANSFER to transfer the PC5 lower layer configurations from gNB-DU to gNB-CU

Moreover, on admission control, the current spec says that

If the DU to CU RRC Container IE is not included in the INITIAL UL RRC MESSAGE TRANSFER, the gNB-CU should reject the UE under the assumption that the gNB-DU is not able to serve such UE. If the gNB-DU is able to serve the UE, the gNB-DU shall include the DU to CU RRC Container IE and the gNB-CU shall configure the UE as specified in TS 38.331 [8]. The gNB-DU shall not include the ReconfigurationWithSync field in the CellGroupConfig IE as defined in TS 38.331 [8] of the DU to CU RRC Container IE.

Observation 3: The presence of the DU to CU RRC Container IE cannot be used to indicate the admission result of gNB-DU for remote UE since this IE might not be included for remote UE

Proposal 10:  The presence of Sidelink Configuration Container can be used to indicate the admission result of remote UE at gNB-DU

## Service continuity

### Inter-gNB-DU mobility when Relay UE is in RRC_CONNECTED

Figure 1

Proposal 11:  Adopt Figure 1 as a baseline call flow for the inter-gNB-DU mobility when relay UE is in RRC_CONNECTED

### Inter-gNB-DU mobility when Relay is in RRC_IDLE or RRC_INACTIVE

Figure 2

Proposal 12:  Adopt Figure 2 as a baseline call flow for the inter-gNB-DU mobility when relay UE is in RRC_IDLE or RRC_INACTIVE

### Intra-gNB-DU mobility for remote UE

TS 38.401 section 8.2.1.2 on intra-gNB-DU mobility doesn’t discusses remote UE mobility and the gNB-DU behavior is not clearly mentioned in such scenarios.

Proposal 13: Stage-2 text should be added in TS 38.401 to clarify the gNB-DU behaviour upon intra-gNB-DU mobility for remote UE, when switching from a direct cell to a cell via relay.

Proposal 14: In case of intra-gNB-DU mobility for a remote UE when switching from a direct cell to a cell via relay, the gNB-DU shall continue sending UL PDCP PDUs to the gNB-CU using the previous UL GTP TEID until a new Uu RLC channel for relaying purpose is successfully established, and vice-versa.

This procedure is used for the case that the UE moves from one cell to another cell within the same gNB-DU or for the case that intra-cell handover is performed during NR operation, and supported by the UE Context Modification (gNB-CU initiated) procedure as specified in TS 38.473 [4]. When the intra-gNB-DU handover is performed (either inter-cell or intra-cell), the gNB-CU provides new UL GTP TEID to the gNB-DU and the gNB-DU provides new DL GTP TEID to the gNB-CU. The gNB-DU shall continue sending UL PDCP PDUs to the gNB-CU using the previous UL GTP TEID until it re-establishes the RLC, and after then start sending using the new UL GTP TEID. In case of intra-gNB-DU mobility for a remote UE when switching from a direct cell to a cell via relay, the gNB-DU shall continue sending UL PDCP PDUs to the gNB-CU using the previous UL GTP TEID until a new Uu RLC channel for relaying purpose is successfully established, and vice versa. The gNB-CU shall continue sending DL PDCP PDUs to the gNB-DU using the previous DL GTP TEID until it performs PDCP re-establishment or PDCP data recovery, and after then start sending using the new DL GTP TEID.

# Conclusion

Proposal 1: Bearer mapping configuration via F1AP signaling of remote UE is sufficient and there is no need to support bearer mapping configuration via the F1AP signaling of relay UE as well

Proposal 2: There is no need to provide the bearer mapping between PC5 RLC channel and remote UE’s E2E bearer to gNB-DU via F1AP signaling

Proposal 3: For the bearer mapping configuration, use an add/remove list in order to reduce the need to send the full list all the time during UE Context setup/modification to establish the PC5/Uu RLC channel

Proposal 4: There is no need to introduce a new IE for DL UP tunnel in UE context setup procedures and the DRB mapping information for sidelink L2 relays can contain the mapping to the DL tunnel corresponding to an existing UL tunnel.

Observation 1: RAN2 agreed that local ID of remote UE is 8 bits. This means that there can be 2^8 = 256 unique remote UE IDs

Proposal 5: RAN3 should confirm that local ID of remote UE is unique per relay UE. This means that there can be 256 unique local ID of remote UEs per relay UE but can be shared across relay UEs.

Observation 2: Simply indicating the local ID of remote UE is not sufficient to uniquely identify a remote UE, hence a relay UE identifier is needed.

Proposal 6: Convert the following WA into agreement:

WA: the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE

Proposal 7: The local ID of remote UE need not be notified to gNB-DU before initial access of remote UE via UE CONTEXT MODIFICATION REQUEST

Proposal 8: The gNB-CU can send the updated local ID of remote UE to gNB-DU after initial access of remote UE along with its old local ID

Proposal 9: Define a separate Sidelink Configuration Container in INITIAL UL RRC MESSAGE TRANSFER to transfer the PC5 lower layer configurations from gNB-DU to gNB-CU

Observation 3: The presence of the DU to CU RRC Container IE cannot be used to indicate the admission result of gNB-DU for remote UE since this IE might not be included for remote UE

Proposal 10:  The presence of Sidelink Configuration Container can be used to indicate the admission result of remote UE at gNB-DU

Proposal 11:  Adopt Figure 1 as a baseline call flow for the inter-gNB-DU mobility when relay UE is in RRC_CONNECTED

Proposal 12:  Adopt Figure 2 as a baseline call flow for the inter-gNB-DU mobility when relay UE is in RRC_IDLE or RRC_INACTIVE

Proposal 13: Stage-2 text should be added in TS 38.401 to clarify the gNB-DU behaviour upon intra-gNB-DU mobility for remote UE, when switching from a direct cell to a cell via relay.

Proposal 14: In case of intra-gNB-DU mobility for a remote UE when switching from a direct cell to a cell via relay, the gNB-DU shall continue sending UL PDCP PDUs to the gNB-CU using the previous UL GTP TEID until a new Uu RLC channel for relaying purpose is successfully established.

# References