3GPP TSG-RAN WG3 Meeting #115-e	R3-221762

E-meeting, 21 Feb – 3 Mar 2022

Agenda item:	14.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to SCG BL CR to 38.463, LTE_NR_DC_enh2-Core)  Enabling information on SCG use

Document for:	Agreement

# 1	Introduction

At the RAN3 #114 and #114-bis meetings, it has been discussed that in case of CP-UP separation, the SN may not know when SCG is actually needed [1-2]. The problem concerns in particular split DRBs, where data may be transferred via either of the nodes.

# 2	Discussion

Possible enhancements to the activity notification were discussed at RAN3 #114-bis, but without conclusion [1]. There are three options considered:

- Using the existing activity notifications from E1AP an F1AP without any changes;

- Enhancing E1AP activity notification (DL Data Notification or Bearer Context Inactivity Notification);

- Enhancing F1AP activity notification (UE INACTIVITY NOTIFICATION).

On F1AP, there is the UE Inactivity Notification, which enables the DU to inform the CU-CP about activity status of particular DRB allocated to a DU within a UE context.

On E1AP, there are two relevant procedures: the Bearer Context Inactivity Notification and the DL Data Notification. The DL Data Notification can only provide information on the arrival of DL data per PDU session. The Bearer Context Inactivity Notification is more useful, as it can indicate data activity situation per DRB within a bearer context.

When the first option is considered, it is needed to address two situations: indication that SCG needs to be activated and indication that SCG needs to be deactivated. In the table below, we compare all the options.

|  | Indication that SCG is needed (data arriving) | Indication is not SCG needed (no data) |
| --- | --- | --- |
| Existing E1AP indication | In case of SCG-only or MCG-only DRB, there is no problem, SCG shall be activated if at least one SCG DRB becomes active. In case of split DRB, neither of the existing procedures can inform the CU-CP that the SCG resources are needed (i.e. that MCG alone is not enough). | In case of SCG-only or MCG-only DRB, there is no problem, SCG may be deactivated if all SCG DRBs becomes inactive. In case of split DRB, neither of the existing procedures can inform the CU-CP that the SCG resources are not needed any more (i.e. that MCG alone is enough to handle the data). |
| Existing F1AP indication | The indication concerns DRBs allocated to resources in given DU, so if there is activity notification from a DU that serves SCG resources, it is sure SCG shall be activated. However, the DU must have data forwarded despite having SG deactivated. | The indication concerns DRBs allocated to resources in given DU, so if there is inactivity notification from a DU that serves SCG resources, it is sure SCG may be deactivated. |

From the above table it can be concluded that:

Observation 1: The existing E1AP can’t provide information that SCG is / is not needed in case of split DRBs.

Observation 2: The existing F1AP can offer activity status per SCG, but activity can be indicated only if there is data forwarded from the CU-UP.

At early stages of the discussion, another method was considered to let the CU-UP know about SCG inactivity: flow control mechanism. In this scenario, the CU-UP, when it needs SCG, tries to send data to the DU. If SCG is deactivated, the DU does not acknowledge it, but it also knows there is data activity on the SCG-related DRB.

Observation 3: If the CU-UP is informed about the SCG status with the flow control, it will have to try to send data to the the DU that serves SCG resources and the DU will know there is DL activity on SCG-related DRBs.

However, at RAN3 #114-bis, signalling has been enabled that allows the CU-CP to indicate to the CU-UP that SCG is deactivated. The only purpose of the signalling is to prevent the CU-UP from sending data to a deactivated SCG. This is all right, but it means that when SCG is deactivated, the DU that serves SCG will not receive any data from the CU-UP.

Of course, the CU-UP may still send some data to the DU (“leak” data), just to let it know SCG is needed. However, such implementation will annihilate any gains from the SCG status indication to the CU-UP – why the CU-CP informs the CU-UP about SCG deactivation, if the latter has to send data to the DU serving SCG anyway?

Observation 4: Since RAN3 preferred to inform the CU-UP about SCG status directly from the CU-CP, so that the CU-UP does not have to try to send data to the DU serving deactivated SCG resources, the DU will not know that SCG is needed.

With the above in mind, we can conclude that both existing procedures have problems:

- The existing E1AP signalling can’t provide information about SCG need in case of split DRBs;

- The existing F1AP signalling, because of decisions made at RAN3 #114-bis, can’t provide information that SCG is needed (once SCG is deactivated, the DU will never discover that there is data activity).

Proposal 1: Because of the limitations of the existing E1AP and F1AP procedures (E1AP has problem with split DRBs, while F1AP can offer information that SCG shall be activated, because there will not be any data sent to the DU once the CU-UP is informed about SCG deactivation), RAN3 must enhance the existing signalling.

The above analysis indicates that F1AP can’t really be enhanced, because the problem is not in the signalling, but in the fact that the source of the signalling (the DU serving SCG resources) does not have sufficient information, once SCG is deactivated. It is therefore reasonable to update the existing E1AP mechanism so that the CU-CP is informed not only when there is DL traffic for a split DRB, but when this traffic is such that SCG resources are needed. The control of the data transmission is in the DRB PDCP in the CU-UP, so it is the place where the information can be fetched.

There are two ways to achieve this: either the UP informs explicitly the CP whenever SCG is needed, or there is a throughput threshold configured per DRB, which indicates that the SCG is or is not needed.

In the 1st case, the PDCP scheduler operates as normally, and only informs the CP when it does not use SCG, or when it wants to start using it again. The 2nd case assumes that the PDCP scheduler in the UP, being aware that SCG is deactivated, will use MCG resources. The threshold shall be set so that it corresponds to the saturation of MCG resources, so when the SCG becomes critical to keep user’s satisfaction. Out of these two options, the first one seems simpler.

Proposal 2: The existing E1AP mechanism for inactivity notification is enhanced so that the CU-UP can inform the CU-CP that in case of a split DRB, there is data activity on SCG resources.

Furthermore, in be sure the information is provided when needed, the existing control mechanism based on the Activity Notification Level IE can be extended so that the CU-CP may inform the CU-UP that the information on SCG use is required. This can also let the PDCP scheduler know that SCG resources may be suspended so that it optimises SCG use (e.g. it enables longer periods of SCG inactivity).

Proposal 2: The existing E1AP mechanism to control the Activity Notification is extended so that the CU-CP can ask the CU-UP to provide information on the use of the SCG resources in case of split DRBs.

# 3	Conclusions

In this paper, we have analysed the mechanism that can help the CU-CP to decide about SCG activation. First, we analyse the existing signalling and we make following observations:

Observation 1: The existing E1AP can’t provide information that SCG is / is not needed in case of split DRBs.

Observation 2: The existing F1AP can offer activity status per SCG, but activity can be indicated only if there is data forwarded from the CU-UP.

Observation 3: If the CU-UP is informed about the SCG status with the flow control, it will have to try to send data to the the DU that serves SCG resources and the DU will know there is DL activity on SCG-related DRBs.

Observation 4: Since RAN3 preferred to inform the CU-UP about SCG status directly from the CU-CP, so that the CU-UP does not have to try to send data to the DU serving deactivated SCG resources, the DU will not know that SCG is needed.

Based on those we make following proposals:

Proposal 1: Because of the limitations of the existing E1AP and F1AP procedures (E1AP has problem with split DRBs, while F1AP can offer information that SCG shall be activated, because there will not be any data sent to the DU once the CU-UP is informed about SCG deactivation), RAN3 must enhance the existing signalling.

Proposal 2: The existing E1AP mechanism for inactivity notification is enhanced so that the CU-UP can inform the CU-CP that in case of a split DRB, there is data activity on SCG resources.

Proposal 3: The existing E1AP mechanism to control the Activity Notification is extended so that the CU-CP can ask the CU-UP to provide information on the use of the SCG resources in case of split DRBs.

The TPs implementing the solution are proposed in below.

# 4	References

- R3-220149, RAN3 #114-bis-e

# 5	Text proposal for BL CR to TS 38.463

The below text proposal is based on the current TS version (no BL CR opened yet). The ASN.1 will be added once the concept of the change is endorsed.

*** First change, omitted text not changed ***

### 8.3.6	Bearer Context Inactivity Notification

#### 8.3.6.1	General

This procedure is initiated by the gNB-CU-UP to indicate the inactivity/resumption of activity related to the UE. The procedure uses UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: Bearer Context Inactivity Notification procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BEARER CONTEXT INACTIVITY NOTIFICATION message to the gNB-CU-CP.

If the Activity Notification Level was set to “DRB” or “DRB-SCG” during the Bearer Context establishment, the gNB-CU-UP shall include the DRB Activity List IE in the BEARER CONTEXT INACTIVITY NOTIFICATION message. If the Activity Notification Level was set to “DRB-SCG”, the gNB-CU-UP shall include the SCG Use IE in the DRB Activity List IE in each DRB configured with SCG resources.

If the Activity Notification Level was set to “PDU Session” during the Bearer Context establishment, the gNB-CU-UP shall include the PDU Session Resource Activity List IE in the BEARER CONTEXT INACTIVITY NOTIFICATION message.

If the Activity Notification Level was set to “UE” during the Bearer Context establishment, the gNB-CU-UP shall include the UE Activity IE in the BEARER CONTEXT INACTIVITY NOTIFICATION message.

#### 8.3.6.3	Abnormal Conditions

Not applicable.

*** Next change, omitted text not changed ***

#### 9.2.2.12	BEARER CONTEXT INACTIVITY NOTIFICATION

This message is sent by the gNB-CU-UP to provide information about the UE activity to the gNB-CU-CP.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE Activity Information | M |  |  |  | YES | reject |
| >DRB Activity List |  | 1 |  | Used if the Activity Notification Level IE is set as “DRB” or "DRB-SCG" in BEARER CONTEXT SETUP Request message | YES | reject |
| >>DRB Activity Item |  | 1 .. <maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>DRB Activity | M |  | ENUMERATED (Active, Not active, …) |  | - | - |
| >>>SCG Use | O |  | ENUMERATED (SCG needed, SCG not needed, ...) |  | YES | ignore |
| >PDU Session Resource Activity List |  | 1 |  | Used if the Activity Notification Level IE is set as “PDU Session” in the BEARER CONTEXT SETUP Request message | YES | reject |
| >>PDU Session Resource Activity Item |  | 1 .. <maxnoofPDUSessionResource> |  |  | - | - |
| >>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>PDU Session Resource Activity | M |  | ENUMERATED (Active, Not active, …) |  | - | - |
| >UE Activity | M |  | ENUMERATED (Active, Not active, …) | Used if the Activity Notification Level IE is set as “UE” in the BEARER CONTEXT SETUP Request message | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB for a UE, the maximum value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

*** Next change, omitted text not changed ***

#### 9.3.1.67	Activity Notification Level

This IE contains information on which level activity notification shall be performed..

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activity Notification Level | M |  | ENUMERATED (DRB, PDU Session, UE, …, DRB-SCG) |  |

*** Next change, omitted text not changed ***