3GPP TSG-RAN WG3 Meeting #115-e	R3-221765

E-meeting, 21.02-03.03.2022

Agenda item:	10.2.1.6

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP to SON BL CRs to 38.423, NR_ENDC_SON_MDT_enh]  MRO for PSCell change failure

Document for:	Endorsement

# 1	Introduction

At RAN3 #114-bis, some decisions were made concerning signalling between the MN and the source SN. However, some points remain open:

Continue to discuss d) Suitable PSCell CGI, e) Mobility Information in the XnAP message from MN to the source SN in the second round.

Continue to discuss the IEs in the message from the MN to the last serving SN.

RAN3 to review if and how to avoid duplication with the Rel.15 S-RLF signalling.

Continue to discuss the IEs from the last serving SN to the MN

Consider how to capture the MN behavior in stage 2.

In this paper, we address the 3rd point from the list, i.e. possible overlap between the existing Rel.15 and the planned Rel.17 signalling.

# 2	Discussion

The reason for the decision of using class-2 procedures in communication between the MN and the last serving SN is that in case the last serving SN is indeed the source node of the last PSCell change, no further message exchange would be required between MN and the last serving SN (or even any other SN). However, when making it, RAN3 did not consider that the MN will have to forward the S-RLF information to the last serving SN, anyway, as part of the Rel-15 SCG failure recovery, which is based on class-1 MN-initiated modification procedure. Furthermore, even though RAN2 has not yet decided, it is very likely that the message used to report S-RLF from the UE since Rel-15 will be reused also for the purposes of SCG MRO. Therefore, if the above agreement is kept, the MN may have to send two procedures: first, the MODIFICATION REQUEST to inform about S-RLF (Rel-15 functionality) and then the new class-2 to ask if the last serving SN has executed any PSCell changes. And the SN will respond to the modification and then, possibly, to the SCG MRO inquiry (in total two times). This is suboptimal and undermines the reasoning to have two class-2 messages.

One may also consider if indeed the message from the last serving SN to the MN can often be avoided? This will be true if the last serving SN executed a PSCell change that failed very soon after an SN change (i.e. within the duration needed to detect too early or wrong cell SN change). It is not impossible, but will likely be rare. Therefore, not only the message towards the last serving SN may be duplicated, but also the response.

Observation 1-1: In most cases, the last serving SN will respond to the MN it did not trigger any PSCell change during the short interval after SN change. Therefore, if no precaution is taken, both communications (from the MN to the last serving SN and the last serving SN to the MN) will be duplicated.

Proposal 1: RAN3 shall agree that using both, the existing Rel.15 and the new Rel.17 signalling shall not be required.

Let us first consider the scenarios where the procedures are needed. The Rel.15 signalling provides the S-RLF report to the last serving SN and enabled the last serving SN to respond with SCG reconfiguration. This is needed in particular in the case when the last serving SN is to keep the UE, i.e. when the failure was due to intra-SN PSCell change. In this case, the Rel.17 does not require response from the last serving SN to the MN.

In case there was no PSCell change in the last serving SN, it does not need to reconfigure the UE anymore. However, it has to inform the MN that there was no PSCell change and thus the problem was due to the last SN change.

Observation 2-1: In both cases, i.e., when the S-RLF was due to a wrong intra-SN PSCell change in the last serving SN and when the S-RLF was due wrong SN change, the last serving SN must respond to the MN: either to provide the SCG reconfiguration, or to let it know there was no PSCell change.

Considering the above analysis, there seem to be two options to help avoid the duplication:

- Replacing Rel.15 signalling with the new Rel.17 functionality; or

- Enhancing Rel.15 signalling for the purpose of Rel.17 functionality.

Let us discuss both options in details.

Replacing Rel.15 signalling with the new Rel.17 functionality

This can be achieved so that in case of S-RLF reported soon after an SN change, the MN sends first the Rel.17 message, with the criticality of the message type set to “reject”. If the last serving SN does not understand the procedure, it signals an error thus letting the MN know it shall use the Rel.15 procedure. If there is no error reported, the MN does not trigger Rel.15 procedure. However, since the last serving SN may need to provide SCG reconfiguration, it shall be allowed to respond to the MN that there was PSCell change, but it still needs the Rel.15 procedure to be initiated in order to reconfigure the UE.

Observation 2-2: If the Rel.17 procedure is meant to be used instead of Rel.15 procedure, there must be an option for the last serving SN to inform the MN that it still needs the Rel.15 procedure in order to reconfigure the UE (despite the fact the problem was due to an intra-SN PSCell change).

Enhancing Rel.15 signalling for the purpose of Rel.17 functionality

In this case, the Rel.17 functionality is added to the MN-initiated modification. In practise, since it is a class-1 procedure, the MN must keep the UE context until the last serving SN responds. Therefore, there is no need to forward all the information discussed for the Rel.17 to the last serving SN – the only needed updates in the signalling are a flag in the MOD REQ to indicate the MN awaits information on the intra-SN PSCell change and a response in the MOD REQ ACK. There is no need to forward to the last serving SN all the information received from the source SN.

Observation 2-3: If the Rel.15 procedure is reused, only two flags need to be added to the existing Rel.15 signalling, one in the MOD REQ and one in the MOD REQ ACK. The MN does not need to forward to the last serving SN all the information received from the source SN. Also, no extra procedures are needed to be executed between the MN and the last serving SN.

Comparing all the cases, we have following situation regarding the signalling needed between the MN and the last serving SN (with a reference to the case where overlap of Rel.15 and Rel.17 signalling is not addressed):

|  | Duplication of Rel.15 and Rel.17 | Replacing Rel.15 with Rel.17 | Enhancing Rel.15 |
| --- | --- | --- | --- |
| Intra-SN failure in the last serving SN | MN > SN: SCG FAILURE INFO MN > SN: SN MOD REQ SN > MN: SN MOD REQ ACK | MN > SN: SCG FAILURE INFO SN > MN SCG FAILURE INFO (info that SCG reconfig needed) MN > SN: SN MOD REQ SN > MN: SN MOD REQ ACK | MN > SN: SN MOD REQ SN > MN: SN MOD REQ ACK (incl. info on PSCell change) |
| SN change failure to wrong SN | MN > SN: SCG FAILURE INFO SN > MN SCG FAILURE INFO MN > SN: SN MOD REQ SN > MN: SN MOD REQ ACK | MN > SN: SCG FAILURE INFO SN > MN SCG FAILURE INFO | MN > SN: SN MOD REQ SN > MN: SN MOD REQ ACK (incl. info on PSCell change) |

The option with enhanced Rel.15 signalling is the most economical in terms of the amount of signalling needed in all cases. If the Rel.17 signalling is defined with criticality “reject”, in the most likely scenario (no intra-SN PSCell failure in the last serving SN), the solution is as good as enhanced Rel.15 signalling. But if there was PSCell change failure in the last serving SN, the solution becomes much worse. The reference case where duplication is allowed is bad in all scenarios.

Observation 2-4: Enhancing Rel.15 signalling is the most economical in all scenarios, while replacing Rel.15 with Rel.17 is good only in case there was no PSCell change in the last serving SN (when no SCG reconfig is needed). Ding nothing requires a lot of signalling in all scenarios.

Proposal 2: For the communication between the MN and the last serving SN, RAN3 agrees to enhance the Rel.15 signalling used for S-RLF handling (MN-initiated modification).

At RAN3 #114-bis, the content of the SN CHANGE REQUIRED message was discussed and it was tentatively agreed to include the Mobility Information and the CGI of the source PSCell. We do not think this requires any further work, hence the agreement can be confirmed.

Proposal 3: RAN3 shall confirm the tentative agreement from RAN3 #114-bis and add the Mobility Information and the CGI of the source PSCell in the SN CHANGE REQUIRED message.

# 3	Conclusions

In this contribution, we have discussed reminded how the Rel.15 solution for S-RLF reporting works and how the Rel.17 solution can be implemented so that unnecessary duplication is avoided. This analysis can be summarised as follows:

Observation 1-1: In most cases, the last serving SN will respond to the MN it did not trigger any PSCell change during the short interval after SN change. Therefore, if no precaution is taken, both communications (from the MN to the last serving SN and the last serving SN to the MN) will be duplicated.

Proposal 1: RAN3 shall agree that using both, the existing Rel.15 and the new Rel.17 signalling shall not be required.

Observation 2-1: In both cases, i.e., when the S-RLF was due to a wrong intra-SN PSCell change in the last serving SN and when the S-RLF was due wrong SN change, the last serving SN must respond to the MN: either to provide the SCG reconfiguration, or to let it know there was no PSCell change.

Observation 2-2: If the Rel.17 procedure is meant to be used instead of Rel.15 procedure, there must be an option for the last serving SN to inform the MN that it still needs the Rel.15 procedure in order to reconfigure the UE (despite the fact the problem was due to an intra-SN PSCell change).

Observation 2-3: If the Rel.15 procedure is reused, only two flags need to be added to the existing Rel.15 signalling, one in the MOD REQ and one in the MOD REQ ACK. The MN does not need to forward to the last serving SN all the information received from the source SN. Also, no extra procedures are needed to be executed between the MN and the last serving SN.

Observation 2-4: Enhancing Rel.15 signalling is the most economical in all scenarios, while replacing Rel.15 with Rel.17 is good only in case there was no PSCell change in the last serving SN (when no SCG reconfig is needed). Ding nothing requires a lot of signalling in all scenarios.

Proposal 2: For the communication between the MN and the last serving SN, RAN3 agrees to enhance the Rel.15 signalling used for S-RLF handling (MN-initiated modification).

Proposal 3: RAN3 shall confirm the tentative agreement from RAN3 #114-bis and add the Mobility Information and the CGI of the source PSCell in the SN CHANGE REQUIRED message.

The above proposals are implemented in TPs for BL CRs to Xn (proposed below) and to X2 [1].

# References

- R3-221766, RAN3 #115e

# Text proposal for TS 38.423

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE MODIFICATION REQUEST message, it shall start the timer TXnDCprep.

-------- skip unchanged part ----------

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted List IE in the PDU Session Resource Setup Response Info – MN terminated IE of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted to be Setup or Modified List IE in the PDU Session Resource Modification Response Info – MN terminated IE of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

Editor’s note: Conditions for the S-NG-RAN node to signal the SCG UE History Information IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message are FFS.

For each DRB configured as SN-terminated split bearer/MCG bearer, if the QoS Mapping Information IE is included in the DRBs To Be Modified List IE in the PDU Session Resource Modification Info – SN terminated IE of the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from S-NG-RAN node to M-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

If the SCG MRO Information Request IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, include the SCG MRO Information Response IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits a modification of the UE context requiring the M-NG-RAN node to report about the success of the RRC connection reconfiguration procedure, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the M-NG-RAN node. The reception of the S-NG-RAN node RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE MODIFICATION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activity Notification procedures, or stop or modify ongoing triggering of these procedures due to a previous request.

Interaction with the Xn-U Address Indication procedure

For QoS flow mapped to DRBs configured with an SN terminated bearer option and removed from the SDAP in the S-NG-RAN node the S-NG-RAN node may provides data forwarding related information in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE within the Data Forwarding and offloading Info from source NG-RAN node IE, in which case the M-NG-RAN node may decide to provide data forwarding addresses to the S-NG-RAN node and trigger the Xn-U Address Indication procedure as specified in TS 37.340 [8].

For QoS flow offloading from the S-NG-RAN node to the M-NG-RAN, the S-NG-RAN node may provide the data forwarding related information in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE within the Data Forwarding and offloading Info from source NG-RAN node IE, in which case the M-NG-RAN node may decide to provide data forwarding addresses to the S-NG-RAN node and trigger the Xn-U Address Indication procedure as specified in TS 37.340 [8].

Interactions with the S-NG-RAN node initiated S-NG-RAN node Modification:

If the SN triggered IE set to "TRUE" is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall consider that the procedure has been initiated in response to the previously initiated S-NG-RAN node initiated S-NG-RAN node Modification procedure.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node including the Target S-NG-RAN node ID IE. When the S-NG-RAN node sends the S-NODE CHANGE REQUIRED message, it shall start the timer TXnDCoverall.

The S-NODE CHANGE REQUIRED message may contain

-	the S-NG-RAN node to S-NG-RAN node Container IE.

If the M-NG-RAN node is able to perform the change requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE CHANGE CONFIRM message to the S-NG-RAN node. For DRBs configured with the PDCP entity in the S-NG-RAN node, the M-NG-RAN node may include data forwarding related information in the Data Forwarding Info from target NG-RAN node IE.

If the S-NODE CHANGE CONFIRM message includes the DRB IDs taken into use IE, the S-NG-RAN node shall, if applicable, act as specified in TS 37.340 [8].

The S-NG-RAN node may start data forwarding and stop providing user data to the UE and shall stop the timer TXnDCoverall upon reception of the S-NODE CHANGE CONFIRM message.

If the S-NODE CHANGE REQUIRED message includes the SCG UE History Information IE, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

If the S-NODE CHANGE REQUIRED message includes the Source PSCell IE, the M-NG-RAN node shall, if supported, store the information and act as specified in TS 37.340 [8].

If the S-NODE CHANGE REQUIRED message includes the Mobility Information IE, the M-NG-RAN node shall, if supported, store the information and act as specified in TS 37.340 [8].

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node |  | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. |  | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.3.49 |  |  | – |  |
| >S-NG-RAN node Security Key | O |  | 9.2.3.51 |  |  | – |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.3.17 |  |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  |  | – |  |
| >Lower Layer presence status change | O |  | 9.2.3.60 |  |  | – |  |
| >PDU Session Resources To Be Added List |  | 0..1 |  |  |  | – |  |
| >>PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  |  | – |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  | – |  |
| >>>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  |  | – |  |
| >>>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  |  | – |  |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. |  | YES | ignore |
| >PDU Session Resources To Be Modified List |  | 0..1 |  |  |  | – |  |
| >>PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE nor the PDU Session Resource Modification Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  | – |  |
| >>>PDU Session Resource Modification Info – SN terminated | O |  | 9.2.1.9 |  |  | – |  |
| >>>PDU Session Resource Modification Info – MN terminated | O |  | 9.2.1.11 |  |  | – |  |
| >>>S-NSSAI | O |  | 9.2.3.21 |  |  | YES | reject |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. |  | YES | ignore |
| >PDU Session Resources To Be Released List | O |  | PDU session List with Cause 9.2.1.26 |  |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10]. |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. |  | YES | ignore |
| Requested Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested to be released. |  | YES | ignore |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  |  | YES | ignore |
| Additional DRB IDs | O |  | DRB List 9.2.1.29 |  | Indicates additional list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) |  | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 |  | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  |  | YES | reject |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) |  | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| Requested Fast MCG recovery via SRB3 Release | O |  | ENUMERATED (true, ...) |  | Indicates that resources for fast MCG recovery via SRB3 are requested to be released. | YES | ignore |
| SN triggered | O |  | ENUMERATED (TRUE ...) |  |  | YES | ignore |
| SCG MRO Information Request | O |  | ENUMERATED (PSCell Change Information, ...) |  |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

#### 9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE is present in a PDU Session Resources Admitted To Be Added Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>PDU Session Resource Setup Response Info – SN terminated | O |  | 9.2.1.6 |  | – |  |
| >>>PDU Session Resource Setup Response Info – MN terminated | O |  | 9.2.1.8 |  | – |  |
| >PDU Session Resources Admitted To Be Modified List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources Admitted To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Response Info – SN terminated IE nor the PDU Session Resource Modification Response Info – MN terminated IE is present in a PDU Session Resources Admitted To Be Modified Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>PDU Session Resource Modification Response Info – SN terminated | O |  | 9.2.1.10 |  | – |  |
| >>>PDU Session Resource Modification Response Info – MN terminated | O |  | 9.2.1.12 |  | – |  |
| >PDU Session Resources Admitted To Be Released List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources admitted to be released List – SN terminated | O |  | PDU session List with data forwarding request info 9.2.1.24 |  | – |  |
| >>PDU Session Resources admitted to be released List – MN terminated | O |  | PDU session List with data Cause 9.2.1.26 |  | – |  |
| PDU Session Resources Not Admitted to be Added List | O |  | PDU session List 9.2.1.27 |  | YES | ignore |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | ignore |
| Admitted Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs release | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | Ignore |
| PDU Session Resources with Data Forwarding List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources with Data Forwarding List – SN terminated | M |  | PDU session List with data forwarding request info 9.2.1.24 |  | – |  |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED {true, ...} | Indicates the fast MCG recovery via SRB3 isenabled. | YES | ignore |
| Release fast MCG recovery via SRB3 | O |  | ENUMERATED {true, ...} | Indicates the fast MCG recovery via SRB3 is released. | YES | ignore |
| SCG UE History Information | O |  | 9.2.3.Y |  | YES | ignore |
| SCG MRO Information Response | O |  | ENUMERATED (PSCell Changed, PSCell not changed, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session SN Change Required List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Required Item |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  | 9.2.1.18 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| SCG UE History Information | O |  | 9.2.3.Y |  | YES | ignore |
| Source PSCell ID | O |  | NG-RAN Cell Identity 9.2.2.9 |  | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the SN change; the source S-NG-RAN node provides it in order to enable later analysis of the conditions that led to a PSCell change failure. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

////////////////////////////////////////////////////////////////////////Next Start ///////////////////////////////////////////////////////////////////////////

[ASN.1 to be added once endorsed.]