# 3GPP TSG-RAN WG3 #115e						                        R3-221790

21st Feb 2022 - 3rd Mar 2022

# Online

Source:               ZTE, Radisys, Reliance JIO, China Telecom

Title:                   Discussion the remaining issue on I-RNTI partitioning

Agenda item:      31.2.1

Document for:    Discussion and Decision

Introduction

In RAN3#114e meeting, the CRs [1][2] on I-RNTI partitioning have been endorsed, but both of them does not specify the I-RNTI structure for I-RNTI partitioning case.

In this paper, we further discuss this issue and give our proposals.

Discussion

In the endorsed CR [1] for TS 38.300, it is specified that the exact encoding of the I-RNTI is specified in TS 38.423 as follows.

Annex xx (normative): Use and structure of the I-RNTI The I-RNTI provides an NG-RAN node with a reference to the UE context and a reference to the NG-RAN node hosting the UE context. To support an NG-RAN node in resolving the Local gNB ID of the NG-RAN node hosting the UE context, the I-RNTI structure is as follows: a first part indicating an I-RNTI profile, a second part indicating a Local NG-RAN Node ID, and a third part to reference the UE context. The exact encoding of the I-RNTI is specified in TS 38.423 [xx]. In case of I-RNTI profile update, the NG-RAN node informs the neighbor NG-RAN nodes about the I-RNTI profile in use and the removal of the old I-RNTI profile.

In the endorsed CR [2] for TS 38.423, it is specified that the I-RNTI structure refers to Annex C in TS 38.300 as follows:

#### 9.2.3.40 UE Context ID This IE is used to address a UE Context within an NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE UE Context ID; M. >RRC Resume. >>I-RNTI; M; 9.2.3.46. >>Allocated C-RNTI; M; BIT STRING (SIZE (16)); Temporary C-RNTI or C-RNTI allocated to the UE by the cell where the RRC connection has been requested to be resumed, contained in the MAC RAR or MAC MSGB as defined in TS 38.321 [35] or in TS 36.321 [36].. >>Access PCI; M; NG-RAN Cell PCI 9.2.2.10; The cell PCI where the RRC connection has been requested to be resumed.. >RRC Reestablishment. >>C-RNTI; M; BIT STRING (SIZE (16)); C-RNTI contained in the RRCReestablishmentRequest message (TS 38.331 [10]) or RRCConnectionReestablishmentRequest message (TS 36.331 [14]).. >>Failure Cell PCI; M; NG-RAN Cell PCI 9.2.2.10. #### 9.2.3.46 I-RNTI The I-RNTI is defined for allocation in an NR or E-UTRA serving cell as a reference to a UE Context within an NG-RAN node. The I-RNTI is partitioned into two parts, the first part identifies the NG-RAN node that allocated the I-RNTI and the second part identifies the UE context stored in this NG-RAN node,or the I-RNTI is partitioned into three parts, the first part indicates the length of NG-RAN Node ID part of the NG-RAN Node that allocated the I-RNTI, the second part identifies the NG-RAN node that allocated the I-RNTI and the third part identifies the UE context stored in this NG-RAN node, refer to Annex C in TS 38.300[9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE I-RNTI. >I-RNTI full. >>I-RNTI full; M; BIT STRING (SIZE (40)); This IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using 40 bits (refer to I-RNTI-Value IE in TS 38.331 [10] and I-RNTI IE in TS 36.331 [14]).. >I-RNTI short. >>I-RNTI short; M; BIT STRING (SIZE (24)); This IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using 24 bits (refer to ShortI-RNTI-Value IE in TS 38.331 [10] and ShortI-RNTI IE in TS 36.331 [14])..

That means both of the endorsed CRs [1][2] on I-RNTI partitioning have not specified the exact I-RNTI structure(e.g. how may bits are occupied by each part has not been specified).

Observation:  Both of the endorsed CRs on I-RNTI partitioning [1][2] have not specified the exact I-RNTI structure(e.g. how may bits are occupied by each part has not been specified).

Usually, the basic rule is specified in stage 2 specification and the detailed description is specified in stage 3 specification, we suggest to specify the exact I-RNTI structure in stage 3 specification [e.g. TS 38.423].

Proposal: specify the exact I-RNTI structure on I-RNTI partitioning in TS 38.423.

Based on the proposal, we provide the related draft CR for TS 38.423[3].

Conclusions

In this contribution, we make the following observation and proposal:

Observation:  Both of the endorsed CRs on I-RNTI partitioning [1][2] have not specified the exact I-RNTI structure(e.g. how may bits are occupied by each part has not been specified).

Proposal: specify the exact I-RNTI structure on I-RNTI partitioning in TS 38.423.

Based on the proposal, we provide the related draft CR for TS 38.423[3].

- References

- 3GPP, R3-216079, Introduction of Local NG-RAN Node IDs for RRC_INACTIVE [RRCInactive], draftCR for 38.300, November 2021

- 3GPP, R3-216187 was 6076 was 4764 CR for TS 38.423 on I-RNTI partitioning, draftCR for 38.423, November 2021

- 3GPP, R3-221791, CR for TS 38.423 on I-RNTI partitioning, February 2022