3GPP TSG-RAN WG3 #115-e																	      R3-221801

Online, 21 Feburary – 03 March 2022

Agenda Item:	24.2

Source:	Ericsson, ZTE, China Telecom

Title:	(TP for CG-SDT BL CR to TS 38.401) Procedures for F1 CG-SDT procedures

Document for:	Approval

# Introduction

This contribution aims to provide the related TS 38.401 changes to reflect the proposals proposed in the F1 TP in R3-221794 with the discussion in R3-221795.

# Annex

TP to TS 38.401:

## 8.x	Overall procedure for Small Data Transmission during RRC Inactive

### 8.x.2	CG based SDT

The procedure for CG based small data transmission in RRC Inactive is shown in Figure 8.x.2-1.

Figure 8.x.2-1: CG based Small Data Transmission in RRC Inactive state.

1.	The gNB-CU decides to change UE into RRC_INACTIVE state.

2.	The gNB-CU-CP decides to configure CG-SDT, it sends UE CONTEXT MODIFICATION REQUEST message by including a query indication for CG-SDT related resource configuration.

3.  The gNB-DU sends UE CONTEXT MODIFICATION RESPONSE message by including the CG-SDT related resource configurations within the DU to CU RRC Information IE.

4. 	The gNB-CU-CP sends UE CONTEXT RELEASE COMMAND message to gNB-DU including the RRCRelease message to UE with the CG-SDT information. The gNB-CU notifies gNB-DU to keep the SDT RLC config and store the CG resource for SDT when the UE is entering RRC inactive with an explicit CG-SDT indicator.

5. 	The gNB-DU sends UE CONTEXT RELEASE COMPLETE message. The gNB-DU shall keep SDT RLC config and store CG resource for SDT when UE entering RRC inactive. The gNB-DU shall store which bearers are CG-SDT bearers and the C-RNTI.

6. 	The gNB-DU sends RRCRelease message to UE.

After a period of time UE in RRC_inactive mode

7.	The UE decides to perform CG based SDT procdure, it sends RRCResumeRequest message together with UL small data/UL NAS PDU. The gNB-DU buffers the UL SDT packets until the gNB-CU initiates the UE CONTEXT MODIFICATION REQUEST message to indicate that UL SDT packets are allowed.

8.	The gNB-DU sends UL RRC MESSAGE TRANSFER message including RRCResumeRequest message.

9/10.	The gNB-CU-CP initiates E1AP BEARER CONTEXT MODIFICATION procedure to resume SDT DRB.