3GPP TSG-RAN WG3#115-e                                             	                     R3-221816

E-Meeting, 21 February – 03 March 2022

Agenda item:	24.2

Source:	Nokia, Nokia Shanghai Bell

Title: 	(TP for TS 38.423) Conclusions on RACH-based SDT

Document for:	Approval

# Introduction

The Work Item on NR “Small data transmission in Inactive Sate” was agreed and is available in [2].

RAN3 started at RAN3#114 and already made a couple of agreements:

- The existing Retrieve UE Context procedure can be reused for both with and without anchor relocation scenarios with possible enhancements.

- Agreed to introduce an “SDT Indicator” in RETRIEVE UE CONTEXT REQUEST message, and the message may include other SDT Assistance Information. For split CU/DU, “SDT indicator” sent by DU to CU in the INITIAL UL RRC MESSAGE TRANSFER message.

- The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision.

- During SDT procedure, SRB PDCP PDU (FFS on the first SDT payload) shall be transferred between new gNB and anchor gNB, either via extending the XnAP RRC TRANSFER message or via defining a new XnAP class-2 procedure

And some agreements specifically for SDT without anchor relocation:

- For RA-SDT without anchor relocation case,

  - SDT related RLC bearer configuration for SDT DRB/SRB should be transferred from anchor gNB to the receiving gNB. How to include those information is FFS.

  - For SDT DRB transfer, the procedure shall enhance either legacy Periodic RNA update without UE context relocation procedure or legacy RNA update with UE context relocation procedure. The receiving gNB forwards PDCP PDUs (RLC SDUs) via GTP-U tunnel per SDT DRB to the anchor gNB.

  - the anchor gNB is not allowed to transfer its security related IEs included in “UE Context Information – Retrieve UE Context Response” IE to the receiving gNB.

  - Enhance RNA update without UE context relocation procedure for RA-SDT without anchor relocation case

At last RAN3#114bis the following complementary agreements were reached:

- WA: Solution 2/3/4 is the basis for further work: exchange partial context using new procedures

- The new procedure may be class 1 or class 2

- Extend the XnAP: RRC TRANSFER message, to forward the UL/DL SRB PDCP PDU during SDT procedure without anchor relocation between new gNB and anchor gNB

- Transfer the first SRB/DRB transfer as the same method as the subsequent SRB/DRB transfer

- The additional SDT assistant information is needed, but it is no need to consult with RAN2. It includes either BSR information or single/multiple packets indication as optional IEs. FFS on others.

And the following items were identified as still open:

- FFS that either new Class 1 message or a new Class 2 message is used to transfer SDT configuration in case of RA-SDT without anchor relocation.

- Whether SDT data handling between gNBs should be per DRB (not per PDU session), different to the fundamental NR mobility design per PDU session basis.

- Whether Xn-U Address Indication should be re-used to deliver DL forwarding TNL from new gNB, or be replaced only for SDT without anchor relocation.

- FFS that the additional SDT assistant information includes either BSR information or single/multiple packets indication as optional IEs, and FFS on other IEs.

- The following IEs in the SDT configuration (from anchor gNB to receiving gNB) are FFS:  SRB, DRB QoS, S-NSSAI, RLC Mode, PDCP SN Length, DL Forwarding (9.2.3.34) or Packet Data Indication, PDU Session ID

This paper addresses the remaining open items to reach conclusion.

# Discussion

We can consider the following open points for RACH-based SDT without anchor relocation

- fetch of partial RLC context

We assume in the following that a class 1 procedure is used.

However, we would like the change the names which are not appropriate with:

- partial context retrieve request from anchor gNB to new gNB,

- partial context retrieve response from new gNB to anchor gNB,

- partial context retrieve failure from new gNB to anchor gNB.

In fact the anchor gNB does not retrieve the context but transfers it to the new gNB.

We propose to rename as follows:

- partial context transfer,

- partial context transfer acknowledge,

- Partial context transfer reject.

Proposal 1: rename the class 1 messages into partial context transfer, partial context transfer acknowledge, partial context transfer reject.

- Assistance information

The following was already agreed at RAN3#114bis:

The additional SDT assistant information is needed, but it is no need to consult with RAN2. It includes either BSR information or single/multiple packets indication as optional IEs. FFS on others

We think that the raw BSR needs not be sent over the Xn interface but it is enough to translate it into single/multiple packets.

Proposal 2: agree single/multiple packets as Assistance Information sent over the Xn interface.

- Other Assistance information

As it was commented at RAN3#114bis, the target gNB may not support the feature of SDT without anchor relocation. In that case it needs to tell the anchor gNB as alternative assistance information to avoid that the anchor gNB decides to build and send the partial context uselessly.

Proposal 3: agree to add “relocation required” as Assistance Information in the XnAP UE Context Retrieve Request message.

- Other Assistance information, end of SDT procedure

About yet other assistance information, it was commented at RAN3#114bis that the termination of the SDT procedure should be assisted as well. The issue is that in the current call flow the anchor gNB has no idea when to terminate the SDT procedure.

The gNB-DU should be able to determine whether there is no more packets to be sent. It should be the one to trigger the end of the SDT procedure.

When DU detects that no more packet is to be sent, the DU can send the F1 Inactivity Notification to target CU CP. Then for target CU CP to inform over XnAP the anchor CU CP there are two possible options:

- Option A: the target CU CP sends a Xn Context Release message including an “end SDT” or  “last packet” indication to anchor CU CP, then anchor CU CP completes the SDT procedure by sending the Xn Retrieve UE Context Failure message including the RRC Release message. The RRC Release is sent to the UE.

- Option B: introduce a new Xn message for target CU CP to indicate “end of SDT” or “last packet” to anchor CU CP. Then Anchor CU CP completes the SDT procedure by sending the Xn Retrieve UE Context Failure message including the RRC Release message. The RRC Release is sent to the UE.

The two options for SDT termination are illustrated in the figure 2 below.

Figure 2: Two possible options for SDT procedure Termination

Option A looks simpler because the UE Context Release message can be reused to indicate the end of SDT procedure or last packet. Then the SDT procedure is completed by anchor gNB sending the final Xn Retrieve UE Context Failure which piggy-backs the RRC Release message.

Proposal 4: new CU CP sends an Xn UE Context release message including a “last packet” or “end SDT” indication which triggers the anchor CU CP to send the final UE Context Retrieve Failure message.

- Handling of DL data

Sending UL TNL information from anchor gNB to receiving gNB is not enough. There may be also some pending DL data at anchor CU UP. The presence of DL data pending at CU UP should be known by the anchor CU CP which can inform target CU CP and target DU in order to retrieve a DL TEID from the target DU. The presence

Proposal 5: include a PDI (Packet Data Indication) in the Partial Context Transfer Acknowledge message from anchor CU CP to target CU CP.

# Conclusion and Proposals

This paper has investigated the remaining RAN3 open points of the RA-SDT with the following proposals:

Proposal 1: rename the class 1 messages into partial context delivery request and partial context delivery acknowledge.

Proposal 2: agree single/multiple packets as assistance information sent over the Xn interface.

Proposal 3: agree to add “relocation required” as assistance information in the XnAP UE Context Retrieve Request message.

Proposal 4: new CU CP sends an Xn UE Context Release message including a “last packet” or “end SDT” indication which triggers the anchor CU CP to send the final UE Context Retrieve Failure message.

Proposal 5: include a PDI (Packet Data Indication) in the Partial Context Deliver Request from anchor CU CP to target CU CP.

Proposal 6: agree the TP below for TS 38.423 and the TP in [3] for TS 38.473.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- RP-212594, Work Item on NR small data transmissions in INACTIVE state

- R3-220204, S(TP for TS 38.473) Support of RACH-based SDT

# TP for Baseline CR TS 38.423

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEGE | PARTIAL UE CONTEXT TRANSFER REJECT |

Not modified

Editor’s note: whether a new Class 1 or Class 2 procedure is FFS. Details are pending.

### 8.2.x	Partial UE Context Transfer

#### 8.2.x.1	General

The purpose of the Partial UE Context Transfer procedure is to transfer part of the UE context from the old NG-RAN node to the new NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Partial UE Context Transfer, successful operation

The old NG-RAN node initiates the procedure by sending the PARTIAL UE CONTEXT TRANSFER message to the old NG-RAN node.

If the new NG-RAN node is able to identify the UE context, it shall respond to the new NG-RAN node with the PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE message.

#### 8.2.x.3	Unsuccessful Operation

Figure 8.2.4.3-1: Partial UE Context Transfer, unsuccessful operation

If the new NG-RAN is not able to accept the SDT transaction without anchor relocation, it shall respond to the old NG-RAN node with the PARTIAL UE CONTEXT TRANSFER REJECT message.

#### 8.2.x.4	Abnormal Condition

Void.

Not modified

Editor’s note: whether a new Class 1 or Class 2 procedure is FFS. Details are pending.

#### 9.1.1.xx	PARTIAL UE CONTEXT TRANSFER

This message is sent by the old NG-RAN node to transfer part of the UE Context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node. | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node. | YES | ignore |
| Partial UE Context Information for SDT | O |  | 9.2.3.y |  | YES | ignore |

#### 9.1.1.xy	PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE

This message is sent by the new NG-RAN node to acknowledge the retrieval of part of the UE context from the old NG-RAN node. This message is also used to provide data forwarding related information for NR SDT.

Direction: new NG-RAN node  old NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| SDT Data Forwarding DRB List |  | 0..1 |  |  | YES | ignore |
| >SDT Data Forwarding DRB Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>DRB ID | M |  | 9.2.3.33 |  |  |  |
| >>UL TNL Information | O |  | UP Transport Layer Information 9.2.3.30 |  |  |  |
| >>Packet Data Indication | O |  | ENUMERATED (true,…) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs. Value is 32. |

#### 9.1.1.yy	PARTIAL UE CONTEXT TRANSFER REJECT

This message is sent by the old NG-RAN node to inform the new NG-RAN node that the Retrieve UE Context procedure has failed.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node. | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

Not modified

#### 9.1.1.5	UE CONTEXT RELEASE

This message is sent by the target NG-RAN node to the source NG-RAN node to indicate that resources can be released. It is also sent by the target NG-RAN node to the Anchor NG-RAN node to indicate that resources related to an ongoing SDT procedure can be released.

Direction: target NG-RAN node  source NG-RAN node, M-NG-RAN node  S-NG-RAN node, target NG-RAN node  anchor NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated for handover at the source NG-RAN node or for dual connectivity at the S-NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated for handover at the target NG-RAN node or for dual connectivity at the M-NG-RAN node. | YES | reject |
| SDT Status | M |  | ENUMERATED (end SDT,…) |  | YES | reject |

Not modified

#### 9.2.3.x	SDT Support Request

This IE indicates that the UE requested for SDT and may include additional assistance information (FFS).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SDT Indicator | M |  | ENUMERATED (true,…) |  |
| SDT assistant information | O |  | ENUMERATED (single SDT, multiple SDT, relocation required …) |  |

Not modified

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Not modified

LTEV2XServicesAuthorized,

MR-DC-ResourceCoordinationInfo,

SDT-STATUS,

ServedCells-E-UTRA,

ServedCells-NR,

Not modified

id-RespondingNodeTypeConfigUpdateAck,

id-RRCResumeCause,

id-SDT-STATUS,

id-selectedPLMN,

id-ServedCellsToActivate,

-- **************************************************************

--

#### -- UE CONTEXT RELEASE

--

-- **************************************************************

UEContextRelease ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{UEContextRelease-IEs}},

...

}

UEContextRelease-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-sDT-Status					CRITICALITY ignore					TYPE SDT-STATUS							PRESENCE optional},

...

}

-- **************************************************************

Not modified

FFS on the new procedure details

-- **************************************************************

--

#### -- Partial UE CONTEXT TRANSFER

--

-- **************************************************************

-- **************************************************************

--

#### -- Partial UE CONTEXT TRANSFER ACKNOWLEDGE

--

-- **************************************************************

-- **************************************************************

--

#### -- Partial UE CONTEXT RTRANSFER REJECT

--

-- **************************************************************

Not modified

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

Not modified

#### -- S

SDT-STATUS	::= ENUMERATED {end SDT, ...}

SecondarydataForwardingInfoFromTarget-Item::= SEQUENCE {

secondarydataForwardingInfoFromTarget				DataForwardingInfoFromTargetNGRANnode,

iE-Extensions		ProtocolExtensionContainer { { SecondarydataForwardingInfoFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

SecondarydataForwardingInfoFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Not modified

SymbolAllocation-in-Slot-BothDLandUL ::= SEQUENCE {

numberofDLSymbols	INTEGER (0..13),

numberofULSymbols	INTEGER (0..13),

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-BothDLandUL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-BothDLandUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SDTSupportRequest ::= SEQUENCE {

sdtindicator					SDTIndicator,

sdtAssistantInfo				SDTAssistantInfo,

iE-Extensions				ProtocolExtensionContainer { { SDTSupportRequest-ExtIEs} } OPTIONAL,

...

}

SDTSupportRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SDTIndicator ::= ENUMERATED {true, ...}

SDTAssistantInfo	::= ENUMERATED {single SDT, multiple SDT, relocation required, ...}

#### -- T

Not modified

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Not modified

id-UESpecificDRX														ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour								ProtocolIE-ID ::= 249

id-QoS-Mapping-Information												ProtocolIE-ID ::= 250

id-AdditionLocationInformation											ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode								ProtocolIE-ID ::= 252

id-SDT-STATUS															ProtocolIE-ID ::= xxx

END

-- ASN1STOP