3GPP TSG-RAN WG3#115-e                                             	                     R3-221817

E-Meeting, 21 February – 03 March 2022

Agenda item:	24.2

Source:	Nokia, Nokia Shanghai Bell

Title: 	(TP for TS 38.473) Conclusions on RACH-based SDT

Document for:	Approval

# Introduction

The Work Item on NR “Small data transmission in Inactive Sate” was agreed and is available in [2].

RAN3 started at RAN3#114 and already made a couple of agreements:

- The existing Retrieve UE Context procedure can be reused for both with and without anchor relocation scenarios with possible enhancements.

- Agreed to introduce an “SDT Indicator” in RETRIEVE UE CONTEXT REQUEST message, and the message may include other SDT Assistance Information. For split CU/DU, “SDT indicator” sent by DU to CU in the INITIAL UL RRC MESSAGE TRANSFER message.

- The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision.

- During SDT procedure, SRB PDCP PDU (FFS on the first SDT payload) shall be transferred between new gNB and anchor gNB, either via extending the XnAP RRC TRANSFER message or via defining a new XnAP class-2 procedure

And some agreements specifically for SDT without anchor relocation:

- For RA-SDT without anchor relocation case,

  - SDT related RLC bearer configuration for SDT DRB/SRB should be transferred from anchor gNB to the receiving gNB. How to include those information is FFS.

  - For SDT DRB transfer, the procedure shall enhance either legacy Periodic RNA update without UE context relocation procedure or legacy RNA update with UE context relocation procedure. The receiving gNB forwards PDCP PDUs (RLC SDUs) via GTP-U tunnel per SDT DRB to the anchor gNB.

  - the anchor gNB is not allowed to transfer its security related IEs included in “UE Context Information – Retrieve UE Context Response” IE to the receiving gNB.

  - Enhance RNA update without UE context relocation procedure for RA-SDT without anchor relocation case

At last RAN3#114bis the following complemntary agreements were reached:

- WA: Solution 2/3/4 is the basis for further work: exchange partial context using new procedures

- The new procedure may be class 1 or class 2

- Extend the XnAP: RRC TRANSFER message, to forward the UL/DL SRB PDCP PDU during SDT procedure without anchor relocation between new gNB and anchor gNB

- Transfer the first SRB/DRB transfer as the same method as the subsequent SRB/DRB transfer

- The additional SDT assistant information is needed, but it is no need to consult with RAN2. It includes either BSR information or single/multiple packets indication as optional IEs. FFS on others.

And the following items were identified as still open:

- FFS that either new Class 1 message or a new Class 2 message is used to transfer SDT configuration in case of RA-SDT without anchor relocation.

- Whether SDT data handling between gNBs should be per DRB (not per PDU session), different to the fundamental NR mobility design per PDU session basis.

- Whether Xn-U Address Indication should be re-used to deliver DL forwarding TNL from new gNB, or be replaced only for SDT without anchor relocation.

- FFS that the additional SDT assistant information includes either BSR information or single/multiple packets indication as optional IEs, and FFS on other IEs.

- The following IEs in the SDT configuration (from anchor gNB to receiving gNB) are FFS:  SRB, DRB QoS, S-NSSAI, RLC Mode, PDCP SN Length, DL Forwarding (9.2.3.34) or Packet Data Indication, PDU Session ID

This paper addresses the remaining open items to reach conclusion.

# Discussion

We can consider the following open points for RACH-based SDT without anchor relocation

- fetch of partial RLC context

We assume in the following that a class 1 procedure is used.

However, we would like the change the names which are not appropriate with:

- partial context retrieve request from anchor gNB to new gNB,

- partial context retrieve response from new gNB to anchor gNB,

- partial context retrieve failure from new gNB to anchor gNB.

In fact the anchor gNB does not retrieve the context but transfers it to the new gNB.

We propose to rename as follows:

- partial context transfer,

- partial context transfer acknowledge,

- Partial context transfer reject.

Proposal 1: rename the class 1 messages into partial context transfer, partial context transfer acknowledge, partial context transfer reject.

- Assistance information

The following was already agreed at RAN3#114bis:

The additional SDT assistant information is needed, but it is no need to consult with RAN2. It includes either BSR information or single/multiple packets indication as optional IEs. FFS on others

We think that the raw BSR needs not be sent over the Xn interface but it is enough to translate it into single/multiple packets.

Proposal 2: agree single/multiple packets as assistance information sent over the Xn interface.

- Other Assistance information

As it was commented at RAN3#114bis, the target gNB may not support the feature of SDT without anchor relocation. In that case it needs to tell the anchor gNB as alternative assistance information to avoid that the anchor gNB decides to build and send the partial context uselessly.

Proposal 3: agree to add “relocation required” as assistance information in the XnAP UE Context Retrieve Request message.

- Other Assistance information, end of SDT procedure

About yet other assistance information, it was commented at RAN3#114bis that the termination of the SDT procedure should be assisted as well. The issue is that in the current call flow the anchor gNB has no idea when to terminate the SDT procedure.

The gNB-DU should be able to determine whether there is no more packets to be sent. It should be the one to trigger the end of the SDT procedure.

When DU detects that no more packet is to be sent, the DU can send the F1 Inactivity Notification to target CU CP. Then for target CU CP to inform over XnAP the anchor CU CP there are two possible options:

- Option A: the target CU CP sends a Xn Context Release message including an “end SDT” or  “last packet” indication to anchor CU CP, then anchor CU CP completes the SDT procedure by sending the Xn Retrieve UE Context Failure message including the RRC Release message. The RRC Release is sent to the UE.

- Option B: introduce a new Xn message for target CU CP to indicate “end of SDT” or “last packet” to anchor CU CP. Then Anchor CU CP completes the SDT procedure by sending the Xn Retrieve UE Context Failure message including the RRC Release message. The RRC Release is sent to the UE.

The two options for SDT termination are illustrated in the figure 2 below.

Figure 2: Two possible options for SDT procedure Termination

Option A looks simpler because the UE Context Release message can be reused to indicate the end of SDT procedure or last packet. Then the SDT procedure is completed by anchor gNB sending the final Xn Retrieve UE Context Failure which piggy-backs the RRC Release message.

Proposal 4: new CU CP sends an Xn UE Context release message including a “last packet” or “end SDT” indication which triggers the anchor CU CP to send the final UE Context Retrieve Failure message.

- Handling of DL data

Sending UL TNL information from anchor gNB to receiving gNB is not enough. There may be also some pending DL data at anchor CU UP. The presence of DL data pending at CU UP should be known by the anchor CU CP which can inform target CU CP and target DU in order to retrieve a DL TEID from the target DU. The presence

Proposal 5: include a PDI (Packet Data Indication) in the Partial Context Transfer Acknowledge message from anchor CU CP to target CU CP.

# Conclusion and Proposals

This paper has investigated the remaining RAN3 open points of the RA-SDT with the following proposals:

Proposal 1: rename the class 1 messages into partial context delivery request and partial context delivery acknowledge.

Proposal 2: agree single/multiple packets as assistance information sent over the Xn interface.

Proposal 3: agree to add “relocation required” as assistance information in the XnAP UE Context Retrieve Request message.

Proposal 4: new CU CP sends an Xn UE Context release message including a “last packet” or “end SDT” indication which triggers the anchor CU CP to send the final UE Context Retrieve Failure message.

Proposal 5: include a PDI (Packet Data Indication) in the Partial Context Deliver Request from anchor CU CP to target CU CP.

Proposal 6: agree the TP in [3] for TS 38.423 and the TP below for TS 38.473.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- RP-212594, Work Item on NR small data transmissions in INACTIVE state

- R3-22xxxx, (TP for TS 38.423) Support of RACH-based SDT

# TP for Baseline CR TS 38.473

#### 9.2.2.12	UE INACTIVITY NOTIFICATION

This message is sent by the gNB-DU to provide information about the UE activity to the gNB-CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Activity List |  | 1 |  |  | YES | reject |
| >DRB Activity Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB Activity | O |  | ENUMERATED (Active, Not active) |  | - |  |
| UE Activity Status | M |  | ENUMERATED (end SDT, …) | If present the DRB Activity List IE shall be ignored. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

Not modified

#### 9.3.1.x	SDT Information

This IE is used to indicate an SDT transaction and to provide the assistant information from the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SDT Indicator | M |  | ENUMERATED (true,…) |  |
| SDT assistant information | O |  | ENUMERATED (single SDT, multiple SDT, …) |  |

Not modified

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Not modified

id-MeasurementBeamInfoRequest,

id-E-CID-ReportCharacteristics,

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-UE-Activity-Status,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

Not modified

-- **************************************************************

--

##### -- UE Inactivity Notification

--

-- **************************************************************

UEInactivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ UEInactivityNotificationIEs}},

...

}

UEInactivityNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-DRB-Activity-List							CRITICALITY reject	TYPE DRB-Activity-List							PRESENCE mandatory	}|

{ ID id-UE-Activity-Status							CRITICALITY reject	TYPE UE-Activity-Status							PRESENCE optional	},

...

}

DRB-Activity-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRB-Activity-ItemIEs } }

DRB-Activity-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Activity-Item			CRITICALITY reject	TYPE DRB-Activity-Item		PRESENCE mandatory},

...

}

UE-Activity-Status	::=	ENUMERATED {end SDT, ...}

-- **************************************************************

Next change

SCS-SpecificCarrier ::=            SEQUENCE {

offsetToCarrier                     INTEGER (0..2199,...),

subcarrierSpacing                   ENUMERATED {kHz15, kHz30, kHz60, kHz120,...},

carrierBandwidth                    INTEGER (1..275,...),

iE-Extensions						ProtocolExtensionContainer { { SCS-SpecificCarrier-ExtIEs } } OPTIONAL

}

SCS-SpecificCarrier-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTInformation ::= SEQUENCE {

sdtIndicator					ENUMERATED(true, ...),

sdtAssistantInformation			SDTAssistantInformation,

iE-Extensions					ProtocolExtensionContainer { { SDTInformation-ExtIEs } } OPTIONAL

}

SDTInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTAssistantInformation ::=  ENUMERATED {singleSDT, multipleSDT, ...}

Search-window-information ::= SEQUENCE {

expectedPropagationDelay		INTEGER (-3841..3841,...),

delayUncertainty				INTEGER (1..246,...),

iE-Extensions					ProtocolExtensionContainer { { Search-window-information-ExtIEs } } OPTIONAL

}

Search-window-information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Not modified

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Not modified

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-UE-Activity-Status								ProtocolIE-ID ::= xxx

END

-- ASN1STOP