3GPP TSG-RAN WG3#115-e                                             	                     R3-221823

E-Meeting, 21 February – 03 March 2022

Agenda item:	30.3

Source:	Nokia, Nokia Shanghai Bell

Title: 	(TP for TS 38.473) Stage 3 conclusions for UE Power Saving Enhancements

Document for:	Approval

# Introduction

The Work Item on “UE Paging Subgrouping” was agreed and is available in [2].

This work or at least the intention is similar to the work done on WUS (Wake Up Signal) in the past where UEs in the same paging occasion are divided into multiple subgroups. A UE will monitor its paging occasion only if it receives a notification that its subgroup is paged. There are two flavors of the feature: CN-based subgrouping and UE-ID based subgrouping.

At last RAN3#114bis meeting, the following agreements were reached:

- The CN provides subgroup ID included in subgroup information.

- Include UE paging subgroup information into the NG Paging message for CN paging.

- Introduce UE subgroup information into NGAP Core Network Assistance Information.

- Introduce UE paging subgroup information into F1 Paging message for paging a UE in RRC_INACTIVE/RRC_IDLE.

- Adopt the terminology "PEIPS Assistance Information" as the IE name to align with SA2 terminology, containing a CN subgroup ID.

- The CN assigned subgroup ID is encoded as 0…7. (note: further checking maybe needed)

- WA: NG-RAN node can know the total number of subgroups supported by CN via OAM.

And the following open issues remain:

- XnAP impact and BL CR are subject to RAN2 decision concerning the applicability of the PEI to the last used cell

- F1AP impact due to UE ID based subgrouping capability

# Discussion

# UE ID based subgrouping and F1AP

At the last RAN3 meeting, the following remained an open issue:

- F1AP impact due to UE ID based subgrouping capability

RAN2 has agreed that the capability of supporting UE-ID based subgrouping will be included in the Radio Capability for Paging IE which is therefore received by the gNB in the Paging message or Initial Context Setup Request message. However, this IE is decoded by the gNB-CU CP in case of split architecture. This means that the gNB-DU would not know whether it is applicable for a particular UE.

If the gNB-DU receives a F1 Paging message including a CN subgroup ID and it does not support the CN based subgrouping method, it will need to either:

- page using the UE ID based method for the UE if the UE supports it,

- Otherwise, page using the legacy paging method.

It is therefore necessary that the gNC-CU informs the gNB-DU whether the UE supports the UE ID based method. There are two ways for this:

- Option 1: Include this UE capability in every F1 paging message,

- Option 2: Include this UE capability in the F1 initial context setup request message.

Since this is a rather static capability, we prefer option 2.

Proposal 1: gNB-CU sends the UE capability for UE ID based subgrouping feature to the gNB-DU.

Proposal 2: agree the TP below for TS 38.473 and the TP in tdoc [3] for TS 38.470.

# Conclusion and Proposals

This paper has investigated the remaining RAN architecture impacts associated with the subgrouping feature and made the following proposals:

Proposal 1: gNB-CU sends the UE capability for UE ID based subgrouping feature to the gNB-DU.

Proposal 2: agree the TP below for TS 38.473 and the TP in tdoc [3] for TS 38.470.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- RP-200938, Work Item on UE Power Saving

- R3-221822, (TP for TS 38.470) Support of UE ID paging subgrouping

# TP for TS 38.473

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure.

Not modified

If the Serving NID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall combine the Serving NID IE with the Serving PLMN IE to identify the serving NPN, and may take it into account for UE context establishment.

If the Estimated Arrival Probability IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If the UE CONTEXT SETUP REQUEST message contains the UE ID Subgrouping Support Indication IE, the gNB-DU shall, if supported, store it in the UE context and take it into account when paging the UE as specified in TS 38.300 [6].

Not modified

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| Trace Activation | O |  | 9.3.1.88 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS Information | M |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| Configured BAP Address | O |  | 9.3.1.111 | The BAP address configured for the corresponding child IAB-node. | YES | reject |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier |  |  | 9.3.1.121 |  | - |  |
| >>RLC mode | M |  | 9.3.1.27 |  | - |  |
| Conditional Inter-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | - | - |
| >Target gNB-DU UE F1AP ID | C-ifCHOmod |  | 9.3.1.5 | Allocated at the target gNB-DU | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.151 |  | YES | ignore |
| Serving NID | O |  | 9.3.1.155 |  | YES | reject |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| UE ID Subgrouping Support Indication | O |  | ENUMERATED (supported, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

Not modified

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Not modified

MeasurementBeamInfoRequest,

E-CID-ReportCharacteristics,

Extended-GNB-CU-Name,

Extended-GNB-DU-Name,

F1CTransferPath,

SCGIndicator,

SpatialRelationPerSRSResource,

UEIDSubgroupingSupportIndication

FROM F1AP-IEs

Not modified

id-TRP-MeasurementRequestList,

id-MeasurementBeamInfoRequest,

id-E-CID-ReportCharacteristics,

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-UEIDSubgroupingSupportIndication,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

Not modified

-- **************************************************************

--

##### -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional 	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE NRCGI												PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE mandatory	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE mandatory}|

{ ID id-Candidate-SpCell-List					CRITICALITY ignore	TYPE Candidate-SpCell-List							PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-SCell-ToBeSetup-List					CRITICALITY ignore	TYPE SCell-ToBeSetup-List								PRESENCE optional	}|

{ ID id-SRBs-ToBeSetup-List						CRITICALITY reject	TYPE SRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-DRBs-ToBeSetup-List						CRITICALITY reject	TYPE DRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY ignore	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional	}|

{ ID id-ServingPLMN								CRITICALITY ignore	TYPE PLMN-Identity										PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE conditional }|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-new-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional }|

{ ID id-RANUEID									CRITICALITY ignore	TYPE RANUEID											PRESENCE optional	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-BHChannels-ToBeSetup-List				CRITICALITY reject	TYPE BHChannels-ToBeSetup-List						PRESENCE optional	}|

{ ID id-ConfiguredBAPAddress					CRITICALITY reject	TYPE BAPAddress											PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetup-List					CRITICALITY reject	TYPE SLDRBs-ToBeSetup-List							PRESENCE optional	}|

{ ID id-ConditionalInterDUMobilityInformation	CRITICALITY reject	TYPE ConditionalInterDUMobilityInformation		PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE 		MDTPLMNList									PRESENCE optional }|

{ ID id-ServingNID								CRITICALITY reject	TYPE NID												PRESENCE optional }|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-UEIDSubroupingSupportIndication			CRITICALITY ignore	TYPE UEIDSubgroupingSupportIndication				PRESENCE optional },

...

}

Candidate-SpCell-List::= SEQUENCE (SIZE(1..maxnoofCandidateSpCells)) OF ProtocolIE-SingleContainer { { Candidate-SpCell-ItemIEs} }

SCell-ToBeSetup-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetup-ItemIEs} }

SRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetup-ItemIEs} }

DRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetup-ItemIEs} }

BHChannels-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeSetup-ItemIEs} }

SLDRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeSetup-ItemIEs} }

Candidate-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Candidate-SpCell-Item					CRITICALITY ignore	TYPE Candidate-SpCell-Item						PRESENCE mandatory	},

...

}

SCell-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetup-Item						CRITICALITY ignore	TYPE SCell-ToBeSetup-Item					PRESENCE mandatory	},

...

}

SRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetup-Item		CRITICALITY reject		TYPE SRBs-ToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetup-Item					CRITICALITY reject	TYPE DRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

BHChannels-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeSetup-Item					CRITICALITY reject	TYPE BHChannels-ToBeSetup-Item					PRESENCE mandatory},

...

}

SLDRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeSetup-Item					CRITICALITY reject	TYPE SLDRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

-- **************************************************************

Not modified

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Not modified

UEIdentityIndexValue ::= CHOICE {

indexLength10			BIT STRING (SIZE (10)),

choice-extension		ProtocolIE-SingleContainer { {UEIdentityIndexValueChoice-ExtIEs} }

}

UEIdentityIndexValueChoice-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UEIDSubgroupingSupportIndication ::= ENUMERATED {supported, ...}

UL-AoA ::= SEQUENCE {

azimuthAoA					INTEGER (0..3599),

zenithAoA					INTEGER (0..1799)	OPTIONAL,

lCS-to-GCS-TranslationAoA	LCS-to-GCS-TranslationAoA		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { UL-AoA-ExtIEs } }

}

Not modified

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Not modified

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-UEIDSubgroupingSupportIndication							ProtocolIE-ID ::= xxx

END

-- ASN1STOP