3GPP TSG-RAN WG3 Meeting #115-e	R3-221841

# Online, February 21st – March 3rd 2022

Agenda Item:	13.2.1.1

Source:	Qualcomm

Title:	(TP for IAB BL CR for TS 38.401): IAB Inter-Donor St2 Procedures

Document for:	Discussion

- Introduction

In TSG RAN Meeting #114-bis-e, the following agreements have been achieved [1]:

IP address assignment, the existing signaling, i.e., RRC container in Xn HO Req, is used to provide the non-F1-terminating CU with the boundary-node’s old IP address configuration. The new Xn procedure can be used after the boundary-IAB-MT’s Xn HO or SN Addition procedure. The new Xn procedure to include a boundary-node ID, which is retained after reception of the UE Context Release message. This identifier can be the XnAP UE ID or the BAP address. Further selection expected in next meeting. One common Xn procedure is used for the QoS/L2/IP-address info exchange of the boundary node and the descendent nodes. After partial migration, the non-F1-terminating CU can use the Xn HO procedure to migrate all offloaded traffic back to the F1-terminating-CU’s topology. The non-F1-terminating CU of a dual-connected boundary node can request partial release of offloaded traffic back to the F1-terminating-CU’s topology. For XnHO, NRDC setup, and UE Context Retrieval, a new indication IE is introduced to indicate that the IAB-MT does not have a PDU session activated. If non-F1-terminating CU is not able to guarantee the per topology fragment QoS requirement, it should reject the request from F1-terminating CU. Both full and partial rejection are supported. For inter-donor RLF recovery, the information about IP addresses requested for the boundary node is included by the F1-terminating CU in the RRC Container of the Retrieve UE Context Response message. For inter-donor RLF recovery, the transport migration via the new Xn procedure is conducted by the F1-terminating CU after it receives the Retrieve UE Context request.

This paper addresses remaining Stage-2 for all procedures under this agenda item. A TP for the BL CR for TS 38.401 is included in the appendix.

- Traffic offload for descendant IAB-nodes

RAN3#114-e agreed that for IP address reconfiguration of descendant nodes, the new Xn procedure between the F1-terminating and non-F1-terminating CUs is used, and the F1-terminating CU adds, replaces or releases the IP addresses on the descendent node via RRC.

RAN3#114-e also agreed that the same Xn procedure is also used for the transfer of the descendent node’s QoS info/L2 info.

RAN3#114-e also agreed that the F1-terminating CU includes into the Xn procedure the TNL information necessary for the non-F1-terminating CU to configure the DL mapping on its Donor-DU.

The above agreements imply that the new Xn procedure must be run a first time to obtain new IP addresses for the descendant IAB-node, and a second time to transfer context of the descendant traffic along with the associated (per-traffic) new IP addresses.

Proposal 1: The new Xn procedure needs to be run a first time to obtain new IP addresses for the descendant node and a second time to transfer the traffic context with per-traffic new IP addresses included.

- Traffic offload for dual-connected boundary IAB-node

Traffic offload to the non-F1-terminating CU’s topology may not occur promptly after dual connectivity is established for the boundary IAB-node. In the meantime, the boundary IAB-node may acquire new IP addresses from the non-F1-terminating CU via RRC (as agreed by RAN3#113-e). Before traffic offload of the dual-connected boundary IAB-node is initiated to the non-F1-terminating CU’s topology, the dual-connected boundary IAB-node has no reason to report the new IP addresses to the F1-terminating CU.

Observation 1: A dual-connected boundary IAB-node may not report IP addresses received from the non-F1-terminating CU before traffic offload is initiated by the F1-terminating CU.

Consequently, by the time the F1-terminating CU sends the new Xn message with the context of traffic to be offloaded, the F1-terminating CU may not be aware of the new IP addresses that will be used by the boundary node for the traffic. The following is proposed:

Proposal 2: The F1-terminating CU may provide context for traffic offload on behalf of a dual-connected boundary IAB-node without including per-traffic new TNL address info if the latter is unknown.

Upon receiving the new Xn request message, the non-F1-terminating CU recognizes the identifier of the boundary IAB-node. The non-F1-terminating CU selects a proper IAB-donor-DU based on the traffic context and the boundary node’s ID and returns UL mappings including the selected IAB-donor-DU’s BAP address to the F1-terminating CU. If it hasn’t done so already, the non-F1-terminating CU configures the boundary IAB-node with IP addresses anchored at the selected IAB-donor-DU.

Proposal 3: The non-F1-terminating CU configures the dual-connected boundary IAB-node with new IP addresses pertaining to the donor-DU selected upon receiving the traffic context from the F1-terminating CU.

The F1-terminating CU configures the new UL mappings at the boundary node. The boundary node recognizes the BAP address of the donor-DU in the UL mappings. The boundary node selects proper IP addresses anchored at the latter donor-DU. The boundary node reports these IP addresses (for F1-U) to the F1-terminating CU.

Proposal 4: The dual-connected boundary IAB-node selects a new IP address (configured by the non-F1-terminating CU) corresponding to the donor-DU whose BAP address is included in the updated UL mapping of the offloaded traffic.

Upon receiving the new IP addresses, the F1-terminating CU may modify the context of the offloaded traffic via the new Xn procedure.

- Revocation procedures

For migration and recovery:

As agreed in last meeting, CU2 can initiate the XnAP HO for the boundary MT to realize full release of traffic offload. Use of the new Xn procedure for the purpose of revoking is not required.

For redundancy:

Offloaded traffic may be returned to the F1-terminating CU’s topology without release of SN. This implies revocation in the case of redundancy is based on the new Xn procedure. Two options were discussed in the last meeting:

- Option 1: CU2 initiates the new procedure to request modification (of L2 info only) and full/partial release (e.g., for the purpose of revoking)

- Option 2: CU2 initiates a new class-2 procedure to provide serving status of CU2’s topology, which can help CU1’s decision making wrt modification and release (e.g. for the purpose of revoking) of traffic migration.

If Option 2 is used, there are two sub-options:

- Sub-option 2a: CU2 provides general status of its traffic condition. CU1 decides which traffic to modify or flip back to its topology. In this sub-option, since CU1 does not know which traffic traverses what path in CU2’s topology nor the condition of each path, CU1 may make an inadequate decision.

- Sub-option 2b: CU2 specifies which traffic it wants to modify or release. CU1 modifies or releases the traffic indicated by CU2. This sub-option involves additional signaling compared to Option 1 with no additional benefit.

We propose Option 1.

Proposal 5: The new Xn procedure is used by the non-F1-terminating CU to request modification (of L2 info only) and full/partial release (e.g., for the purpose of revoking).

- Summary

A TP for the BL CR for TS 38.401 is provided in the appendix based on the above discussion.

Proposal 6: Agree to the TP for the BL CR for TS 38.401 in the appendix.

- Conclusion

This paper addressed remaining Stage-2 for all procedures under this agenda item. A TP for the BL CR for TS 38.401 is included in the appendix. The following observation and proposals have been made:

Observation 1: A dual-connected boundary IAB-node may not report IP addresses received from the non-F1-terminating CU before traffic offload is initiated by the F1-terminating CU.

Proposal 1: The new Xn procedure needs to be run a first time to obtain new IP addresses for the descendant node and a second time to transfer the traffic context with per-traffic new IP addresses included.

Proposal 2: The F1-terminating CU may provide context for traffic offload on behalf of a dual-connected boundary IAB-node without including per-traffic new TNL address info if the latter is unknown.

Proposal 3: The non-F1-terminating CU configures the dual-connected boundary IAB-node with new IP addresses pertaining to the donor-DU selected upon receiving the traffic context from the F1-terminating CU.

Proposal 4: The dual-connected boundary IAB-node selects a new IP address (configured by the non-F1-terminating CU) corresponding to the donor-DU whose BAP address is included in the updated UL mapping of the offloaded traffic.

Proposal 5: The new Xn procedure is used by the non-F1-terminating CU to request modification (of L2 info only) and full/partial release (e.g., for the purpose of revoking).

Proposal 6: Agree to the TP for the BL CR for TS 38.401 in the appendix.

- References

- Chairman report, 3GPP RAN TSG WG3 Meeting #114-bis-e, Electronic Meeting, January 17 – 26, 2022

# Appendix. Text Proposal for IAB BL CR for TS 38.401

-------------------------------------------Change 1-------------------------------------------

## 8.x	IAB Inter-gNB-CU Topological Redundancy

### 8.x.1	IAB Inter-CU topological redundancy procedure

The inter-CU topological redundancy procedure enables the establishment and release of redundant paths in IAB-topologies underneath different IAB-donor-CUs. Since topological redundancy uses NR-DC for the IAB-MT, it is only supported for IAB-nodes operating in SA mode.

Figure 8.x.1-1 shows an example of the inter-CU topological redundancy procedure, where a second path for the F1 connection of the dual-connecting IAB-node with the F1-terminating IAB-donor-CU is established in the topology of the non-F1-terminating IAB-donor-CU. By retaining the F1-connection of the dual-connecting IAB-node’s IAB-DU with the F1-terminating IAB-donor-CU and having a second RRC connection for the dual-connecting IAB-node’s IAB-MT with the non-F1-terminating IAB-donor-CU, this procedure renders the dual-connecting IAB-node as a boundary IAB-node.

Figure 8.x.1-1 IAB inter-donor topology redundancy procedure

1. The NR-DC establishment procedure is performed for the IAB-MT of the dual-connecting IAB node.

2. The non-F1-terminating IAB-donor-CU sends a DL RRC MESSAGE TRANSFER message to the second parent IAB-node’s IAB-DU, which includes an RRCReconfiguration message for the dual-connecting IAB-node’s IAB-MT.  The RRC configuration includes a BAP address for the boundary node in the non-F1-terminating IAB-donor-CU’s topology, default BH RLC channel and a default BAP routing ID configuration for UL F1-C/non-F1 traffic mapping on the second path. The RRC configuration may include new TNL addresses anchored at the second-path IAB-donor-DU for the dual-connecting IAB-node. This step (and steps 3 to 5) may be integrated into step 1 or triggered after step 6.

3. The second parent IAB-node’s IAB-DU forwards the received RRCReconfiguration message to the dual-connecting IAB-MT.

4. The dual-connecting IAB-MT responds to the second parent IAB-node’s IAB-DU with an RRCReconfigurationComplete message.

5. The second parent IAB-node’s IAB-DU sends an UL RRC MESSAGE TRANSFER message to the non-F1-terminating IAB-donor-CU to convey the received RRCReconfigurationComplete message.

6. The F1-terminating IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1-terminating IAB-donor-CU to provide the context of the traffic to be offloaded.

7. The non-F1-terminating IAB-donor-CU may configure or modify BH RLC channels and BAP-sublayer routing entries on the second path between the second parent IAB-node and second-path IAB-donor-DU as well as DL mappings on the second-parent IAB-donor-DU for the dual-connecting IAB-node’s second path. The DL mappings may be based on the TNL addresses allocated to the dual-connecting IAB-node in step 2. These configurations may support the transport of UP and non-UP traffic on the second path.

8. The non-F1-terminating IAB-donor-CU responds with an IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message to the F1-terminating IAB-donor-CU to provide the mapping information for the traffic to be offloaded. The message includes the L2 info necessary to configure the dual-connecting IAB-node with the UL mappings of traffic included in step 6. The message includes the DSCP/IPv6 Flow Label values used to configure the DL mappings of traffic included in step 6.

9. The F1-terminating IAB-donor-CU provides updated UL BH information for the traffic included in step 6 based on the UL BH information received in step 8. The dual-connecting IAB-node switches the traffic to new TNL address(es) received in step 3. The dual-connecting IAB-node reports the TNL address information it wants to use for its F1-U tunnels to the F1-terminating IAB-donor-CU.

10. The F1-terminating IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1-terminating IAB-donor-CU to modify the context of the descendant IAB-node’s offloaded traffic. The message includes the DL TNL address information necessary received in step 9 for the non_F1-terminating IAB-donor-CU to configure or modify DL mappings on the second-path IAB-donor-DU.

11. The non-F1-terminating IAB-donor-CU may configure or modify BH RLC channels and BAP-sublayer routing entries on the second path between the second parent IAB-node and second-path IAB-donor-DU as well as DL mappings on the second-parent IAB-donor-DU for the dual-connecting IAB-node’s second path. The DL mappings may be based on the TNL addresses received in step 10. These configurations may support the transport of UP and non-UP traffic on the second path.

12. The non-F1-terminating IAB-donor-CU responds with an IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message to the F1-terminating IAB-donor-CU.

13. Repetition of steps 6 to 12, as needed, where the F1-terminating IAB-donor-CU can request addition, modification or release of QoS information for non-UP and UP traffic. The non-F1-terminating IAB-donor-CU can fully or partially reject addition or modification requests by the F1-terminating IAB-donor-CU.

Traffic offload for descendent nodes follows the same procedure as that of clause 8.xx.2.

The traffic offload through the inter-CU topological redundancy procedure described in steps 1 to 20 can be revoked. In this case, the SN of the dual-connecting IAB-node does not have to be released. Instead, the second path for traffic offloaded to the non-F1-terminating IAB-donor-CU’s topology is released. The traffic may be returned to the F1-terminating IAB-donor-CU’s topology.

Editor’s NOTE: Capture the outcome of the down-selection (based on agreements) of the following options:

- Whether CU2 can initiate the new procedure to request modification (of L2 info only) and full/partial release (e.g. for the purpose of revoking), OR

- Whether CU2 can initiate a new class-2 procedure to provide serving status of CU2’s topology, which can help CU1’s decision making wrt modification and release (e.g. for the purpose of revoking) of traffic migration.

-------------------------------------------Change 2-------------------------------------------

## 8.xx		IAB Inter-gNB-CU Topology Adaptation

### 8.xx.1 	IAB inter-CU topology adaptation procedure

During the inter-CU topology adaptation for single-connected IAB-node, the IAB-MT switches connection from an old parent node to a new parent node, where the old and the new parent nodes are served by different IAB-donor-CUs. Without loss of generality, the old parent node can be referred to as source parent node, and the new parent node can be referred to as target parent node.

Figure 8.xx.1-1 shows an example of the topology adaptation procedure where the migrating IAB-MT is migrated from one IAB-donor-CU to another IAB-donor-CU. In case the IAB-DU of the migrating IAB-node retains its F1 connection with the first IAB-donor-CU (i.e. the source IAB-donor-CU) after the migrating IAB-MT connects to the second IAB-donor-CU (i.e. the target IAB-donor-CU), this procedure renders the migrating IAB-node as a boundary IAB-node.

Figure 8.xx.1-1: IAB inter-CU topology adaptation procedure

1. The source IAB-donor-CU sends a HANDOVER REQUEST message to the target IAB-donor-CU over the Xn interface. This message may include the migrating IAB-node’s TNL address information in the RRC container.

2. The target IAB-donor-CU sends a UE CONTEXT SETUP REQUEST message to the target parent node IAB-DU to create the UE context for the migrating IAB-MT and set up one or more bearers. These bearers can be used by the migrating IAB-MT for its own signalling, and, optionally, data traffic.

3. The target parent node IAB-DU responds to the target IAB-donor-CU with a UE CONTEXT SETUP RESPONSE message.

4. The target IAB-donor-CU performs admission control and provides the new RRC configuration as part of the HANDOVER REQUEST ACKNOWLEDGE message. The RRC configuration includes a BAP address for the boundary node in the target IAB-donor-CU’s topology, default BH RLC channel and a default BAP routing ID configuration for UL F1-C/non-F1 traffic mapping on the target path. The RRC configuration may include the new TNL addresses anchored at IAB-donor-DU2 for the migrating node.

5. The source IAB-donor-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source parent node IAB-DU, which includes the received RRCReconfiguration message from the target IAB-donor-CU.

6. The source parent node IAB-DU forwards the received RRCReconfiguration message to the migrating IAB-MT.

7. The source parent node IAB-DU responds to the source IAB-donor-CU with the UE CONTEXT MODIFICATION RESPONSE message.

8. A random access procedure is performed at the target parent node IAB-DU.

9. The migrating IAB-MT responds to the target parent node IAB-DU with an RRCReconfigurationComplete message.

10. The target parent node IAB-DU sends an UL RRC MESSAGE TRANSFER message to the target IAB-donor-CU to convey the received RRCReconfigurationComplete message.

11. The target IAB-donor-CU triggers the path switch procedure for the migrating IAB-MT, if needed.

12. The target IAB-donor-CU sends UE CONTEXT RELEASE message to the source IAB-donor-CU.

NOTE: FFS whether the XnAP UE ID of the migrating node is retained at target and source IAB-donor-CU.

13. The source IAB-donor-CU may release BH RLC channels and BAP-sublayer routing entries on the source path between source parent IAB-node of the migrating IAB-node and the source IAB-donor-DU.

14. The target IAB-donor-CU configures BH RLC channels and BAP-sublayer routing entries on the target path between the target parent IAB-node and target IAB-donor-DU as well as DL mappings on the target IAB-donor-DU for the migrating IAB-node’s target path. These configurations support the transport of F1-C traffic on the target path.

15.  The F1-C connection between the migrating IAB-node and the source IAB-donor-CU is switched to the target path using the new TNL address information of the migrating IAB-node. The migrating IAB-node reports the TNL address information it wants to use for its F1-U tunnels to the source IAB-donor-CU.

16.  The source IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the target IAB-donor-CU to provide the context of the traffic to be offloaded. The message includes the DL TNL address information necessary for the target IAB-donor-CU to configure or modify DL mappings on the target IAB-donor-DU.

17. The target IAB-donor-CU may configure or modify BH RLC channels and BAP-sublayer routing entries on the target path between the target parent IAB-node and target IAB-donor-DU as well as DL mappings on the target IAB-donor-DU for the migrating IAB-node’s target path. These configurations may support the transport of UP and non-UP traffic on the target path.

18. The target IAB-donor-CU responds with an IAB TRANSPORT MIGRATION RESPONSE message to the source IAB-donor-CU to provide the mapping information for the traffic to be offloaded. The message includes the L2 info necessary to configure the migrating IAB-node with the UL mappings of traffic included in step 16. The message includes the DSCP/IPv6 Flow Label values used to configure the DL mappings of traffic included in step 16.

19. The F1-U connections of the migrating IAB-node with the source IAB-donor-CU are switched to the migrating IAB-node’s new TNL address(es). The source IAB-donor-CU provides updated UL BH information for the traffic included in step 16 based on the UL BH information received in step 18. The source IAB-donor-CU may also update the UL BH information associated with non-UP traffic. This step may use UE associated signaling or non-UE associated signaling in E1 and/or F1 interface. Implementation must ensure the avoidance of potential race conditions, i.e., no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures.

20. Repetition of steps 16 to 19, as needed, where the source IAB-donor-CU can request addition, modification or release of QoS information for non-UP and UP traffic. The target IAB-donor-CU can fully or partially reject addition or modification requests by the source IAB-donor-CU.

The traffic offload through the inter-CU topology adaptation described in steps 1 to 20 can be revoked. In this case, the migrating IAB-MT is handed over in reverse direction, i.e., from the former target IAB-donor-CU to the former source IAB-donor-CU, and the traffic of the migrating IAB-DU and the descendant IAB-DUs is routed again along the former source path. The former target IAB-donor-CU can trigger this return of offloaded traffic back to the source IAB-donor-CU by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the former source IAB-donor-CU.

### 8.xx.2 	IAB inter-CU topology adaptation procedure with descendant IAB-node

Figure 8.xx.2-1 shows an example of the topology adaptation procedure where the migrating IAB-MT is migrated from one IAB-donor-CU to another IAB-donor-CU, the IAB-DU of the migrating IAB-node retains its F1 connection with the first IAB-donor-CU and the descendant IAB-node retains both its RRC connection and F1 connection with the first IAB-donor-CU.

Figure 8.xx.2-1: IAB inter-CU topology adaptation procedure with descendant IAB-node

0. The topology adaptation procedure of clause 8.xx.1 is performed.

1. The source IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the target IAB-donor-CU to request new TNL addresses anchored at a target IAB-donor-DU for the descendant IAB-node. The source IAB-donor-CU includes an identifier of the migrating IAB-node in the request message.

2. The target IAB-donor-CU determines the target IAB-donor-DU based on the identifier of the migrating IAB-node. The target IAB-donor-CU obtains the new TNL addresses from the target IAB-donor-DU,

3. The target IAB-donor-CU returns the new TNL addresses to the source IAB-donor-CU.

4. The source IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the target IAB-donor-CU to provide the context of the descendant IAB-node’s traffic to be offloaded. The message includes the DL TNL address information necessary for the target IAB-donor-CU to configure or modify DL mappings on the target IAB-donor-DU.

5. The target IAB-donor-CU may configure or modify BH RLC channels and BAP-sublayer routing entries on the target path between the target parent IAB-node and target IAB-donor-DU as well as DL mappings on the target IAB-donor-DU for the migrating IAB-node’s target path. These configurations may support the transport of UP and non-UP traffic on the target path.

6. The target IAB-donor-CU responds with an IAB TRANSPORT MIGRATION RESPONSE message to the source IAB-donor-CU to provide the mapping information for the traffic to be offloaded. The message includes the L2 info necessary to configure the migrating IAB-node with the BAP-sublayer routing, header-rewriting and BH RLC CH mapping entries of traffic included in step 4. The message includes the DSCP/IPv6 Flow Label values used to configure the DL mappings of traffic included in step 4.

7. The source IAB-donor-CU configures the BAP-sublayer routing, header-rewriting and BH RLC CH mapping entries of the migrating IAB-node.

8. The source IAB-donor-CU sends a DL RRC MESSAGE TRANSFER message to the migrating IAB-node’s IAB-DU, which includes an RRCReconfiguration message for the descendant IAB-node’s IAB-MT. The RRC configuration includes the new TNL addresses received in step 3.

9. The migrating IAB-node’s IAB-DU forwards the received RRCReconfiguration message to the descendant IAB-MT.

10. The descendant IAB-MT responds to the migrating IAB-node’s IAB-DU with an RRCReconfigurationComplete message.

11. The migrating IAB-node’s IAB-DU sends an UL RRC MESSAGE TRANSFER message to the source IAB-donor-CU to convey the received RRCReconfigurationComplete message.

12. The descendant IAB-node redirects its traffic with the source IAB-donor-CU to the new TNL addresses.

13. Repetition of steps 4 to 12, as needed, where the source IAB-donor-CU can request addition, modification or release of QoS information for non-UP and UP traffic of the descendant IAB-node. The target IAB-donor-CU can fully or partially reject addition or modification requests by the source IAB-donor-CU.

-------------------------------------------Change 3-------------------------------------------

## 8.x.y	IAB Inter-CU Backhaul RLF recovery for single connected IAB-node

The inter-CU backhaul RLF recovery procedure for IAB-nodes in SA mode enables recovery of an IAB-node to another parent node underneath a different IAB-donor-CU, when the IAB-MT declares a backhaul RLF.

Figure 8.x.y-1 shows an example of the BH RLF recovery procedure for an IAB-node in SA mode. In this example, the IAB-node changes from its initial parent node to a new parent node, where the new parent node is served by an IAB-donor-CU different than the one serving its initial parent node. In case the IAB-DU of the recovery IAB-node retains its F1 connection with the initial IAB-donor-CU after the recovering IAB-MT connects to the new IAB-donor-CU, this procedure renders the recovery IAB-node as a boundary IAB-node

Figure 8.x.y-1: IAB inter-CU backhaul RLF recovery procedure for an IAB-node in SA mode

1. The IAB-MT of the IAB node declares BH RLF.

2. The IAB-MT undergoing recovery performs Random Access towards a new parent IAB-DU.

3. The IAB-MT undergoing recovery sends RRCReestablishmentRequest to the new parent IAB-DU.

4. The new parent IAB-DU sends INITIAL UL RRC MESSAGE to the new IAB-donor-CU to convey the received RRCReestablishmentRequest message.

5. The new IAB-donor-CU retrieves the UE Context for the IAB-MT undergoing recovery, through the Retrieve UE Context procedure in the Xn interface. The initial IAB-donor-CU may include the TNL address information of the IAB-node undergoing recovery in the RRC container of the RETRIEVE UE CONTEXT RESPONSE message.

6. The new IAB-donor-CU sends DL RRC MESSAGE TRANSFER message to the new parent IAB-DU to convey the generated RRCReestablishment message.

7. The new parent IAB-DU sends RRCReestablishment to the IAB-MT undergoing recovery.

8. The IAB-MT undergoing recovery sends RRCReestablishmentComplete to the new parent IAB-DU.

9. The new parent IAB-DU sends UL RRC MESSAGE TRANSFER message to the new IAB-donor-CU to covey the received RRCReestablishmentComplete message.

10. The new IAB-donor-CU triggers the UE Context Setup procedure toward the new parent IAB-DU to create the UE context for the IAB-MT undergoing recovery and set up one or more bearers. These bearers can be used by the IAB-MT undergoing recovery for its own signalling, and, optionally, data traffic.

11. The new IAB-donor-CU triggers the path switch procedure for the IAB-MT undergoing recovery, if needed.

12. The new IAB-donor-CU sends UE CONTEXT RELEASE message to the initial IAB-donor-CU.

NOTE: FFS whether the XnAP UE ID of the recovery IAB-node is retained at initial and new IAB-donor-CU.

13. The initial IAB-donor-CU may release the BH RLC channels and BAP-sublayer routing entries on the initial path between initial parent IAB-node and initial IAB-donor-DU.

14. The new IAB-donor-CU sends a DL RRC MESSAGE TRANSFER message to the new parent IAB-DU, which includes an RRCReconfiguration message for the IAB-MT undergoing recovery. The RRC configuration includes the new TNL addresses received in step 5.

15. The new parent IAB-DU forwards the received RRCReconfiguration message to the IAB-MT undergoing recovery.

16. The IAB-MT undergoing recovery responds to the new parent IAB-DU with an RRCReconfigurationComplete message.

17. The new parent IAB-DU sends an UL RRC MESSAGE TRANSFER message to the new IAB-donor-CU to convey the received RRCReconfigurationComplete message.

18. The remaining part of the procedure follows the steps 14-20 of the inter-CU topology adaptation procedure defined in clause 8.xx.1.

Traffic offload for descendent nodes follows the same procedure as that of clause 8.xx.2.

The traffic offload due to inter-CU RLF recovery described in steps 1 to 14 can be revoked. In this case, the IAB-MT that previously underwent RLF recovery is handed over in reverse direction, i.e., from the former new IAB-donor-CU to the former initial IAB-donor-CU, and the traffic of the IAB-DU whose collocated IAB-MT underwent RLF recovery, and the traffic of descendant IAB-DUs, is routed again along the path under the former old IAB-donor-CU. The former new IAB-donor-CU can trigger this return of offloaded traffic back to the source IAB-donor-CU by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the former initial IAB-donor-CU.

-------------------------------------------End of changes-------------------------------------------