3GPP TSG-RAN WG3 Meeting #115-e	R3-221857

Online, 21 February – 03 March 2022

Title:	(TP for SL Relay BL CR to TS 38.401) Updates on overall procedures

Source:	Ericsson

Agenda Item:	23.3

Document for:	Discussion

# 1	Introduction

This contribution provides the TP to TS 38.401 for support of Sidelink Relay.

- TP to TS 38.401:

## 8.x Overall procedures for L2 UE-to-Network Relay

### 8.x.1	Remote UE initial access

The signalling flow for remote UE Initial access is shown in Figure 8.x.1-1.

Figure 8.x.1-1: Remote UE Initial Access procedure

- The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

- The U2N Remote UE sends an RRCSetupRequest message to the U2N Relay UE via PC5 RLC Channel. The gNB configures SRB0 relaying Uu RLC channel to the U2N Relay UE.

- The U2N Relay UE withholds the received RRC message and sends the SidelinkUEInformation message to the gNB-DU. Before that, if the U2N Relay UE is in RRC_IDLE/RRC_INACTIVE status, it should trigger the RRC establishment/resume procedure to enter RRC_CONNECTED status.

- The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the SidelinkUEInformation message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

- The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the U2N Relay UE to gNB-DU. Such message may request the establishment of Uu RLC channel for SRB0.

- The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the U2N Relay UE to gNB-CU

Editor’s Notes: FFS on whether including local ID of U2N Remote UE in the UE CONTEXT MODIFICATION REQUEST message

- The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the U2N Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE.

- The gNB-DU sends the RRCReconfiguration message to the U2N Relay UE to configure the local ID of the U2N Remote UE.

- The U2N Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

- The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

- After receiving the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU. The local ID of the U2N Remote UE is conveyed in the SRAP header

- The gNB-DU sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE is included in the INITIAL UL RRC MESSAGE TRANSFER message .

- The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 RLC channel at least for the transmission of U2N Remote UE’s SRB.

Editor’s Notes: FFS on the inclusion of the configurations of PC5 RLC channel at least for the transmission of U2N Remote UE’s SRB1 in Step 12&13.

- The gNB-DU sends the RRCSetup message to the U2N Remote UE via the U2N Relay UE.

- The gNB-CU and the U2N Relay UE perform relaying channel setup procedure. According to the configuration from gNB-CU, the U2N Relay establishes a PC5 RLC channel for relaying of SRB1 over PC5 and establish Uu RLC channel for relaying of SRB1 towards gNB-DU. This step may be performed earlier, e.g., via steps 5~8. The gNB-CU also configures PC5 RLC Channel for the U2N Remote UE.

Note 1: Step 15 may be performed earlier.

- The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the U2N Relay UE.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

- The gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

- The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

- The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU.

- The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via U2N Relay UE.

- The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the PC5 link configurations.

- The U2N Remote UE responds with the SecurityModeComplete message.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

- The gNB-CU generates the RRCReconfiguration message and encapsulates it in the DL RRC MESSAGE TRANSFER message.

- The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the U2N Relay UE.

- The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the U2N Relay UE.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

- The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

- The gNB-CU establishes additional Uu RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5 Uu RLC channels for the U2N Relay/Remote UE for the relay traffic of DRBs and SRBs belonging to the U2N Remote UE. Also, such step may configure the mapping between DRB/SRB and PC5/Uu RLC channel at the U2N Relay UE. This step may be performed earlier, e.g., right after step 11.

Note 2: Step 30 may be performed earlier.

### 8.x.2	Remote UE RRC Reestablishment

The signalling flow for remote UE RRC Reestablishment is shown in Figure 8.x.2-1.

Figure 8.x.2-1: Remote UE RRC Reestablishment procedure

- The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure. This step may be omitted if PC5 connection was established.

- The U2N Remote UE sends an RRCSetupRequest message to the U2N Relay UE via PC5 RLC Channel.

3~10. The gNB-CU allocates the local ID of the U2N Remote UE if the U2N Relay UE does not have it. The details of those steps can be referred to Section 8.x.1.

- After receiving the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCReestablishmentRequest message of the U2N Remote UE to gNB-DU.

- The gNB-DU sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCReestablishmentRequest message of the U2N Remote UE.

- The gNB-CU and U2N Relay UE perform relaying channel setup procedure. According to the configuration from gNB-CU, the U2N Relay UE establishes a PC5 RLC channel for relaying of SRB1 over PC5 and establishes Uu RLC channel for relaying of SRB1 towards gNB-DU. This step may be performed earlier, e.g., via steps 5~8.

14~23. The details of those steps can be referred to Steps 5~14 in Section 8.7. For L2 U2N relay, the RRC message(s) between the U2N Remote UE and the gNB-DU are relayed via the U2N Relay UE; Step18~19 may additionally perform the configurations of PC5 RLC channel(s)

- The gNB-CU establishes additional Uu RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5 Uu RLC channels for the U2N Relay/Remote UE for the relay traffic of DRBs and SRBs belonging to the U2N Remote UE. Also, such step may configure the mapping between DRB/SRB and PC5/Uu RLC channel at the U2N Relay UE. This step may be performed earlier, e.g., right after step 11.

Note 1: Step 24 may be performed earlier.

### 8.x.3	Remote UE RRC Inactive to other states

The signalling flow for remote UE from RRC Inactive to other states is shown in Figure 8.x.3-1.

Figure 8.x.3-1: Remote UE RRC Resume procedure

- The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR Prose procedure. This step may be omitted if PC5 connection was established.

- The U2N Remote UE sends an RRCResumeRequest message to the U2N Relay UE via PC5 RLC Channel.

3~10. The gNB-CU allocates the local ID of the U2N Remote UE if the U2N Relay UE does not have it. The details of those steps can be referred to Section 8.x.1.

- After receiving the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCResumeRequest message of the U2N Remote UE to gNB-DU.

- The gNB-DU sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCResumeRequest message of the U2N Remote UE.

- The gNB-CU and U2N Relay UE perform relaying channel setup procedure. According to the configuration from gNB-CU, the U2N Relay UE establishes a PC5 RLC channel for relaying of SRB1 over PC5 and establishes Uu RLC channel for relaying of SRB1 towards gNB-DU. This step may be performed earlier, e.g., via steps 5~8.

14~19. The details of those steps can be referred to Steps 5~10 in Section 8.6.2. For L2 U2N relay, the RRC message(s) between the U2N Remote UE and the gNB-DU are relayed via the U2N Relay UE; Step14~15 may additionally perform the configurations of PC5 RLC channel(s)

- The gNB-CU establishes additional Uu RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5 Uu RLC channels for the U2N Relay/Remote UE for the relay traffic of DRBs and SRBs belonging to the U2N Remote UE. Also, such step may configure the mapping between DRB/SRB and PC5/Uu RLC channel at the U2N Relay UE. This step may be performed earilier, e.g., right after step 11.

Note 1: Step 30 may be performed earlier.