3GPP TSG-RAN WG3 Meeting #115-e	R3-221870

E-meeting, 21 February – 3 March, 2022

Agenda item:	9.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Avoiding delayed HO in deployments supporting dynamic ACL for data forwarding

Document for:	Discussion and Decision

# 1	Introduction

This paper provides further discussion on dynamic ACL support, in particular with focus on avoiding delayed HO in deployments supporting dynamic ACL.

# 2	Discussion

## 2.1	Delayed HO problem

Figure 1 illustrates interfaces and some termination points involved in the basic scenario of inter-gNB handover with NR-DC in the source node. For data forwarding using dynamic ACL, the source gNB shall, if the discussed Rel-16 correction is supported, provide UP source IP addresses for MN terminated bearers (IP@ C) and for SN terminated bearers (IP@ D). We also indicate on Fig. 1 the source IP address used for data forwarding from the MN to the SN during the SN addition (IP@ A), as well as the IP address used by the gNB-CU-UP to send data to the gNB-DU (IP@ B).

Fig. 1: Interfaces and some termination points involved in inter-gNB handover with NR-DC in the source node. IP@ A is the source IP address used for data forwarding from the MN to the SN during the SN addition.

IP@ B is the source IP address used by the gNB-CU-UP to send data to the gNB-DU.

IP@ C is the source IP address used by the source NG-RAN node for data forwarding towards the target node for MN-terminated bearers.

IP@ D is the source IP address used by the source NG-RAN node for data forwarding towards the target node for SN-terminated bearers.

As per XnAP CR submitted to RAN3#114bis-e in [1], the proposed signalling enables the MN to request the SN to inform about IP@ B using the M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure (Source DL Forwarding IP Address Query IE). This query will need to happen before the HO Request is sent, and HO triggering is therefore delayed. It was further commented by proponents during online discussion that a similar query functionality was intended on E1 (while not included in the proposed E1AP CR in [2]) in support of the Xn query in case of CP/UP split. This cascaded Xn/E1 query will of course add even further delay of the HO triggering.

Fig. 2: Signalling flow for establishment of NR-DC followed by outgoing handover.

It can be seen in figure 2 that as per signalling proposed at last meeting, the S-gNB-CU-CP will be informed about IP@ B, which is used for UP signalling connection towards the gNB-DU. However, RAN3 didn't so far discuss e.g. whether in case of a subsequent handover, the same IP address will be used for direct data forwarding towards a target gNB, or whether the standard needs to support the use of a different IP address (IP@ D).

We see three possible options relative to source IP address handling over E1, avoiding the described HO delay problem:

- Option 1: The S-gNB-CU-CP considers the provided IP@ B is also used as IP@ D, i.e. the source IP address the CU-UP has allocated for UP signalling towards the DU is also used as source IP address for data forwarding to target gNB in case of subsequent outgoing handover.

- Option 2: Separate signalling of source IP@ B and source IP@ D during the Bearer setup procedure.

- Option 3: The source IP@ contained in the BEARER CONTEXT SETUP RESPONSE message corresponds to the IP@ D, and the gNB-CU-CP is therefore not informed about IP@ B.

For option 3 we would like to observe that current RAN3 agreements correspond to data forwarding only, and therefore transfer of source IP address for F1-U is not covered by these agreements (data forwarding is defined at PDCP level, hence between CU-UPs and not between CU-UP and DU). So option 3 is the option that best follows RAN3's agreement and seems to be a reasonable solution.

Proposal 1: RAN3 to discuss available options for E1 and agree on option 3.

Based on any of the solutions in option 1, 2 or 3, the HO delay problem can then be solved by adding the Source DL Forwarding IP Address to the SN ADDITION REQUEST ACKNOWLEDGE message.

Proposal 2: Add the Source DL Forwarding IP Address to the SN ADDITION REQUEST ACKNOWLEDGE message.

A corresponding text proposal is provided in annex of this paper.

As a consequence of the solution based on the discussion above, we also believe that there is no need for the source IP address query functionality on XnAP that was proposed in [1].

Proposal 3: Source IP address query functionality is not needed on XnAP.

In principle, option 3 also implies that the F1AP CR [3] and W1AP CR [4] agreed at last meeting are not needed. However, as we commented at RAN3#114bis-e, RAN3 has not yet  taken any specific decision on, beyond data forwarding, whether to provide dynamic ACL support for MN-terminated SCG bearer and SN-terminated MCG bearer. The MN-terminated SCG bearer scenario, with Xn-U interface set up between the CU-UP in the MN and the DU in the SN, is included in Fig. 1, using source IP@ B2. Because this IP address will belong to an external node (e.g. the MN as in Fig. 1), we suggest that the F1AP CR [3] and W1AP CR [4] are kept for the purpose of transfer of IP@ B2.

Proposal 4: Transfer of source IP address over F1-C and W1-C serves the purpose of dynamic ACL for MN-terminated SCG bearers and SN-terminated MCG bearers.

# 3	Conclusion

Proposal 1: RAN3 to discuss available options for E1 and agree on option 3.

Proposal 2: Add the Source DL Forwarding IP Address to the SN ADDITION REQUEST ACKNOWLEDGE message. (See TP in annex of this paper)

Proposal 3: Source IP address query functionality is not needed on XnAP.

Proposal 4: Transfer of source IP address over F1-C and W1-C serves the purpose of dynamic ACL for MN-terminated SCG bearers and SN-terminated MCG bearers.

# References

[1]		R3-220304, Dynamic ACL over Xn CR 38.423

[2]		R3-220301, Dynamic ACL over E1 CR 38.463

[3]		R3-220178, Support of dynamic ACL during handover and dual connectivity, Rel-16 CR to TS 38.473

[4]		R3-221444, Support of dynamic ACL during handover and dual connectivity, Rel-16 CR to TS 37.473

# Annex	- TP for TS 38.423 (proposal 2)

The TP is based on [1]

<<< start of changes >>>

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

<<< skip unchanged text >>>

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted List IE in the PDU Session Resource Setup Response Info – MN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

If for a given QoS Flow the Source DL Forwarding IP Address IE is included within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resource Setup Info – SN terminated IE contained in the S-NODE ADDITION REQUEST message, the target NG-RAN node shall, if supported, store this information and use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given QoS Flow the Source DL Forwarding IP Address IE is included within the PDU Session Resource Setup Response Info – SN terminated IE contained in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, store this information and use it for subsequent handover preparation, if ACL functionality is deployed.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall.

<<< next change >>>

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

<<< skip unchanged text >>>

For each DRB configured as SN-terminated split bearer/MCG bearer, if the QoS Mapping Information IE is included in the DRBs To Be Modified List IE in the PDU Session Resource Modification Info – SN terminated IE of the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from S-NG-RAN node to M-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits a modification of the UE context requiring the M-NG-RAN node to report about the success of the RRC connection reconfiguration procedure, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the M-NG-RAN node. The reception of the S-NG-RAN node RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall.

If for a given QoS Flow the Source DL Forwarding IP Address IE is included within the PDU Session Resource Setup Response Info – SN terminated IE contained in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, store this information and use it for subsequent handover preparation, if ACL functionality is deployed.

<<< next change >>>

#### 9.2.1.6	PDU Session Resource Setup Response Info – SN terminated

This IE contains the result of the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information at NG-RAN | M |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – |  |
| DRBs To Be Setup List |  | 0..1 |  |  | – |  |
| >DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>SN UL PDCP UP TNL Information | M |  | UP Transport Parameters 9.2.3. 76 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>PDCP SN Length | O |  | 9.2.3.63 | Indicates the PDCP SN length of the DRB. | – |  |
| >>RLC Mode | M |  | 9.2.3.28 | Indicates the RLC mode to be used in the assisting node. | – |  |
| >>secondary SN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | – |  |
| >>Duplication Activation | O |  | 9.2.3.71 | Information on the initial state of UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | – |  |
| >>UL Configuration | O |  | 9.2.3.75 | Information about UL usage in the M-NG-RAN node. This IE is used when the concerned DRB has both MCG resource and SCG resource configured i.e. the concerned DRB is configured as split bearer. | – |  |
| >>QoS Flows Mapped To DRB List |  | 1 |  |  | – |  |
| >>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>MCG requested GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.3.6 | This IE contains GBR QoS Flow Information necessary for the MCG part. | – |  |
| >>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>>>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.2.3.103 |  | YES | ignore |
| >>>>Source DL Forwarding IP Address | O |  | Transport Layer Address 9.2.3.29 |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | Ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |
| >>RLC Duplication Information | O |  | 9.2.3.111 | . | – |  |
| Data Forwarding Info from target NG-RAN node | O |  | 9.2.1.16 |  | – |  |
| QoS Flows Not Admitted List | O |  | QoS Flow List with Cause 9.2.1.4 |  | – |  |
| Security Result | O |  | 9.2.3.67 |  | – |  |
| DRB IDs taken into use | O |  | DRB List 9.2.1.29 | Indicating the DRB IDs taken into use by the target NG-RAN node, as specified in TS 37.340 [8]. | YES | reject |
| Redundant DL NG-U UP TNL Information at NG-RAN | O |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs for the redundant transmission. | YES | ignore |
| Used RSN Information | O |  | Redundant PDU Session Information 9.2.3.112 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |
| maxnoofQoSFlows | Maximum no. of QoS flows. Value is 64 |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional PDCP Duplication TNL. Value is 2. |

<<< end of changes >>>