3GPP TSG-RAN WG3 Meeting #115-e	R3-221874

Online, February 21st – March 3rd, 2022

Agenda item:	21.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR_IIOT_URLLC_enh BL CR for TS 38.473) Time synchronization: resolution of F1AP open issues

Document for:	Discussion and Decision

# 1	Introduction

For time synchronisation over F1AP, the following open issues were listed for further discussion in RAN3:

Further discussion needed on the following (open issues): - usefulness of Uu Time Synchronisation Budget (cell level?) at the DU. - whether/how the Uu Time Synchronisation Error Budget affects RTI delivery (e.g. periodicity). Further discussion of gNB-based PDC is postponed to next meeting to allow further RAN2 progress. The following may need further discussion depending on RAN2 discussion outcome: - For TA-based PDC at the gNB, it is FFS whether DU reports compensated RTI or the TA-value. - For RTT-based PDC, DU may need to report gNB Rx-Tx time different to the CU (FFS)

In this paper, we analyse the remaining open issues for time synchronization over F1AP. A text proposal for F1AP is also provided.

# 2	Discussion

## 2.1	Uu Time Synchronisation Error Budget over F1AP

Time Synchronization Assistance Information (i.e., Uu Time Synchronisation Error Budget and Time Distribution Indication) can be received at the gNB-CU from another gNB (due to handover) or from the core network. However, it is still open if the gNB-DU should also receive the Uu time synchronization error budget (cell level?) or if the budget will influence RTI delivery periodicity.

Periodic delivery of RTI to the UE is needed to ensure clock alignment at the UE with the 5G clock, in the presence of UE clock drift and mobility as described in section 4.1 of [1]. The accuracy of the device clock depends on implementation and other factors (e.g., environmental). Therefore, the Uu Synchronisation Error Budget is one factor that affects the needed interval/periodicity of RTI delivery, i.e., smaller budget requires more frequent RTI updates than a larger budget.

At a minimum, the Uu time synchronization error budget influences the PDC configuration the UE will have with the serving gNB to improve time synchronization accuracy even if the gNB and the UE are far away from each other. To do so, as already agreed in RAN3, the gNB-CU is responsible for controlling whether/how to perform PDC. However, PDC cannot be used for broadcast-based RTI delivery. To support broadcast-based RTI delivery, the gNB-DU must determine the SIB9 periodicity in either of two ways:

1)	The gNB-DU decides the needed SIB9 periodicity based on information that includes the cell-level Uu time synchronisation error budget received from the gNB-CU over F1AP; or

2)	The gNB-DU is configured via OAM with the SIB9 periodicity.

If OAM is assumed as baseline, the consequence is that if the configured SIB9 periodicity cannot achieve the required timing accuracy, the gNB-CU must decide to use unicast-based RTI with greater frequency and/or with PDC. This baseline may be sufficient for Rel-17, considering that we must conclude the work item at this meeting.

Proposal 1:	There is no need to signal the Uu Time Synchronisation Error Budget over F1AP in Rel-17.

## 2.2	Propagation Delay Compensation over F1AP

RAN3 has received an LS from RAN2 in [2], providing a long list of agreements on PDC for time synchronization. Of particular relevance to RAN3 are the following agreements from RAN2#116bis-e:

1.	Both RTT-based PDC and legacy TA-based PDC are supported.

2.	Both RTT-based UE side PDC and RTT-based gNB side PDC are supported. RTT-based gNB side PDC has to be a simple solution and converge by February meeting.

4.	For RTT-based UE side PDC, gNB Rx-Tx time difference, e.g., gNB Rx-Tx, shall be provided to UE via DLInformationTransfer signaling.

6.	For RTT-based gNB side PDC, RRC measurement framework can be reused as baseline to provide UE Rx-Tx time difference report.

7.	For RTT-based gNB side PDC, besides UE Rx-Tx time difference, no additional information needs to be reported to NW.

In addition, the following agreement from RAN2#115-e is related to TA-based PDC:

1.	RAN2 assumes that gNB can perform pre-compensation.  RAN2 agrees to introduce signalling to enable/disable UE-side PDC.

Based on the above RAN2 agreements, there are RAN3 impacts to support TA-based gNB side PDC, RTT-based gNB side PDC, and RTT-based UE side PDC. The impacts are as follows:

-	For RTT-based PDC, the gNB-CU must obtain gNB Rx-Tx time difference measurements from the gNB-DU.

-	For TA-based PDC, the gNB-CU must obtain either TA measurement or Compensated RTI from the gNB-DU.

In case of gNB-side PDC, it is up to the gNB to perform the compensation for each UE and provide that dedicated time information to the specific UE it applies. The PDC measurement estimation or the compensated RTI is UE specific, so UE-associated procedure is preferred to control PDC via F1AP.

For RTT-based PDC, RAN2 agreed gNB Rx-Tx time difference is reported to the UE via DLInformationTransfer signalling and RRC measurement framework can be reused as baseline to provide UE Rx-Tx time difference report to the gNB.

The gNB-CU is responsible for configuring the RRC measurements to be performed by the UE. That is, the IE MeasConfig is part of the RRC information that is sent from the gNB-CU to the gNB-DU via F1AP. Therefore, the role of the gNB-DU for RTT-based PDC is to determine gNB Rx-Tx time difference measurement and provide it to the gNB-CU. To configure this reporting, UE-associated F1AP signalling is preferred.

Proposal 2:	For RTT-based PDC, UE-associated signalling is needed over F1AP to provide gNB Rx-Tx time difference measurement from gNB-DU to gNB-CU.

For TA-based PDC, it remains open if the gNB-DU reports TA measurement or compensated RTI to gNB-CU via F1AP. To evaluate both alternatives, two metrics can be considered, achievable accuracy considering granularity and the signalling load in F1AP, as summarized in Table 1 below.

| Metric | - TA measurement | - Compensated RTI |
| --- | --- | --- |
| Accuracy | Depends on the granularity of the new IE in F1AP that defines the NR timing advance. | 10ns (granularity of RTI IE structure). |
| F1AP signalling | 1) gNB-DU reports gNB-CU the uncompensated RTI periodically (non UE-associated signalling). This signalling is the conventional RTI reporting control the gNB-CU needs. 2) gNB-DU reports TA estimate to gNB-CU per UE (UE-associated signalling). The reporting from gNB-DU can be configured to be done when the TA estimate changes beyond X threshold, event-triggered from gNB-CU, or periodically. | 1) gNB-DU reports gNB-CU the uncompensated RTI periodically (non UE-associated signalling). This signalling is the conventional RTI reporting control the gNB-CU needs. 2) gNB-DU reports compensated RTI to gNB-CU per UE (UE-associated signalling). The reporting from gNB-DU needs to be done following the required RTI periodicity per UE. |

Table 1: TA-based PDC reporting attributes in F1AP comparison

Comparing the signalling needed in case a) and b), there is not a significant difference. However, for RTT-based PDC at gNB, the gNB-CU must perform the compensation and request measurements from the UE and gNB.  Therefore, it is beneficial to align TA-based PDC at gNB with this, i.e. compensation is a function of the gNB-CU, and the gNB-CU uses a common procedure to get TA or gNB Rx-Tx measurements from the gNB-DU.

Proposal 3:	For TA-based PDC, UE-associated signalling is needed over F1AP to provide TA measurement from gNB-DU to gNB-CU. gNB-CU is responsible for compensating RTI with the received PD estimation.

From our perspective, to have a signalling solution for all PDC methods, we propose the following key principles for F1AP (Figure 1 illustrates an example of signalling flow for gNB-side PDC):

-	The gNB-CU controls gNB-DU to report PDC measurement values (either TA value or gNB Rx-Tx time difference measurement). To make the signalling general for all methods, the new IE with the result can be named “PDC Measurement Result”.

-	The gNB-CU is responsible for compensating RTI using the last reported PDC measurement results from the gNB-DU for a UE.

-	New F1AP procedures for PDC using UE-associated signalling are introduced, modelled after existing procedures for E-CID measurements.

-	The DLInformationTransfer procedure is used to send gNB Rx-Tx measurement (for UE-side RTT-based PDC) or compensated RTI from gNB-CU to the UE (no RAN3 impact).

-	The new F1AP procedures for PDC are independent of positioning procedures already defined in F1AP. Nevertheless, if Rx-Tx time difference is already available at the gNB-CU due to positioning-related procedures, and the configuration of the positioning measurement is compatible with PDC procedure, the gNB-CU can opportunistically make use of the measurement for PDC also. It is up to the gNB-CU to pick up the right measurement to use.

Figure 1: Example of proposed new UE-associated F1AP procedure for gNB-side PDC case.

Proposal 4:	Introduce a common set of UE-associated F1AP procedures to support both TA-based and RTT-based PDC: PDC Measurement Initiation (class 1) procedure and PDC Measurement Report (class 2) procedure.

# 3	Conclusions

In this paper, we analysed the remaining open issues for time synchronization assistance information over F1AP. The following is proposed:

Proposal 1:	There is no need to signal the Uu Time Synchronisation Error Budget over F1AP in Rel-17.

Proposal 2:	For RTT-based PDC, UE-associated signalling is needed over F1AP to provide gNB Rx-Tx time difference measurement from gNB-DU to gNB-CU.

Proposal 3:	For TA-based PDC, UE-associated signalling is needed over F1AP to provide TA measurement from gNB-DU to gNB-CU. gNB-CU is responsible for compensating RTI with the received PD estimation.

Proposal 4:	Introduce a common set of UE-associated F1AP procedures to support both TA-based and RTT-based PDC: PDC Measurement Initiation (class 1) procedure and PDC Measurement Report (class 2) procedure.

A TP for F1AP is provided in Annex A reflecting the necessary changes for the above proposals.

# References

- R3-221112 Summary of offline: Propagation delay compensation enhancements, Nokia (moderator)

- R3-221666 LS on PDC for Time Synchronization, ZTE

# Annex A: Text Proposal for TS 38.473

Start of Text Proposal for TS 38.473

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

CAG	Closed Access Group

CN	Core Network

CG	Cell Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

NID	Network Identifier

NPN	Non-Public Network

NSSAI	Network Slice Selection Assistance Information

PDC	Propagation Delay Compensation

posSIB	Positioning SIB

PNI-NPN	Public Network Integrated NPN

RANAC	RAN Area Code

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RRC	Radio Resource Control

RSRP	Reference Signal Received Power

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

TRP	Transmission-Reception Point

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

Z-AoA	Zenith Angles of Arrival

Next Change

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup |  | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update |  | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update |  | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup |  | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) |  | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) |  | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) |  | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning |  | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel |  | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination |  | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal |  | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  | BAP MAPPING CONFIGURATION FAILURE |  |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  | GNB-DU RESOURCE CONFIGURATION FAILURE |  |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  | IAB TNL ADDRESS FAILURE |  |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |  |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |  |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |  |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |  |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |  |
| PDC Measurement Initiation |  | PDC MEASUREMENT INITIATION REQUEST |  | PDC MEASUREMENT INITIATION RESPONSE |  | PDC MEASUREMENT INITIATION FAILURE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| PDC Measurement Report |  | PDC MEASUREMENT REPORT |  |

Next Change

### 8.12.x	PDC Measurement Initiation

#### 8.12.x.1	General

The purpose of the PDC Measurement Initiation procedure is to enable the gNB-CU to request the gNB-DU to report measurements used for propagadation delay compensation at the gNB-CU or UE. The procedure uses UE-associated signalling.

#### 8.12.x.2	Successful Operation

Figure 8.12.x.2-1: PDC Measurement Initiation procedure, successful operation

The gNB-CU initiates the procedure by sending a PDC MEASUREMENT INITIATION REQUEST message. If the gNB-DU is able to initiate the requested PDC measurements, it shall reply with the PDC MEASUREMENT INITIATION RESPONSE message.

If the PDC Report Type IE is set to "OnDemand", the gNB-DU shall return the result of the measurement in the PDC MEASUREMENT INITIATION RESPONSE message including the PDC Measurement Result IE, and the gNB-CU shall consider that the PDC measurements for the UE have been terminated by the gNB-DU.

Interaction with the PDC Measurement Report procedure:

If the PDC Report Type IE is set to "Periodic", the gNB-DU shall initiate the requested measurements and shall reply with the PDC MEASUREMENT INITIATION RESPONSE message without including the PDC Measurement Result IE in this message. The gNB-DU shall then periodically initiate the PDC Measurement Report procedure for the measurements, with the requested reporting periodicity.

#### 8.12.x.3	Unsuccessful Operation

Figure 8.12.x.3-1: PDC Measurement Initiation procedure, unsuccessful operation

If the gNB-DU is not able to initiate at least one of the requested PDC measurements, the gNB-DU shall respond with a PDC MEASUREMENT INITIATION FAILURE message.

### 8.12.y	PDC Measurement Report

#### 8.12.y.1	General

The purpose of the PDC Measurement Report procedure is for the gNB-DU to provide the PDC measurements for the UE to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.12.y.2	Successful Operation

Figure 8.12.y.2-1: PDC Measurement Report procedure, successful operation

The gNB-DU initiates the procedure by sending a PDC MEASUREMENT REPORT message. The PDC MEASUREMENT REPORT message contains the PDC measurement results according to the measurement configuration in the respective PDC MEASUREMENT INITIATION REQUEST message.

#### 8.12.y.3	Unsuccessful Operation

Not applicable.

Next Change

#### 9.2.11.x1	PDC MEASUREMENT INITIATION REQUEST

This message is sent by gNB-CU to initiate PDC measurements.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PDC Report Type | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| PDC Measurement Periodicity | C-ifReportTypePeriodic |  | INTEGER (0..512, …) | Unit in radio frame. | YES | reject |
| PDC Measurement Quantities |  | 1 .. <maxnoMeasPDC> |  |  | EACH | reject |
| >PDC Measurement Quantities Item | M |  | ENUMERATED (NR TADV, gNB RX-TX, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeasPDC | Maximum no. of PDC measured quantities that can be configured and reported with one message. Value is 16. |

| Condition | Explanation |
| --- | --- |
| ifReportTypePeriodic | This IE shall be present if the PDC Report Type IE is set to the value "Periodic". |

#### 9.2.11.x2	PDC MEASUREMENT INITIATION RESPONSE

This message is sent by gNB-DU to indicate that the requested PDC measurement is successfully initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PDC Measurement Result | O |  | 9.3.1.y1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.11.x3	PDC MEASUREMENT INITIATION FAILURE

This message is sent by gNB-DU to indicate that the requested PDC measurement cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.11.x4	PDC MEASUREMENT REPORT

This message is sent by gNB-DU to report the results of the requested PDC measurement.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PDC Measurement Result | M |  | 9.3.1.y1 |  | YES | ignore |

Next Change

#### 9.3.1.y1	PDC Measurement Result

The purpose of this IE is to provide the PDC measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDC Measured Results List |  | 1 |  |  |
| >PDC Measured Results Item |  | 1 .. <maxnoMeasPDC> |  |  |
| >>CHOICE Measured Results Value | M |  |  |  |
| >>>NR Timing Advance | M |  | FFS |  |
| >>>gNB Rx-Tx Time Difference | M |  | 9.3.1.170 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeasPDC | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16. |

Next Change

### 9.4.3	Elementary Procedure Definitions

** unchanged text skipped **

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

PDCMeasurementInitiationRequest,

PDCMeasurementInitiationResponse,

PDCMeasurementInitiationFailure,

PDCMeasurementReport

** unchanged text skipped **

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-PDCMeasurementInitiationRequest,

id-PDCMeasurementInitiationResponse,

id-PDCMeasurementInitiationFailure,

id-PDCMeasurementReport

** unchanged text skipped **

F1AP-ELEMENTARY-PROCEDURES-CLASS-1 F1AP-ELEMENTARY-PROCEDURE ::= {

reset							|

f1Setup							|

gNBDUConfigurationUpdate		|

gNBCUConfigurationUpdate		|

uEContextSetup					|

uEContextRelease				|

uEContextModification			|

uEContextModificationRequired	|

writeReplaceWarning				|

pWSCancel					|

gNBDUResourceCoordination		|

f1Removal						|

bAPMappingConfiguration			|

gNBDUResourceConfiguration		|

iABTNLAddressAllocation			|

iABUPConfigurationUpdate		|

resourceStatusReportingInitiation	|

positioningMeasurementExchange	|

tRPInformationExchange			|

positioningInformationExchange	|

positioningActivation			|

e-CIDMeasurementInitiation		|

pDCMeasurementInitiation,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

pDCMeasurementReport,

...

}

** unchanged text skipped **

positioningInformationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationUpdate

PROCEDURE CODE			id-PositioningInformationUpdate

CRITICALITY				ignore

}

pDCMeasurementInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementInitiationRequest

SUCCESSFUL OUTCOME		PDCMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	PDCMeasurementInitiationFailure

PROCEDURE CODE			id-PDCMeasurementInitiation

CRITICALITY				reject

}

pDCMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementReport

PROCEDURE CODE			id-PDCMeasurementReport

CRITICALITY				ignore

}

Next Change

### 9.4.4	PDU Definitions

** unchanged text skipped **

SpatialRelationPerSRSResource,

PDCMeasurementPeriodicity,

PDCMeasurementQuantities

PDCMeasurementResult,

PDCReportType

** unchanged text skipped **

id-SRSSpatialRelationPerSRSResource,

id-PDCMeasurementPeriodicity,

id-PDCMeasurementQuantities,

id-PDCMeasurementResult,

id-PDCReportType,

maxCellingNBDU,

** unchanged text skipped **

E-CIDMeasurementTerminationCommand ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementTerminationCommand-IEs}},

...

}

E-CIDMeasurementTerminationCommand-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- PDC MEASUREMENT PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- PDC Measurement Initiation Request

--

-- **************************************************************

PDCMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PDCMeasurementInitiationRequest-IEs}},

...

}

PDCMeasurementInitiationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-PDCReportType					CRITICALITY reject	TYPE PDCReportType						PRESENCE mandatory	}|

{ ID id-PDCMeasurementPeriodicity		CRITICALITY reject	TYPE PDCMeasurementPeriodicity			PRESENCE conditional	}|

-- The above IE shall be present if the PDCReportType IE is set to “periodic” –-

{ ID id-PDCMeasurementQuantities		CRITICALITY reject	TYPE PDCMeasurementQuantities			PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- PDC Measurement Initiation Response

--

-- **************************************************************

PDCMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PDCMeasurementInitiationResponse-IEs}},

...

}

PDCMeasurementInitiationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-PDCMeasurementResult		CRITICALITY ignore	TYPE PDCMeasurementResult		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

-- PDC Measurement Initiation Failure

--

-- **************************************************************

PDCMeasurementInitiationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{PDCMeasurementInitiationFailure-IEs}},

...

}

PDCMeasurementInitiationFailure-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- PDC MEASUREMENT REPORT PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PDC Measurement Report

--

-- **************************************************************

PDCMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{PDCMeasurementReport-IEs}},

...

}

PDCMeasurementReport-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-PDCMeasurementResult		CRITICALITY ignore	TYPE PDCMeasurementResult		PRESENCE mandatory	},

...

}

END

-- ASN1STOP

Next Change

### 9.4.5	Information Element Definitions

** unchanged text skipped **

id-SRSSpatialRelationPerSRSResource,

id-PDCMeasurementQuantities-Item,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

maxCellingNBDU,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofslots,

maxnoofNonUPTrafficMappings,

maxnoofServingCells,

maxnoofServedCellsIAB,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofSymbols,

maxnoofDUFSlots,

maxnoofHSNASlots,

maxnoofEgressLinks,

maxnoofMappingEntries,

maxnoofDSInfo,

maxnoofQoSParaSets,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofPhysicalResourceBlocks-1,

maxnoofPRACHconfigs,

maxnoofRACHReports,

maxnoofRLFReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationState,

maxnoofCHOcells,

maxnoofMDTPLMNs,

maxnoofCAGsupported,

maxnoofNIDsupported,

maxnoofExtSliceItems,

maxnoofPosMeas,

maxnoofTRPInfoTypes,

maxnoofSRSTriggerStates,

maxnoofSpatialRelations,

maxnoBcastCell,

maxnoofTRPs,

maxnoofAngleInfo,

maxnooflcs-gcs-translation,

maxnoofPath,

maxnoofMeasE-CID,

maxnoofSSBs,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxnoofPRS-ResourceSets,

maxnoofPRS-ResourcesPerSet,

maxNoOfMeasTRPs,

maxnoofPRSresourceSets,

maxnoofPRSresources,

maxnoofMeasPDC

** unchanged text skipped **

PDCMeasurementPeriodicity ::= INTEGER (0..512, ...)

PDCMeasurementQuantities ::= SEQUENCE (SIZE (1.. maxnoofMeasPDC)) OF ProtocolIE-SingleContainer { {PDCMeasurementQuantities-ItemIEs} }

PDCMeasurementQuantities-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-PDCMeasurementQuantities-Item	CRITICALITY reject	TYPE PDCMeasurementQuantities-Item		PRESENCE mandatory}

}

PDCMeasurementQuantities-Item ::= SEQUENCE {

PDCmeasurementQuantitiesValue				PDCMeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { PDCMeasurementQuantitiesValue-ExtIEs} } OPTIONAL

}

PDCMeasurementQuantitiesValue-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCMeasurementQuantitiesValue ::= ENUMERATED {

nr-tadv,

gNB-rx-tx,

...

}

PDCMeasurementResult ::= SEQUENCE {

pDCMeasuredResultsList		PDCMeasuredResultsList,

iE-Extensions					ProtocolExtensionContainer { { PDCMeasurementResult-ExtIEs} } OPTIONAL

}

PDCMeasurementResult-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCMeasuredResultsList ::= SEQUENCE (SIZE(1..maxnoofMeasPDC)) OF PDCMeasuredResults-Item

PDCMeasuredResults-Item ::= SEQUENCE {

PDCMeasuredResults-Value 	PDCMeasuredResults-Value,

iE-Extensions			ProtocolExtensionContainer {{ PDCMeasuredResults-Item-ExtIEs }}	 OPTIONAL

}

PDCMeasuredResults-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCMeasuredResults-Value ::= CHOICE {

tADV-NR					TADV-NR,

rxTxTimeDiff			GNBRxTxTimeDiffMeas,

choice-extension		ProtocolIE-SingleContainer { { PDCMeasuredResults-Value-ExtIEs} }

}

PDCMeasuredResults-Value-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PDCReportType ::= ENUMERATED {

onDemand,

periodic,

...

}

** unchanged text skipped **

TADV-NR ::= OCTET STRING	-- encoding FFS

Next Change

### 9.4.7	Constant Definitions

** unchanged text skipped **

id-ReferenceTimeInformationReportingControl	ProcedureCode ::= 58

id-PDCMeasurementInitiation					ProcedureCode ::= x1

id-PDCMeasurementReport						ProcedureCode ::= x2

** unchanged text skipped **

maxnoofPRSresources						INTEGER ::= 64

maxnoofMeasPDC							INTEGER ::= 16

** unchanged text skipped **

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-PDCMeasurementPeriodicity						ProtocolIE-ID ::= xx1

id-PDCMeasurementQuantities							ProtocolIE-ID ::= xx2

id-PDCMeasurementQuantities-Item					ProtocolIE-ID ::= xx3

id-PDCMeasurementResult								ProtocolIE-ID ::= xx4

id-PDCReportType									ProtocolIE-ID ::= xx5

End of Text Proposal for TS 38.473