3GPP TSG-RAN WG3 Meeting #115-e	R3-221875

Online, February 21st – March 3rd, 2022

Agenda item:	21.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Time synchronization: resolution of mobility open issues

Document for:	Discussion and Decision

# 1	Introduction

RAN3 has been discussing assistance information that should be passed to a target gNB during handover to maintain the timing accuracy required by the UE. The following agreements / open issues are already captured in the Chair’s minutes:

Introduce the Time Synchronisation Assistance Information IE as an optional UE-level parameter in - XnAP (HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE) It is FFS on whether assistance information (e.g., UE TSN timing reference, referenceTimeInfo delivery periodicity, timestamp) should be delivered during HO.

As described in the moderator’s summary of R3-221112 [1], there was no consensus yet that any additional parameters are needed during handover beyond the Time Synchronisation Assistance Information IE that was already agreed. Therefore, the current FFS still exists.

In this paper, we discuss this issue and propose a way forward to reach a conclusion.

# 2	Discussion

As described in R3-221112 [1], two use cases were identified for passing additional assistance information from source gNB to target gNB:

Use Case #1:	Assist the target gNB in deciding the RTI delivery configuration.

Use Case #2:	Assist the target gNB in determining the level of urgency/reliability to deliver the first RTI following handover.

For supporting Use Case #1, in our understanding the Time Synchronization Assistance Information IE already agreed to be exchanged over XnAP is enough for the target gNB to determine the delivery configuration of RTI to the UE in the new cell. That is, this IE together with the UEAssistanceInformation containing the referenceTimeInfoPreference from the UE is the same input the source gNB had when determining RTI prior to the handover event. Therefore, we see no need to add any other parameters in XnAP to support Use Case #1 (like uncertainty, Time Information type, RTI distribution, or periodicity listed in other previous contributions in RAN3). From our view, these parameters are not essential to be exchanged in XnAP:

-	timeInfoType attribute is unlikely to change between the source gNB and target gNB (i.e., GPS or a local clock as per TS 38.331).

-	uncertainty attribute is specific per gNB implementation. The target gNB does not have a use for the value at the source gNB.

-	ReferenceTimeInfo periodicity used by the source gNB. The same way the source gNB was able to determine RTI periodicity prior to the handover, the same can be done by the target gNB (using for example the Time Synchronization Assistance Information IE). Therefore, this parameter is redundant if sent via XnAP.

-	ReferenceTimeInfo distribution used by the source gNB. The SIB9 configuration at the target gNB may differ significantly from the source gNB. Additionally, at the source gNB the gNB-CU may complement SIB9 updates with RRC signalling to deliver RTI with enough updates towards the UE. Therefore, it is not clear how RTI distribution (i.e., broadcast or unicast) at the source gNB for a UE can assist the target gNB RTI determination.

Therefore, the Time Synchronization Assistance Information IE is sufficient to assist the target gNB in deciding the RTI delivery configuration

Observation 1: No additional assistance information is needed to support Use Case #1.

For supporting Use Case #2, additional information is needed to be exchanged between source gNB and target gNB. This use case could be considered an optimization to enable the target gNB to determine the level of urgency/reliability to deliver the first RTI following the handover. By default, the target gNB can decide to deliver the RTI as soon as possible after handover but this requirement could be relaxed if the source gNB provides the timestamp of when, at the latest, the next RTI needs to be delivered to the UE at the target gNB. However, considering that this optimisation may not be essential and that we must conclude the work item at this meeting, we propose not to support Use Case #2 in Rel-17.

Observation 2: Support for Use Case #2 can be considered in a future release.

# 3	Conclusions

In this paper, we analysed the remaining open issues for time synchronization mobility (XnAP open issues). The following is proposed:

Proposal: No additional time synchronisation assistance information is needed over XnAP in Release 17.

# References

- R3-221112 Summary of offline: Propagation delay compensation enhancements, Nokia (moderator)