3GPP TSG-RAN WG3 Meeting #115	R3-221887

# E-meeting, 21 Feb – 3 Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0859 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.473 on Measurement Amount |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-17 |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For periodical positioning events, the reporting amount is always included along with the periodicity information, both in service request and LPP. The amount information is missing when periodic reporting is required in NRPPa Measurement Request message. The LMF would always need to send a Measurement Abort message to stop the gNB measurements. To support SCG activation and deactivation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new IE Measurement Amount |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be waste of ressource in the gNB reporting and extra Abord message is needed from the LMF, when the number of report is configured by the LMF into the UE, but not know from the gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.12.3, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.455 CR0055 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------Start of the First Change-----------------------------

#### 9.2.12.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.3.1.197 |  |  |  |
| >>Search Window Information | O |  | 9.3.1.204 |  |  |  |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Positioning Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| Positioning Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms) |  | YES | reject |
| Positioning Measurement Quantities |  | 1 |  |  | YES | reject |
| > Positioning Measurement Quantities Item |  | 1..<maxnoofPosMeas> |  |  | EACH |  |
| >> Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …) |  |  | - |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [38] |  |  |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Amount | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (ra1, ra2, ra4, ra8, ra16, ra32, ra64, ra-Infinity,…) | reportingAmount as defined in TS 37.355 [14] | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one measurement message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Positioning Report Characteristics IE is set to the value "Periodic". |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofPRSresource,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-MeasurementAmount,

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- Positioning Measurement Request

--

-- **************************************************************

PositioningMeasurementRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningMeasurementRequestIEs} },

...

}

PositioningMeasurementRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory}|

{ ID id-LMF-MeasurementID						CRITICALITY reject	TYPE LMF-MeasurementID								PRESENCE mandatory}|

{ ID id-RAN-MeasurementID						CRITICALITY reject	TYPE RAN-MeasurementID								PRESENCE mandatory}|

{ ID id-TRP-MeasurementRequestList				CRITICALITY reject	TYPE TRP-MeasurementRequestList					PRESENCE mandatory}|

{ ID id-PosReportCharacteristics				CRITICALITY reject	TYPE PosReportCharacteristics				PRESENCE mandatory}|

{ ID id-PosMeasurementPeriodicity				CRITICALITY reject	TYPE MeasurementPeriodicity					PRESENCE conditional }|

-- The above IE shall be present if the PosReportCharacteristics IE is set to “periodic” --

{ ID id-PosMeasurementQuantities				CRITICALITY reject	TYPE PosMeasurementQuantities					PRESENCE mandatory}|

{ ID id-SFNInitialisationTime					CRITICALITY ignore	TYPE RelativeTime1900	PRESENCE optional	}|

{ ID id-SRSConfiguration						CRITICALITY ignore	TYPE SRSConfiguration								PRESENCE optional}|

{ ID id-MeasurementBeamInfoRequest				CRITICALITY ignore	TYPE MeasurementBeamInfoRequest	PRESENCE optional	}|

{ ID id-SystemFrameNumber						CRITICALITY ignore	TYPE SystemFrameNumber		PRESENCE optional}|

{ ID id-SlotNumber								CRITICALITY ignore	TYPE SlotNumber				PRESENCE optional}||

{ ID id-MeasurementAmount				CRITICALITY ignore	TYPE MeasurementAmount			PRESENCE conditional },

-- The above IE shall be present if the PosReportCharacteristics IE is set to “periodic” --

...

}

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

#### -- M

MeasurementAmount ::= ENUMERATED {

ra1,

ra2,

ra4,

ra8,

ra16,

ra32,

ra64,

ra-Infinity,

...

}

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-ResponseTime										ProtocolIE-ID ::= x8

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

-------------------End of the Change-------------------