3GPP TSG-RAN WG3 Meeting #115-e 	   R3-221899

Online, 21st Feb – 3rd Mar 2022

Title:	[DRAFT] Reply LS on RAN3 impacts for non-SDT handling

Reply to:	R3-221667/R2-2201977 LS on RAN3 impacts for non-SDT handling

Release:	Release 17

Work Item:	NR_SmallData_INACTIVE

Source:	CATT (to be RAN3)

To:	RAN2

Cc:

Contact Person:

#### Name:	Jiancheng Sun

E-mail Address:	sunjiancheng@catt.cn

Attachments:	n/a

1. Overall Description:

RAN3 thanks RAN2 for the LS on non-SDT handling aspects. RAN3 discussed how to handle the second RRCResumeRequest message during SDT procedure, and provided the answers to the questions accordingly.

Q1: Which node (old anchor gNB or serving gNB) will process the second RRCResumeRequest message with I-RNTI associated to the old anchor gNB and will perform ResumeMAC-I verification and key derivation?

A1: two use cases should be considered to decide the network behaviors when the new gNB receives the second RRCResumeRequest message:

- In case of SDT without anchor relocation, the old anchor gNB should identify the UE context with I-RNTI and perform ResumeMAC-I verification and key derivation, and then send the UE to inactive via RRCRelease message. An additional Retrieval UE context procedure may need to be initiated during the ongoing SDT procedure when the receiving gNB receives the second RRCResumeRequest message.

- In case of SDT with anchor relocation, the new serving gNB should be able to handle the second RRCResumeRequest totally, e.g. resume the UE to RRC connected for the non-SDT transmission. This requires the receiving gNB to keep the old I-RNTI and use it to identify the retrieved UE context.

Q2: From RAN3 point of view, does the old anchor gNB and/or the serving gNB need to distinguish the second RRCResumeRequest message via any explicit indication sent from UE?

A2: RAN3 understands it’s no need to distinguish the second RRCResumeRequest message via any explicit indication sent from UE.

RAN3 kindly requests RAN2 to take above answers into consideration, and provide feedback if any.

2. Actions:

To RAN2:

ACTION: RAN3 kindly requests RAN2 to take above answers into consideration, and provide feedback if any.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#116-e	       16 - 27 May, 	2022	Online