3GPP TSG-RAN3 Meeting # 115-e	R3-221909

# Online, 21 Feb - 3 Mar, 2022

Agenda Item:	15.3

Source:	China Unicom

Title:	(TP for QoE BL CR for TS 38.423): RAN visible QoE for NR QoE Measurement Collection

Document for:	Agreement

1 Introduction

This TP proposes the changes for the QoE BL CR of TS 38.423, according to the agreements of RAN3#114bis-e meeting.

In RAN3#114bis-e meeting, RAN3 has agreed:

During handover preparation, source NG-RAN node sends to the target NG-RAN node:

- in XnAP/NGAP IEs: available RVQoE metrics (received as part of QMC configuration);

The RAN visible QoE metrics configured from the OAM should be propagated from source NG-RAN Node to target NG-RAN node, via HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE message.

TP for QoE BL CR for TS 38.423

Editor’s note: The alignment with the QoE BL CR for TS 38.413 is FFS.

Start of the change

9.2.3.b UE Application Layer Measurement Information

The IE defines the information about the QoE Measurement Collection (QMC) configuration.

Editor’s note: The presence of configuration container in the IE is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Container for S-based Application Layer Measurement Configuration | O (WA) |  | OCTET STRING (SIZE(1..FFS)) | Contains the s-based QoE measurement configuration, see Annex L in [xx]. |
| QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the trace activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. |
| Measurement Configuration Application Layer ID | M |  |  | This IE indicates the identity of the application layer measurement configuration, as defined in TS 38.331 [10]. |
| Service Type | M |  | ENUMERATED (QMC for DASH streaming, QMC for MTSI, QMC for VR, ...) | This IE indicates the service type of QoE measurements. |
| QoE Measurement Status (WA) | M |  | ENUMERATED (ongoing, configured...) | Indicates whether the QoE measurement has been activated. |
| CHOICE MDT Alignment Information | O |  |  | Indicates the MDT measurements with which alignment is required. |
| >S-based MDT |  |  |  |  |
| >>NG-RAN Trace ID | M |  | 9.2.3.97 | Indicates the MDT measurements with which alignment is required. |
| >M-based MDT |  |  |  |  |
| >>Trace Reference | M |  | OCTET STRING (SIZE(6)) | Trace Reference defined in TS 32.422 [23]. |
| Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| CHOICE Area Scope of QMC | O |  |  |  |
| >Cell based |  |  |  |  |
| >>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |
| >>>NR CGI | M |  | 9.2.2.7 |  |
| >TA based |  |  |  |  |
| >>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. |
| >TAI based |  |  |  |  |
| >>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |
| >>>TAI | M |  | 9.2.3.20 |  |
| >PLMN based |  |  |  |  |
| >>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |
| >>>PLMN Identity | M |  | 9.2.2.4 |  |
| S-NSSAI List | O | 0..1 |  |  |
| >S-NSSAI Item |  | 1 .. <maxnoofSNSSAIforQMC> |  |  |
| >>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |
| RAN Visible QoE Metric Indication | O |  | 9.2.3.x |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforQMC | Maximum no. of Cell IDs comprising the QMC scope. Value is 32. |
| maxnoofTAforQMC | Maximum no. of TA comprising the QMC scope. Value is 8. |
| maxnoofPLMNforQMC | Maximum no. of PLMNs in the PLMN list for QMC scope. Value is 16. |
| maxnoofSNSSAIforQMC | Maximum no. of S-NSSAIs comprising the QMC scope. Value is 16. |

#### 9.2.3.x RAN Visible QoE Metric Indication

This IE defines which RAN visible QoE metrics can be configured by the NG-RAN in the RAN visible QoE measurement.

| Buffer Level Indication | O |  | ENUMERATED (true, ...) | The IE defines whether the Buffer Level can be collected as a RAN visible QoE metric by NG-RAN from UE, for DASH streaming and VR service types. | - | - |
| --- | --- | --- | --- | --- | --- | --- |
| Playout Delay Indication | O |  | ENUMERATED (true, ...) | The IE defines whether the Playout delay can be collected as a RAN visible QoE metric by NG-RAN from UE, for DASH streaming and VR service types. |  |  |

End of the change