3GPP TSG-RAN WG3 #115-e																	      R3-221937

21th Feb – 3rd Mar 2022

Online

Title: 	(TP for RA-SDT BLCR to TS 38.300) Left issues for RA-SDT without anchor relocation case

Source: 	 ZTE, Ericsson

Agenda item: 	24.2

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #114bis e-meeting, we have the following progress.

WA: Solution 2/3/4 is the basis for further work. Agree to include the SDT related UE content in TP, the details need to be further checked. SRB is FFS. Extend the XnAP: RRC TRANSFER message, to forward the UL/DL SRB PDCP PDU during SDT procedure without anchor relocation between new gNB and anchor gNB Transfer the first SRB/DRB transfer as the same method as the subsequent SRB/DRB transfer The additional SDT assistant information is needed, but it is no need to consult with RAN2. It includes either BSR information or single/multiple packets indication as optional IEs. FFS on others. It is no need to send LS to SA2/RAN2 for the issue indicated in section 4.6.1 and 4.6.2. It is FFS that either new Class 1 message or a new Class 2 message is used to transfer SDT configuration in case of RA-SDT without anchor relocation. Whether SDT data handling between gNBs should be per DRB (not per PDU session), which is different to the fundamental NR mobility design per PDU session basis. Whether Xn-U Address Indication should be re-used to deliver DL forwarding TNL from new gNB, or be replaced only for SDT without anchor relocation. It is FFS that the additional SDT assistant information includes either BSR information or single/multiple packets indication as optional IEs, and FFS on other IEs. Other IEs in the SDT configuration (from anchor gNB to receiving gNB) are FFS Finalize signaling design, SDT related UE context, to be continued…

This paper addresses the left issues based on above progress.

- Discussion

  - Change the WA into agreement

WA: Solution 2/3/4 is the basis for further work

During a SDT progress, when gNB-DU does not receive UL SDT packet, it sends UE Inactivity Notification message to gNB-CU-CP, and when gNB-CU-UP does not receive DL SDT packtet, it also sends Bearer Context Inactivity Notification message to gNB-CU-CP, so that the anchor gNB-CU-CP sends Retrive UE context failure message to the receiving gNB to stop current SDT progress.

Proposal 1: Change the “WA: Solution 2/3/4 is the basis for further work” to agreement.

  - Downselection Class 1 vs Class 2

It is FFS that either new Class 1 message or a new Class 2 message is used to transfer SDT configuration in case of RA-SDT without anchor relocation.

During the online and offline discussion in the last meeting, all companies agree that both class 1 and class 2 are feasible and the RLC configuration informations sent from anchor gNB to receiving gNB are essential. Half of companies prefer to class 1 and another half prefer to class 2.

Some companies think class 1 message can satisfy the essential progress, but some companies think class 2 message criticality reject can also satify this requirement.

Some companies think class 1 message can support potential failure case, but some companies does not agree with it.

Observation 1: Both class 1 and class 2 are feasible and the RLC configuration informations sent from anchor gNB to receiving gNB are essential.

Proposal 2: It is proposed to select class 1 messag to support RA-SDT without anchor reloction case.

Whether Xn-U Address Indication should be re-used to deliver DL forwarding TNL from new gNB, or be replaced only for SDT without anchor relocation.

If class 2 message is selected, RAN3 shall further down select either solution 3 or solution 4.

Solution 3:

Figure 1: Solution 3 overall procedure

Solution 4:

Figure 2: Solution 4 overall procedure

Proposal 2.1: If class 2 message is selected, RAN3 shall further select either solution 3 or solution 4.

  - The additional SDT assistant information

The additional SDT assistant information is needed, but it is no need to consult with RAN2. It includes either BSR information or single/multiple packets indication as optional IEs. FFS on others.

It is FFS that the additional SDT assistant information includes either BSR information or single/multiple packets indication as optional IEs, and FFS on other IEs.

We have already agreed that “SDT indictor” IE is essential for SDT progress, and other additional SDT assistant information is not essential.

Proposal 3: RAN3 shall decide to include either BSR information or single/multiple packets indication as additional SDT assistant information.

  - Other IEs in the SDT configuration

Agree to include the SDT related UE content in TP, the details need to be further checked. SRB is FFS.

Whether SDT data handling between gNBs should be per DRB (not per PDU session), which is different to the fundamental NR mobility design per PDU session basis.

Other IEs in the SDT configuration (from anchor gNB to receiving gNB) are FFS

The following IEs in the SDT configuration (from anchor gNB to receiving gNB) are FFS

- SRB

- DRB QoS (QoS Flow Level QoS Parameters 9.2.3.5)

- S-NSSAI

- RLC Mode

- PDCP SN Length

- DL Forwarding (9.2.3.34) or Packet Data Indication (Indicates downlink data available)

- PDU Session ID

In our view, the DL Forwarding, Packet Data Inidcation and PDU session ID are not needed.

Proposal 4: The SDT configuration shall include SRB, DRB QoS, S-NSSAI, RLC Mode and PDCP SN length.

- Conclusion

After the above analysis, we provide the following observations and proposals

Proposal 1: Change the “WA: Solution 2/3/4 is the basis for further work” to agreement.Proposal 2: RAN3 shall select either class 1 or class 2 messag to support RA-SDT without anchor reloction case.Proposal 2.1: If class 2 message is selected, RAN3 shall further select either solution 3 or solution 4.

Observation 1: Both class 1 and class 2 are feasible and the RLC configuration informations sent from anchor gNB to receiving gNB are essential.

Proposal 2: It is proposed to select class 1 messag to support RA-SDT without anchor reloction case.

Proposal 3: RAN3 shall decide to include either BSR information or single/multiple packets indication as additional SDT assistant information.

Proposal 4: The SDT configuration shall include SRB, DRB QoS, S-NSSAI, RLC Mode and PDCP SN length.

- Text Proposal to TS38.300

### XX.3	RA-based SDT without UE context relocation

Editor’s Note: Further discussion is needed for the overall procedure for RACH based SDT without anchor relocation.

Figure XX.3-1. RA-based SDT without UE context relocation

1.  	The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.  	The receiving gNB locates the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE requested for SDT and may also provide SDT assistance information.

Editor’s Note: Details of assistance information is pending to future discussion.

3.  	The last serving gNB decides not to relocate UE context for SDT.

4.  	The last serving gNB transfers the SDT related RLC context.

5. 	The receiving gNB acknowledges receiving SDT related RLC context and forwards associated DL TNL address, if needed. After Partial UE Context Retrieval procedure, the UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB, then the UL SDT data is delivered to the UPF, the UL NAS PDU is delivered to AMF.

Editor’s Note: The definition of Partial UE Context Retrieval signalling/procedure can be refined.

Editor’s Note: whether receiving the SDT related RLC context is acknowledged is FFS.

6. 	After SDT transmission is completed, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes Suspend Indication.

7.	The receiving gNB forwards the RRCRelease message to the UE, and releases SDT RLC context of this UE.

8. 	The UE moves to RRC_INACTIVE mode.