3GPP TSG-RAN WG3 Meeting #115-e	R3-221949

21 February – 3 March 2022

Agenda item:	19.3

Source: 	Ericsson

Title: 	Discussion on positioning PRS processing window signalling and latency gain aspects

Document for:	Discussion and Approval

# 1. Introduction

In this contribution, we raise concerns regarding last meeting agreements, with FFS, that have been taken on the signalling of the PRS Processing Window (PPW) in NRPPa and F1AP and that may jeopardize the latency reduction goal.

# Discussion

In the last meeting, the following agreements were taken regarding the signalling of the PPW:

LMF provides a full PRS configuration to gNB as assistance information, and the gNB determines the pre-configuration of MG. A UE-associated class 1 procedure is used to provide a full PRS configuration to gNB as assistance information of the pre-configuration of MG. FFS on using new defined or existing signaling procedure. Support to transfer the information related to the PRS measurement with MG over F1AP, similar to the legacy MG, but detail waits for RAN2. Support the MG activation request by the LMF. The signaling procedure of the MG activation request uses an UE-associated class 2 signaling procedure. FFS on whether to use new defined or existing signaling procedure? Include the similar information to that in RRC LocationMeasurementIdication message in the MG activation request message. LMF provides the assistance information to help gNB determine the PRS Processing Window configuration. For activation request procedure initiated by non-LMF, an unified signaling procedure over NRPPa can be adopted for the delivery of pre-configured MG and PRS processing Window configuration information. Support to transfer the information related to the PRS processing window configuration over F1AP, similar to the legacy MG, but detail waits for RAN2. FFS on whether to transfer the UE’s positioning capability to the gNB via NRPPa, pending to the progress of the other groups. FFS on transferring some important assistance informations to gNB via NRPPa, e.g., the response time IE, Prioritisation IE. FFS on whether to define a new NRPPa class 2 procedure for LMF-initiated PRS Processing Window activation request.

The addition of the new class 1 “Measurement Preconfiguration Information” and class 2 “Measurement Activation Information” procedures, with FFS, were considered based on the input from the contribution in [1], where a stage 2 signalling description was provided. We copy it below from [2]:

Figure 1 MG/PPW Preconfiguration and Activation Request by the LMF - LMF obtains the TRP information required for positioning services from the gNBs. - LMF provides the PRS information of the neighbour TRPs to the serving gNB and requests the serving gNB to pre-configure MG/PPW(s). - The serving gNB determines the MG/PPW configurations and preconfigures the UE(s) that may have low-latency positioning service demands. - The gNB sends the confirmation message to the LMF to indicate the success of the preconfiguration. Note: the preconfiguration of MG/PPW(s) should be performed before the location services initiate for the purpose of latency improvements. - LMF sends the LPP Provide Assistance Data message to the target device. The message includes any required assistance data for the target device to perform necessary DL-PRS measurements. - LMF may send the NRPPa message to request for MG/PPW activation. This message can also be transmitted along with the LPP Request Location Information message. - The serving gNB decides the MG/PPW configuration to be activated and sends activation request to the target UE. - The LMF sends a LPP Request Location Information message to request for DL-PRS measurements.

First, we must highlight that Step 1 in Figure1 has been agreed to be UE specific, contrary to what was initially proposed in [1], where the procedure was described to be cell-specific with 65535 TRPs that can be signalled in the new MEASUREMENT PRECONFIGURATION REQUIRED message. Thus, we must reconsider the logic of the signalling in [1].

Since Step 1 is UE-specific, then the UE context has already been set up in NG-RAN and the UE has been configured to perform measurements. Hence, the gNB knows already what kind of PRS configuration is there for the UE and the gNB can activate it by itself.

In fact, if we follow the RAN1 agreements below, then there is no need for more signalling from the LMF and it is up to gNB to configure using either RRC or RRC+MAC CE for the activation of the PPW:

Agreement PRS processing window request to the gNB by the LMF is supported from RAN1 perspective. - It is up to RAN3 to design the necessary information to be transferred in the NRPPa message. - Note: It is up to gNB to determine the usage of measurement gap or PRS processing window - Include it in the LS to RAN2 and RAN3. Agreement For PRS processing window configuration and indication, at least the following mechanism is supported - RRC (pre-)configuration for PRS processing window configuration and DL MAC CE activation for PRS processing window, respectively. Include it in the LS to RAN2 and request RAN2 to decide whether DL MAC CE is feasible for this indication.

Observation 1: Considering that the message in step 1 is UE-specific, then the gNB already knows during that step that UE has been configured with PRS configuration and it is up to gNB to decide on the PPW activation either via RRC or RRC+MAC CE.

In this respect, step 5 would not be necessary, and the confirmation of step 3 for the LMF is not necessary either. Otherwise, if RAN3 introduces too many sequential behaviours in NRPPa (3 messages) and F1AP (up to 6 messages in total), then we achieve the opposite of the original goal of the PPW configuration, i.e. we will be adding more latency to the network, instead of reducing it as it was targeted.

For information, as studied during the Release-17 Positioning SI phase in section 8.1.3 of TR 38.857 [2], the NRPPa Processing delays is 9 ms and the NRPPa signalling delay:4-20 ms. Besides, gNB split architecture adds F1AP processing latency and CU-DU signalling propagation delay (the latter latency values have not been mentioned in the TR).

Observation 2: Considering that the whole goal of PPW is to reduce network latency, if we add too many sequential messages over F1AP/NRPPa for PPW pre-configuration, confirmation and activation, then instead of saving latency, we create more latency, which goes against the initial objective of this feature.

Therefore, we must revise the proposed signalling to be in-line with the objective of latency reduction and the RAN1 agreements by removing unnecessary signalling. One solution is that the message MEASUREMENT ACTIVATION in 9.1.1.a7 of the NRPPA BL CR becomes a combined message for both PPW pre-configuration and MGs activation with two distinct IEs. This way, a single class2 procedure can be used for providing the PPW information to gNB for gap-less measurements pre-configuration, and the MG activation for gap-based measurements.

We note that some proposals were proposed previously in RAN3#114-bis-e in this same spirit, such as in [3]. We can further discuss if a class1 procedure with failure cause could be more adequate to inform LMF of the MG activation or PPW pre-configuration failure. In this case, re-using the Positioning Information Exchange procedure can be a possible option.

Proposal 1: Revise the MEASUREMENT ACTIVATION message in 9.1.1.a7 of NRPPA BL CR to provide PPW information pre-configuration and MG activation as two distinct IEs in one single UE-associated message

On the other hand, step 2 of Figure 1 mentions that “2. The serving gNB determines the MG/PPW configurations and preconfigures the UE(s) that may have low-latency positioning service demands”.

The proposal in [1] did not provide any solution on how gNB can know that the UE has low-latency positioning service demands. In order to address this and help gNB with the configuration and prioritization of resources, it is needed to signal Positioning QoS information with the UE LCS latency demands in the same message for PPW pre-configuration (and also in the Positioning Information Request). We note that the SA2 specification TS 23.273 broadly categorizes the LCS latency requirements in three categories: no delay, low delay and delay tolerant; and the accuracy requirements in two categories: Horizontal Accuracy and Vertical Accuracy [4].

Proposal 2: LMF to provide the UE LCS latency and accuracy positioning demands as a Positioning QoS IE when signalling the PPW information and positioning information to gNB

A TP to NRPPa BL CR is provided in the annex.

A TP to F1AP BL CR can be provided upon convergence.

# 3. Conclusion

Observation 1: Considering that the message in step 1 is UE-specific, then the gNB already knows during that step that UE has been configured with PRS configuration and it is up to gNB to decide on the PPW activation either via RRC or RRC+MAC CE.

Observation 2: Considering that the whole goal of PPW is to reduce network latency, if we add too many sequential messages over F1AP/NRPPa for PPW pre-configuration, confirmation and activation, then instead of saving latency, we create more latency, which goes against the initial objective of this feature.

Proposal 1: Revise the MEASUREMENT ACTIVATION message in 9.1.1.a7 of NRPPA BL CR to provide PPW information pre-configuration and MG activation as two distinct IEs in one single UE associated message

Proposal 2: LMF to provide the UE LCS latency and accuracy positioning demands as a Positioning QoS IE when signalling the PPW information and positioning information to gNB

A TP to NRPPa BL CR is provided in the annex.

A TP to F1AP BL CR can be provided upon convergence.

# 4. References

- R3-220513 (TP for POS BL CR for TS 38.455, TS 38.473) on Latency improvement in positioning

- TR 38.587, Study on NR Positioning Enhancements; (Release 17), section 8.1.3

- R3-220354 (TP for NR_pos_enh BL CR for TS 38.455) Latency reduction related to the measurement gap, Nokia

- TS 23.273, clause 4.1b	LCS Quality of Service

# 5. Annex : TP to NRPPa BL CR

<<START OF CHANGES>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413:"NG-RAN; NG Application Protocol (NGAP)".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[4]	Void.

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[6]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Base Station (BS) radio transmission and reception".

[8]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[9]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[10]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Physical Channels and Modulation".

[11]	IEEE Std 802.11™-2012, IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area network.

[12]	3GPP TS 36.455: " Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 37.355: " Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

[15]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[16]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[17]	3GPP TS 36:214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer (PHY); Measurements".

[18]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[19]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

<<NEXT CHANGE>>

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-

-	Measurement Activation Information Transfer. This function allows the LMF to indicate the NG-RAN node to activate the preconfigured measurement gap or to pre-configure PRS processing window.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Activation Information Transfer | Measurement Activation |

Editor Notes: The procedure name is FFS.

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |

Editor Notes: The procedure name is FFS.

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |

<<NEXT CHANGE>>

### 8.2.Z	Measurement Activation

#### 8.2.Z.1	General

The Measurement Activation procedure is initiated by the LMF to indicate the NG-RAN node to activate the preconfigured measurement gap or to pre-configure PRS processing window for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.Z.2	Successful Operation

Figure 8.2.y.2-1: Measurement Activation procedure, successful operation

The LMF initiates the procedure by sending a MEASUREMENT ACTIVATION message.

If the Requested PRS Processing Window IE is included in the MEASUREMENT ACTIVATION message, the NG-RAN node may take this information into account when configuring PRS processing window for the UE.

If the Requested Measurement Gap IE is included in the MEASUREMENT ACTIVATION message, the NG-RAN node may take this information into account when configuring measurement gaps for the UE.

If the Positioning QoS IE is included in the MEASUREMENT ACTIVATION message, the NG-RAN node may take this information into account when configuring measurement gaps and/or pre-configuration PRS processing window for the UE.

#### 8.2.Z.3	Unsuccessful Operation (FFS)

Not Applicable.

<<NEXT CHANGE>>

#### 9.1.1.a7	MEASUREMENT ACTIVATION

This message is sent by the LMF to indicate the NG-RAN node to activate the preconfigured measurement gap or to pre-configure PRS processing window for the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested PRS Processing Window | O |  | 9.2.b1 |  | YES | ignore |
| Requested Measurement Gap | O |  | 9.2.b2 |  | YES | ignore |
| Positioning QoS | O |  | 9.2.b3 |  | YES | ignore |

<<NEXT CHANGE>>

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.x8 |  | YES | ignore |
| Positioning QoS | O |  | 9.2.b3 |  | YES | ignore |

<<NEXT CHANGE>>

### 9.2.b1	Requested PRS Processing Window

This IE contains the requested PRS processing window for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP PRS Information List |  | 1 |  |  |
| >TRP PRS Information Item |  | 1 .. <maxnoTRPsforPPW> |  |  |
| >>TRP ID | M |  | 9.2.24 |  |
| >>NR PCI | M |  | INTEGER (0..1007) |  |
| >>NR CGI | O |  | 9.2.9 |  |
| >>PRS Configuration | M |  | 9.2.44 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPforPPWs | Maximum no. of TRPs for PRS processing window pre-configuration. Value is 256 |

### 9.2.b2	Requested Measurement Gap

This IE contains the requested measurement gap for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PRS Measurement Info List |  | 1 |  |  |
| >PRS Measurement Info Item |  | 1..<maxnoFreqLayers> |  |  |
| >>DL PRS PointA | M |  | INTEGER (0..3279165) | NR ARFCN |
| >>MeasPRS Periodicity | M |  | ENUMERATED (ms20, ms40, ms80, ms160, …) | Measurement gap periodicity in units of ms |
| >>MeasPRS Offset | M |  | INTEGER (0..159, …) | Measurement gap offset in units of subframes |
| >>MeasPRS Length | M |  | ENUMERATED (ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20) | Measurement gap length as defined in TS 38.133 [16], in units of ms |

| Range bound | Explanation |
| --- | --- |
| maxnoFreqLayers | Maximum no of frequency layers. Value is 4. |

### 9.2.b3	Positioning QoS

This IE contains the UE Positioning QoS information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Latency demand | M |  | ENUMERATED (no delay, low delay, delay tolerant, …) | As defined in TS 22.071 [19] |
| Accuracy demand | O |  | ENUMERATED (vertical accuracy, horizontal accuracy, …) | As defined in TS 22.071 [19] |