3GPP TSG-RAN WG3 #115e	R3-221983

# Online, 21st Feb – 3rd March 2022

Agenda Item:	14.2

Source:	Lenovo, Motorola Mobility

Title: 	         (TP to SCG BLCR to TS38.401)  CU-CP CU-UP DU coordination for SCG (de)activation

Document for:	Discussion and Approval

# 1	Introduction

In this paper we further discuss the miscellaneous issues left from previous RAN3 meetings about SCG activation and deactivation.

# 2	Discussion

## 2.1 When to notify CU-UP about SCG status

Another remaining FFS is when to perform the SCG status notification from CU-CP to CU-UP, i.e., whether it is before or after CU-CP requests SCG (de)activation to DU. In our understanding, both CU-UP and DU may reject the SCG (de)activation and fail the corresponding bearer/UE context setup/modification procedure, so we don’t see strong reason to coordinate with one before the other. Thus, it seems fine to leave it to NW implementation if CU-CP shall inform CU-UP about SCG status before or after CU-CP sends SCG (de)activation request to DU.

### R3-221580 SCG BL CR to TS 38.401 ### 8.4.x SCG Deactivation and Activation This clause gives the NR SCG deactivation and activation in EN-DC and MR-DC with NR SN given that the en-gNB and SgNB consists of a gNB-CU and gNB-DU(s). Editor’s note: among the procedures listed in this section, when to perform the SCG status notification from CU-CP to CU-UP is FFS. #### 8.4.x.1 SN Addition with SCG Deactivation Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

It is up to NW implementation if CU-CP shall inform CU-UP about SCG status before or after CU-CP sends SCG (de)activation request to DU.

# 3	Conclusion

Based on the discussion above, we propose:

Proposal 1	It is up to NW implementation if CU-CP shall inform CU-UP about SCG status before or after CU-CP sends SCG (de)activation request to DU.

# TP for TS 38.401 SCG BLCR

--------------------------------Start of the First Change-----------------------------

### 8.4.x	SCG Deactivation and Activation

This clause gives the NR SCG deactivation and activation in EN-DC and MR-DC with NR SN given that the en-gNB and SgNB consists of a gNB-CU and gNB-DU(s).

Editor’s note: among the procedures listed in this section, it is up to NW implementation if CU-CP shall inform CU-UP about SCG status before or after CU-CP sends SCG (de)activation request to DU.

#### 8.4.x.1	SN Addition with SCG Deactivation

Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG deactivation.

2. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and notify the deactivated status of SCG.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG deactivation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

6. The SN sends the SN addition Request Acknowledge towards the MN, indicates that the SCG is deactivated.

#### 8.4.x.2	SN Addition with SCG Activation

Figure 8.4.x.2-1: SCG Activation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG activation.

2. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and notify the activated status of SCG.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG activation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU, indicates that the SCG is activated.

6. The SN sends the SN addition Request Acknowledge towards the MN, indicates that the SCG is activated.

#### 8.4.x.3	MN initiated SN Modification with SCG Deactivation

Figure 8.4.x.3-1: SCG Deactivation in MN initiated SN Modification procedure

1. SCG is activated.

2. The MN sends the SN Modification Request towards the SN, indicates the request of SCG deactivation.

3. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the deactivated status of SCG.

4. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

5. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

6. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

7. The SN sends the SN Modification Request Acknowledge towards the MN, indicates that the SCG is deactivated.

#### 8.4.x.4	MN initiated SN Modification with SCG Activation

Figure 8.4.x.4-1: SCG Activation in MN initiated SN Modification procedure

1. SCG is deactivated.

2. MN sends the SN Modification Request towards the SN, indicates the request of SCG activation.

3. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the activated status of SCG.

4. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP

5. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

6. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is activated.

7. The SN sends the SN Modification Request Acknowledge towards the MN, indicates that the SCG is activated.

#### 8.4.x.5	SN initiated SN Modification with SCG Deactivation

Figure 8.4.x.5-1: SCG Deactivation in SN initiated SN Modification procedure

1. SCG is activated.

2. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the deactivated status of SCG.

3. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

5. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

6. The SN sends the SN Modification Required towards the MN, indicates the request of SCG deactivation.

7. The MN sends the SN Modification Confirm towards the SN.

#### 8.4.x.6	SN initiated SN Modification with SCG Activation

Figure 8.4.x.6-1: SCG Activation in SN initiated SN Modification procedure

1. SCG is deactivated.

2. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the activated status of SCG.

3. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

5. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is activated.

6. The SN sends the SN Modification Required towards the MN, indicates the request of SCG activation.

7. The MN sends the SN Modification Confirm towards the SN.

-------------------End of the Change-------------------