3GPP TSG-RAN WG3 #115-e	R3-221986

# Online, 21st Feb – 3rd March 2022

Agenda Item:	18.4.1

Source:	Lenovo, Motorola Mobility

Title: 	        (TP to TR 37.817) On Energy Saving

Document for:	Discussion and Approval

# 1	Introduction

This paper provides TP to TR 37.817 subclause 5.1.

# TP to TR 37.817

-----------------------------------Start of Changes----------------------------------

## 5.1	Network Energy Saving

### 5.1.1	Use case description

To meet the 5G network requirements of key performance and the demands of the unprecedented growth of the mobile subscribers, millions of base stations (BSs) are being deployed. Such rapid growth brings the issues of high energy consumption, CO2 emissions and operation expenditures (OPEX). Therefore, energy saving is an important use case which may involve different layers of the network, with mechanisms operating at different time scales.

Cell activation/deactivation is an energy saving scheme in the spatial domain that exploits traffic offloading in a layered structure to reduce the energy consumption of the whole radio access network (RAN). When the expected traffic volume is lower than a fixed threshold, the cells may be switched off, and the served UEs may be offloaded to a new target cell.

Efficient energy consumption can also be achieved by other means such as reduction of load, coverage modification, or other RAN configuration adjustments. The optimal energy saving decision depends on many factors including the load situation at different RAN nodes, RAN nodes capabilities, KPI/QoS requirements, number of active UEs and UE mobility, cell utilization, etc.

However, the identification of actions aimed at energy efficiency improvements is not a trivial task. Wrong switch-off of the cells may seriously deteriorate the network performance since the remaining active cells need to serve the additional traffic. The conventional energy saving schemes are vulnerable to potential issues listed as follows:

- Lacking the ability of cell load prediction. Conventionally, energy saving decisions rely on measured traffic load without considering future traffic load.

- Trade-off between network performance and energy efficiency. Maximizing the system’s key performance indicator (KPI) is usually done at the expense of energy efficiency. Similarly, improving energy efficiency may sacrifice network performance. Thus, there is a need to balance and manage the trade-off between the two.

- Suboptimal energy saving  parameter adjustment. Energy saving  parameters are conventionally set in suboptimal way, e.g., based on different thresholds of cell load for cell switch on/off which is somewhat a rigid mechanism since it is difficult to set a reasonable threshold.

- A local (e.g. limited to a single RAN node) energy efficiency improvement may lead to a global (e.g. involving multiple RAN nodes) energy efficiency deterioration.

To deal with issues listed above, ML techniques could be utilized to optimize energy saving decisions leveraging the data collected in the RAN network. ML algorithms may predict the future energy efficiency and load state, which can be used to make better decisions on cell activation/deactivation for energy saving. Based on the predicted load, the system may dynamically configure the energy saving strategy (e.g. the switch-off timing and granularity, offloading actions) to keep a balance between network performance and energy efficiency and to reduce the energy consumption.

### 5.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 5.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following deployments can be considered for supporting AI/ML-based network energy saving:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM

In case of CU-DU split architecture, the following deployments are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 5.1.2.2	AI/ML Model Training at OAM and AI/ML Model Inference at NG-RAN

In this solution, NG-RAN makes energy saving decisions using AI/ML model trained from OAM.

Figure 5.1.2.1-1. Model Training at OAM, Model Inference at NG-RAN

Step 1: NG-RAN node 1 configures the UE measurement configuration and sends the configuration message to UE.

Step 2: The UE collects the required measurement, e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends measurement report message to NG-RAN node 1 including the required measurement results.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM. NG-RAN node 2 (assumed to have an AI/ML model optionally) also sends input data for Model Training to OAM.

Step 5: Model Training at OAM. Required measurements are leveraged to train AI/ML models for network energy saving.

Step 6: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-17 scope.

Step 7: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 8: UE sends UE measurement report to NG-RAN node 1.

Step 9: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2 and UE, NG-RAN node 1 generates model inference output(s) (e.g. predicted traffic load, energy saving strategy, handover strategy, etc).

Step 10: NG-RAN node 1 sends Model Performance Feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 11: NG-RAN node 1 may take action and handover the UE to NG-RAN node 2.

Step 12: NG-RAN node 2 provides feedback to OAM.

Step 13: NG-RAN node 1 provides feedback to OAM.

#### 5.1.2.3	AI/ML Model Training and AI/ML Model Inference at NG-RAN

In this solution, NG-RAN is responsible for model training and generates energy saving decisions.

Figure 5.1.2.2-1. Model Training and Model Inference at NG-RAN

Step 1: NG-RAN node 1 configures the UE measurement configuration and sends the configuration message to UE.

Step 2: The UE collects the required measurement, e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends measurement report to NG-RAN node 1 including the required measurement results.

Step 4: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model training of AI/ML-based network energy saving.

Step 5: NG-RAN node 1 trains AI/ML model for AI/ML-based energy saving based on collected data. NG-RAN node 2 is assumed to have AI/ML model for AI/ML-based energy saving optionally, which can also generate energy saving predictions or decisions.

Step 6: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 7: UE sends UE measurement report to NG-RAN node 1.

Step 8: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2 and UE, NG-RAN node 1 generates model inference output (e.g. predicted traffic load, energy saving strategy, handover strategy, etc).

Step 9: NG-RAN node 1 may take action and handover the UE to NG-RAN node 2.

Step 10: NG-RAN node 2 provides feedback to NG-RAN node 1.

#### 5.1.2.4	Input of AI/ML-based Network Energy Saving

The following information may be needed as input data for AI/ML-based network energy saving:

Input information from local NG-RAN node:

- UE mobility/trajectory prediction

- Current/Predicted energy efficiency

- Current/Predicted resource status

Input information from UE:

-      UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

Input from neighbouring NG-RAN nodes:

- Current/Predicted energy efficiency

- Current/Predicted resource status

- Current/Predicted energy state (e.g., active, high, low, inactive)

- FFS Accept/reject of offloading plan to transfer a certain number of UEs to a neighboring RAN node for energy saving reasons

If existing UE measurements are needed by a gNB for AI/ML-based network energy saving, RAN3 shall reuse the existing framework (including MDT and RRM measurements).

#### 5.1.2.5	Output of AI/ML-based Network Energy Saving

AI/ML-based network energy saving model can generate following information as output:

- Energy saving strategy, such as recommended cell activation/deactivation.

- Handover strategy, including recommended candidate cells for taking over the traffic

- Predicted energy efficiency

- Predicted energy state (e.g., active, high, low, inactive)

- FFS Validity time of the predicted energy saving decisions

#### 5.1.2.6	Feedback of AI/ML-based Network Energy Saving

The following information may be needed as feedback for AI/ML-based network energy saving:

- Resource status of neighbouring NG-RAN nodes

- Energy efficiency

- UE performance affected by the energy saving action (e.g. handed-over UEs), including bitrate, packet loss, latency.

- System KPIs (e.g. throughput, delay, RLF of local and neighbouring NG-RAN nodes)

#### 5.1.2.7	Standard Impact

Potential Xn interface impact:

- New signalling procedure or enhanced existing procedure to collect the input data information

  - Predicted energy efficiency between neighbouring NG-RAN nodes and local NG-RAN node

  - Predicted resource status between neighbouring NG-RAN nodes and local NG-RAN node

New signalling procedure or enhanced existing procedure to retrieve feedback information.

-----------------------------------End of Changes----------------------------------