3GPP TSG-RAN WG3 #115-e	R3-221988

# Online, 21st Feb – 3rd March 2022

Agenda Item:	18.4.3

Source:	Lenovo, Motorola Mobility

Title: 	        (TP to TR 37.817) On Mobility Optimization

Document for:	Discussion and Approval

# 1	Introduction

This paper provides TP to TR 37.817 subclause 5.3.

# TP to TR 37.817

-----------------------------------Start of Changes-----------------------------------

## 5.3	Mobility Optimization

### 5.3.1	Use case description

Mobility management is the scheme to guarantee the service-continuity during the mobility by minimizing the call drops, RLFs, unnecessary handovers, and ping-pong. For the future high-frequency network, as the coverage of a single node decreases, the frequency for UE to handover between nodes becomes high, especially for high-mobility UE. In addition, for the applications characterized with the stringent QoS requirements such as reliability, latency etc., the QoE is sensitive to the handover performance, so that mobility management should avoid unsuccessful handover and reduce the latency during handover procedure. However, for the conventional method, it is challengeable for trial-and-error-based scheme to achieve nearly zero-failure handover. The unsuccessful handover cases are the main reason for packet dropping or extra delay during the mobility period, which is unexpected for the packet-drop-intolerant and low-latency applications. In addition, the effectiveness of adjustment based on feedback may be weak due to randomness and inconstancy of transmission environment. Besides the baseline case of mobility, areas of optimization for mobility include dual connectivity, CHO, and DAPS, which each have additional aspects to handle in the optimization of mobility.

Mobility aspects of SON that can be enhanced by the use of AI/ML include:

- Reduction of the probability of unintended events

- UE Location/Mobility/Performance prediction

- Traffic Steering

Reduction of the probability of unintended events associated with mobility.

Examples of such unintended events are:

- Intra-system Too Late Handover: A radio link failure (RLF) occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

- Intra-system Too Early Handover: An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

- Intra-system Handover to Wrong Cell: An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

- Successful Handover: During a successful handover, there is underlying issue.

RAN intelligence could observe multiple HO events with associated parameters, use this information to train its ML model and try to identify sets of parameters that lead to successful HOs and sets of parameters that lead to unintended events.

UE Location/Mobility/Performance Prediction

Predicting UE’s location is a key part for mobility optimisation, as many RRM actions related to mobility (e.g. selecting handover target cells) can benefit from the predicted UE location/trajectory. UE mobility prediction is also one key factor in the optimization of early data forwarding particularly for CHO. UE performance prediction when the UE is served by certain cells is a key factor in determining which is the best mobility target for maximisation of efficiency and performance.

Traffic Steering

Efficient resource handling can be achieved by adjusting handover trigger points and selecting optimal combination of PCell/PSCell/Scells to serve a user.

Existing traffic steering can also be improved by providing a RAN node with information related to mobility or dual connectivity.

For example, before initiating a handover, the source gNB, could use feedbacks on UE performance collected for successful handovers occurred in the past and received from neighbouring gNBs.

Similarly, for the case of dual connectivity, before triggering the addition of a secondary gNB or triggering SN change, an eNB could use information (feedbacks) received in the past from the gNB for successfully completed SN Addition or SN Change procedures.

In the two reported examples, the source RAN node of a mobility event, or the RAN node acting as Master Node (a eNB for EN-DC, a gNB for NR-DC) can use feedbacks received from the other RAN node, as input to an AI/ML function supporting traffic related decisions (e.g. selection of target cell in case of mobility, selection of a PSCell / Scell(s) in the other case), so that future decisions can be optimized.

### 5.3.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 5.3.2.1			Locations for AI/ML Model Training and AI/ML Model Inference

Considering the locations of AI/ML Model Training and AI/ML Model Inference for mobility solution, the following two deployments are considered:

- The AI/ML Model Training function is deployed in OAM, while the Model Inference function resides within the RAN node

- Both the AI/ML Model Training function and the AI/ML Model Inference function reside within the RAN node

Furthermore, for CU-DU split scenario, following deployment is possible:

- AI/ML Model Training is located in CU-CP or OAM, and AI/ML Model Inference function is located in CU-CP

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM.

#### 5.3.2.2	AI/ML Model Training in OAM and AI/ML Model Inference in NG-RAN node

Figure 5.3.2-1  AI/ML Model Training in OAM and AI/ML Model Inference in NG-RAN node

Step 0. NG-RAN node 2 is assumed to optionally have an AI/ML model, which can generate required input such as resource status and utilization prediction/estimation etc.

Step 1. The NG-RAN node configures the UE measurement configuration and sends configuration message to UE.

Step 2. The UE collects the required measurement, e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3. The UE sends measurement report message to NG-RAN node 1 including the required measurement.

Step 4. The NG-RAN node 1 sends the input data for training to OAM, where the input data for training includes the required input information from the NG-RAN node 1 and the measurement from UE.

Step 5. The NG-RAN node 2 sends the input data for training to OAM, where the input data for training includes the required input information from the NG-RAN node 2.

Step 6. Model Training. Required measurements are leveraged to training AI/ML model for UE mobility optimization.

Step 7. OAM sends AI/ML Model Deployment Message to deploy the trained/updated AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-17 scope.

Step 8. The NG-RAN node 1 obtains the measurement report as inference data for UE mobility optimization.

Step 9. The NG-RAN node 1 obtains the input data for inference from the NG-RAN node 2 for UE mobility optimization, where the input data for inference includes the required input information from the NG-RAN node 2.

Step 10. Model Inference. NG-RAN node 1 performs model inference and generate Mobility Optimization predictions or decisions (e.g.  UE trajectory prediction, target cell prediction, target NG-RAN node prediction, handover strategy etc.)

Step 11. The NG-RAN 1 sends the model performance feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 12. According to the prediction, recommended actions or configuration are executed for Mobility Optimization.

Step 13. The NG-RAN node 1, the target NG-RAN node (represented by NG-RAN node 2 of this step in the flowchart), and UE do the handover procedure to hand over UE from NG-RAN node 1 to the target NG-RAN node.

Step 14. The NG-RAN node 1 sends the feedback information to OAM.

#### 5.3.2.3			AI/ML Model Training and AI/ML Model Inference in a NG-RAN node

Figure 5.3-2: Model Training and Model Inference both located in RAN node

Step 0. NG-RAN node 2 is assumed to optionally have an AI/ML model, which can generate required input such as resource status and utilization prediction/estimation etc.

Step 1. NG-RAN node1 configures the UE measurement configuration and sends configuration message to UE.

Step 2. UE collects the indicated measurement, e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3. UE sends measurement report message to NG-RAN node1 including the required measurement.

Step 4. The NG-RAN node 1 obtains the input data for training from the NG-RAN node2, where the input data for training includes the required input information from the NG-RAN node 2. Step 5. Model training. Required measurements are leveraged to training AI/ML model for mobility optimization.

Step 6. NG-RAN node1 obtains the measurement report as inference data for real-time UE mobility optimization.

Step 7. The NG-RAN node 1 obtains the input data for inference from the NG-RAN node 2 for UE mobility optimization, where the input data for inference includes the required input information from the NG-RAN node 2.

Step 8. Model Inference. Required measurements are leveraged into Model Inference to output the prediction, including e.g., UE trajectory prediction, target cell prediction, target NG-RAN node prediction, etc.

Step 9. According to the prediction, recommended actions are executed for Mobility Optimization.

Step 10. The NG-RAN node 1, the target NG-RAN node (represented by NG-RAN node 2 of this step in the flowchart), and UE do the handover procedure to hand over UE from NG-RAN node 1 to the target NG-RAN node.

Step 11. The NG-RAN node 2 sends feedback information after mobility optimization action to the NG-RAN node 1.

Note: UE mobility information for training purposes is only sent to gNBs that request such information or when triggered.

#### 5.3.2.4	Input data

The following data may be needed as input data for mobility optimization.

Input Information from the UE:

- UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available.

- Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

- UE historical serving cells and their locations

- Moving velocity

Input Information from the neighbouring RAN nodes:

- UE’s successful handover information in the past and received from neighbouring RAN nodes

- UE’s history information from neighbouring RAN nodes

- Position, resource status, QoS parameters of historical HO-ed UE (e.g., loss rate, delay, etc.)

- Resource status and utilization prediction/estimation

- SON Reports of handovers that are successful, too-early, too-late, or handover to wrong (sub-optimal) cell

- Information about the performance of handed over UEs

Input Information from the local node:

- UE trajectory prediction output (will be used by the RAN node internally)

- Local resource status prediction

- Current/predicted UE traffic

If existing UE measurements are needed by a gNB for AI/ML-based mobility optimization, RAN3 shall reuse the existing framework (including MDT and RRM measurements). FFS on whether new UE measurements are needed.

#### 5.3.2.5	Output data

- UE trajectory prediction (Latitude, longitude, altitude, cell ID of UE over a future period of time)

- Note:FFS whether the UE trajectory prediction is an internal output to the node hosting the Model Inference function

- Estimated arrival probability in CHO and relevant confidence interval

- Predicted handover target node, candidate cells in CHO, may together with the confidence of the prediction

- Priority, handover execution timing, predicted resource reservation time window for CHO

          - 5.3.2.6	Feedback

The following data is required as feedback data for mobility optimization.

- QoS parameters such as throughput, packet delay of the handed-over UE, etc

- Resource status information updates from target NG-RAN

- Performance information from target NG-RAN, FFS on performance information details

#### 5.3.2.7	Standard impact

To improve the mobility decisions at a gNB (Gnb-CU), a gNB can request mobility feedback from a neighbouring node. Details of the procedure are FFS.

- Potential Xn interface impact:

  - New signaling procedure or existing procedure to retrieve input information via Xn interface.

  - New signaling procedure or existing procedure to retrieve feedback information via Xn interface.