3GPP TSG-RAN WG3 #115-e	R3-222016

E-meeting, 21st February – 3rd March 2022

Source:	CATT

Title:	(TP on SON for 36.300) Description for RACH optimisation in EN-DC

Agenda Item:	10.2.1.7

Document for:	Discussion and decision

- Introduction

Currently there is no Stage 2 description of RACH optimisation for en-gNBs in any specification. In this document we propose adding some description in TS 36.300, mainly by aligning with the description in TS 38.300 and TS 38.401.

- Annex: TP for TS 36.300

////////////////////////////////////////////////////////////////////////skip unrelated text////////////////////////////////////////////////////////////////////////

### 22.4.3	Support for RACH Optimisation

#### 22.4.3.1	E-UTRA cell case

The setting of RACH parameters that can be optimized are:

-	RACH configuration (resource unit allocation);

-	RACH preamble split (among dedicated, group A, group B, RSRP level, NRSRP level (for NB-IoT), NPRACH resource pools (for NB-IoT), EDT);

-	RACH backoff parameter value;

-	RACH transmission power control parameters.

RACH optimisation is supported by UE reported information and by PRACH parameters exchange or NPRACH parameters (for NB-IoT) between eNBs.

UEs which receive polling signalling shall report the below information:

-	Number of RACH preambles sent until the successful RACH completion;

-	Contention resolution failure;

-	For BL UE or UE in enhanced coverage or NB-IoT UE, the RSRP (NRSRP for NB-IoT) level in which the UE started the random access procedure;

-	For BL UE or UE in enhanced coverage or NB-IoT UE, an EDT fallback indication.

UE reporting of RACH information is not supported for a NB-IoT UE using the Control Plane CIoT EPS Optimisation,

#### 22.4.3.2	EN-DC cell case

RACH optimization is supported by PRACH parameters exchange indirectly between en-gNBs.

To perform RACH optimisation at the en-gNB, the en-gNB signals the PRACH configuration per-cell to the eNB. The eNB may forward a limited set of neighbour cell’s PRACH configurations received from neighbour eNBs and other en-gNBs to the en-gNB to resolve the configuration conflict.

////////////////////////////////////////////////////////////////////////end////////////////////////////////////////////////////////////////////////