3GPP TSG-RAN WG3 #115-e	R3-222024

E-meeting, 21st February – 3rd March 2022

Source:	CATT

Title:	Discussion on remaining issues on F1-U

Agenda Item:	22.2.4

Document for:	Discussion and decision

- Introduction

This document focuses on the remaining issue on F1-U.

- Discussion

During the discussion last meeting, there was some divergence on how many UP tunnels should be established between the gNB-CU(-UP) and the gNB-DU. There was one voice that there should be always one UP tunnel, whereas another voice is that UE-specific tunnels should be separated from the non-UE-specific tunnels.

Our preference is the latter, since it has less impact on specs. The major motivation is PDCP-based retransmission.

PDCP-based retransmission occurs mainly in two scenarios:

- After handover between MBS-supporting gNBs.

- Used as a guarantee for lossless delivery if lower layer uses PTM delivery and thus the RLC is working in UM mode.

That is to say, if lower layer uses PTP delivery during normal transmission, the RLC entity can be set to AM mode and thus PDCP-based retransmission is not needed.

For the case of handover, the most common case is that only one UE is handed over, so the most cost-efficient way to deliver the NACKed packet is through a PTP method over Uu, where the MCS can be set to a value suitable for the UE, taking the radio link quality between the UE and the gNB into consideration.

For the case of guaranteeing lossless delivery upon normal PTM delivery, it is also common that only one UE within a cell missed a DL packet (as emphasised repeatedly by some company, the MCS for PTM delivery tends to be set in a conservative way thus L2 NACK is typically quite rare). So the most cost-efficient way is also to use a PTP method for PDCP retransmission.

Proposal 1: It should be confirmed that PDCP retransmission for multicast packets is performed in a PTP manner over Uu (this is within the scope of RAN2 anyhow).

And next we should check how such PDCP retransmission works in split gNB scenario.

The gNB-DU does not host any PDCP entity. When a gNB-DU receives a Uu packet containing a PDCP Status Report, it will—according to the current specs of MAC and RLC—forward the packet toward the gNB-CU(-UP) without decoding it. That is to say, the gNB-DU doesn’t know what UE missed what packet unless major revise on the MAC or RLC layer.

So the gNB-CU(-UP) should decode the PDCP Status Report and get aware of what UE misses what DL packet, or otherwise the whole network will be unable to decide what packet to be retransmitted to what UE.

There are two method to make the gNB-CU(-UP) aware as mentioned during the last meeting:

- To use a UE-specific F1-U tunnel.

- To enhance the NR UP spec so that the UL packet can be tagged with a UE ID.

Obviously the latter method has much more impact. So we prefer the former one.

The case of delivering the retransmitted packet over F1-U is similar. We prefer to use a UE-specific F1-U tunnel rather than tagging the UP packet with a UE ID.

As a result, a bi-directional UE-specific F1-U tunnel should be established between the gNB-DU and the gNB-CU, if PDCP retransmission is enabled.

Generally speaking, PDCP retransmission is enabled if lossless delivery is expected. One may argue that the lower layer may decide to use PTP delivery mode with RLC AM mode, where the PDCP retransmission is not used. This is true, but for this case the gNB-DU should provide a DDDS frame toward the gNB-CU(-UP) when handover is triggered, thus the UE-specific F1-U tunnel is anyhow needed.

So we can conclude that, a UE-specific F1-U tunnel should be established for an MRB if and only if lossless delivery is expected for this MRB.

Proposal 2: A UE-specific F1-U tunnel should be established for an MRB if and only if lossless delivery is expected for this MRB. No need to enhance the NR UP spec.

- Conclusion

Proposal 1: It should be confirmed that PDCP retransmission for multicast packets is performed in a PTP manner over Uu (this is within the scope of RAN2 anyhow).

Proposal 2: A UE-specific F1-U tunnel should be established for an MRB if and only if lossless delivery is expected for this MRB. No need to enhance the NR UP spec.