3GPP TSG-RAN3 Meeting #115-e	R3-222030

E-meeting, 21 Jan – 3 Feb 2022

Title: 	Discussion on some remaining issues for alignment of sections within TR 37.817 (incl. TP)

Source: 	Deutsche Telekom

Agenda item:	18.2

Document Type:	Discussion

# 1. Introduction

The Rel-17 SI called “Study on enhancement for data collection for NR and ENDC” was agreed by RAN Plenary with SID in [1] and inputs were discussed in RAN3#110-e - RAN3#114bis-e meetings. Agreements achieved so far are covered in TR 37.817 [2].

This paper is addressing some minor corrections needed in Sec. 4 “General Framework” and alignments of the definitions given in Sec. 3.1 “Terms” with those used for the functional framework for RAN intelligence in Sec. 4.2. Furthermore, it includes a TP how to cover those aspects in TR 37.817.

# 2. Discussion

## 2.1 Correction of Sec. 4 “General Framework” in TR 37.817

At last RAN3#114bis-e meeting it was agreed to keep the Model Performance Feedback in the Functional Framework for RAN Intelligence and to remove the FFSs. During discussion on use cases, it was noted that the arrow titled by Model Performance Feedback in the different figures should be drawn with a dashed line to highlight that this is only optional and the content is out of RAN3 scope. This was also captured in following chairman’s note in the meeting minutes.

The rapporteur to clean the figure with dash line on Model Performance Feedback when merge the TP to TR

To be consistent within the TR we should also change the arrow in Fig. 4.2.1 describing the functional framework for RAN intelligence to a dashed line.

Proposal 1: To change the arrow for Model Performance Feedback in Figure 4.2-1 of current draft of TR 37.817 to a dashed line.

In the High-Level Principles in Sec. 4.1 there is the following bullet point listed:

- NG-RAN is prioritized; EN-DC is included in the scope. FFS on whether MR-DC should be down-prioritized.

As we are at the end of the SI phase, we should clarify this statement and remove the FFS. Our proposal is to rephrase the bullet point as:

- NG-RAN is prioritized; EN-DC and MR-DC are not excluded from the functional framework.

and to put it at the end of the bullet list.

Proposal 2: To change the wording for the bullet point “NG-RAN is prioritized; EN-DC and MR-DC are not excluded from the functional framework.” to “NG-RAN is prioritized; EN-DC and MR-DC are not excluded from the functional framework.” and put it at the end of the list.

The Editor’s note at the beginning of Sec. 4 of current draft TR 37.817 is no more needed and should be removed.

Proposal 3: To remove following editor’s note at the beginning of Sec. 4 of current draft of TR 37.817: “Editor Note: high level principles for RAN intelligence enabled by AI, the functional framework (e.g. the AI functionality and the input/output of the component for AI enabled optimization)”.

Related modifications of the figure and the text (covering also some minor editorial changes) for Sec. 4 of TR 37.817 can be found in the TP in Sec. 5 of present tdoc.

## 2.2 Alignment between definitions in “Terms” and in “Functional framework for RAN intelligence”

In Sec. 3.1 of draft TR 37.817 there is a list of terms which were entered at an early stage of the SI phase:

- Data collection: Data collected from the network nodes, management entity or UE, as a basis for ML model training, data analytics and inference.

- ML Model: A data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information, based on a set of inputs

- ML Training: An online or offline process to train an ML model by learning features and patterns that best present data and get the trained ML model for inference.

- ML Inference: A process of using a trained ML model to make a prediction or guide the decision based on collected data and ML model.

In the meantime, definitions of elements of the functional framework described in Sec. 4.2 were agreed which are partially overlapping with the terms in Sec. 3.1. Therefore, we propose to align those definitions with each other as follows:

- “ML” used in the 4 bullet points in Sec. 3.1 should be replaced by “AI/ML” to be consistent with the terminology applied in Sec. 4.2.

- For “ML model” it is stated that the output consists of predicted information only. According to current description the “ML inference” is using the model to make predictions or guide the decision, i.e., the output is more than just prediction results. Therefore, we would propose to extend the AI/ML Model description by “… predicted information and/or decision parameters, based on a set of inputs.”

Proposal 4: To change the description of the bullet list in Sec. 3.1 according to above proposed changes to align with the definitions given in Sec. 4.2 of current draft TR 37.817.

The Editor’s note at the end of Sec. 3.1 of current draft TR 37.817 is no more needed and should be removed.

Proposal 5: To remove following editor’s note at the end of Sec. 3.1 of current draft of TR 37.817: “Editor Note: Definition of each terminology might be updated to align with other working groups, in order to have common or unified definition on AI/ML related terminology.”.

Related modifications of the text (covering also some minor editorial changes) for Sec. 3.1 of TR 37.817 can be found in the TP in Sec. 5 of present tdoc.

# 3. Conclusion

Based on the discussion in this paper, we can summarize the proposals as listed in the following:

Proposal 1: To change the arrow for Model Performance Feedback in Figure 4.2-1 of current draft of TR 37.817 to a dashed line.

Proposal 2: To change the wording for the bullet point “NG-RAN is prioritized; EN-DC and MR-DC are not excluded from the functional framework.” to “NG-RAN is prioritized; EN-DC and MR-DC are not excluded from the functional framework.” and put it at the end of the list.

Proposal 3: To remove following editor’s note at the beginning of Sec. 4 of current draft of TR 37.817: “Editor Note: high level principles for RAN intelligence enabled by AI, the functional framework (e.g. the AI functionality and the input/output of the component for AI enabled optimization)”.

Proposal 4: To change the description of the bullet list in Sec. 3.1 according to above proposed changes to align with the definitions given in Sec. 4.2 of current draft TR 37.817.

Proposal 5: To remove following editor’s note at the end of Sec. 3.1 of current draft of TR 37.817: “Editor Note: Definition of each terminology might be updated to align with other working groups, in order to have common or unified definition on AI/ML related terminology.”.

A TP covering the proposed changes in draft TR 37.817 (covering also some minor editorial changes) is given in Sec. 5 of present tdoc.

# 4. Reference

- 3GPP RP-201620: SID “Study on enhancement for data collection for NR and ENDC”.

- 3GPP R3-221610: TR 37.817: Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on enhancement for Data Collection for NR and EN-DC; Rel-17, V1.2.0.

# 5. TP for TR 37.817

<skip unchanged parts>

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

- Data collection: Data collected from the network nodes, management entity or UE, as a basis for AI/ML model training, data analytics and inference.

- AI/ML Model: A data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information and/or decision parameters, based on a set of inputs.

- AI/ML Training: An online or offline process to train an AI/ML model by learning features and patterns that best present data and get the trained AI/ML model for inference.

- AI/ML Inference: A process of using a trained AI/ML model to make a prediction or guide the decision based on collected data and AI/ML model.

<skip unchanged parts>

# 4	General Framework

## 4.1	High-level Principles

The following high-level principles should be applied for AI-enabled RAN intelligence:

- The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope.

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs.

- The input/output and the location of the Model Training and Model Inference function should be studied case by case.

- The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope.

- The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope.

- Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases.

- The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm.

- The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g. via subscription), or nodes that take actions based on the output from Model Inference.

- An AI/ML model used in a Model Inference function has to be initially trained, validated and tested before deployment.

- A general framework and workflow for AI/ML optimization should be defined and captured in the TR. The generalized workflow should not prevent to “think beyond” the workflow if the use case requires so.

- User data privacy and anonymisation should be respected during AI/ML operation.

- NG-RAN is prioritized; EN-DC and MR-DC are not excluded from the functional framework.

- A general framework and workflow for AI/ML optimization should be defined and captured in the TR. The generalized workflow should not prevent to “think beyond” the workflow if the use case requires so.

- User data privacy and anonymisation should be respected during AI/ML operation.

## 4.2	Functional Framework

Figure 4.2-1: Functional Framework for RAN Intelligence

This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1. For the functions and data/information flows shown in the Figure 4.2-1, whether there is any standardization impact and what is the standardization impact are discussed in clause 5.

- Data Collection is a function that provides input data to Model Training and Model Inference functions. AI/ML algorithm specific data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) is not carried out in the Data Collection function.  
Examples of input data may include measurements from UEs or different network entities, feedback from Actor, output from an AI/ML model.

  - Training Data: Data needed as input for the AI/ML Model Training function.

  - Inference Data: Data needed as input for the AI/ML Model Inference function.

- Model Training is a function that performs the AI/ML model training, validation, and testing which may generate model performance metrics as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

  - Model Deployment/Update: Used to initially deploy a trained, validated, and tested AI/ML model to the Model Inference function or to deliver an updated model to the Model Inference function.

    - Note: Details of the Model Deployment/Update process as well as the use case specific AI/ML models transferred via this process are out of RAN3 Rel-17 study scope. The feasibility to single-vendor or multi-vendor environment has not been studied in RAN3 Rel-17 study.

- Model Inference is a function that provides AI/ML model inference output (e.g. predictions or decisions). The Model Inference function may provide model performance feedback to Model Training function when applicable. The Model Inference function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

  - Output: The inference output of the AI/ML model produced by a Model Inference function.

    - Note: Details of inference output are use case specific.

  - Model Performance Feedback:  It may be used for monitoring the performance of the AI/ML model.

    - Note: Details of the Model Performance Feedback process are out of RAN3 scope.

- Actor is a function that receives the output from the Model Inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

  - Feedback: Information that may be needed to derive training or inference data or performance feedback.

<skip unchanged parts>