3GPP TSG-RAN WG3 Meeting #115-e	R3-222036

# Online Meeting, 21st February– 3rd March 2022

Agenda Item:	30.2

Source:	Ericsson

Title: 	MUSIM reaming issues

Document for:	Discussion and Approval

# 1		Information

As in [1], there are some remaining issues in WI MUSIM, before it can be closed.

- “Paging Cause support indication” IE: naming: Using “Paging Cause Indication for Voice Service Supported”, which is aligned with SA2; and Coding: One code point with extension mark is agreeable. Further: Using “supported, …” which is aligned with SA2. - Paging Cause IE Coding. Choose from the below two Options: Option 1: “voice, …” Option 2: A new IE structure as in below: This IE indicates the paging cause for paging a UE. - New Cause value over NGAP “Release due to MUSIM”, discuss if needed / what is the usage.

This paper propose a way forward on the remaining issues..

# 2		Discussion

## 2.1	Paging Cause Indication for Voice Service Supported

When we introduce new features, it is in general a good practice to using the same termilogy as the other groups are using, such as SA2. The advantage is when people searching for the feature in different specification, it is much easier.

In TS 23.501, it says:

If a UE operating two or more USIMs, supports and intends to use one or more Multi-USIM features (see clause 5.38) in a PLMN for a USIM, it shall indicate in the UE 5GMM Core Network Capability for this USIM in this PLMN that it supports these one or more Multi-USIM features with the following indications: - Connection Release Supported. - Paging Cause Indication for Voice Service Supported. - Reject Paging Request Supported. - Paging Restriction Supported.

Proposal 1: RAN3 to agree to use “Paging Cause Indication for Voice Service Supported”

## 2.2	Paging Cause IE coding

Option 1: “voice, …”

Option 2: A new IE structure as in below: This IE indicates the paging cause for paging a UE.

The argumentation to introducing an IE struction is said to align with RAN2.

Our understanding is that RAN2 strcuture is indicating if NG-RAN node support the Rel 17 MUSIM or not. This should not require any input from AMF.

Option 1 is the solution we prefer.

Proposal 2: RAN3 to agree to endcode Pagining Cause IE as “voice, …”.

## 2.3	New cause value

There is a proposal to introduce a new cause value “Release due to MUSIM”. It is unclear how this cause value should be used/ can contribute.

Proposal 3: It is proposed that no new cause value is needed.

# 3		Proposal

Proposal 1: RAN3 to agree to use “Paging Cause Indication for Voice Service Supported”

Proposal 2: RAN3 to agree to endcode Pagining Cause IE as “voice, …”.

Proposal 3: It is proposed that no new cause value is needed.

[2] is submitted for XnAP.

# 4		Reference

[1]		R3-221146			Summary of offline discussion MUSIM

[2]		R3-22	2035			TP on Introduction of MultiSIM support over Xn, Ericsson