3GPP TSG-RAN WG3 #115-e	R3-222067

Electronic meeting, February 21st – March 3rd 2022

Agenda Item:	10.2.1.4

Source:	Ericsson

Title:	(TP for SON BL CR for TS 38.423, TS 38.413, TS 36.413) UE History Information for Secondary Node

Document for:	Discussion, Decision

# Introduction

In RAN3#114-bis-e, the following agreements and FFS were agreed:

Add a new IE, e.g. PSCell history information retrieve in the SN modification request message to indicate the retrieving of SN UHI.

Add optional UE history information from the UE IE in the SN addition request message.

Add optional UE history information from the UE IE in the SN modification request message.

Remove the Last Visited PSCell List IE in the XnAP and X2AP BLCRs.

Discuss the UHI transfer during CHO in Rel-18.

WA：Add a flag indicating subscription of PSCell changes in the SN addition request

No need to add a flag indicating subscription of PSCell changes in the SN modification request message

No need to add a “subscription stop” codepoint in the subscription indicator

WA：If MN subscribes to PSCell changes, SN shall send the full SN UHI during each PSCell change to the MN via the SN modification required message.

SN is responsible for collecting the SN UHI.

Whether MN can correlate MN and SN UHI only based on time stay in PSCell?

Whether to include Time spent without SCG and/or Time stamp?

Which option below could be pursued to achieve the subscription mechanism?

Option 1:Add a subscription indicator in the SN addition request message to indicate the subscription of PSCell changes. SN sends the full SN UHI to MN during each PSCell change.

Option 2: Use the existing Location Information at S-NODE reporting IE to indicate the the subscription of PSCell changes. SN informs MN with the new PSCell ID during each PSCell change.

To summarize the current landscape of UE history information discussions:

Usage of SN UHI for target node: (RAN3#113-e)

- SN node ping pong issue

- Assist target MN to select the appropriate SN

- Assist target MN in requirement of dual connectivity

Collection of UE history information:

- SN is responsible for collecting SN UHI. (RAN3#112-e WA).

Correlation of MN UE history information and SN UE history information:

- Correlation of MN UHI and SN UHI is beneficial (RAN3#109-e),

- Realized via two-dimensional structure for UHI (RAN3#112-e WA).

- Correlation is done at the MN (RAN3#114-e).

- Whether MN can correlate MN and SN UHI only based on time stay in PSCell? (RAN3#114-bis-e)

Transfer of correlated UE history information between MN and SN:

- SN UE history information is collected at SN and transferred to MN during Handover (RAN3#114-e).

- Correlated UE history information is sent from MN to SN (RAN3#114-e).

- Add a new IE, e.g. PSCell history information retrieve in the SN modification request message to indicate the retrieving of SN UHI. (RAN3#114-bis-e)

UE history information is transferred from SN and MN in the following messages: (RAN3#114-e).

- SN Change procedure (S-NODE CHANGE REQUIRED, SGNB CHANGE REQUIRED)

- SN Modification procedure

  - MN-initiated: S-NODE MODIFICATION REQUEST ACKNOWLEDGE, SGNB MODIFICATION REQUEST ACKNOWLEDGE

  - SN-initiated: S-NODE MODIFICATION REQUIRED and SGNB MODIFICATION REQUIRED messages

- SN Release procedure

  - MN-initiated: S-NODE RELEASE REQUEST ACKNOWLEDGE, SGNB RELEASE REQUEST ACKNOWLEDGE

  - SN-initiated: S-NODE RELEASE REQUIRED, SGNB RELEASE REQUIRED

Content of SN UE history information:

- includes PSCell list, time UE stayed in cell. (RAN3#109-e).

- does not include HO clause. (RAN3#110-e).

- No enough benefit to introduce Cell type. (RAN3#113-e)

- Including originating node of PSCell change is FFS. (RAN3#112-e WA)

- Whether to use timestamp or time spent without SCG (RAN3#114-e) (RAN3#114-bis-e)

MN subscription to PSCell changes:

- WA：Add a flag indicating subscription of PSCell changes in the SN addition request (RAN3#114-bis-e)

- No need to add a flag indicating subscription of PSCell changes in the SN modification request message (RAN3#114-bis-e)

- No need to add a “subscription stop” codepoint in the subscription indicator (RAN3#114-bis-e)

- WA：If MN subscribes to PSCell changes, SN shall send the full SN UHI during each PSCell change to the MN via the SN modification required message. (RAN3#114-bis-e)

- Which option below could be pursued to achieve the subscription mechanism? (RAN3#114-bis-e)

- Option 1:Add a subscription indicator in the SN addition request message to indicate the subscription of PSCell changes. SN sends the full SN UHI to MN during each PSCell change. (RAN3#114-bis-e)

- Option 2: Use the existing Location Information at S-NODE reporting IE to indicate the the subscription of PSCell changes. SN informs MN with the new PSCell ID during each PSCell change. (RAN3#114-bis-e)

UE history information from the UE (MHI):

- Add optional UE history information from the UE IE in the SN addition request message. (RAN3#114-bis-e)

- Add optional UE history information from the UE IE in the SN modification request message. (RAN3#114-bis-e)

Rel-18 topics:

- Discuss the UHI transfer during CHO in Rel-18. (RAN3#114-bis-e)

In this contribution, we discuss the open issues and give proposals on the way forward. A complete solution considering the proposals in the paper are also included before the conclusion for clarity.

# Discussion

## Subscription of SN UE history information

During RAN3#115-e meeting, companies discussed two options to enable MN subscription to PSCell changes from SN.

- Through the use of existing Location Information at S-NODE Reporting and Location Information at S-NODE IE. SN informs MN the new PSCell ID during each PSCell change.

- Add a subscription indicator in SN ADDITION message to indicate subscription. The SN sends SN UHI during each PSCell change

As a result, the following WA was taken:

WA：Add a flag indicating subscription of PSCell changes in the SN addition request

The use of Location Information IE is not suitable for the purpose of subscription as it is a CN feature. The procedural text for SNODE ADDITION says the following

”If the Location Information at S-NODE Reporting IE set to "pscell" is included in the S-NODE ADDITION REQUEST, the S-NG-RAN node shall, start providing information about the current location of the UE. If the Location Information at S-NODE IE is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE, the M-NG-RAN node shall store the included information so that it may be transferred towards the AMF.”

This clearly indicates that the MN is expected to forward the information to the AMF. This is however a not feasible as the UHI should not be transmitted out of the RAN. SA3 has used the same reporting procedure for legal interception and there are strong constraints on the use of feature.

The existing LOCATION INFORMATION IE is not suitable for subscription as it is a CN feature with strong constraints of usage.

RAN3 also acknowledged that having the trigger in the modification message is not needed, while for location information we have it.

As there is no technical issue with the WA taken last meeting, we propose to convert it into an agreement

- Add a flag indicating subscription of PSCell changes to the SN addition request sent from the master node to the secondary node.

The information sent from the secondary node to the master node in case of PSCell changes should include the Global Cell ID of the new PSCell, and other feasible information of the previous PSCell. Three alternative solutions for how to send this were considered last meeting:

- Introduce a new optional IE PSCell change information containing Global Cell ID of the new PSCell and other available SN UHI content in the SN MODIFICATION REQUIRED message.

- Introduce a dedicated, mandatory class 2 procedure message PSCELL CHANGE INFORMATION including the Global Cell ID of the new PSCell.

- Send the full SN UHI during each PSCell change from the SN to the MN. This can be sent through the SN MODIFICATION REQUIRED message which has already been agreed to contain SN UHI

As a result, the following WA was taken:

WA：If MN subscribes to PSCell changes, SN shall send the full SN UHI during each PSCell change to the MN via the SN modification required message

It is clear that sending the full SN UHI has a low specification impact (it is already present in S-NODE MODIFICATION REQUIRED message) and is also future proof, allowing accurate SN UHI transfer even in scenarios with future IE additions to the SN UHI

Sending the full SN UHI has low specification impact and is also future proof, allowing accurate SN UHI transfer even in scenarios with future IE additions to the SN UHI.

As there is no technical issue with the WA taken last meeting, we propose to convert it into an agreement.

If MN subscribes to PSCell changes, SN shall send the full SN UHI during each PSCell change to the MN via the SN modification required message

## Correlated UE history information between MN and SN

According to the current discussions, the MN collects PCell related UHI (legacy UHI) and the SN collects PSCell related UHI (SN UHI). The SN sends SN UHI to the MN for correlation. During correlation, the MN puts each PSCell entry under the appropriate PCell entry during which both cells were active. This can be accomplished through the use of the IE ”time UE stayed in cell” [4] present in legacy UHI and (expected to be agreed for) the SN UHI.

The IE ”time UE stayed in cell” can take values from 0 to 4095, upon exceeding this time it is set to 4095. This can cause problems with correlation when this IE has overflowed as the MN cannot determine the exact time and thus cannot perform correlation. Consider the figure below that illustrates the different Pcells and PSCells the UE connects from 10:00 to 13:00. The top and bottom rows correpond to the current PCell and PSCell repectively.

PSCell A

PCell A

PCell B

10:00:00

11:00:00

12:00:00

13:00:00

PCell C

PCell B

PSCell B

PSCell C

PSCell A

Consider the scenario where the UE has connected to a series of PCells for the following times:

- PCell A: From time 10:00:00 to 10:50:00

- PCell B: From time 10:50:00 to 12:00:00

- PCell C: From time 12:00:00 to 12:40:00

- PCell B: From time 12:40:00 to time of correlation (12:50:00)

During the same time, the UE is connected to the following PSCells:

- PSCell A: From time 10:00:00 to 10:02:00

- PSCell B: From time 10:02:00 to 10:42:00

- PSCell C: From time 10:42:00 to 12:10:00

- PSCell A: From time 12:10:00 to SN release at time 12:40:00

The (simplified) PCell UHI (left) at the MN and (simplified) PSCell UHI (right) look as shown below,

| PCell ID | Time UE stayed in PCell(s) |
| --- | --- |
| PCell A | 3000 |
| PCell B | 4095* |
| PCell C | 2400 |
| PCell B | 600 |

| PSCell ID | Time UE stayed in PSCell(s) |
| --- | --- |
| PSCell A | 100 |
| PSCell B | 2000 |
| PSCell C | 4095* |
| PSCell A | 1200 |

The entries for PCell B and PSCell C exceed the limits and thus make accurate correlation of the two UHIs impossible.

Correlation of PCell and PSCell entries is not possible using the current standardized IEs in the legacy UHI

### Correlation of PCell and PSCell entries using optional timestamp

Two options were discussed during RAN3#114-bis-e for correlation of PCell and PSCell information in addition with the already agreed ”time UE stayed in a cell”

- Optional Time Stamp of PSCell successful creation

- Time spent without SCG

The below example is from email discussions from RAN3#114-bis-e which shows a scenario where a single PSCell exceeds the time stayed.

According to the discussions, it is possible to perform correlation and get the actual time stayed in PSCell 2 through the use of Time Stamp 3(time of successful connection of PSCell 3) and the ”time stayed without SCG” recorded at the MN.

However consider the scenario, where 2 consecutive PSCells have exceeded the ¨time stayed in cell” IE as shown below.

In such scenarios with no subscription, the exact time t4 cannot be determined and when there is inter-MN handovers it is not possible to correlate UHI. In such scenarios, an optional time stamp becomes vital.It is not possible to accurately correlate UHI using only ”time without SCG” when 2 consecutive PSCells have exceeded their threshold and no subscription mechanism exists.

The above problem can be solved through the use of an optional timestamp in the UHI that is only used when the IEs ”time UE stayed in the cell” and ”time UE stayed in secondary cell” exceed their limits. In the above example, the UHIs will contain the following information,

| PCell ID | Time UE stayed in PCell(s) | Optional time stamp |
| --- | --- | --- |
| PCell A | 3000 |  |
| PCell B | 4095* | 10:50:00 |
| PCell C | 2400 |  |
| PCell B | 600 |  |

| PSCell ID | Time UE stayed in PSCell(s) | Optional time stamp |
| --- | --- | --- |
| PSCell A | 100 |  |
| PSCell B | 2000 |  |
| PSCell C | 4095* | 10:42:00 |
| PSCell A | 1200 |  |

The timestamps can then be used to perform correlation. This would however require the clocks in MN and SN to be synchronized. This can be avoided by also sending the timestamp of SN release. Then MN can then use  the timestamps in the SN UHI together with SN release time and correlate it with MN UHI and the current time at MN.

Thus we propose

- Introduce an optional timestamp indicating the time of successful PSCell connection in UE History Information that is used when the IE ”time UE stayed in cell” exceeds the limit.

- Introduce an optional timestamp indicating SN release time in S-NODE RELEASE REQUEST ACKNOWLEDGE and S-NODE RELEASE REQUIRED messages to facilitate UHI correlation.

### Information content of SN UE History Information to facilitate correlation

UE history information should include the amount of time spent with no PSCell to correctly interpret the correlated information. This information can be used to enhance understanding of previous UE choices in terms of DC. For example, A UE may not be connected to SCGs for a certain period as it is moving at a high speed. A node can then use this information to deduce that the UE is travelling too fast to configure a reliable DC connection. The MN determines the feasibility of initiating DC for a UE. It can decide to initiate SN addition due to obtained coverage, or due to additional bandwidth requirements of the UE. Thus we propose the following

- Add the following IEs to the (correlated) UE history information under relevant PCell entry.

  - Time spent with no PSCell.

  - Reason for no PSCell.

## UE history information during SCG activation/deactivation

- There are currently some discussions in 3GPP on the possibility of activation/deactivation of SCG to save power when the UE does not need additional bandwidth through additional SCells of the the SCG. The activation/deactivation request can come from the MN or the UE. This network state has to be accurately captured by the UHI as to not give wrong information about UE mobility and usage. This information is important for SN mobility optimization, as it is related to the “real” usage of the SCG-leg. This can be done e.g. by collecting information about the duration for which the SCG was deactivated.

- We recognise there is not enough time in the current meeting to discuss in detail about required enhancements and propose to discuss the issue in release 18.

Discuss impact of SCG activation/deactivation on UE history information in release 18.

## Time with no PSCell

The UE history information should also include the amount of time spent with no PSCell to correctly interpret the correlated information. Do note that this is however not used for correlation as it is not suitable (see observation above).

This information can be used to enhance understanding of previous UE choices in terms of DC. For example, A UE may not be connected to SCGs for a certain period as it is moving at a high speed. A node can then use this information to deduce that the UE is travelling too fast to configure a reliable DC connection.

- Time spent with no PSCell should be included in the UE history information.

In addition, we need to add a reason for why SN was not configured. For example, the MN might not configure SN for a UE as there might not be any SN coverage. The MN might also not configure SN for a UE in scenarios where the UE does not require the high data rate to achieve better battery capacity even though SN coverage existed. Thus we propose

Reason for no PSCell should be added to UE history information

# Conclusion

In the above we have made the following observations:

Observation 1	The existing LOCATION INFORMATION IE is not suitable for subscription as it is a CN feature with strong constraints of usage.

Observation 2	Sending the full SN UHI has low specification impact and is also future proof, allowing accurate SN UHI transfer even in scenarios with future IE additions to the SN UHI.

Observation 3	Correlation of PCell and PSCell entries is not possible using the current standardized IEs in the legacy UHI

Observation 4	It is not possible to accurately correlate UHI using only ”time without SCG” when 2 consecutive PSCells have exceeded their threshold and no subscription mechanism exists.

We propose the following:

Proposal 1	Add a flag indicating subscription of PSCell changes to the SN addition request sent from the master node to the secondary node.

Proposal 2	If MN subscribes to PSCell changes, SN shall send the full SN UHI during each PSCell change to the MN via the SN modification required message

Proposal 3	Introduce an optional timestamp indicating the time of successful PSCell connection in UE History Information that is used when the IE ”time UE stayed in cell” exceeds the limit.

Proposal 4	Introduce an optional timestamp indicating SN release time in S-NODE RELEASE REQUEST ACKNOWLEDGE and S-NODE RELEASE REQUIRED messages to facilitate UHI correlation.

Proposal 5	Add the following IEs to the (correlated) UE history information under relevant PCell entry.

a.	Time spent with no PSCell.

b.	Reason for no PSCell.

Proposal 6	Discuss impact of SCG activation/deactivation on UE history information in release 18.

Proposal 7	Time spent with no PSCell should be included in the UE history information.

Proposal 8	Reason for no PSCell should be added to UE history information

# Appendix A: Text Proposal to 38.423

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

-------- skip unchanged part ----------

For each QoS flow which has been successfully established in the S-NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and, if supported, use it for RAN part delay reporting. In case such a QoS flow is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring. If the QoS Monitoring Reporting Frequency IE is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it for RAN part delay reporting.

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted List IE in the PDU Session Resource Setup Response Info – MN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

Upon reception of the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, start collecting the SCG information and continue for as long as the UE stays in one of its cells.

If the UE History Information from the UE IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information.

If the S-NODE ADDITION REQUEST message contains the PSCell change subscription IE, the S-NG-RAN node shall, if supported, signal the latest SCG UE History Information upon each PSCell change, to the M-NG-RAN node, using the S-NG-RAN node initiated S-NG-RAN node Modification procedure.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits a modification of the UE context requiring the M-NG-RAN node to report about the success of the RRC connection reconfiguration procedure, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the M-NG-RAN node. The reception of the S-NG-RAN node RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall.

-- TEXT OMITTED –

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell change subscription | O |  | ENUMERATED (start, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

-- TEXT OMITTED –

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

-- TEXT OMITTED –

UESpecificDRX,

SuccessfulHOReportInformation,

PSCellHistoryInformationRetrieve,

Coverage-Modification-List,

SCGFailureReportContainer,

PSCellChangeSubscribe

FROM XnAP-IEs

-- TEXT OMITTED –

id-PDUSessionExpectedUEActivityBehaviour,

id-SuccessfulHOReportInformation,

id-PSCellHistoryInformationRetrieve,

id-NG-RANnode1SSBOffsets,

id-NG-RANnode2SSBProposedOffsets,

id-NG-RANnode2SSBOffsetsModificationRange,

id-Coverage-Modification-List,

id-PSCellCGI,

id-FailedPSCellCGI,

id-SCGFailureReportContainer,

id-PSCellChangeSubscribe

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows

FROM XnAP-Constants;

-- TEXT OMITTED –

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- The IE shall be present if there is at least one  PDUSessionResourceSetupInfo-SNterminated included --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional}|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional}|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional}|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional}|

{ ID id-TraceActivation						CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional}|

{ ID id-UERadioCapabilityID					CRITICALITY reject		TYPE UERadioCapabilityID							PRESENCE optional}|

{ ID id-UEHistoryInformation				CRITICALITY ignore		TYPE UEHistoryInformation						PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE	CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-PSCellChangeSubscribe	CRITICALITY ignore	TYPE PSCellChangeSubscribe				PRESENCE optional },

...

}

-- TEXT OMITTED –

ProtectedE-UTRAFootprintTimePattern ::= SEQUENCE {

protectedFootprintTimeperiodicity			INTEGER (1..320, ...),

protectedFootrpintStartTime					INTEGER (1..20, ...),

iE-Extensions					ProtocolExtensionContainer { {ProtectedE-UTRAFootprintTimePattern-ExtIEs} } 	OPTIONAL,

...

}

ProtectedE-UTRAFootprintTimePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PSCellChangeSubscribe ::= ENUMERATED {start, ...}

#### -- Q

-- TEXT OMITTED –

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-SuccessfulHOReportInformation																	ProtocolIE-ID ::= xxx

id-SliceRadioResourceStatus-List																	ProtocolIE-ID ::= yyy

id-CompositeAvailableCapacitySupplementaryUplink													ProtocolIE-ID ::= zzz

id-SCGUEHistoryInformation																	ProtocolIE-ID ::= www

id-NG-RANnode1SSBOffsets																			ProtocolIE-ID ::= x01

id-NG-RANnode2SSBProposedOffsets																	ProtocolIE-ID ::= x02

id-NG-RANnode2SSBOffsetModificationRange															ProtocolIE-ID ::= x03

id-Coverage-Modification-List																		ProtocolIE-ID ::= xxx

id-NR-U-Channel-List					                                                          ProtocolIE-ID ::= xx1

id-PSCellCGI						                                                              ProtocolIE-ID ::= xxx

id-FailedPSCellCGI				                                                                  ProtocolIE-ID ::= yyy

id-SCGFailureReportContainer		                                                              ProtocolIE-ID ::= zzz

id-PSCellChangeSubscribe																			ProtocolIE-ID ::= aaa

END

-- ASN1STOP

-- END OF CHANGES –

# Appendix B: Text Proposal to 38.413

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |  |  |  |  |
| Cell Type | M |  | 9.3.1.98 |  |  |  |  |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |  |  |  |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |  |  |  |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. |  |  |  |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES |  | ignore |  |
| >Last Visited PSCell Information | M |  | 9.2.3.X | The PSCell related information. | YES |  | ignore |  |
| Time stamp | O |  | OCTET STRING (SIZE(4)) | This time stamp is used in case the IE ”Time UE Stayed in cell” is set to the maximum. Used for correlation of UE History Information. Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 |  |  |

-- TEXT OMITTED –

#### 9.2.3.X1	Last Visited NG-RAN PSCell Information

The Last Visited NG-RAN PSCell Information contains information on the PSCell used and the time the UE accessed the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | M |  | NR CGI 9.3.1.7 |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |
| Time stamp | O |  | OCTET STRING (SIZE(4)) | This time stamp is used in case the IE ”Time UE Stayed in cell” is set to the maximum. Used for correlation of UE History Information. Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 |

#### 9.2.3.X2	Last Visited E-UTRAN PSCell Information

The Last Visited NG-RAN PSCell Information contains information on the PSCell used and the time the UE accessed the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | M |  | E-UTRA CGI 9.3.1.9 |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. |
| Time stamp | O |  | OCTET STRING (SIZE(4)) | This time stamp is used in case the IE ”Time UE Stayed in cell” is set to the maximum. Used for correlation of UE History Information. Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 |

-- TEXT OMITTED –

NG-RAN-LastVisitedPSCell ::= SEQUENCE {

nGRAN-PSCell-CGI 		NGRAN-CGI,

timeUEStayedInCell 	TimeUEStayedInCell,

timestamp			OCTET STRING (SIZE(4))			OPTIONAL,

...

}

NG-RAN-LastVisitedPSCell-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-LastVisitedPSCell  ::= SEQUENCE {

eUTRA-PSCell-CGI 		EUTRA-CGI,

timeUEStayedInCell 	TimeUEStayedInCell,

timestamp			OCTET STRING (SIZE(4))			OPTIONAL,

...

}

EUTRAN-LastVisitedPSCell-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- END OF CHANGES –

# Appendix  C: Text Proposal to 36.413

#### 9.2.1.43a	Last Visited E-UTRAN Cell Information

The Last Visited E-UTRAN Cell Information contains information about a cell that is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  | - |  |
| Cell Type | M |  | 9.2.1.66 |  | - |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the cell in seconds. If the UE stays in a cell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. | YES | ignore |
| HO Cause Value | O |  | 9.2.1.3 | The cause for the handover from the E-UTRAN cell. | YES | ignore |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.2.1.X | The PSCell related information. | YES | ignore |
| Time stamp | O |  | OCTET STRING (SIZE(4)) | This time stamp is used in case the IE ”Time UE Stayed in cell” is set to the maximum. Used for correlation of UE History Information. Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is FFS. |

#### 9.2.1.X	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell Information | M |  | 9.2.1.141 |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |
| Time stamp | O |  | OCTET STRING (SIZE(4)) | This time stamp is used in case the IE ”Time UE Stayed in cell” is set to the maximum. Used for correlation of UE History Information. Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 |

-- TEXT OMITTED –

LastVisitedPSCellInformation	:: SEQUENCE {

pSCellInformation							PSCellInformation,

timeStayedInCell		INTEGER (0..40950),

timestamp			OCTET STRING (SIZE(4))			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- END OF CHANGES –