3GPP TSG-RAN3#115-e	R3-222073

Online, 21 February – 3 March 2022

Online

Agenda Item:	10.2.6

Source:	Ericsson

Title:	MRO for CHO and DAPS

Document for:	Discussion and decision

# Introduction

In RAN#86 the new WID on enhancement of data collection for SON/MDT in NR was approved. In the following we will discuss the objectives of the WID and in particular focus on Conditional handover (CHO), bringing forward our proposals on topics to be studied, specifically in connection to failure cases and message forwarding.

# Discussion

## CHO-related open issues

Considering agreements from RAN3 meetings (114e, 114bis-e), there are several CHO-related open issues that should be addressed.

The following open issues are to be continued in the next meeting:

- which network-based solution is adopted, e.g. Option a-1/a-2/b/c, or combination of at least one of them;

- whether to introduce a new initiating condition for CHO recovery procedure in FAILURE INDICATION message;

- whether the FAILURE INDICATION message may be initiated without RLF report for CHO, if yes, whether to include an explicit CHO recovery cell ID in FAILURE INDICATION message and whether to include an explicit CHO recovery Cell CGI in HANDOVER REPORT message, in case of without RLF Report.

### Network-based solution

For the failure case when RLF occurs in the target cell after completing CHO, there is a need for network-based solution in order to get the list of CHO candidates.

Following options are considered in RAN3:

Option a: Derive candidate cell list and CHO execution condition(s) based on Mobility Information.

- Option a-1: Source node transmits the Mobility Information to the target node when CHO is completed, i.e. in the SN STATUS TRANSFER message, and the target node sends the Mobility Information back to the source node via HANDOVER REPORT message.

- Option a-2: Source node transmits the Mobility Information to each candidate target node in the HO request message, and the target node sends the Mobility Information back to the source node via HANDOVER REPORT message.

- Option a-3: Including the Mobility Information in the UE RLF-report. RAN3 asks RAN2 to consider feasibility of adding the Mobility Information to the CHO configuration.

Option b: Source node sends candidate cell list and CHO execution condition(s) to the target node after receiving Handover Success message, e.g. in SN Status Transfer message, and then the target transmits the info back to the source node in HANDOVER REPORT message.

Option c: Source node stores the CHO related configuration.

The network solution has been agreed based on the following scenario.

for the case that a RLF occurred in CHO target cell after completing handover

This is not the most common scenario. Therefore, we prefer to limit the signaling impact and to keep as much as possible the complexity in the node which needs to optimize the CHO configuration (i.e. the source node). Solutions a and b all have additional signaling and impact the target node for all Conditional Handovers, including the ones which will not lead to RLF. Option c does not create additional signaling for all the Conditional Handovers, and has a straightforward implementation, which only impact the source node.

- RAN3 to agree on option c: Source node stores the CHO related configuration

- Keeping the UE context in the source after successful CHO is a matter of implementation. From a memory point of view, it is beneficial to keep it for a time as short as possible. Some simple optimizations could be foreseen e.g.:

- During the retrieve UE Context procedure, if the UE’s time of stay in the target node is less than a threshold, the target candidate node will inform the reestablishment node that the UE Context is for CHO (using RETRIEVE UE CONTEXT RESPONSE message). The reestablishment node should then try to fetch the RLF Report as soon as possible and send it to the source node hosting the source cell of the last successfully executed handover.

- While receiving the RETRIEVE UE CONTEXT REQUEST message from the reestablishment node, and if the UE’s time of stay in the target node is less than a threshold, the target candidate node understands that an RLF was declared by the UE just after CHO. Therefore, it can warn the source node that an RLF has occurred, so the source node keeps the UE context a bit longer (i.e. until receiving the RLF report).

- These optimizations can both be introduced to lower the time the source keep the UE context in memory. The extra signaling will be used only if an RLF occurred in CHO target cell after completing handover and therefore does not bring extra complexity in most of the cases.

- Add a CHO flag in the RETRIEVE UE CONTEXT RESPONSE message

- Introduce a new message from target candidate to source node indicating that RLF Report is on its way

### Failure Indication message enhancements

There is also an ongoing discussion on initiating conditions for FAILURE INDICATION for CHO and whether to include an explicit CHO recovery cell ID in FAILURE INDICATION message.

It seems that the need of introducing the CHO Recovery Cell ID is for the case where the RLF report is not received. But in that case, it is unclear what are the possible optimizations that the source node could do, with only the CHO Recovery Cell ID and no other information. Also, if no additional IE is necessary, there is little benefit of changing or introducing new initiating conditions for FAILURE INDICATION.

The benefits of changing or introducing new initiating conditions for FAILURE INDICATION in case of CHO recovery are unclear, therefore, it brings little value for CHO optimization

- There is no need to change or introduce new initiating conditions for FAILURE INDICATION in case of CHO recovery

- There is no need to introduce the CHO Cell Recovery ID

# Conclusion

Based on the discussion, we propose the following:

Proposal 1	RAN3 to agree on network-based solution (Option c: Source node stores the CHO related configuration)

- Proposal 2	Add a CHO flag in the RETRIEVE UE CONTEXT RESPONSE message

- Proposal 3	Introduce a new message from target candidate to source node indicating that RLF Report is on its way

- The benefits of changing or introducing new initiating conditions for FAILURE INDICATION in case of CHO recovery are unclear, therefore, it brings little value for CHO optimization

Proposal 4	There is no need to change or introduce new initiating conditions for FAILURE INDICATION in case of CHO recovery

Proposal 5	There is no need to introduce the CHO Cell Recovery ID