3GPP TSG-RAN WG3 Meeting #115-e	R3-222120

E-meeting, 21 February – 3 March 2022

Agenda item:	18.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TR 37.817) AI/ML Framework final remarks

Document for:	Text Proposal

# 1	Introduction

In the last meeting, RAN3 agreed the last remaining FFS point of the AI/ML Framework by agreeing to have Model Performance Feedback sent from Model Inference to Model Training and by redefining its meaning. In this contribution, we discuss one last aspect related to the Model Performance Feedback arrow. We also propose to re-define the scope of the study to reflect current agreements with respect to DC aspects.

# 2	Discussion

Model Performance Feedback arrow is an important arrow that visually and conceptually completes the ML Functional Framework in Figure 1.

Figure 1 Functional Framework as captured in TR 37.817.

It was captured in the TR that Model Performance Feedback may be used for monitoring the performance of the AI/ML Model. However, there is not a unique way of monitoring Model Performance, as it is an implementation specific aspect. It depends on the details of an ML Model and the availability of ground truth information, which may not be available at all times if for example only delayed ground truth can be obtained.

Observation 1: Model Performance Feedback may not be available at all times.

In order for Model Inference to obtain Model Performance Feedback to be sent to Model Training, it needs to have access to the ground truth information. This needs to be reflected in the Input Data to Model Inference.

Proposal 1: Inference Data comprises necessary information for Model Inference to calculate Model Performance Feedback in addition to Output.

In addition, according to our agreement from the last meeting, performance monitoring may be supported also in Model Training in which case Model Performance Feedback arrow from Model Inference to Model Training may not be needed. Besides, in cases where ground truth information cannot be obtained Model Performance Feedback may be triggered periodically at Model Training. To more accurately reflect this we propose to show Model Performance Feedback with a dashed arrow in the ML Framework.

Proposal 2: Make Model Performance Feedback a dashed arrow in the ML Framework to capture the case it is optionally sent from Model Inference to Model Training, e.g., when performance monitoring of the trained and deployed AI/ML Model is supported in Model Training.

As another topic, in the high level principles of TR 37.817 we have captured the following regarding the scope of the study:

- NG-RAN is prioritized; EN-DC is included in the scope. FFS on whether MR-DC should be down-prioritized

However, during the study neither EN-DC or MR-DC aspects were discussed. Still, we propose that EN-DC could be included in the scope but down-prioritize MR-DC.

Proposal 3: Down-prioritize MR-DC in the scope of Rel.17/Rel.18 work.

# 3	Conclusion

We make the following observations and proposals:

Observation 1: Model Performance Feedback may not be available at all times.

Proposal 1: Inference Data comprises necessary information for Model Inference to calculate Model Performance Feedback in addition to Output.

Proposal 2: Make Model Performance Feedback a dashed arrow in the ML Framework to capture the case it is optionally sent from Model Inference to Model Training, e.g., when performance monitoring of the trained and deployed AI/ML Model is supported in Model Training.

Proposal 3: Down-prioritize MR-DC in the scope of Rel.17/Rel.18 work.

# Annex - TP for TR 37.817

# 4	General Framework

Editor Note: high level principles for RAN intelligence enabled by AI, the functional framework (e.g. the AI functionality and the input/output of the component for AI enabled optimization)

## 4.1	High-level Principles

The following high-level principles should be applied for AI-enabled RAN intelligence:

- The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope.

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs.

- The input/output and the location of the Model Training and Model Inference function should be studied case by case.

- The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope.

- The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope.

- Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases.

- The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm.

- The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g. via subscription), or nodes that take actions based on the output from Model Inference.

- An AI/ML model used in a Model Inference function has to be initially trained, validated and tested before deployment.

- NG-RAN is prioritized; EN-DC is included in the scope. MR-DC is down-prioritized.

- A general framework and workflow for AI/ML optimization should be defined and captured in the TR. The generalized workflow should not prevent to “think beyond” the workflow if the use case requires so.

- User data privacy and anonymisation should be respected during AI/ML operation.

## 4.2	Functional Framework

Figure 4.2-1: Functional Framework for RAN Intelligence

This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1. For the functions and data/information flows shown in the Figure 4.2-1, whether there is any standardization impact and what is the standardization impact are discussed in clause 5.

- Data Collection is a function that provides input data to Model training and Model inference functions. AI/ML algorithm specific data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) is not carried out in the Data Collection function.  
Examples of input data may include measurements from UEs or different network entities, feedback from Actor, output from an AI/ML model.

  - Training Data: Data needed as input for the AI/ML Model Training function.

  - Inference Data: Data needed  as input for the AI/ML Model Inference function to provide Output and Model Performance Feedback.

- Model Training is a function that performs the ML model training, validation, and testing which may generate model performance metrics as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

  - Model Deployment/Update: Used to initially deploy a trained, validated, and tested AI/ML model to the Model Inference function or to deliver an updated model to the Model Inference function.

    - Note: Details of the Model Deployment/Update process as well as the use case specific AI/ML models transferred via this process are out of RAN3 Rel-17 study scope. The feasibility to single-vendor or multi-vendor environment has not been studied in RAN3 Rel-17 study.

- Model Inference is a function that provides AI/ML model inference output (e.g. predictions or decisions) . Model Inference function may provide model performance feedback to Model Training function when applicable. The Model inference function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

  - Output: The inference output of the AI/ML model produced by a Model Inference function.

    - Note: Details of inference output are use case specific.

  - Model Performance Feedback:  It may be used for monitoring the performance of the AI/ML model, when available.

    - Note: Details of the Model Performance Feedback process are out of RAN3 scope.

- Actor is a function that receives the output from the Model inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

  - Feedback: Information that may be needed to derive training or inference data or performance feedback.