3GPP TSG-RAN WG3 Meeting #115-e	R3-222121

E-meeting, 21 February – 3 March 2022

Agenda item:	18.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TR 37.817) A few final aspects on AI/ML Energy Saving

Document for:	Text Proposal

# 1	Introduction

In this contribution, we address the last two remaining open aspects captured in AI/ML Energy Saving, namely:

FFS Accept/reject of offloading plan to transfer a certain number of UEs to a neighboring RAN node for energy saving reasons (from neighboring NG-RAN node) as input for AI/ML based network energy saving use case.

FFS Validity time of the predicted energy saving decisions to be considered as output for AI/ML based network energy saving use case.

We also discuss a minor alignment issue in the flowcharts with respect to AI/ML Energy Saving and the other use cases related to step 0.

# 2	Energy Saving Decision Making

In current network implementations, a gNB has the freedom to independently switch-off its cells when it considers that this is the best decision for the network energy efficiency. The gNB would typically take such decision after evaluating resource status information from itself and from its neighbours and determining that a neighbour can support the additional load after a cell switch-off decision.

Observation 1: Currently energy saving decisions are taken independently by a network node that decides to switch-off its cell(s) once it has evaluated that this is to improve energy efficiency.

The introduction of AI/ML in the network can introduce intelligence into the energy saving decisions at a node by utilizing a larger amount of information through measurements and predictions of measurements both at the local node, making the switching-off decisions, as well as at the neighbouring node(s) that undertake the offloaded traffic. For example, input information to the AI/ML energy saving functionality can include current and predicted resource status, as well as current and predicted energy efficiency from the local and neighbouring nodes. In addition, a neighbour’s predicted energy state can be provided in the input. Having access to all this information, a capacity cell is enabled to make better guesses on the expected load at the neighbour at the time of cell switch-off to avoid neighbour overloading.

Observation 2: Introduction of AI/ML at a capacity cell can assist the latter in taking more intelligent guesses in its energy saving/switch-off decisions.

Observation 3: Current and predicted resource status information from the local node and from the neighbours enables a capacity cell to predict whether an offloading decision is feasible at a given candidate target.

Despite the fact that introduction of AI/ML is a powerful tool, energy saving decisions are still made locally and in a distributed way in a RAN architecture. This means that it is always possible that the local best decision to switch-off certain cells is not the best energy saving strategy for the overall network energy efficiency. As a possible solution it was discussed that a gNB can indicate its planned energy saving strategies to another gNB under the assumption that the second gNB can reject the offloading plan from the first gNB and therefore in this way allowing two gNBs to have a negotiation with respect to their energy saving actions. However, this problem cannot still be resolved through local negotiations with neighbouring nodes.

Observation 4: The problem that energy saving decisions at a local node may not be the best for the overall network energy efficiency cannot be addressed using negotiations with neighbouring nodes as long as the solution is distributed.

Towards this end, to achieve network-wide energy efficiency, support and information from OAM is necessary. An AI/ML solution in the OAM can further provide better support and information at the local node with a more global view on the system energy efficiency e.g., by prohibiting cell switching-off if this decision doesn’t seem to provide gains system-wide.

Observation 5: Network-wide energy efficiency can be achieved through the support of OAM energy saving decisions.

Proposal 1: A new procedure where an offloading plan to transfer a certain number of UEs to a neighboring RAN node for energy saving reasons can be accepted or rejected from a neighbor is not beneficial.

# 3	Validity Time of Predictions

Another aspect that has been discussed in several meetings is that of validity time for predictions. Even though we have discussed validity time over several meetings, we haven’t captured its definition in the TR. Validity time seems to be a model specific parameter related to how long a prediction is valid. How it can be defined exactly is unclear:

- Is validity time a time instant in the horizon that a prediction is expected to be valid?

- Is validity time a time period/window inside which a prediction is valid?

Different definitions of this time will affect the result of the prediction since the statistics of the predicted result depend on it.

Observation 6: There doesn’t seem to exist a unique definition of what validity time means.

The problem is essentially to identify Model output validity. Model output validity though may not necessarily depend on time, but may depend on other information as well, e.g., related to the area where the prediction is made.

Observation 7: Model output validity may depend on information other than time.

Furthermore, defining a time of validity of a prediction may not be a simple task. It is perhaps more natural to assume that once a prediction becomes invalid, Model Inference sends a new prediction.

Observation 8: A prediction can be assumed to be valid until a new prediction is sent.

It is therefore a bit ambiguous still how validity time can be used to indicate validity of Output information.

Proposal 2: Do not send a validity time in Model Inference Output. Model output validity can be discussed in normative phase per inference output.

# 4	Discussion on the flowcharts

In all the use cases, it is assumed that NG-RAN node 2 has a ML model optionally available. This step is reflected by Step 0.

Observation 9: Step 0 is missing from the description under Figure 5.1.2.1-1 and Figure 5.1.2.2-1.

Proposal 3: Introduce step 0 and adjust the text in the descriptions under Figure 5.1.2.1-1 and Figure 5.1.2.2-1.

# 5	Way Forward for Work Item Phase

In this study, a number of predictions were captured in order to introduce AI/ML in the RAN. A big part of this information is based on predictions, calculated at a gNB, e.g., resource status and utilization predictions. When applicable we should use legacy SON/MDT as a baseline in normative work. As an example, load predictions can be based on existing metrics used for load reporting such as CAC, PRB, etc. Furthermore, the prediction granularity for the load as defined in SON/MDT can be considered e.g., per SSB area, per cell, etc.. We believe that the already defined granularity can also be supported when it comes to predictions of information.

Proposal 4: Use existing SON/MDT procedures as a baseline when possible in normative work, i.e., use predictions of existing metrics (e.g., CAC, PRB) with their defined granularities (e.g., per SSB area, per cell).

During the Rel. 17 study, slicing hasn’t been discussed as part of the prioritized use cases. Legacy SON/MDT mechanisms inherently support slicing information.

Observation 10: Even though slicing is part of existing procedures, slicing hasn’t been explicitly discussed in AI/ML Rel.17 study.

Load related measurements can be reported per slice in Resource Status Reporting. Slice Available Capacity indicates the network resources on a slice in the Uplink and Downlink directions  Predicted load information could include slicing information so that reporting is on a per slice basis. Slicing enhancements have also been part of Rel-17. There are several possible ways forward on how to handle slicing information in this Rel.17 study and subsequent Rel.18 work:

- Include in the scope of the study predictions of existing metrics that may be made on a per slice granularity and in this way introduce slicing aspects into the energy saving use case.

- Exclude slicing aspects completely from the study and revisit those under a new use case covering slicing enhancements for a possible Rel.18 SI.

Proposal 5:  RAN3 is invited to discuss and make a decision on the preferred way forward between the following 2 options with respect to slicing information in the AI/ML Rel.17 study/Rel.18 work:

- Include in the scope of the study predictions of existing metrics that may be made on a per slice granularity and in this way introduce slicing aspects into the energy saving use case.

- Exclude slicing aspects completely from the study and revisit those under a new use case covering slicing enhancements for a possible Rel.18 SI.

We have a preference to the first approach.

# 6	Conclusion

We make the following observations and proposals:

Observation 1: Currently energy saving decisions are taken independently by a network node that decides to switch-off its cell(s) once it has evaluated that this is to improve energy efficiency.

Observation 2: Introduction of AI/ML at a capacity cell can assist the latter in taking more intelligent guesses in its energy saving/switch-off decisions.

Observation 3: Current and predicted resource status information from the local node and from the neighbours enables a capacity cell to predict whether an offloading decision is feasible at a given candidate target.

Observation 4: The problem that energy saving decisions at a local node may not be the best for the overall network energy efficiency cannot be addressed using negotiations with neighbouring nodes as long as the solution is distributed.

Observation 5: Network-wide energy efficiency can be achieved through the support of OAM energy saving decisions.

Proposal 1: A new procedure where an offloading plan to transfer a certain number of UEs to a neighboring RAN node for energy saving reasons can be accepted or rejected from a neighbor is not beneficial

Observation 6: There doesn’t seem to exist a unique definition of what validity time means.

Observation 7: Model output validity may depend on information other than time.

Observation 8: A prediction can be assumed to be valid until a new prediction is sent.

Proposal 2: Do not send a validity time in Model Inference Output. Model output validity can be discussed in normative phase per inference output.

Observation 9: Step 0 is missing from the description under Figure 5.1.2.1-1 and Figure 5.1.2.2-1.

Proposal 3: Introduce step 0 and adjust the text in the descriptions under Figure 5.1.2.1-1 and Figure 5.1.2.2-1.

Proposal 4: Use existing SON/MDT procedures as a baseline when possible in normative work, i.e., use predictions of existing metrics (e.g., CAC, PRB) with their defined granularities (e.g., per SSB area, per cell).

Observation 10: Even though slicing is part of existing procedures, slicing hasn’t been explicitly discussed in AI/ML Rel.17 study.

Proposal 5:  RAN3 is invited to discuss and make a decision on the preferred way forward between the following 2 options with respect to slicing information in the AI/ML Rel.17 study/Rel.18 work:

- Include in the scope of the study predictions of existing metrics that may be made on a per slice granularity and in this way introduce slicing aspects into the energy saving use case.

- Exclude slicing aspects completely from the study and revisit those under a new use case covering slicing enhancements for a possible Rel.18 SI.

# Annex - TP for TR 37.817

## 5.1	Network Energy Saving

### 5.1.1	Use case description

To meet the 5G network requirements of key performance and the demands of the unprecedented growth of the mobile subscribers, millions of base stations (BSs) are being deployed. Such rapid growth brings the issues of high energy consumption, CO2 emissions and operation expenditures (OPEX). Therefore, energy saving is an important use case which may involve different layers of the network, with mechanisms operating at different time scales.

Cell activation/deactivation is an energy saving scheme in the spatial domain that exploits traffic offloading in a layered structure to reduce the energy consumption of the whole radio access network (RAN). When the expected traffic volume is lower than a fixed threshold, the cells may be switched off, and the served Ues may be offloaded to a new target cell.

Efficient energy consumption can also be achieved by other means such as reduction of load, coverage modification, or other RAN configuration adjustments. The optimal EE decision depends on many factors including the load situation at different nodes, RAN nodes capabilities, KPI/QoS requirements, number of active Ues and UE mobility, cell utilization, etc.

However, the identification of actions aimed at energy efficiency improvements is not a trivial task. Wrong switch-off of the cells may seriously deteriorate the network performance since the remaining active cells need to serve the additional traffic. Wrong traffic offload actions may lead to a deterioration of Energy Efficiency instead of an improvement. The current energy-saving schemes are vulnerable to potential issues listed as follows:

- Inaccurate cell load prediction. Currently, energy-saving decisions rely on current traffic load without considering future traffic load.

- Conflicting targets between system performance and energy efficiency. Maximizing the system’s key performance indicator (KPI) is usually done at the expense of energy efficiency. Similarly, the most energy efficient solution may impact system performance. Thus, there is a need to balance and manage the trade-off between the two.

- Conventional energy-saving related parameters adjustment. Energy-saving related parameters configuration is set by traditional operation, e.g., based on different thresholds of cell load for cell switch on/off which is somewhat a rigid mechanism since it is difficult to set a reasonable threshold.

- Actions that may produce a local (e.g. limited to a single RAN node) improvement of Energy Efficiency, while producing an overall (e.g. involving multiple RAN nodes) deterioration of Energy Efficiency.

To deal with issues listed above, ML techniques could be utilized to leverage on the data collected in the RAN network. ML algorithms may predict the energy efficiency and load state of the next period, which can be used to make better decisions on cell activation/deactivation for ES. Based on the predicted load, the system may dynamically configure the energy-saving strategy (e.g. the switch-off timing and granularity, offloading actions) to keep a balance between system performance and energy efficiency and to reduce the energy consumption.

### 5.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 5.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network energy saving:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 5.1.2.2	Model Training at OAM and Model Inference at NG-RAN

In this solution, NG-RAN makes energy decisions using AI/ML model trained from OAM.

Figure 5.1.2.1-1. Model Training at OAM, Model Inference at NG-RAN

Step 0: NG-RAN node 2 may have an AI/ML model, which can generate required input such as resource status and utilization prediction/estimation etc.

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement, e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends measurement report message to NG-RAN node 1.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM. NG-RAN node 2 (assumed to have an AI/ML model optionally) also sends input data for Model Training to OAM. If NG-RAN node 2 executes the AI/ML model, the input data for Model Training can include the corresponding inference result from NG-RAN node 2.

Step 5: Model Training at OAM. Required measurements are leveraged to train AI/ML models for network energy saving.

Step 6: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-17 scope.

Step 7: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 8: UE sends UE measurement report to NG-RAN node 1.

Step 9: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output(s) (e.g. energy saving strategy, handover strategy, etc).

Step 10: NG-RAN node 1 sends Model Performance Feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 11: NG-RAN node 1 may select the most appropriate target cell for each UE before it performs handover, if the output is handover strategy.

Step 12: NG-RAN node 2 provides feedback to OAM.

Step 13: NG-RAN node 1 provides feedback to OAM.

#### 5.1.2.3	Model Training and Model Inference at NG-RAN

In this solution, NG-RAN is responsible for model training and generates energy saving decisions.

Figure 5.1.2.2-1. Model Training and Model Inference at NG-RAN

Step 0: NG-RAN node 2 may have an AI/ML model, which can generate required input such as resource status and utilization prediction/estimation etc.

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement, e.g. UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.s

Step 3: The UE sends measurement report to NG-RAN node 1 including the required measurement result.

Step 4: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model training of AI/ML-based network energy saving. If NG-RAN node 2 executes the AI/ML model, the input data for Model Training can include the corresponding inference result from NG-RAN node 2.

Step 5: NG-RAN node 1 trains AI/ML model for AI/ML-based energy saving based on collected data. NG-RAN node 2 is assumed to have AI/ML model for AI/ML-based energy saving optionally, which can also generate predicted results/actions.

Step 6: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 7: UE sends UE measurement report to NG-RAN node 1.

Step 8: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g. energy saving strategy, handover strategy, etc).

Step 9: NG-RAN node 1 may select the most appropriate target cell for each UE before it performs handover, if the output is handover strategy.

Step 10: NG-RAN node 2 provides feedback to NG-RAN node 1.

#### 5.1.2.4	Input of AI/ML-based Network Energy Saving

To predict the optimized network energy saving decisions, NG-RAN may need following information as input data for AI/ML-based network energy saving:

Input Information from Local node:

- UE mobility/trajectory prediction

- Current/Predicted Energy efficiency

- Current/Predicted resource status

Input Information from UE:

-      UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

Input from neighbouring NG-RAN nodes:

- Current/Predicted energy efficiency

- Current/Predicted resource status

- Current energy state (e.g., active, high, low, inactive)

If existing UE measurements are needed by a gNB for AI/ML-based network energy saving, RAN3 shall reuse the existing framework (including MDT and RRM measurements).

#### 5.1.2.5	Output of AI/ML-based Network Energy Saving

AI/ML-based network energy saving model can generate following information as output:

- Energy saving strategy, such as recommended cell activation/deactivation.

- Handover strategy, including recommended candidate cells for taking over the traffic

- Predicted energy efficiency

- Predicted energy state (e.g., active, high, low, inactive)

Model output validity can be discussed in normative phase per inference output.

#### 5.1.2.6	Feedback of AI/ML-based Network Energy Saving

To optimize the performance of AI/ML-based network energy saving model, following feedback can be considered to be collected from NG-RAN nodes:

- Resource status of neighbouring NG-RAN nodes

- Energy efficiency

- UE performance affected by the energy saving action (e.g. handed-over UEs), including bitrate, packet loss, latency.

- System KPIs (e.g. throughput, delay, RLF of current and neighboring NG-RAN node)

#### 5.1.2.7	Standard Impact

Potential Xn interface impact:

- New signalling procedure or enhanced existing procedure to collect the input data information

  - Predicted energy efficiency between neighboring NG-RAN nodes and source NG-RAN node

  - Predicted resource status between neighboring NG-RAN nodes and source NG-RAN node

- New signalling procedure or enhanced existing procedure to retrieve feedback information