3GPP TSG-RAN WG3 Meeting #115-e	R3-222141

21 Feb – 3 Mar 2022

Agenda item:	13.2.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS38.401 BL CR) discussion on Inter-Donor IAB Node Migration

Document for:	Discussion and Decision

# Introduction

This contribution discusses the open issues from last RAN3 meeting on inter-Donor migration.

# Discussion

## 2.1 procedure for descendant IAB migration

The main issue is when the IP address for the descendant IAB is assigned.

RAN3 agreed following:

If the IP addresses of the descendants of the boundary node in partial migration, RLF recovery or topology redundancy need to be changed to enable traffic migration, the new XnAP procedure can be used for IP address request, if needed.

In Stage-2 BL CR, the inter-CU migration call flow is as below:

Option 1 is the new IP address for the descendant IAB is assigned when CU1 initiate the transport migration request (Step 16/18) for the boundary’s F1 traffic. The REQUEST message (Step 16) only includes the DL TNL address for the boundary IAB. After CU1 receive the new IP address for the descendant IAB in Step 18, CU1 has to initiate a 2nd IAB TRANSPORT MIGRATION REQUEST for the F1-C/U traffic terminated at the descendant IAB. (This is not shown in the figure). This option uses 2 XnAP procedures.

As discussed during the CB#1304 discussion in last RAN3 meeting ([1]),

The Moderator proposes a way forward based on Option 2, given that it requires one, rather than 2 runs of the new XnAP procedure.

- Option 2: CU1 provides to CU2 the DL IP addresses together with the QoS info. In this case, CU1 indicates to CU2 the old DL IP addresses.

In option 2, the descendant’s F1-C/U traffic information can be included in Step 16. The REQUEST message (Step 16) includes the old/existing DL TNL address for the descendant IAB. When CU2 send the RESPONSE message in Step 18, CU2 can provide a 1:1 update for the IP address, i.e. an old IP address for descendant IAB received in Step 16 is replaced with a new IP address sent in Step 18.  This only requires 1 XnAP procedure, and it is better than using 2 XnAP procedure in Option 1. We proposed to adopt Option 2.

Proposal 1: the REQUEST message in Step 16 can include the traffic information for the descendant IAB, and the TNL address of the descendant IAB assigned by CU1. The RESPONSE message in Step 18 can include the new TNL address assigned by CU2 to the descendant IAB.

One may ask how CU2 can know whether it need to assign a new IP address when it receives the REQUEST message in Step 16. Of course, CU1 may provide an explicit indication informing CU2 to assign a new IP address, but this may be unnecessary. CU2 can make the decision based on whether the received TNL address is known. In case the received TNL address is the one previously assigned, i.e. the TNL address assigned to the boundary IAB in Step 4, CU2 does not need to assign new IP address. CU2 only need to assign the new TNL address if the received TNL address is unknown, e.g. the one assigned by CU1 and anchored in source IAB-donor-DU.

This also requires a small change in the Stage-3 CR that the RESPONSE message includes the new IP address assigned to the descendant IAB.

Proposal 2: Update Stage-3 to include the new TNL address in the Non-F1-terminating Topology BH Information IE.

Similar mechanism can also be reused for redundancy:

- CU2 configures (outer) IP addresses/prefixes on boundary node together with donor-DU2’s BAP address via RRC.

- Boundary IAB report its new IP address to CU1 via F1AP.

- CU1 initiates the Xn transport migration request, including the traffic information for the boundary IAB (with the new TNL address assigned to the boundary IAB), and the traffic information for the descendant IAB (with the old TNL address). Then CU2 replies with the new TNL address for the descendant IAB.

- CU1 configures the descendant IABs

Similar mechanism for RLF recovery:

- CU2 assigns the new IP address to the boundary IAB during the RRC re-establishment procedure

- CU2 initiates the XnAP Retrieve UE Context procedure

- Boundary IAB report its new TNL address to CU1.

- CU1 initiates the Xn transport migration request, including the traffic information for the boundary IAB (with the new TNL address assigned to the boundary IAB), and the traffic information for the descendant IAB (with the old TNL address). Then CU2 replies with the new TNL address for the descendant IAB.

- CU1 configures the descendant IABs

Proposal 3: during the redundancy or RLF recovery procedure, the REQUEST message can include the traffic information for the descendant IAB, and the TNL address of the descendant IAB assigned by CU1. The RESPONSE message can include the new TNL address assigned by CU2 to the descendant IAB.

An example for the migration of the descendant IAB’s transport can be found in the annex section. If above proposals are accepted, similar update is needed for topology redundancy section, and RLF recovery section.

## 2.2 IAB Node indication in XnAP

In current XnAP spec and BL CR, the IAB Node Indication IE is only included in the XnAP HANDOVER REQUEST message. The BL CR has a mixed information on how the peer NG-RAN node detect the UE is an IAB-MT, for example,

### 8.2.1	Handover Preparation

If the IAB Node Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the handover is for an IAB node.

If the Activated Cells List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the handover is for an IAB node, and use this IE for the inter-donor resource coordination for the IAB node.

Editor’s Note: FFS the final wording.

If the No PDU Session Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider the UE as an IAB node which does not have a PDU session activated, and ignore the PDU Session Resources To Be Setup List IE, and shall not take any action with respect to PDU session setup. Subsequently, the source NG-RAN node shall, if supported, ignore the PDU Session Resources Admitted To Be Added List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

### 8.2.4	Retrieve UE Context

If the No PDU Session Indication IE is contained in the UE Context Information – Retrieve UE Context Response IE of the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, consider the UE as an IAB node which does not have a PDU session activated,  ignore the PDU Session Resources To Be Setup List IE in the UE Context Information – Retrieve UE Context Response IE, and shall not take any action with respect to PDU session setup.

It needs to be consistent, i.e. using the IAB Node Indication IE to determine the UE is an IAB node, during the Xn Retrieve UE Context procedure and Xn S-NG-RAN node Addition Preparation. The new IAB related IEs also needs to be clarified in the semantics description that the IEs are only applicable to IAB.

Proposal 4: introduce the IAB Node Indication IE in the XnAP RETRIEVE UE CONTEXT RESPONSE message, and S-NODE ADDITION REQUEST message.

Proposal 5: use semantics description to describe the new IEs that only applicable when the UE is an IAB-MT.

# Conclusion

In this contribution, we have analysed the technical detail on inter-Donor topology adaptation. Our proposal is:

Proposal 1: the REQUEST message in Step 16 can include the traffic information for the descendant IAB, and the TNL address of the descendant IAB assigned by CU1. The RESPONSE message in Step 18 can include the new TNL address assigned by CU2 to the descendant IAB.

Proposal 2: Update Stage-3 to include the new TNL address in the Non-F1-terminating Topology BH Information IE.

Proposal 3: during the redundancy or RLF recovery procedure, the REQUEST message can include the traffic information for the descendant IAB, and the TNL address of the descendant IAB assigned by CU1. The RESPONSE message can include the new TNL address assigned by CU2 to the descendant IAB.

Proposal 4: introduce the IAB Node Indication IE in the XnAP RETRIEVE UE CONTEXT RESPONSE message, and S-NODE ADDITION REQUEST message.

Proposal 5: use semantics description to describe the new IEs that only applicable when the UE is an IAB-MT.

The update to Stage-2 BL CR can be found at the annex section. The update to TS38.423 BL CR can be found at ([4]).

# References

- R3-221317, CB: # 1304_IAB_Top_Red - Summary of email discussion

- R3-221595, BL CR to TS 38.401 on support of eIAB

- R3-216078, CB: # 1302_IAB_Inter_Donor_Mig

- R3-222142, (TP for TS38.423 BL CR) discussion on Inter-Donor IAB Node Migration

TP for TS 38.401 BL CR:

### 8.xx.z.1  IAB inter-CU topology adaptation procedure

During the inter-CU topology adaptation for a single-connected IAB-node, the IAB-MT switches connection from an old parent node to a new parent node, where the old and the new parent nodes are served by different IAB-donor-CUs. Without loss of generality, the old parent node can be referred to as source parent node, and the new parent node can be referred to as target parent node.

Figure 8.xx.1-1 shows an example of the topology adaptation procedure where the migrating IAB-MT is migrated from one IAB-donor-CU to another IAB-donor-CU. In case the IAB-DU of the migrating IAB-node retains its F1 connection with the first IAB-donor-CU (i.e. the source IAB-donor-CU) after the migrating IAB-MT connects to the second IAB-donor-CU (i.e. the target IAB-donor-CU), this procedure renders the migrating IAB-node as a boundary IAB-node.

Figure 8.xx.1-1: IAB inter-CU topology adaptation procedure

1.	The source IAB-donor-CU sends a HANDOVER REQUEST message to the target IAB-donor-CU over the Xn interface. This message may include the migrating IAB-node’s TNL address information in the RRC container.

2.	The target IAB-donor-CU sends a UE CONTEXT SETUP REQUEST message to the target parent node IAB-DU to create the UE context for the migrating IAB-MT and set up one or more bearers. These bearers can be used by the migrating IAB-MT for its own signaling, and, optionally, data traffic.

3.	The target parent node IAB-DU responds to the target IAB-donor-CU with a UE CONTEXT SETUP RESPONSE message.

4.	The target IAB-donor-CU performs admission control and provides the new RRC configuration as part of the HANDOVER REQUEST ACKNOWLEDGE message. The RRC configuration includes a BAP address for the boundary node in the target IAB-donor-CU’s topology, default BH RLC channel and a default BAP routing ID configuration for UL F1-C/non-F1 traffic mapping on the target path. The RRC configuration may include the new TNL addresses anchored at IAB-donor-DU2 for the migrating node.

5.	The source IAB-donor-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source parent node IAB-DU, which includes the received RRCReconfiguration message from the target IAB-donor-CU.

6.	The source parent node IAB-DU forwards the received RRCReconfiguration message to the migrating IAB-MT.

7.	The source parent node IAB-DU responds to the source IAB-donor-CU with the UE CONTEXT MODIFICATION RESPONSE message.

8.	A random access procedure is performed at the target parent node IAB-DU.

9.	The migrating IAB-MT responds to the target parent node IAB-DU with an RRCReconfigurationComplete message.

10.	The target parent node IAB-DU sends an UL RRC MESSAGE TRANSFER message to the target IAB-donor-CU to convey the received RRCReconfigurationComplete message.

11.	The target IAB-donor-CU triggers the path switch procedure for the migrating IAB-MT, if needed.

12.	The target IAB-donor-CU sends UE CONTEXT RELEASE message to the source IAB-donor-CU.

NOTE: FFS whether the XnAP UE ID of the migrating node is retained at target and source IAB-donor-CU.

13.	The source IAB-donor-CU may release BH RLC channels and BAP-sublayer routing entries on the source path between source parent IAB-node of the migrating IAB-node and the source IAB-donor-DU.

14.	The target IAB-donor-CU configures BH RLC channels and BAP-sublayer routing entries on the target path between the target parent IAB-node and target IAB-donor-DU as well as DL mappings on the target IAB-donor-DU for the migrating IAB-node’s target path. These configurations support the transport of F1-C traffic on the target path.

15.	The F1-C connection between the migrating IAB-node and the source IAB-donor-CU is switched to the target path using the new TNL address information of the migrating IAB-node. The migrating IAB-node reports the TNL address information it wants to use for its F1-U tunnels to the source IAB-donor-CU.

16.	The source IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the target IAB-donor-CU to provide the context of the traffic to be offloaded. The message includes the DL TNL address information necessary for the target IAB-donor-CU to configure or modify DL mappings on the target IAB-donor-DU. This message may also include the context of the traffic terminated at the descendant IAB-DU(s), and the DL TNL address information is the descendant IAB’s address anchored in the source IAB-donor-DU.

17.	The target IAB-donor-CU may configure or modify BH RLC channels and BAP-sublayer routing entries on the target path between the target parent IAB-node and target IAB-donor-DU as well as DL mappings on the target IAB-donor-DU for the migrating IAB-node’s target path. These configurations may support the transport of UP and non-UP traffic on the target path.

18.	The target IAB-donor-CU responds to the source IAB-donor-CU with an IAB Transport Migration Management Response message to provide the mapping information for the traffic to be offloaded. The message includes the L2 info necessary to configure the migrating IAB-node with the UL mappings of traffic included in step 16. The message includes the DSCP/IPv6 Flow Label values used to configure the DL mappings of traffic included in step 16.

19.	The F1-U connections of the migrating IAB-node with the source IAB-donor-CU are switched to use the migrating IAB-node’s new TNL address(es). The source IAB-donor-CU provides updated UL BH information for the traffic included in step 16 based on the UL BH information received in step 18. The source IAB-donor-CU may also update the UL BH information associated with non-UP traffic. This step may use UE associated signaling or non-UE associated signaling in E1 and/or F1 interface. Implementation must ensure the avoidance of potential race conditions, i.e., no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures.

20.	Repetition of steps 16 to 19, as needed, where the source IAB-donor-CU can request addition, modification or release of QoS information for non-UP and UP traffic. The target IAB-donor-CU can fully or partially reject addition or modification requests by the source IAB-donor-CU.

The traffic offload through the inter-CU topology adaptation described in steps 1 to 20 can be revoked. In this case, the migrating IAB-MT is handed over in reverse direction, i.e., from the former target IAB-donor-CU to the former source IAB-donor-CU, after which the traffic of the migrating IAB-DU and the descendant IAB-DUs is routed again along the former source path. The former target IAB-donor-CU can trigger this return of offloaded traffic back to the source IAB-donor-CU by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the former source IAB-donor-CU.

For the migration of the descendent nodes’ transport, the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message in Step 16 may also include the traffic information for the descendant IAB(s), and the DL TNL address is the descendant IAB’s existing IP address anchored in source IAB-donor-DU. The target IAB-donor-CU assign the new TNL address(es) for the descent IAB(s), and include the new TNL address(es) in the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message in Step 18. Accordingly, the source IAB-donor-CU reconfigure the descendant IAB to use the new TNL address. Procedures similar to Step 19 and 20 are performed to migrate the descendant IAB’s transport to target path.

.

Editor’s NOTE: The initiation of revocation of partial migration by the target IAB-donor-CU needs to be captured. FFS if same or separate procedure.