3GPP TSG-RAN WG3 Meeting #115-e	R3-222145

21 Feb – 3 Mar 2022

Agenda item:	13.2.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS38.423 BL CR) on Inter-Donor-DU tunnel

Document for:	Discussion and Decision

Per the discussion in ([1]), this contribution proposes TP for TS 38.423 BL CR.

# References

- R3-222144, (TP for TS38.473 BL CR) discussion on Inter-Donor-DU tunnel

TP for TS 38.423 BL CR:

-------------------------------------------Start of Change-------------------------------------------

### 8.x.a	IAB Transport Migration Management

#### 8.x.a.1	General

The purpose of the IAB Transport Migration Management procedure is to exchange information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, for the purpose of managing the migration of the boundary and descendant IAB-node traffic between the topologies managed by the two IAB-donor-CUs.

The procedure is applicable to inter-donor partial migration, inter-donor RLF recovery and inter-donor topology redundancy cases. The procedure can be initiated by the F1-terminating IAB-donor-CU of the boundary IAB-node. The procedure can be used to set up, modify and release (e.g., for the purpose of revoking) the resources under the non-F1-terminating IAB-donor-CU used for serving the offloaded traffic.

Editor’s NOTE: FFS on initiation by non-F1-terminating IAB-donor-CU of the boundary IAB-node via the same procedure.

Editor’s NOTE: FFS on NUA or UA.

Editor’s NOTE: FFS on whether using “F1-terminating donor”&“non-F1-terminating donor” or using “NG-RAN Node 1”&”NG-RAN Node 2”

Editor’s NOTE: FFS on message name

#### 8.x.a.2	Successful Operation

Figure 8.x.a.2-1: IAB Transport Migration Management triggered by the F1-terminating donor, successful operation

The F1-terminating donor initiates the procedure by sending the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1-terminating IAB-donor.

The non-F1-terminating donor may respond with the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message by indicating:

-	Traffic accepted for offloading, within the Traffic Added Item IE;

-	Already offloaded traffic accepted for modification , within the Traffic Modified Item IE;

If the Traffic to Be Released Information IE is contained in the IAB TRANSPORT MIGRATION MANAGEMENTREQUEST message, the non-F1-terminating donor should release all offloaded traffic if the All Traffic Indication IE in the Traffic to Be Released Information IE is set to “true”, or release only the offloaded traffic indicated by the Traffic to Be Released Item IE in the Traffic to Be Released Information IE.

If the IAB TNL Address Request IE is contained in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, the non-F1-terminating donor should provide the allocated TNL address via the IAB TNL Address Response IE in the IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message.

If the IAB DU Tunnel Information IE is contained in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, the non-F1-terminating donor should configure the related IAB-donor-DU to enable the inter-DU tunnel.

-------------------------------------------Next Change-------------------------------------------

#### 9.1.x.a	IAB TRANSPORT MIGRATION MANAGEMENT REQUEST

This message is sent by a F1-terminating IAB-donor-CU to a non-F1-terminating IAB-donor-CU of a boundary IAB-node, for the purpose of setting up, modifying, or releasing (e.g., for the purpose of revoking) the configuration for the migration of boundary and descendant node traffic between two IAB-donor-CUs.

Direction: F1-terminating donor CU  non-F1-terminating donor CU.

FFS: Direction: Non-F1-terminating donor CU  F1-terminating donor CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| IAB Boundary Node ID | M |  | FFS |  |  |  |
| Traffic To Be Added List |  | 0..1 |  |  | YES | reject |
| >Traffic To Be Added Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.x0 |  | – |  |
| >>Traffic Profile | M |  | 9.2.2.x1 |  | – |  |
| >>F1-terminating Topology BH Information | M |  | 9.2.2.x2 |  | – |  |
| Traffic To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Traffic To Be Modified Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.x0 |  | – |  |
| >>Traffic Profile | O |  | 9.2.2.x1 |  | – |  |
| >>F1-terminating Topology BH Information | O |  | 9.2.2.x2 |  | – |  |
| Traffic To Be Released Information | O |  | 9.2.2.x4 |  | YES | reject |
| IAB TNL Address Request | O |  | 9.2.2.x5 |  | YES | reject |
| IAB DU Tunnel Information | O |  | 9.2.2.xx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor-CU. The value is FFS. |

-------------------------------------------Next Change-------------------------------------------

#### 9.2.2.xx	IAB DU Tunnel Information

This IE indicates the list of TNL addresses related to the UL packets to be forwarded over the Inter-DU tunnel, e.g. the UL packets with source IP address field matches the ones indicated in this IE will be forwarded over the associated tunnel by the IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| IAB DU Tunnel Information |  | 1 |  |  | YES | Ignore |
| >IAB DU Tunnel Information Item |  | 1..< maxnoofTunnels> |  |  |  |  |
| >>IAB DU tunnel identity |  |  |  |  |  |  |
| >>>Source-DU TNL Address | M | 9.2.3.29 |  |  |  |  |
| >>>GTP-TEID | O | OCTET STRING (4) |  |  |  |  |
| >>IAB TNL Address List |  | 0..1 |  |  |  |  |
| >>>IAB TNL Address Item |  | 1..< maxnoofTLAsIAB> |  |  |  |  |
| >>>>IAB TNL Address |  | 9.2.2.x12 |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTunnels | Maximum no. of tunnel information in one procedure execution. The value is 16 (FFS). |
| maxnoofTLAsIAB | Maximum no. of individual IPv4/IPv6 addresses or IPv6 address prefixes in one procedure execution. The value is 1024 (FFS). |

-------------------------------------------Next Change-------------------------------------------

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-------------------------------------------Unaffected part is skipped-------------------------------------------

F1-TerminatingTopologyBHInformation,

Non-F1-TerminatingTopologyBHInformation,

IABDUTunnelInfo

FROM XnAP-IEs

-------------------------------------------Unaffected part is skipped-------------------------------------------

id-TrafficModifiedList,

id-TrafficNotAddedList,

id-TrafficNotModifiedList,

id-IABDUTunnelInfo,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries

FROM XnAP-Constants;

-------------------------------------------Unaffected part is skipped-------------------------------------------

-- **************************************************************

--

#### -- IAB TRANSPORT MIGRATION MANAGEMENT REQUEST

--

-- **************************************************************

IABTransportMigrationManagementRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ IABTransportMigrationManagementRequest-IEs}},

...

}

IABTransportMigrationManagementRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-IABBoundaryNodeID					CRITICALITY reject		TYPE IABBoundaryNodeID					PRESENCE mandatory}|

{ ID id-TrafficToBeAddedList				CRITICALITY reject		TYPE TrafficToBeAddedList				PRESENCE optional	 }|

{ ID id-TrafficToBeModifiedList				CRITICALITY reject		TYPE TrafficToBeModifiedList				PRESENCE optional	 }|

{ ID id-TrafficToBeReleaseInformation		CRITICALITY reject		TYPE TrafficToBeReleaseInformation			PRESENCE optional	 }|

{ ID id-IAB-TNL-Address-Request				CRITICALITY reject		TYPE IAB-TNL-Address-Request				PRESENCE optional	 }|

{ ID id-IABDUTunnelInfo						CRITICALITY ignore		TYPE IABDUTunnelInfo						PRESENCE optional	},

...

}

TrafficToBeAddedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficToBeAdded-Item

-------------------------------------------Next Change-------------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-------------------------------------------Unaffected part is skipped-------------------------------------------

maxnoofNonAnchorCarrierFreqConfig,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofServingCells,

maxnoofBHInfo,

maxnoofTLAsIAB,

maxnoofTrafficIndexEntries,

maxnoofTunnels

FROM XnAP-Constants

-------------------------------------------Unaffected part is skipped-------------------------------------------

IABNodeIndication ::= ENUMERATED {true,...}

IAB-TNL-Address-Request ::= SEQUENCE {

iABIPv4AddressesRequested				IABTNLAddressesRequested,

iABIPv6RequestType						IABIPv6RequestType,

iABTNLAddressToRemove-List				IABTNLAddressToRemove-List,

iE-Extensions							ProtocolExtensionContainer { {IAB-TNL-Address-Request-ExtIEs} }	OPTIONAL,

...

}

IAB-TNL-Address-Request-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABIPv6RequestType	 ::= CHOICE {

iPv6Address						IABTNLAddressesRequested,

iPv6Prefix						IABTNLAddressesRequested,

choice-extension				ProtocolIE-Single-Container { {IABIPv6RequestType-ExtIEs} }

}

IABIPv6RequestType-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

IAB-TNL-Address-Response ::= SEQUENCE {

iABAllocatedTNLAddress-List		IABAllocatedTNLAddress-List,

iE-Extensions					ProtocolExtensionContainer { {IAB-TNL-Address-Response-ExtIEs} }	OPTIONAL,

...

}

IAB-TNL-Address-Response-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABAllocatedTNLAddress-List	::= SEQUENCE (SIZE(1..maxnoofTLAsIAB))	OF IABAllocatedTNLAddress-Item

IABAllocatedTNLAddress-Item ::=	SEQUENCE {

iABTNLAddress				IABTNLAddress,

iABTNLAddressUsage			IABTNLAddressUsage	 	OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {IABAllocatedTNLAddress-Item-ExtIEs} }	OPTIONAL,

...

}

IABAllocatedTNLAddress-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddress ::= CHOICE {

iPv4Address						BIT STRING (SIZE(32)),

iPv6Address						BIT STRING (SIZE(128)),

iPv6Prefix						BIT STRING (SIZE(64)),

choice-extension				ProtocolIE-Single-Container { {IABTNLAddress-ExtIEs} }

}

IABTNLAddress-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

IABTNLAddressesRequested ::= SEQUENCE {

tNLAddressesOrPrefixesRequestedAllTraffic	INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedF1-C			INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedF1-U			INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedNoNF1		INTEGER (1..256) 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {IABTNLAddressesRequested-ExtIEs} } OPTIONAL

}

IABTNLAddressesRequested-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressToRemove-List	::= SEQUENCE (SIZE(1..maxnoofTLAsIAB))	OF IABTNLAddressToRemove-Item

IABTNLAddressToRemove-Item ::=	SEQUENCE {

iABTNLAddress			IABTNLAddress,

iE-Extension			ProtocolExtensionContainer { {IABTNLAddressToRemove-Item-ExtIEs} }	OPTIONAL,

...

}

IABTNLAddressToRemove-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressUsage ::= ENUMERATED {

f1-c,

f1-u,

non-f1,

...

}

IABDUTunnelInfo ::= SEQUENCE (SIZE(1..maxnoofTunnels)) OF IABTunnelInfo-Item

IABTunnelInfo-Item ::= SEQUENCE {

iABDUTunnelIdentifier			IABDUTunnelIdentifier,

iABTNLAddressList		          IABTNLAddressList	    OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { IABTunnelInfo-ItemExtIEs } }	OPTIONAL

}

IABTunnelInfo-ItemExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABDUTunnelIdentifier ::= SEQUENCE {

sourceDUTNLAddress				TransportLayerAddress,

gTP-TEID 						GTP-TEID	    OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { IABDUTunnelIdentifier-ExtIEs } }	OPTIONAL

}

IABDUTunnelIdentifier-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressList ::= SEQUENCE (SIZE(1.. maxnoofTLAsIAB)) OF IABTNLAddress

IAB-ULTraffic-Info-Request ::= SEQUENCE {

ingressBAPRoutingID				BAPRoutingID,

ingressBHRLCCHID				BHRLCChannelID,

iE-Extensions					ProtocolExtensionContainer { {IAB-ULTraffic-Info-Request-ExtIEs} }	OPTIONAL,

...

}

IAB-ULTraffic-Info-Request-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

IAB-ULTraffic-Info-Response ::= SEQUENCE {

egressBAPRoutingID					BAPRoutingID,

egressBHRLCCHID					BHRLCChannelID,

next-hopBAPAddress				BAPAddress,

qoSMappingInformation			QoS-Mapping-Information,

iE-Extensions					ProtocolExtensionContainer { {IAB-ULTraffic-Info-Response-ExtIEs} }	OPTIONAL,

...

}

IAB-ULTraffic-Info-Response-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ImmediateMDT-EUTRA ::= OCTET STRING

-------------------------------------------Next Change-------------------------------------------

### 9.3.7 Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

-------------------------------------------Unaffected part is skipped-------------------------------------------

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN    	INTEGER ::= 256

maxnoofServedCellsIAB					INTEGER ::= 512

maxnoofServingCells						INTEGER ::= 32

maxnoofBHInfo							INTEGER ::= FFS

maxnoofTrafficIndexEntries					INTEGER	::=	FFS

maxnoofTLAsIAB								INTEGER	::=	1024

maxnoofTunnels								INTEGER ::= xx

-------------------------------------------Unaffected part is skipped-------------------------------------------

id-TrafficAddedList																					ProtocolIE-ID ::= xxx

id-TrafficModifiedList																				ProtocolIE-ID ::= xxx

id-TrafficNotAddedList																				ProtocolIE-ID ::= xxx

id-TrafficNotModifiedList																			ProtocolIE-ID ::= xxx

id-IABDUTunnelInfo																					ProtocolIE-ID ::= xxx

END

-- ASN1STOP

-------------------------------------------End of change-------------------------------------------