3GPP TSG-RAN WG3 Meeting #115-e	R3-222158

E-meeting, 21 Feb – 3 Mar 2022

Title: 	(TP to CPAC BL CR for TS 37.340) SN initiated inter-SN CPC

Source: 	Huawei

Agenda item:	14.3

Document Type:	other

# 1. Introduction

For SN initiated inter-SN CPC, the following agreements/WAs were made during RAN3#114bis-e meeting:

SN Change Confirm message is sent from the MN to the S-SN after CPC reconfiguration, but before CPC execution.

RAN3 will enable signalling so that it is optional for the MN, e.g., when the target SN accepts all or some of the proposed PSCells, to inform the SN about the accepted PSCells (for the 2nd step).

WA: If the MN decides to trigger the 2nd step (optionally, as proposed in the LS from RAN2), the MN uses MN-initiated modification procedure. If the SN decides to update the CPC configuration, it provides the update in the response to the SN MOD REQ message.

In this document we further discuss the leftover issues for SN initiated inter-SN CPC.

# 3. Discussion

- There were quite a lot of discussion on the achieve WA last meeting, in this paper, firstly we propose to turn the WA to agreement.

- Proposal 1: turn the WA of using MN initiated modification procedure as agreement.

- As we discussed several meetings before, about if and how to prepare multiple T-SNs during SN initiated inter-SN CPC, there are several options. In [1], two options are listed on how to prepare multiple T-SNs:

- Single SN Change Required message for all T-SNs to be prepared.

- Single procedure used for the signalling related to the 2nd step is combined for all T-SNs.

- Based on the inputs from companies, we see that many companies prefer to use a single message if it possible, therefore we try to analyses how to prepare multiple T-SNs by using option 1).

- We take the following call flow as the basis of single T-SN preparation in SN initiated inter-SN CPC:

- Figure 1, Single T-SN preparation in SN initiated inter-SN CPC

- Therefore the multiple T-SN preparation by using single SN Change Required message could be illustrated as below:

- Figure 2, Multiple T-SN preparation in SN initiated inter-SN CPC

- Considering that currently the SN Change Required message includes the T-SN ID as mandatory IE, in order to carry multiple T-SN IDs, it is needed to introduce an additional T-SN ID IE in the message to be used in case of multiple T-SN preparation.

- Take XnAP spec as an example, the S-NODE CHANGE REQUIRED message could be updated as follows:

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.3.1 |  | YES |  | reject |  |
| M-NG-RAN node UE XnAP ID | M |  |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES |  | reject |  |
| S-NG-RAN node UE XnAP ID | M |  |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES |  | reject |  |
| Target S-NG-RAN node ID | M |  |  | Global NG-RAN Node ID 9.2.2.3 | This IE is ignored if the Target SN List to be prepared IE is present. | YES |  | reject |  |
| Cause | M |  |  | 9.2.3.2 |  | YES |  | ignore |  |
| PDU Session SN Change Required List |  |  | 0..1 |  |  | YES |  | ignore |  |
| >PDU Session SN Change Required Item |  |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |  |  |
| >>PDU Session ID | M |  |  | 9.2.3.18 |  | – |  |  |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  |  | 9.2.1.18 |  | – |  |  |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES |  | reject |  |
| Target SN List to be prepared |  | 0.. <maxnoofTargetSNs> |  |  | This IE would be present only if the preparation of multiple candidate target SNs is required in case of SN initiated inter-SN CPC. |  | YES |  | Reject |
| >Target S-NG-RAN node ID | M |  |  | Global NG-RAN Node ID 9.2.2.3 |  |  | - |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of candidate target SNs in case of SN initiated inter-SN CPC, Value is 4. |

- Proposal 2: introduce a T-SN List to be prepared IE in the SN Change Required message, to be used in case of multiple T-SN preparation, and the existing Target SN ID IE is ignored if this new IE is present.

- In legacy SN Change Confirm message, there is no T-SN ID, in case of multiple T-SN preparation, we need to introduce a list of T-SN IDs in that message, and furthermore, it is noticed that the data forwarding address is currently provided in that message for a single T-SN which should be extended as well.

- Take XnAP spec as an example, the S-NODE CHANGE CONFIRM message could be updated as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session SN Change Confirm List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Confirm Item |  | 1 .. <maxnoof PDUsessions> |  | NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present in a PDU Session SN Change Confirm Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Confirm Info – SN terminated | O |  | 9.2.1.19 | This IE is ignored if the Additional List of PDU Session Resource Change Confirm Info – SN terminated IE is present. | – |  |
| >>Additional List of PDU Session Resource Change Confirm Info – SN terminated |  | 0..1 |  | This IE would be present only if multiple candidate target SNs are prepared in case of SN initiated inter-SN CPC. | YES | ignore |
| >>> Additional Item of PDU Session Resource Change Confirm Info – SN terminated |  | 1 .. <maxnoofTargetSNs> |  |  | - |  |
| >>>>PDU Session Resource Change Confirm Info – SN terminated | M |  | 9.2.1.19 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Prepared Target SN List |  | 0.. <maxnoofTargetSNs> |  | This IE would be present only if multiple candidate target SNs are prepared in case of SN initiated inter-SN CPC. | YES | ignore |
| >Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of candidate target SNs in case of SN initiated inter-SN CPC, Value is 4. |

- Proposal 3: introduce a Prepared T-SN List IE in the SN Change Confirm message, to be used in case of multiple T-SN is prepared.

- Proposal 4: introduce an Additional List of PDU Session Resource Change Confirm Info – SN terminated IE in the SN Change Confirm message, to carry different data forwarding address for different T-SNs, the existing PDU Session Resource Change Confirm Info – SN terminated IE is ignored if this new IE is present.

- In case the CPC is executed by the UE, the MN also need to send SN Release Request message towards the other T-SNs to cancel CPC, which is not clear in current BL CR.

- Proposal 5: in case the CPC is executed by the UE, the MN sends SN Release Request message to cancel CPC in the target SNs.

# 4. Conclusion

- In this contribution, the leftover issues for SN initiated inter-SN CPC are discussed, got the following proposals:

- Proposal 1: turn the WA of using MN initiated modification procedure as agreement.

- Proposal 2: introduce a T-SN List to be prepared IE in the SN Change Required message, to be used in case of multiple T-SN preparation, and the existing Target SN ID IE is ignored if this new IE is present.

- Proposal 3: introduce a Prepared T-SN List IE in the SN Change Confirm message, to be used in case of multiple T-SN is prepared.

- Proposal 4: introduce an Additional List of PDU Session Resource Change Confirm Info – SN terminated IE in the SN Change Confirm message, to carry different data forwarding address for different T-SNs, the existing PDU Session Resource Change Confirm Info – SN terminated IE is ignored if this new IE is present.

- Proposal 5: in case the CPC is executed by the UE, the MN sends SN Release Request message to cancel CPC in the target SNs.

- The corresponding TP to TS37.340 is provided in section 6. And the corresponding XnAP TP is provided in [2].

# 5. Reference

- R3-221034 Summary of Offline Discussion on CPAC (CB #MRDC3), Nokia (moderator)

- R3-222346 (TP to CPAC BL CR for TS 38.423) Preparation of Multiple T-SNs in SN initiated inter-SN CPC, Huawei

# 6. TP to TS 37.340 BL CR

## ----------Start of the Next Change--------------

## 10.5	Secondary Node Change (MN/SN initiated)

### 10.5.1	EN-DC

//skip unchanged part

SN initiated conditional inter-SN Change

The SN initiated conditional inter-SN change procedure is used to configure CPC configuration.

The SN initiated conditional inter-SN change procedure may also be initiated by the source SN, to modify the existing CPC configuration, or to trigger the release of the target SN by cancellation of all the prepared PSCells at the target SN and releasing the CPC related UE context at the target SN.

Figure 10.5.1-4: Conditional SN Change – SN initiated

Figure 10.5.1-4 shows an example signalling flow for the Conditional Secondary Node Change initiated by the SN:

1.	The source SN initiates the conditional SN change procedure by sending SgNB Change Required message which contains a CPC initiation indication. The message also contains target SN ID(s) information and may include the SCG configuration (to support delta configuration), and contains the measurement results related to the target candidate SN(s). The message also includes a list of proposed PSCell candidates recommended by the source SN, including execution conditions, the upper limit for the number of PSCells, and may also include the SCG measurement configurations for CPC (e.g. measId(s) to be used for CPC).

2/3.	The MN requests the target candidate SN(s)  to allocate resources for the UE by means of the SgNB Addition procedure(s), including a CPC initiation indication, and the measurements results related to the target candidate SN and indicates the list of proposed PSCell candidates received from the source SN, but not including execution conditions. Within the list of PSCells, the SN decides the list of  PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration****) contained in the SgNB Addition Request Acknowledge message. If forwarding is needed, the target SN provides forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration, and the list of prepared PSCell IDs to the MN. The target SN can either accept or reject each of the candidate cells suggested by the source -SN, i.e. it cannot come up with any alternative candidates.

4. The MN may indicate the candidate PSCells accepted by the target SN to the source SN via SgNB Modification Request message, if needed, e.g., when T-SN does not acknowledge all candidate PSCells, otherwise step 4 and 5 are skipped, e.g. when T-SN accepts all candidate PScells.

5. The source SN may provide updated measurement configurations for CPC to the MN via SgNB Modification Request Acknowledge message.

7.	The MN sends to the UE an RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) including the CPC configuration, (i.e. a list of RRCConnectionReconfiguration messages RRCConnectionReconfiguration***) and associated execution conditions, in which RRCConnectionReconfiguration*** contains RRCReconfiguration**** received from the candidate SN. Besides, the RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements, as well as the NR RRC configuration message (RRCReconfigutation**) generated by the source-SN.

8.  The UE applies the RRC configuration (in RRCConnectionReconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRCConnectionReconfigurationComplete message (RCConnectionReconfigurationComplete*), which can include an NR RRC response message (RRCReconfigurationComplete**). In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration* message, it performs the reconfiguration failure procedure.

8a. If an NR RRC response message is included, the MN informs the source SN with the NR RRC response message (RRCReconfigutationComplete**) for the source SN via SgNB Reconfiguration Complete message.

The MN sends the SgNB Change Confirm message towards the Source SN to indicate that CPC is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the target SN, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding. In case multiple Target SNs are prepared, the MN includes a list of Target SgNB ID and list of data forwarding addresses to the source SN.

9.  The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCConnectionReconfiguration message (RRCConnectionReconfiguration***) corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete message (RRCConnectionReconfigurationComplete***), including an NR RRC message (RRCReconfigurationComplete****) for the selected candidate PSCell, and the selected PSCell information to the MN.

10: The MN triggers the MeNB initiated SgNB Release procedure to inform source SN to stop providing user data to the UE, and provide the address of the selected target SN and if applicable, start late data forwarding.

11.	If the RRC connection reconfiguration procedure was successful, the MN informs the target SN via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete**** response message for the target SN. The MN sends the SgNB Release Request messages to cancel CPC in the other target candidate SNs, if configured.

12.	The UE synchronizes to the target SN indicated in RRCConnectionReconfiguration***.

13.	For SN terminated bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target SN, if needed.

14.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding message from the MN.

15.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 7:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SgNB may send the report when the transmission of the related bearer is stopped.

16-20.	If applicable, a path update is triggered by the MN.

21.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

### 10.5.2	MR-DC with 5GC

//skip unchanged part

SN initiated conditional inter-SN Change

The SN initiated conditional inter-SN change procedure is used to configure CPC configuration.

The SN initiated conditional inter-SN change procedure may also be initiated by the source SN, to modify the existing CPC configuration, or to trigger the release of the target SN by cancellation of all the prepared PSCells at the target SN and releasing the CPC related UE context at the target SN.

Figure 10.5.2-4: Conditional SN change procedure - SN initiated

Figure 10.5.2-4 shows an example signalling flow for the conditional SN Change initiated by the SN:

- The source SN initiates the conditional SN change procedure by sending the SN Change Required message, which a CPC initiation indication.The message also contains candidate target node ID(s)  and may include the SCG configuration (to support delta configuration) , and contains the measurements results which may include cells that are not CPC candidates. The message also includes a list of proposed PSCell candidates recommended by the source SN, including execution conditions, the upper limit for the number of PSCells, and may also include the SCG measurement configurations for CPC (e.g. measId(s)to be used for CPC).

2/3.	The MN requests the target SN(s)  to allocate resources for the UE by means of the SN Addition procedure(s), including a CPC initiation indication, and the measurements results which may include cells that are not CPC candidates received from the source SN to the target SN, and indicates the list of proposed PSCell candidates received from the source SN, but not including execution conditions. Within the list of PSCells, the SN decides the list of  PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration****) contained in the SgNB Addition Request Acknowledge message. If data forwarding is needed, the target SN provides data forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration, and the list of prepared PSCell IDs to the MN. The target-SN can either accept or reject each of the candidate cells suggested by the Source-SN, i.e., it cannot come up with any alternative candidates.

4. The MN may indicate the candidate PSCells accepted by the target SN to the source SN via SN Modification Request message, if needed,, e.g., when T-SN does not acknowledge all candidate PSCells, , otherwise step 4 and 5 are skipped, e.g. when T-SN accepts all candidate PScells.

5. The source SN may provide the updated measurement configurations to the MN via SgNB Modification Request Acknowledge message.

7.	The MN sends to the UE an RRC reconfiguration message (RRC reconfiguration*) including the CPC configuration, (i.e. a list of RRC reconfiguration messages RRC reconfiguration***) and associated execution conditions, in which RRC reconfiguration*** contains RRCReconfiguration**** received from the candidate SN. Besides, the RRC reconfiguration message (RRC reconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements, as well as the NR RRC configuration message (RRCReconfigutation**) generated by the source-SN.

8.  The UE applies the RRC configuration (in RRC reconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRC reconfiguration complete message (RRC reconfiguration complete*), which can include an NR RRC response message (RRCReconfigutationComplete**). In case the UE is unable to comply with (part of) the configuration included in the RRC reconfiguration* message, it performs the reconfiguration failure procedure.

8a.	If an SN RRC response message is included, the MN informs the source SN via SN Reconfiguration Complete message with the SN RRC response message (RRCReconfigutationComplete**) for the source SN via SN Reconfiguration Complete message

The MN sends the SN Change Confirm message towards the Source SN to indicate that CPC is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the target SN, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding. In case multiple Target SNs are prepared, the MN includes a list of Target SN ID and list of data forwarding addresses to the source SN.

9.  The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRC reconfiguration message (RRC reconfiguration***) corresponding to the selected candidate PSCell, and sends an RRC reconfiguration complete message (RRC reconfiguration complete***), including an NR RRC message (RRCReconfigurationComplete****) for the selected candidate PSCell, and the selected PSCell information to the MN.

10: The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and triggers the Xn-U Address Indication procedure to inform the source SN the address of the selected target SN and if applicable, starts late data forwarding.

11.	If the RRC connection reconfiguration procedure was successful, the MN informs the target SN via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete**** response message for the target SN. The MN sends the SN Release Request messages to cancel CPC in the other target candidate SNs, if configured.

12.	The UE synchronizes to the target SN indicated in RRCConnectionReconfiguration***.

13.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target SN, if needed.

14.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the data forwarding address related information from the MN.

15.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 6:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

16-20.	If applicable, a PDU Session path update procedure is triggered by the MN.

21.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

## ----------End of the Changes--------------