3GPP TSG-RAN WG3 Meeting #115-e	R3-222169

E-meeting, 21st February – 03rd March 2022

Title: 	(TP to TS 38.300 BL CR) Broadcast service continuity

Source: 	Huawei, CBN, China Unicom, China Telecom

Agenda item:	22.4

Document Type:	other

# 1. Introduction

In last RAN3 meeting, the following agreements were made on broadcast service continuity:

MBS xxx ID/SAI is used to identify a preconfigured MBS area and it could be provided to RAN node via OAM.

Name the MBS xxx ID/SAI as SAI in the interim before SA2 gives final name. As to the length of SAI, it depends on the decision of SA2.RAN3 send LS to SA2 on the conclusion in RAN3

Exchange the list of SAI that neighbor cells supported via Xn interface. In case of CU/DU split, SAIs of each cell should be configured in DU by OAM and be provided from DU to CU.

Check the view of SA2 on the necessity of indication of session start success or failure with cell accuracy.

In this paper, we further discuss the potential impacts of the Broadcast Service Continuity.

# 2. Discussion

As one of the remaining issues on broadcast service continuity, how to let the NG-RAN node know the TMGI of the broadcast service which is ongoing in the neighbor cells is still under discussion. There are two potential methods to transfer the information of the cells that provides the broadcast service:

- Method 1: exchange the broadcast services which are ongoing in neighbor cells via Xn interface;

- Method 2: transfer cell list containing all of the cells that provide the broadcast service over NG.

The neighbour broadcast information from Xn interface is not workable in case the Xn interface is absence between two NG-RAN nodes, but the same MBS service may be provided in both of these two NG-RAN nodes. Hence Method 1 only is not enough and Method 2 shall be considered. To support neighbor cell broadcasting status for ongoing services and NR MBS interesting indication report, the CN needs to provide the MBS Service Area (i.e. cell list and/or TAI list) in which the NR MBS session to be broadcasted to the NG-RAN node during Broadcast Session Start/Update, and the MBS Service Area includes all the involved cells and or TAIs for that broadcast session, it is not a big burden from signaling point of view as these are non UE associated messages. With above NG based Method 2 as baseline, the Xn based Method 1 can be added on the top, to provide more accurate information especially in case if the session start has failed in some cells.

- For broadcast service, CN shall provide the MBS Service Area to be transferred over the NG interface includes all the cells and/or TAIs for that broadcast session.

- In order to get more accurate information, it is also needed to exchange the broadcast services which are ongoing in neighbour cells via Xn interface.

In the last meeting, there is proposals to introduce a new concept i.e. 5G-MBS Session areas to represent geographical area where a MBS session should be maintained. In our opinion, the existing cell list and/or TAIs are flexible enough to indicate the areas wherein the broadcast session is maintained. In case of small geographical area like several cells wherein a broadcast session is provided, only cell list would be OK. While in case of larger area to be supported, the TAIs or a combination of TAIs and cell lists would also work. A flexible representation on geographical area of a MBS session is accomplished by above method. On the other hand, it is proposed that when the DU receives a request from the CU-CP to start a session in a 5G-MBS Session area, it starts the service in suitable cells according to its configuration. Once the geographical areas that the broadcast session is maintained has changed, the CN shall be notified and update the mapping between the 5G-MBS Session area with gNB-ID/TAI. Such way may be static and complicates the whole system. Hence, it is proposed to use the existing cell list and/or TAIs, instead of introducing a new concept i.e. 5G-MBS Areas to represent geographical area where a MBS session should be maintained.

- Use the existing cell list and/or TAIs, instead of introducing a new concept i.e. 5G-MBS Areas to represent geographical area where a MBS session should be maintained.

Moreover, to avoid repeated MII report after handover, the MII has been agreed in RAN2 to be exchanged between source gNB and target gNB at handover. In LTE, the MII is included in the inter-node message HandoverPreparationInformation, which is contained by the RRC context in HANDOVER REQUEST message and with limited RAN3 specification impacts. The same approach is also applied in NR and MII is included in inter-node message HandoverPreparationInformation as captured in the RAN2 running CR of TS38.331.

- No need for RAN3 to include MII as NGAP/XnAP IE during HO, as RAN2 has already included it in inter-node message HandoverPreparationInformation.

# 3. Proposals

In this contribution, we discuss the potential RAN3 impacts to support Service Continuity for broadcast sessions, and get the following proposals:

- For broadcast service, CN shall provide the MBS Service Area to be transferred over the NG interface includes all the cells and/or TAIs for that broadcast session.

- In order to get more accurate information, it is also needed to exchange the broadcast services which are ongoing in neighbour cells via Xn interface.

- Use the existing cell list and/or TAIs, instead of introducing a new concept i.e. 5G-MBS Areas to represent geographical area where a MBS session should be maintained.

- No need for RAN3 to include MII as NGAP/XnAP IE during HO, as RAN2 has already included it in inter-node message HandoverPreparationInformation.

The Text Proposal to TS 38.300 BL CR is provided in section 4.

# 4. Text proposal to TS 38.300 BL CR

--------------------------------   Start of Change   -----------------------------

### 16.x.6	Broadcast Handling

#### 16.x.6.1	Session Management

Editor’s Note: RAN3 to provide Session management aspects here.

For delivery of location dependent contents of a broadcast session, Area session ID related information is included in the NGAP broadcast session resource setup procedure associated with MBS service area information and per Area Session ID NG-U tunnels are established.

#### 16.x.6.4	Service Continuity

Editor’s Note: Mobility related aspects to be covered here.

Mobility principles builds on existing functionality including functions described in section 9.2.

NR MBS supports MBS frequency layer prioritization for broadcast MBS sessions.

### The NG-RAN nodes may be configured with the SAI(s) supported by each of their cell. The NG-RAN nodes may exchange this information with their neighbors within Xn Setup messages and subsequent Xn Configuration Update messages to help for frequency layer prioritization.

Editor’s Note: SAI is the interim name to designate the geographical area used in NG-RAN nodes for frequency layer prioritization pending the final name and definition from SA2.

The core network provides the MBS Service Area (i.e. cell list and/or TAI list) in which the NR MBS session to be broadcasted to the NG-RAN node during Broadcast Session Start/Update, and the MBS Service Area includes all the involved cells and or TAIs for that broadcast session. And the NG-RAN nodes also exchange with their neighbors over Xn about which broadcast services are being delivered.

--------------------------------   End of Change   -----------------------------