3GPP TSG-RAN WG3#115-e                                             	                     R3-222183

E-Meeting, 21 Feb – 3 Mar 2022

Agenda item:	8.3.1

Source:	ZTE

Title: 	(TP for TS 36.423) X2AP Handling of UE Security Capabilities

Document for:	Approval

# Discussion

The description part need to be updated to align with S1AP CR.

# Conclusion and Proposal

Proposal : Update description for UE Security Capabilities in X2AP CR 1624.

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1624 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for mapping complete security capabilities from NAS [UE_Sec_Caps] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA3 has requested RAN3 (in S3-213272) “to modify their specifications to ensure that all of MME, eNB, AMF and ng-RAN node copy on the complete UE security capabilities” from release 17 onwards, to prevent the network not selecting what would be the preferred security algorithm if the full UE security capabilities were available at the eNB or ng-RAN node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All currently defined bits in the NAS security capability IEs (both 4G and 5G) are mapped to bits in the corresponding RAN IEs, irrespective of whether they are currently assigned to represent support for a security algorithm. A statement is also added that the “Security Capabilities received from NAS signaling shall not be modified or truncated when forwarded to eNBs”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the requested functionality in S3-213272. RAN may not be aware of UE’s support for new security algorithms. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.29, 9.2.107 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS36.413 CR1835, TS 38.413 CR0669, TS 38.423 CR0676 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.2.29	UE Security Capabilities

The UE Security Capabilities IE defines the supported algorithms for encryption and integrity protection in the UE.The Security Capabilities received from NAS signaling shall not be modified or truncated when forwarded to eNBs, and the eNBs store and send the complete bitmaps without modification or truncation as specified in TS 36.300 [14].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Encryption Algorithms | M |  | BIT STRING (16, ...) | Each position in the bitmap represents an encryption algorithm: "all bits equal to 0" - UE supports no other algorithm than EEA0 "first bit" - 128-EEA1, "second bit" - 128-EEA2, "third bit" - 128-EEA3, “fourth to seventh bit” are mapped from bit 4 to bit 1 of octet 3 in the UE Security Capability IE defined in TS 24.301 [14], other bits reserved for future use. Value ‘1’ indicates support and value "0" indicates no support of the algorithm. Algorithms are defined in TS 33.401 [18]. |
| Integrity Protection Algorithms | M |  | BIT STRING (16, ...) | Each position in the bitmap represents an integrity protection algorithm: all bits equal to 0" - UE supports no other algorithm than EIA0 (TS 33.401 [18]) "first bit" - 128-EIA1, "second bit" - 128-EIA2, "third bit" - 128-EIA3, “fourth to seventh bit” are mapped from bit 4 to bit 1 of octet 4 in the UE Security Capability IE defined in TS 24.301 [14], other bits reserved for future use. Value '1' indicates support and value "0" indicates no support of the algorithm. Algorithms are defined in TS 33.401 [18]. |

>>> NEXT CHANGE <<<

9.2.107	NR UE Security Capabilities

This IE defines the supported algorithms for encryption and integrity protection in NR as defined in TS 33.401 [18]. The Security Capabilities received from NAS signaling shall not be modified or truncated when forwarded to eNBs, and the eNBs store and send the complete bitmaps without modification or truncation as specified in TS 36.300 [14].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR Encryption Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an encryption algorithm: "all bits equal to 0" – UE supports no other NR algorithm than NEA0, "first bit" – 128-NEA1, "second bit" – 128-NEA2, "third bit" – 128-NEA3, “fourth to seventh bit” are mapped from bit 4 to bit 1 of octet 3 in the UE Additional Security Capability IE defined in TS 24.301 [14], “eighth to fifteenth bit” are mapped from bit 8 to bit 1 of octet 4 in the UE Additional Security Capability IE defined in TS 24.301 [14], other bits reserved for future use. Value '1' indicates support and value '0' indicates no support of the algorithm. Algorithms are defined in TS 33.401 [18]. |
| NR Integrity Protection Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an integrity protection algorithm: "all bits equal to 0" – UE supports no other NR algorithm than NIA0, "first bit" – 128-NIA1, "second bit" – 128-NIA2, "third bit" – 128-NIA3, “fourth to seventh bit” are mapped from bit 4 to bit 1 of octet 5 in the UE Additional Security Capability IE defined in TS 24.301 [14], “eighth to fifteenth bit” are mapped from bit 8 to bit 1 of octet 6 in the UE Additional Security Capability IE defined in TS 24.301 [14], other bits reserved for future use. Value '1' indicates support and value '0' indicates no support of the algorithm. Algorithms are defined in TS 33.401 [18]. |