3GPP TSG-RAN WG3 Meeting #115-e	R3-222192

21 Feb – 3 Mar 2022

# Online

Agenda Item:	10.4

Source:	ZTE

Title:	(TP for MDT BL CR for TS37.320) M6 calculation for split bearer in MR-DC

Document for:	Discussion and agreement

TP for TS37.320

Start of Change

5.1.2.2	Measurement reporting

For Immediate MDT, the UE provides detailed location information (e.g. GNSS location information) if available. The UE also provides available neighbour cell measurement information that may be used to determine the UE location (RF fingerprint). ECGI, Cell-Id, or CellIdentity of the serving cell when the measurement was taken is always assumed known in E-UTRAN, UTRAN or NR respectively.

The location information which comes with UE radio measurements for MDT can be correlated with other MDT measurements, e.g. RAN measurements. For MDT measurements where UE location information is provided separately, it is assumed that the correlation of location information and MDT measurements should be done in the TCE based on time-stamps.

When the gNB becomes aware of an in-device coexistence interference problem for NR reported by the UE, this information should be forwarded to the TCE which may correlate impacted measurements (e.g. RAN measurements M4, M5, M6, M7) with the in-device coexistence interference problem.

When the gNB calculates M6 calculation for split bearer in MR-DC for the UE, when PDCP duplication is enabled, MN provides number of packets has send to the UE to the TCE at the end of each M6 measurement interval, While when PDCP duplication is not enabled, MN and SN provides number of packets has send to the UE to the TCE at the end of each M6 measurement interval respectively. The information is used by TCE to calculate weighted average between MN and SN and to compute the total RAN delay.

End of Change